| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G521 |
| --- | --- |

| Title: | Non-square hadamard transform for motion estimation and merge estimation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhen Zheng, HiSilicon Lingzhi Liu, HiSilicon Yuan Yuan, Tsinghua Yun He, Tsinghua | Tel: Email: | zhengxiaozhen@huawei.com |
| Source: | HiSilicon Technologies, Tsinghua University |  |  |

_____________________________

# Abstract

This contribution provides a non-square hadamard transform solution which is used at motion estimation and merge estimation process. Different configurations of non-square hadamard transform are also discussed at the contribution. Experimental results show that non-square hadamard can achieve the average gain of 0.2% for RA, 0.2% for RA_LC, 0.2% for LD_B, 0.3% for LD_B_LC, 0.2% for LD_P and 0.3% for LD_P_LC when inter 2x8 and 8x2 transform is used at residual coding.

# Introduction

At current HM software, hadamard transform at motion estimation and merge estimation process which can help to estimate the distortion between the current coding block and prediction block. In details, square hadamard with the size of 4x4 or 8x8 is used at half-pixel and quarter-pixel motion estimation, and merge estimation.

At inter residual transformation, non-square quadtree transform (NSQT) [1] are used depending on inter partition modes. If the partition modes is non-square type, e.g. 2NxN, Nx2N and asymmetric partitions, non-square transforms with the size of 32x8, 8x32, 16x4 and 4x16 will be chose at the RQT’s second and third subdivision level. NSQT structure solves the problem of that TU cross PU’s boundary, and considers a coding block’s texture and contour information. Meanwhile, inter 2x8 and 8x2 transform under NSQT structure is evaluating at CE2 [2].

Based on the characteristic of NSQT, this contribution explores the approach of applying non-square hadamard transform to motion estimation and merge estimation. The basic idea is to replace NxN hadamard transform with 2Nx0.5N or 0.5Nx2N hadamard transform is the inter partition mode belongs to non-square type.

# Algorithm description

Considering that NS-HAD (non-square hadamard) has same functionality as S-HAD (square hadamard), NS-HAD is used at hale-pixel and quarter-pixel motion estimation, and merge estimation functions, which is same as S-HAD. In detail, it is applied to the functions listed as follow to calculate HAD distortion.

- TEncSearch::xPatternRefinement()

- TEncSearch::xGetInterPredictionError()

At the proposed algorithm, the size of HAD depends on prediction block’s size. In general, if the width of the block is equal to its height, S-HAD is chose for HAD calculation. By contrast, NS-HAD is applied to HAD calculation for the case of that the width of block is different to its height. Table 1 describes HAD sizes used at HM4.0 and proposed algorithm.

Table 1: Relation between prediction block’s size and HAD size

| Prediction block’s size | HAD size at HM4.0 | HAD size at proposed |
| --- | --- | --- |
| 4x4 | 4x4 | 4x4 |
| 8x4 | 4x4 | 8x2 |
| 4x8 | 4x4 | 2x8 |
| 8x8 | 8x8 | 8x8 |
| 16x4 | 4x4 | 16x4 |
| 16x8 | 8x8 | 16x4 |
| 16x12 | 4x4 | 16x4 |
| 16x16 | 8x8 | 8x8 |
| 12x16 | 4x4 | 4x16 |
| 8x16 | 8x8 | 4x16 |
| 4x16 | 4x4 | 4x16 |
| 32x8 | 8x8 | 16x4 |
| 32x16 | 8x8 | 16x4 |
| 32x24 | 8x8 | 16x4 |
| 32x32 | 8x8 | 8x8 |
| 24x32 | 8x8 | 4x16 |
| 16x32 | 8x8 | 4x16 |
| 8x32 | 8x8 | 4x16 |
| 64x16 | 8x8 | 16x4 |
| 64x32 | 8x8 | 16x4 |
| 64x48 | 8x8 | 16x4 |
| 64x64 | 8x8 | 8x8 |
| 48x64 | 8x8 | 4x16 |
| 32x64 | 8x8 | 4x16 |
| 16x64 | 8x8 | 4x16 |

The pseudo code for the determination of HAD’s size is as followings.

if( ( iRows % 8 == 0) && (iCols % 8 == 0) && ( iRows == iCols ) )

{

Int  iOffsetOrg = iStrideOrg<<3;

Int  iOffsetCur = iStrideCur<<3;

for ( y=0; y<iRows; y+= 8 )

{

for ( x=0; x<iCols; x+= 8 )

{

uiSum += xCalcHADs8x8( &piOrg[x], &piCur[x*iStep], iStrideOrg, iStrideCur, iStep );

}

piOrg += iOffsetOrg;

piCur += iOffsetCur;

}

}

else if ( ( iCols > 8 ) && ( iCols > iRows ) )

{

Int  iOffsetOrg = iStrideOrg<<2;

Int  iOffsetCur = iStrideCur<<2;

for ( y=0; y<iRows; y+= 4 )

{

for ( x=0; x<iCols; x+= 16 )

{

uiSum += xCalcHADs16x4( &piOrg[x], &piCur[x*iStep], iStrideOrg, iStrideCur, iStep );

}

piOrg += iOffsetOrg;

piCur += iOffsetCur;

}

}

else if ( ( iRows > 8 ) && ( iCols < iRows ) )

{

Int  iOffsetOrg = iStrideOrg<<4;

Int  iOffsetCur = iStrideCur<<4;

for ( y=0; y<iRows; y+= 16 )

{

for ( x=0; x<iCols; x+= 4 )

{

uiSum += xCalcHADs4x16( &piOrg[x], &piCur[x*iStep], iStrideOrg, iStrideCur, iStep );

}

piOrg += iOffsetOrg;

piCur += iOffsetCur;

}

}

else if ( ( iCols <= 8 ) && ( iCols > iRows ) )

{

Int  iOffsetOrg = iStrideOrg<<1;

Int  iOffsetCur = iStrideCur<<1;

for ( y=0; y<iRows; y+= 2 )

{

for ( x=0; x<iCols; x+= 8 )

{

uiSum += xCalcHADs8x2( &piOrg[x], &piCur[x*iStep], iStrideOrg, iStrideCur, iStep );

}

piOrg += iOffsetOrg;

piCur += iOffsetCur;

}

}

else if ( ( iRows <= 8 ) && ( iCols < iRows ) )

{

Int  iOffsetOrg = iStrideOrg<<3;

Int  iOffsetCur = iStrideCur<<3;

for ( y=0; y<iRows; y+= 8 )

{

for ( x=0; x<iCols; x+= 2 )

{

uiSum += xCalcHADs2x8( &piOrg[x], &piCur[x*iStep], iStrideOrg, iStrideCur, iStep );

}

piOrg += iOffsetOrg;

piCur += iOffsetCur;

}

}

else if( ( iRows % 4 == 0) && (iCols % 4 == 0) )

{

Int  iOffsetOrg = iStrideOrg<<2;

Int  iOffsetCur = iStrideCur<<2;

for ( y=0; y<iRows; y+= 4 )

{

for ( x=0; x<iCols; x+= 4 )

{

uiSum += xCalcHADs4x4( &piOrg[x], &piCur[x*iStep], iStrideOrg, iStrideCur, iStep );

}

piOrg += iOffsetOrg;

piCur += iOffsetCur;

}

}

# Experimental results

The proposed method is implemented on HM4.0_rev.1354 software. Considering that HM4.0 does not use 2x8 and 8x2 transforms for residual coding, the implementation on HM4.0 excludes 2x8 and 8x2 HADs, which means that 8x4 and 4x8 prediction blocks still use 4x4 HADs. The other configurations of HAD size are same as Table 1. The coding efficiency is evaluated in RA_HE, RA_LC, LD_B_HE, LD_B_LC, LD_P_HE and LD_P_LC configurations based on the common conditions defined in JCTVC-F900 [3]. The coding efficiency is measured by using BD-Rate. The experimental results are shown in Table 2

Table 2: HM4.0 vs. HM4.0 + NS-HAD

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.2% | -0.1% | -0.1% | 0.0% |
| Class B | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% |
| Class C | 0.0% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% |
| Class D | 0.0% | -0.1% | 0.1% | 0.0% | -0.3% | -0.1% |
| Class E |  |  |  |  |  |  |
| Class F | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |
| Overall (without classF) | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% |
|  | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% |
| Enc Time[%] (without classF) | 101% |  |  | 101% |  |  |
| Dec Time[%] (without classF) | 99% |  |  | 100% |  |  |
| Overall (with classF) | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% |
|  | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% |
| Enc Time[%] (with classF) | 101% |  |  | 101% |  |  |
| Dec Time[%] (with classF) | 100% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -0.2% | 0.0% | -0.1% | -0.1% | -0.2% |
| Class C | -0.1% | -0.3% | 0.0% | -0.1% | 0.0% | -0.1% |
| Class D | -0.1% | 0.0% | -0.1% | -0.1% | 0.2% | -0.1% |
| Class E | -0.1% | 0.2% | 0.3% | -0.2% | -0.2% | 0.4% |
| Class F | -0.1% | -0.2% | -0.2% | 0.1% | 0.0% | -0.1% |
| Overall (without classF) | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% |
|  | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% |
| Enc Time[%] (without classF) | 101% |  |  | 101% |  |  |
| Dec Time[%] (without classF) | 97% |  |  | 98% |  |  |
| Overall (with classF) | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% | -0.1% |
|  | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% |
| Enc Time[%] (with classF) | 102% |  |  | 101% |  |  |
| Dec Time[%] (with classF) | 98% |  |  | 99% |  |  |

Table 3: HM4.0 + 2x8 & 8x2 transform vs. HM4.0 + 2x8 & 8x2 transform + NS-HAD

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% | 0.0% |
| Class B | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% |
| Class C | -0.2% | -0.1% | -0.1% | -0.2% | 0.0% | -0.1% |
| Class D | -0.3% | -0.1% | 0.0% | -0.4% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Class F | -0.2% | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% |
| Overall (without classF) | -0.2% | -0.1% | -0.1% | -0.2% | 0.0% | 0.0% |
|  | -0.1% | -0.1% | -0.1% | -0.2% | 0.0% | 0.0% |
| Enc Time[%] (without classF) | 101% |  |  | 101% |  |  |
| Dec Time[%] (without classF) | 101% |  |  | 100% |  |  |
| Overall (with classF) | -0.2% | -0.1% | -0.1% | -0.2% | 0.0% | 0.0% |
|  | -0.2% | 0.0% | -0.1% | -0.2% | 0.0% | 0.0% |
| Enc Time[%] (with classF) | 101% |  |  | 101% |  |  |
| Dec Time[%] (with classF) | 102% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -0.1% | 0.0% | -0.2% | 0.0% | -0.1% |
| Class C | -0.2% | -0.2% | -0.4% | -0.3% | -0.1% | -0.2% |
| Class D | -0.4% | -0.4% | -0.4% | -0.4% | 0.5% | -0.2% |
| Class E | -0.2% | -0.3% | -0.4% | -0.2% | -0.3% | -0.2% |
| Class F | -0.3% | -0.3% | -0.4% | -0.4% | -0.3% | -0.4% |
| Overall (without classF) | -0.2% | -0.3% | -0.2% | -0.3% | 0.1% | -0.2% |
|  | -0.2% | -0.3% | -0.2% | -0.3% | 0.0% | -0.2% |
| Enc Time[%] (without classF) | 101% |  |  | 102% |  |  |
| Dec Time[%] (without classF) | 101% |  |  | 102% |  |  |
| Overall (with classF) | -0.2% | -0.3% | -0.3% | -0.3% | 0.0% | -0.2% |
|  | -0.2% | -0.3% | -0.3% | -0.3% | 0.0% | -0.2% |
| Enc Time[%] (with classF) | 101% |  |  | 102% |  |  |
| Dec Time[%] (with classF) | 101% |  |  | 102% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -0.1% | -0.3% | -0.2% | 0.0% | 0.2% |
| Class C | -0.3% | -0.1% | -0.3% | -0.3% | -0.1% | -0.2% |
| Class D | -0.3% | -0.7% | -0.4% | -0.5% | -0.4% | 0.1% |
| Class E | -0.1% | -0.1% | -0.2% | -0.1% | 0.4% | 0.0% |
| Class F | -0.2% | -0.2% | 0.2% | -0.4% | -0.3% | -0.1% |
| Overall (without classF) | -0.2% | -0.2% | -0.3% | -0.3% | -0.1% | 0.0% |
|  | -0.2% | -0.2% | -0.3% | -0.3% | 0.0% | 0.1% |
| Enc Time[%] (without classF) | 101% |  |  | 101% |  |  |
| Dec Time[%] (without classF) | 101% |  |  | 101% |  |  |
| Overall (with classF) | -0.2% | -0.2% | -0.2% | -0.3% | -0.1% | 0.0% |
|  | -0.2% | -0.2% | -0.2% | -0.3% | -0.1% | 0.0% |
| Enc Time[%] (with classF) | 101% |  |  | 101% |  |  |
| Dec Time[%] (with classF) | 103% |  |  | 102% |  |  |

Table 4: HM4.0 without NSQT vs. HM4.0 + without NSQT + NS-HAD

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.1% | -0.1% | 0.1% | 0.0% | 0.1% |
| Class B | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% |
| Class C | 0.1% | 0.1% | 0.1% | 0.2% | 0.0% | 0.0% |
| Class D | 0.2% | 0.2% | 0.1% | 0.3% | 0.1% | 0.1% |
| Class E |  |  |  |  |  |  |
| Class F | -0.4% | -0.2% | -0.2% | -0.4% | -0.1% | -0.2% |
| Overall (without classF) | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% |
|  | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% |
| Enc Time[%] (without classF) | 101% |  |  | 101% |  |  |
| Dec Time[%] (without classF) | 99% |  |  | 100% |  |  |
| Overall (with classF) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] (with classF) | 104% |  |  | 103% |  |  |
| Dec Time[%] (with classF) | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.1% | 0.2% | 0.1% | 0.0% | 0.1% |
| Class C | 0.1% | 0.0% | -0.1% | 0.1% | 0.1% | 0.1% |
| Class D | 0.2% | -0.7% | 0.4% | 0.3% | 0.4% | 0.3% |
| Class E | 0.0% | -0.9% | -0.3% | 0.1% | 0.4% | 0.0% |
| Class F | -0.9% | -0.8% | -0.2% | -0.8% | -0.3% | -0.2% |
| Overall (without classF) | 0.1% | -0.4% | 0.1% | 0.1% | 0.2% | 0.1% |
|  | 0.1% | -0.4% | 0.1% | 0.1% | 0.2% | 0.1% |
| Enc Time[%] (without classF) | 102% |  |  | 101% |  |  |
| Dec Time[%] (without classF) | 100% |  |  | 99% |  |  |
| Overall (with classF) | -0.1% | -0.4% | 0.0% | 0.0% | 0.1% | 0.1% |
|  | -0.1% | -0.5% | 0.0% | 0.0% | 0.1% | 0.0% |
| Enc Time[%] (with classF) | 103% |  |  | 103% |  |  |
| Dec Time[%] (with classF) | 101% |  |  | 100% |  |  |

Besides the implementation on HM4.0, the proposed NS-HAD is also added to NSQT with 2x8 and 8x2 transform, whose software comes from [2]. The configurations of HAD size are same as the description at Table 2. It can be observed that NS-HAD can achieve higher coding gain at HM4.0 + 2x8 and 8x2 transform than HM4.0 platform. The test conditions are same as JCTVC-F900 [3].

To help understanding the relationship between NS-HAD and NSQT, NS-HAD is also implemented on HM4.0 without NSQT. During the tests, the macro “NSQT” is disabled. The test conditions are same as JCTVC-F900 [3] except that NSQT is disabled. It can be found that no coding gain can be got on this test case.

# Conclusion

The proposed non-square hadamard transform can achieve the average coding gain of 0.1% at HM4.0 platform with around 1% encoding time increase. To implement non-square hadamard to HM4.0 + 2x8 and 8x2 transform, the coding gain is increased to 0.2% ~ 0.3% on average with around 1% ~ 2% encoding time increase. The proposed method is an encoder-side optimization, and does not require changing the decoder process. It is recommended to incorporate the proposed non-square hadamard transform into HM5 platform.

# References

[1] Y. Yuan (Tsinghua), X. Zheng (HiSilicon), X. Peng (USTC), J. Xu (Microsoft), I.-K Kim (Samsung), L. Liu (HiSilicon), Y. Wang, X. Cao (Tsinghua Univ.), C. Lai, J. Zheng (HiSilicon), Y. He (Tsinghua Univ.), H. Yu (Huawei), “CE2: Non-Square Quadtree Transform for symmetric and asymmetric motion partitions”, Doc. JCTVC-F412, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Torino, IT, July 2011.

[2] Y. Yuan (Tsinghua), X. Zheng (HiSilicon), Y. He (Tsinghua Univ.), “CE2 subtest C.1: Non-square quadtree (NSQT) with 2x8 and 8x2 transform”, Doc. JCTVC-F518, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, CH, November 2011.

[3] Frank Bossen, “Common test conditions and software reference configurations”, Doc. JCTVC-F900, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Torino, IT, July 2011.

# Patent rights declaration(s)

Huawei Technologies, HiSilicon Technologies and Tsinghua University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).