| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G522 WG11 Number: m22086 |
| --- | --- |

| Title: | Non-CE11:Context reduction for coefficient level |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kengo Terada Hisao Sasai Takahiro Nishi 1006 Kadoma, Kadoma Osaka, Japan | Tel: Email: | +81 6 6900 9689 { terada.kengo, sasai.hisao, nishi.takahiro }@jp.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This proposal presents a technique of context reduction of coefficient level for CABAC. There are two proposal points in this technique. First one is to share contexts of luma and chroma. This proposal shares contexts of luma and chroma for syntax elements coeff_abs_level_greater1/2_flag when neighboring coefficient level is big. Second one is to reduce the context that is not frequently used when neighboring coefficient level is big. This proposal was implemented on HMv4.0 and evaluated. Contexts of coeff_abs_level_greater1/2_flag were reportedly reduced from 120 to 60 without major impact on coding efficiency.

# Introduction

There is a lot of context of coefficient level in the current HEVC[1] as discussed in [2].

In this contribution, there are two proposal techniques. First one is to reduce ctxSet by sharing context of luma and chroma. This proposal shares contexts of luma and chroma for syntax elements coeff_abs_level_greater1/2_flag when neighboring coefficient level is big. Second one is to reduce the context in ctxSet. This proposal is to reduce the context that is not frequently used when neighboring coefficient level is big.

# Proposed Solution

## Reducing ctxSet by sharing context of luma and chroma

This proposal shares contexts of luma and chroma for syntax elements coeff_abs_level_greater1/2_flag when neighboring coefficient level is big. When neighboring coefficient level is big, 1 occurring probability of current coefficient is uniformly high about luma and chroma for coeff_abs_level_greater1/2_flag.

In the current HEVC, the variable ctxSet is a value from 0 to 5, and there are 5 contexts in each ctxSet. The ctxSet is decided by block position and neighboring coefficient situation. The ctxSet is set equal to 2 or 5, when there are a lot of big coefficients around. Table 1 “currnet HEVC” line shows used context for coeff_abs_level_greater1/2_flag on current HEVC. A total of context is 120 contexts (24 ctxSets * 5 contexts).

In proposed solution, luma and chroma use the same contexts by neighboring coefficients. We ran tests that changed a sharing condition. Table 1 “proposal (1)” line shows the used context for coeff_abs_level_greater1/2_flag when the sharing condition is that neighboring coefficient level is big and middle (ctxSet is 1, 2, 4 or 5). And Table 1 “proposal (2)” line is used context when the sharing condition is that neighboring coefficient level is big (ctxSet is 2 or 5). This contribution proposes the proposal (1). The proposal (1) reduces 40 contexts (8 ctxSets * 5 contexts).

Table 1 : Used context for coeff_abs_level_greater1/2_flag

## Reducing context in ctxSet

In the current HEVC, the context of coeff_abs_level_greater1_flag in the ctxSet is decided depending on the number of 0 continuations in current 16 coefficient subset. However, when neighboring coefficient level is big, 1 occurs immediately. The number of 0 continuations is little. And, the context of coeff_abs_level_greater2_flag is decided depending on coefficient scan position. However, when neighboring coefficient level is big, 1 occurs immediately same as coeff_abs_level_greater1_flag. Therefore, coeff_abs_level_greater2_flag except the neighborhood of the top is approximately 1.

This proposal reduces contexts in ctxSet by neighboring coefficient situation. This proposal reduces 2 contexts per ctxSet for coeff_abs_level_greater1_flag and reduces 3 contexts per ctxSet for coeff_abs_level_greater2_flag when neighboring coefficient is big. We ran two tests that changed an applying ctxSet. First one is that the applying ctxSet is 2 or 5 (neighboring coefficient is big). Second one is that the applying ctxSet is 1, 2, 4 or 5 (neighboring coefficient is big or middle). This contribution proposes the first one (applying ctxSet is 1, 2, 4 or 5). There are 3 contexts in each ctxSet1, 2, 4, 5 and 5 contexts in each ctxSet0, 3 for coeff_abs_level_greater1_flag. There are 2 contexts in each ctxSet1, 2, 4, 5 and 5 contexts in each ctxSet0, 3 for coeff_abs_level_greater2_flag.

Table 2 : Example of context derivation for coeff_abs_level_greater1_flag

Table 3 : Example of context derivation for coeff_abs_level_greater2_flag

## The benefit of proposed solution

This proposed solution can reduce the memory size for coefficient level contexts. The contexts for coeff_abs_level_greater1/2_flag are reduced 120 to 60. The details of number of contexts are as follows.

Current HEVC	:24 ctxSets(ctxSet0~5)*5 contexts	= 120 contexts

Proposal solution	:8 ctxSets(ctxSet0,3 greater1/2)*5 contexts +

4 ctxSets(ctxSet1,2,4,5 greater1)*3 contexts +

4 ctxSets(ctxSet1,2,4,5 greater2)*2 contexts	= 60 contexts

# Results

The proposed method performance relative to the HMv4.0 software was checked under the common test condition described in JCTVC-F900[3]. The computing platform is Linux 64bits and the executables were compiled on 64-bit Linux with gcc 4.4.3. The cross verification is performed by TI.

Table 4: BD BR for proposed solution (high efficiency)

Table 5 shows results for each proposal technique. “Reducing ctxSet” is explained in 2.1 section. “Reducing context” is explained in 2.2 section.

Table 5: BD BR for each proposal technique (high efficiency)

In additional information, the proposed solution can be combined with JCTVC-F132[2]. The combination method can reduce the context of ctxSet 0 and 3 in comparison with proposed solution. The contexts for coeff_abs_level_greater1/2_flag are reduced 120 to 48 by the combination method. Table 6 shows a result for the combination method compared with HMv4.0.

Table 6: BD BR for proposed solution with JCTVC-F132 (high efficiency)

- Conclusion

In this contribution, a method of context reduction is proposed. Contexts for coeff_abs_level_greater1/2_flag were reportedly reduced from 120 to 60. This proposed solution can reduce the memory size for coefficient level contexts without major impact on coding efficiency. It is suggested to consider the inclusion of this proposal in the HM/WD of HEVC.

# References

- JCT-VC, “WD4: Working Draft 4 of High-Efficiency Video Coding”, JCTVC-F803, JCT-VC 6th Meeting, Torino, July, 2011.

- V.Sze, “Reduction in contexts used for significant_coeff_flag and coefficient level”, JCTVC-F132, JCT-VC 6th Meeting, Torino, July, 2011.

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, JCT-VC 6th Meeting, Torino, July, 2011.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Text for Working Draft

###### 9.3.3.1.1.5 Derivation process of ctxIdxInc for the syntax element coeff_abs_level_greater1_flag

….

The context index increment ctxIdxInc is derived using the current context set ctxSet and the current context greater1Ctx as follows.

- The variable ctxSet is equal to 0 or 3, the following applies.

ctxIdxInc  =  ( ctxSet  *  5 )  +  Min( 4, greater1Ctx )	(9-1)

- Otherwise (the variable ctxSet is equal to 1, 2, 4 or 5), the following applies.

ctxIdxInc  =  ( ctxSet  *  5 )  +  Min( 2, greater1Ctx )	(9-2)

When cIdx is greater than 0 and the variable ctxSet is equal to 0 or 3, ctxIdxInc is modified as follows.

ctxIdxInc =  ctxIdxInc  +  30		(9-3)

###### 9.3.3.1.1.6 Derivation process of ctxIdxInc for the syntax element coeff_abs_level_greater2_flag

….

The context index increment ctxIdxInc is derived using the current context set ctxSet and the current context greater2Ctx as follows.

- The variable ctxSet is equal to 0 or 3, the following applies.

ctxIdxInc  =  ( ctxSet  *  5 )  +  Min( 4, greater2Ctx )	(9-48)

- Otherwise (the variable ctxSet is equal to 1, 2, 4 or 5), the following applies.

ctxIdxInc  =  ( ctxSet  *  5 )  +  Min( 1, greater2Ctx )	(9-59)

When cIdx is greater than 0 and the variable ctxSet is equal to 0 or 3, ctxIdxInc is modified as follows.

ctxIdxInc =  ctxIdxInc  +  30		(9-6)