| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G523 |
| --- | --- |

| Title: | Cross-check of Division-free MV scaling (JCTVC-G223) from MediaTek |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Patrice ONNO Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33(2)99876800 patrice.onno@crf.canon.fr |
| Source: | Canon Research Centre France |  |  |

_____________________________

# Abstract

This document is a crosscheck report of contribution JCTVC-G223. The results perfectly match those reported in JCTVC-G223 [1]. Source code also corresponds to the described modification.

# Introduction

The method proposed by MediaTek is about a division-free MV scaling to replace the general divider by a look-up table and simple arithmetic operations. The scaling operation is used during the derivation of spatial and temporal motion vector predictors (MVPs) and the proposed method is used to compute the scaling factor.

- Code verification

MediaTek has provided a modified source code based on HM4.0. The source code has been analyzed and corresponds to the modifications described in [1].

The source code was compiled successfully by using windows32, VC2008 and by using Linux 2.6.18-194.el5, gcc version 4.1.2 20080704 (Red Hat 4.1.2-48) 64 bits.

# Results

## Results with common test conditions

Results have been obtained by using the testing conditions [2] with a Linux cluster. This corresponds to Table 4 of JCTVC-G223. The associated excel file is Results_HM4.0_DFREE_1_600f.xls

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.2% | -0.2% | 0.0% | 0.1% | 0.1% |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% |
| Class D | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 102% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% | -0.2% |
| Class C | -0.1% | -0.1% | -0.3% | -0.1% | 0.1% | -0.3% |
| Class D | 0.0% | 0.1% | 0.2% | 0.0% | 0.1% | 0.4% |
| Class E | 0.0% | 0.4% | 0.3% | -0.1% | -0.3% | 0.3% |
| Overall | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
|  | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | 0.0% | 0.0% | -0.2% | -0.1% | -0.2% |
| Class C | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class D | 0.0% | -0.2% | -0.3% | -0.1% | -0.3% | -0.4% |
| Class E | -0.1% | -0.4% | -0.2% | 0.0% | -0.2% | 0.1% |
| Overall | -0.1% | -0.2% | -0.1% | -0.1% | -0.2% | -0.2% |
|  | -0.1% | -0.2% | -0.2% | -0.1% | -0.2% | -0.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

## Results with common test conditions but with double GOP size

Results have been obtained by using the testing conditions [2] with a Linux cluster. However testing conditions have been modified as follows: the GOP sizes are doubled. For random access conditions, the GOP size is increased from eight to 16. For low delay conditions, the temporal distance of two high quality pictures and the RateGOPSize are increased from four to eight. Specific anchors corresponding to these testing conditions have been also generated. This corresponds to Table 5 of JCTVC-G223. The associated excel file is Results_HM4.0_DFREE_2_600f.xls

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3% | -0.5% | -0.5% | -0.4% | -0.9% | -1.0% |
| Class C | -0.2% | -0.3% | -0.5% | -0.2% | -0.2% | -0.2% |
| Class D | -0.2% | 0.0% | -0.3% | -0.1% | -0.4% | -0.7% |
| Class E | -0.1% | 0.1% | -0.4% | -0.1% | -0.3% | 0.1% |
| Overall | -0.2% | -0.2% | -0.4% | -0.2% | -0.5% | -0.5% |
|  | -0.2% | -0.2% | -0.5% | -0.2% | -0.5% | -0.6% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.4% | -0.7% | -0.7% | -0.4% | -0.9% | -0.9% |
| Class C | -0.2% | -0.1% | -0.4% | -0.3% | -0.6% | -0.4% |
| Class D | -0.1% | -0.2% | 0.0% | -0.1% | 0.0% | -0.1% |
| Class E | 0.0% | -0.2% | -0.1% | -0.1% | 0.0% | -0.2% |
| Overall | -0.2% | -0.3% | -0.3% | -0.3% | -0.4% | -0.4% |
|  | -0.2% | -0.3% | -0.3% | -0.2% | -0.4% | -0.5% |
| Enc Time[%] | 98% |  |  | 98% |  |  |
| Dec Time[%] | 102% |  |  | 100% |  |  |

# Conclusion

The results presented in Table 4 and Table 5 by MediaTek in document JCTVC-G223 have been confirmed by this cross-check.

# References

- T. Chuang, J.-L. Lin, C.-Y. Chen, Yi-Wen, Y.W. Huang, S. Lei “Non-CE9: Division-free MV scaling”, JCTVC-G223 7th JCT-VC meeting, Geneva, CH, November 2011.

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, 5th Meeting: Torino, IT, July 2011.