| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G524 |
| --- | --- |

| Title: | Cross-checking reports of improved weighted prediction (JCTVC-G065) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Akiyuki Tanizawa Takeshi Chujoh Tomoo Yamakage 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, JAPAN | Tel: Email: | +81-44-549-2152 akiyuki.tanizawa@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

This contribution reports cross-checking results of improved weighted prediction (JCTVC-G065). The modified weighted prediction process for bi-prediction by changing accuracy of the rounding operation was tested on HM common conditions. The verification task has been done successfully and the results match those provided by proponent.

Experimental results show that BD-bitrate gain of the improved weighted prediction is 0.3%/0.6%, and 0.6%/0.8% for black/white-fade sequences on RA-LC and LB-LC cases, respectively. Additional results on HE with 8 internal bit depeth shows that BD-bitrate gain of this scheme is 0.2%/0.5% and 0.6%/0.9% for black/white-fade sequences on RA-HE(8bit) and LB-HE(8bit) cases, respectively. This method does not affect weighted prediction for uni-prediction and the case on low complexity condition. Average encoding and decoding times are 100% on all conditions compared with anchor WP method based on HM-4.0-dev.

# Cross-checking

The software based on HM version 4.0 (HM-4.0-dev including weighted prediction (WP)) and the experimental results were provided on Nov. 8, 2011 by InterDigital Communications. The software was complied fine and the cross-checking was performed on 64bit environments. The simulation has been conducted under common test conditions [1] for only low complexity conditions in the case of random access and low delay B coding structures using black/white-fade sequences provided by [2], since this scheme does not affect the weighted prediction process for input signal with bit depth larger than 8 bits. Additional simulation has been conducted under high efficiency condition with 8 internal bit depth in case of random access and low delay B coding structures.

Proponent adds only one macro as follows;

#define WEIGHT_PRED_IMP  1

We reviewed the source code and confirmed that this modification of source code in original HM-4.0-dev software is just 2 lines and the schemes implemented in the software matches those described in JCTVC-G065 [3].

Table 1 and Table 2 show BD-bitrate and relative encoding/decoding time of this scheme for black-fade and white-fade sequences respectively. The tests on HE conditions use 8 internal bit depth for both anchor and verified method. Through the results, it turns out that the gain is additive and the encoding/decoding time does not change compared with WP anchor. One point is that the encoding/decoding time of our tests does not match one of proponent, but it may come from proponent’s PC cluster fluctuation.

Table 1. BD-Rate[%] and relative encoding/decoding time[%] (Black-fade sequences)

|  | Random Access HE (with 8 internal bit depth for both anchor and proposal) |  |  | Random Access LC (Common condition) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.7% | -1.5% | -0.1% | -0.8% | -1.8% |
| Class B | -0.3% | -1.4% | -0.5% | -0.4% | -1.6% | -1.0% |
| Class C | -0.2% | -0.9% | -0.4% | -0.4% | -1.2% | -0.7% |
| Class D | -0.3% | -1.1% | -1.4% | -0.2% | -1.9% | -2.0% |
| Class E |  |  |  |  |  |  |
| Overall | -0.2% | -1.0% | -0.9% | -0.3% | -1.4% | -1.3% |
|  | -0.2% | -1.1% | -0.9% | -0.3% | -1.4% | -1.3% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE (with 8 internal bit depth for both anchor and proposal) |  |  | Low delay B LC (Common condition) |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.6% | -4.9% | -7.6% | -0.5% | -5.0% | -7.6% |
| Class C | -0.2% | -0.7% | -0.8% | -0.3% | -0.9% | -1.3% |
| Class D | -0.3% | -3.8% | -2.8% | -0.2% | -3.9% | -2.2% |
| Class E | -1.8% | -4.9% | -7.1% | -2.0% | -5.2% | -6.9% |
| Overall | -0.6% | -3.6% | -4.6% | -0.6% | -3.7% | -4.5% |
|  | -0.6% | -3.5% | -4.3% | -0.6% | -3.7% | -4.7% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 2. BD-Rate[%] and relative encoding/decoding time[%] (White-fade sequences)

|  | Random Access HE (with 8 internal bit depth for both anchor and proposal) |  |  | Random Access LC (Common condition) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.4% | -1.0% | -0.8% | -0.4% | -1.3% | -1.2% |
| Class B | -0.7% | -1.1% | -1.5% | -0.9% | -1.1% | -2.1% |
| Class C | -0.3% | -1.1% | -0.9% | -0.4% | -0.8% | -1.2% |
| Class D | -0.5% | -1.4% | -1.6% | -0.5% | -1.2% | -1.2% |
| Class E |  |  |  |  |  |  |
| Overall | -0.5% | -1.2% | -1.2% | -0.6% | -1.1% | -1.5% |
|  | -0.5% | -1.2% | -1.2% | -0.6% | -1.1% | -1.5% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE (with 8 internal bit depth for both anchor and proposal) |  |  | Low delay B LC (Common condition) |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.9% | -4.5% | -8.5% | -0.9% | -5.3% | -8.5% |
| Class C | -0.4% | -1.0% | -1.1% | -0.3% | -0.8% | -0.8% |
| Class D | -0.4% | -3.6% | -3.3% | -0.3% | -3.2% | -2.1% |
| Class E | -2.0% | -5.8% | -6.6% | -1.7% | -5.2% | -6.8% |
| Overall | -0.9% | -3.7% | -5.0% | -0.8% | -3.6% | -4.6% |
|  | -0.9% | -3.5% | -5.0% | -0.8% | -3.6% | -4.5% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# References

[1]	F. Bossen ," Common test conditions and software reference configurations ", Joint Collaborative Team on Video Coding, JCTVC-F900, Torino, July 2011.

[2]	AHG report, " JCT-VC AHG report: Weighted prediction (AHG 18)," Joint Collaborative Team on Video Coding, JCTVC-F018, Torino, July 2011.

[3]	Y. Ye and E-S. Ryu, " Improved Weighted Prediction," Joint Collaborative Team on Video Coding, JCTVC-G065, Geneva, Nov. 2011.

# Patent rights declaration(s)

TOSHIBA Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).