| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G526 |
| --- | --- |

| Title: | AHG21: Combined signaling for reference picture set |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Youngo Park Il-Koo Kim Chanyul Kim Kwang Pyo Choi JeongHoon Park | Email: | youngo.park@samsung.com ilkoo.kim@samsung.com dionism@samsung.com kp5.choi@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution is proposed for AHG21 especially for supporting combined signaling. We suggest a method keeping long distance POC difference (theoretically infinite) between reference picture and current picture as well as providing the possibility to combine relatively short distance picture to reference picture set.

# Introduction

There were many discussions on JCT-VC reflector about absolute signaling (AHG21). The reference picture set (RPS) is signaled in picture parameter set (PPS) explicitly. The reference picture information can be signaled as the same way in slice header only when current picture does not want to use pre-defined RPS. If decoder wants to keep long distance picture or just combine (add) a few pictures into pre-defined RPS, two kinds of methods were proposed in slice header. One is just for representing long-term reference picture and the other is combining reference picture which includes supporting long-distance pictures. Figure 1 describes the syntax for long-term reference pictures and Figure 2 describes the syntax for combined signaling in slice header.

| pic_order_cnt_lsb | u(v) |
| --- | --- |
| short_term_ref_pic_set_pps_flag | u(1) |
| if( !short_term_ref_pic_set_pps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_pics; i++ ) { |  |
| delta_poc_lt_minus1[ i ] | u(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| } |  |

Figure 1 The syntax for supporting long-term picture in slice header

| pic_order_cnt_lsb | u(v) |
| --- | --- |
| ref_pic_set_pps_flag | u(1) |
| if( ! ref_pic_set_pps_flag ) |  |
| ref_pic_set( num_ref_pic_sets ) |  |
| else { |  |
| ref_pic_set_idx | u(v) |
| num_additional_pics | ue(v) |
| for( i = 0; i < num_additional_pics; i++ ) { |  |
| delta_poc_a_minus1[ i ] | ue(v) |
| used_by_curr_pic_a_flag[ i ] | u(1) |
| } |  |
| } |  |

Figure 2 The syntax for supporting combined signaling in slice header

The method1 (Figure 1) can support long distance reference pictures efficiently because the syntax “delta_poc_lt_minus1[i]” is coded as u(v) which of the coding length is defined in PPS. However because the maximum code length of delta_poc_lt_minus1[i] is 16 bits, it is impossible to keep over 2^16 POC difference.

The method2 (Figure 2) also can support long distance reference pictures. For the descriptor of delta_poc_a_minus1[i] is ue(v), it looks inefficient for bit saving at relatively big POC difference.

# Proposed Method

In this contribution we want to propose a new method which can support for combining past and future reference pictures to RPS and can represent long distance picture.

To support both, it could be possible by changing syntax from “delta_poc_a_minus1[i]” to “delta_poc_combined[i]” and by changing its descriptor from “ue(v)” to “se(v)” in Figure 2. Figure 3 describes the proposed syntax.

Figure 3 The syntax for proposed combined signaling

First, regarding future picture signaling, it could be easily understood. Because the descriptor changed from “ue(v)” to “se(v)”, the code word can have the signed value. So we can suppose that if the code word is positive value this is for future picture’s combination and vice versa.

Second, regarding bit efficiency for long-distance picture, if the code word for “se(v)” is equal to 0 (we want to call this “exception case for long distance picture”) then we can suppose that the next bits is coded as “u(32)” and this delta value is only for past for long distance picture (32 bits is for example). In this case we can support up to 2^32 POC distance. If we want to support more than 2^32, we just code this 32 bits to 0 and fork it again. It means that we should read another “u(32)” bits more. Definitely there will be few application that needs over 2^32 POC difference without any IDR picture. Figure 4 explains the described process (we assume that the maximum bit depth for exception case is 8 for simple example).

Figure 4 The proposed combined signaling

# Comments on long-term reference picture

In AVC, long-term reference picture was differently process in DPB and decoding process.

- Marked as “long-term reference picture” in DPB via MMCO

- MV scaling

- Weighted Prediction

- Reference picture list initialization

In HEVC, if there is not any certain reason to have long-term reference picture, it is recommended to unify the decoding process for reference pictures (not short-term and long-term) for implementation simplicity while keeping the functionality that is referring to long-distance reference picture.

- Marked as “long-term reference picture” in DPB via MMCO

  - MMCO command is not valid in AHG21

- MV scaling

  - Consideration for long distance reference picture in MV scaling has no effect

  - JCTVC-G551 [Restriction on motion vector scaling for Merge and AMVP]

    - 0.0%(RAHE) 0.0%(RALC) -0.1%(LDHE) 0.0%(LDLC) -0.1%(LPHE) -0.1%(LPLC)

- Weighted Prediction

  - WP is not ready for HM but simple restriction may solve the problem

- Reference picture list initialization

  - Reference picture reordering can be sufficient

# Conclusion

It is recommended that combined signaling is discussed in AHG21 and the limitation for long-distance picture should be removed. Furthermore, it is highly recommended that simple reference picture buffer management is required. It is understood that one of motivation in AHG21 was for simple and clear process for picture buffer management.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).