| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, 21-30 Nov, 2011 | Document: JCTVC- G528 WG11 Number: m22092 |
| --- | --- |

| Title: | CE4: Cross-Check report for CE4 subset3 of TI'compact representation of quantization matrices |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jianhua Zheng Q20, Shi Chuang Ke Ji Shi Fan Yuan No.156 Beiqing Rd.,Hai-Dian District Beijing 100095, P.R. China | Tel: Email: | +86-10-60612141 zhengjianhua@huawei.com |
| Source: | HiSilicon Technologies |  |  |

_____________________________

# Abstract

This contribution provides a summary of the cross-checks performed by HiSilicon for TI’ CE4 proposal (JCTVC-G083/ F085) on quantization matrix compression. The cross-check rate-distortion performance results and coded matrix bits for TI’ down-sampling tools (QMatxCompressionOptionx = 8) with asymmetric matrix and symmetric matrix cases match those provided by TI.

# Introduction

In JCTVC-F085[1], a compact representation of quantization matrices was proposed. The proposed algorithm contains four tools, 1) matrix down-sampling, 2) 135 degree symmetry processing, 3) 45 degree symmetry processing and 4) unsigned exp-golomb coding. Those four tools can be turned on/off with high-level control parameters and combined arbitrarily. In CE4[2], following tests are conducted to evaluate coding performance.

Test 1: Results of compression performance of matrix itself (matrix only)

Test 2: Its impact on overall HM performance

For Test1, only one frame of all intra is test to evaluate the coded matrix bits and the matrix average error and compression ratio.

For Test2, QMs are changed frame by frame, and are encoded at each frame. The matrices at each frame are generated by the software provided by the proponent. Then the impact on the overall performance can be studied. In this subtest, the ratio of the generated bits for matrix coefficients and the other remaining bits are measured. BD rate/PSNR are measured compared with AVC quantization method (F362)[3]. The proponent generated a dither value 2 for the matrix of chroma U and 4 for the matrix of chroma V.

We have studied the code provided by the proponents, and evaluate the down sampling tools.

QMatxCompressionOptionx = 8 with asymmetric matrix  (down sampling)

QMatxCompressionOptionx = 8 with symmetric matrix  (down sampling)   X = 0, 1, 2, 3

The asymmetric matrix and symmetric matrix used in test are same as in CE4[2].

# Simulation Results

The implementation is on HM 4.0. Tests were conducted on all configurations on the specified QPs (22, 27, 32, 37) [4].

## TEST1 results

The summary for the “dither” case is as follows,

| <Subtest3.1:AVC> |  |  |  | Symmetry |  |  | Asymmetry |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | bits |  |  | bits |  |  |
|  |  |  |  | 26252 |  |  | 28232 |  |  |
| <Subtest3.3:TI> |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Symmetry |  |  | Asymmetry |  |  |
| Options |  |  |  | bits | comp ratio | average error | bits | comp ratio | average error |
| down S. | UEK | sym45 | sym135 |  |  |  |  |  |  |
| 0 | 0 | 0 | 0 | 27226 | 1 | 0 | 29464 | 1 | 0 |
| 0 | 0 | 0 | 1 | 14803 | 1.84 | 0 | 14932 | 1.97 | 3.53 |
| 0 | 0 | 1 | 0 | 15418 | 1.77 | 5.06 | 13639 | 2.16 | 3.14 |
| 0 | 0 | 1 | 1 | 8585 | 3.17 | 5.06 | 8005 | 3.68 | 5.25 |
| 0 | 1 | 0 | 0 | 11168 | 2.44 | 3.69 | 10404 | 2.83 | 6.66 |
| 0 | 1 | 0 | 1 | 6749 | 4.03 | 3.47 | 6836 | 4.31 | 6.76 |
| 0 | 1 | 1 | 0 | 6554 | 4.15 | 7.83 | 5941 | 4.96 | 6.88 |
| 0 | 1 | 1 | 1 | 4223 | 6.45 | 7.47 | 4067 | 7.24 | 6.92 |
| 1 | 0 | 0 | 0 | 11551 | 2.36 | 0.43 | 12126 | 2.43 | 0.32 |
| 1 | 0 | 0 | 1 | 6685 | 4.07 | 0.43 | 6655 | 4.43 | 3.76 |
| 1 | 0 | 1 | 0 | 6996 | 3.89 | 4.73 | 5829 | 5.05 | 3.54 |
| 1 | 0 | 1 | 1 | 4330 | 6.29 | 4.73 | 3699 | 7.97 | 5.52 |
| 1 | 1 | 0 | 0 | 5733 | 4.75 | 3.52 | 4652 | 6.33 | 6.21 |
| 1 | 1 | 0 | 1 | 3683 | 7.39 | 3.16 | 3731 | 7.9 | 6.3 |
| 1 | 1 | 1 | 0 | 3764 | 7.23 | 6.92 | 2819 | 10.45 | 6.76 |
| 1 | 1 | 1 | 1 | 2566 | 10.61 | 6.49 | 2389 | 12.33 | 6.91 |

The summary for the “no dither” case is as follows,

| <Subtest3.1:AVC> |  |  |  | Symmetry |  |  | Asymmetry |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | bits |  |  | bits |  |  |
|  |  |  |  | 26240 |  |  | 28232 |  |  |
| <Subtest3.3: TI> |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Symmetry |  |  | Asymmetry |  |  |
| Options |  |  |  | bits | comp ratio | average error | bits | comp ratio | average error |
| down S. | UEK | sym45 | sym135 |  |  |  |  |  |  |
| 0 | 0 | 0 | 0 | 8856 | 1 | 0 | 9512 | 1 | 0 |
| 0 | 0 | 0 | 1 | 4864 | 1.82 | 0 | 4894 | 1.94 | 3.53 |
| 0 | 0 | 1 | 0 | 5052 | 1.75 | 5.06 | 4558 | 2.09 | 3.14 |
| 0 | 0 | 1 | 1 | 2852 | 3.11 | 5.06 | 2626 | 3.62 | 5.25 |
| 0 | 1 | 0 | 0 | 3390 | 2.61 | 3.78 | 3442 | 2.76 | 6.68 |
| 0 | 1 | 0 | 1 | 2102 | 4.21 | 3.53 | 2126 | 4.47 | 6.83 |
| 0 | 1 | 1 | 0 | 2012 | 4.4 | 7.91 | 1968 | 4.83 | 6.9 |
| 0 | 1 | 1 | 1 | 1352 | 6.55 | 7.52 | 1286 | 7.4 | 6.95 |
| 1 | 0 | 0 | 0 | 3631 | 2.44 | 0.43 | 3784 | 2.51 | 0.32 |
| 1 | 0 | 0 | 1 | 2137 | 4.14 | 0.43 | 2094 | 4.54 | 3.76 |
| 1 | 0 | 1 | 0 | 2263 | 3.91 | 4.73 | 1923 | 4.95 | 3.54 |
| 1 | 0 | 1 | 1 | 1399 | 6.33 | 4.73 | 1249 | 7.62 | 5.52 |
| 1 | 1 | 0 | 0 | 1391 | 6.37 | 3.95 | 1380 | 6.89 | 6.35 |
| 1 | 1 | 0 | 1 | 1001 | 8.85 | 3.47 | 998 | 9.53 | 6.65 |
| 1 | 1 | 1 | 0 | 933 | 9.49 | 7.28 | 905 | 10.51 | 6.81 |
| 1 | 1 | 1 | 1 | 774 | 11.44 | 6.69 | 693 | 13.73 | 7.07 |

## TEST2 results

The anchor in TEST2 is AVC like quantization matrix extension proposed by SONY. The following summarizes the results for down-sampling (QMatxCompressionOptionx = 8) with asymmetric matrix and symmetric matrix, respectively. The results match the data provided by TI.

Table 1 The results of down-sampling with symmetric matrix

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1.9% | -2.6% | -2.5% | -2.2% | -2.6% | -2.5% |
| Class B | -3.0% | -3.9% | -3.9% | -2.9% | -4.3% | -4.4% |
| Class C | -8.2% | -9.4% | -9.5% | -7.6% | -8.9% | -9.1% |
| Class D | -19.7% | -20.0% | -20.1% | -18.8% | -19.4% | -19.3% |
| Class E | -10.7% | -11.0% | -10.8% | -9.9% | -10.5% | -10.5% |
| Class F |  |  |  |  |  |  |
| Overall | -8.3% | -9.0% | -9.0% | -7.9% | -8.8% | -8.9% |
|  | -8.3% | -9.1% | -9.1% | -7.9% | -8.9% | -8.9% |
| Enc Time[%] | 101% |  |  | 99% |  |  |
| Dec Time[%] | 106% |  |  | 96% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -11.9% | -11.5% | -10.3% | -11.2% | -10.3% | -10.1% |
| Class B | -21.6% | -20.3% | -19.5% | -20.4% | -19.2% | -18.6% |
| Class C | -32.9% | -32.0% | -32.1% | -32.0% | -31.3% | -31.4% |
| Class D | -46.3% | -45.6% | -45.7% | -45.9% | -45.3% | -45.5% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | -27.8% | -26.9% | -26.5% | -27.0% | -26.1% | -25.9% |
|  | -27.8% | -26.9% | -26.5% | -27.0% | -26.1% | -26.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 107% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -22.9% | -20.7% | -20.2% | -21.7% | -19.7% | -19.4% |
| Class C | -33.2% | -31.7% | -31.9% | -32.4% | -31.2% | -31.4% |
| Class D | -46.1% | -45.0% | -45.2% | -45.7% | -44.7% | -45.0% |
| Class E | -48.2% | -47.0% | -47.0% | -47.3% | -46.1% | -46.2% |
| Class F |  |  |  |  |  |  |
| Overall | -36.0% | -34.5% | -34.4% | -35.2% | -33.8% | -33.8% |
|  | -36.0% | -34.5% | -34.4% | -35.2% | -33.8% | -33.9% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 104% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -22.3% | -20.3% | -19.6% | -21.0% | -19.1% | -18.7% |
| Class C | -32.8% | -31.2% | -31.3% | -31.9% | -30.6% | -30.8% |
| Class D | -45.7% | -44.5% | -44.7% | -45.4% | -44.3% | -44.7% |
| Class E | -47.8% | -46.5% | -46.5% | -46.8% | -45.3% | -45.7% |
| Class F |  |  |  |  |  |  |
| Overall | -35.6% | -34.0% | -33.8% | -34.6% | -33.2% | -33.3% |
|  | -35.6% | -34.0% | -33.9% | -34.6% | -33.2% | -33.4% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 109% |  |  | 100% |  |  |

Table2 The results of down-sampling with asymmetric matrix

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1.5% | -1.8% | -1.9% | -2.0% | -2.5% | -2.7% |
| Class B | -3.9% | -3.2% | -3.2% | -4.3% | -3.4% | -3.9% |
| Class C | -10.7% | -7.9% | -8.1% | -10.7% | -8.1% | -8.6% |
| Class D | -21.4% | -18.2% | -18.4% | -21.0% | -17.7% | -18.2% |
| Class E | -11.0% | -10.9% | -10.8% | -10.5% | -10.0% | -10.3% |
| Class F |  |  |  |  |  |  |
| Overall | -9.4% | -8.0% | -8.1% | -9.4% | -8.0% | -8.4% |
|  | -9.4% | -8.1% | -8.2% | -9.4% | -8.1% | -8.5% |
| Enc Time[%] | 101% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -11.8% | -10.6% | -10.7% | -11.1% | -10.1% | -10.2% |
| Class B | -22.0% | -19.5% | -18.8% | -20.6% | -18.1% | -17.9% |
| Class C | -33.8% | -31.5% | -31.7% | -33.0% | -30.6% | -30.8% |
| Class D | -46.9% | -45.4% | -45.6% | -46.5% | -45.0% | -45.1% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | -28.2% | -26.3% | -26.2% | -27.4% | -25.5% | -25.5% |
|  | -28.3% | -26.3% | -26.3% | -27.5% | -25.5% | -25.6% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 104% |  |  | 96% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -22.9% | -20.2% | -19.9% | -21.7% | -18.8% | -18.7% |
| Class C | -33.4% | -31.5% | -31.4% | -32.6% | -30.5% | -30.7% |
| Class D | -46.4% | -45.1% | -45.0% | -45.9% | -44.7% | -44.7% |
| Class E | -49.0% | -47.5% | -47.7% | -48.0% | -46.6% | -46.9% |
| Class F |  |  |  |  |  |  |
| Overall | -36.3% | -34.3% | -34.3% | -35.4% | -33.4% | -33.5% |
|  | -36.3% | -34.3% | -34.3% | -35.4% | -33.4% | -33.5% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 93% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -22.2% | -19.5% | -19.2% | -20.9% | -18.1% | -18.0% |
| Class C | -32.9% | -30.8% | -30.8% | -32.1% | -29.9% | -30.2% |
| Class D | -45.9% | -44.5% | -44.6% | -45.5% | -44.1% | -44.1% |
| Class E | -48.5% | -47.1% | -47.3% | -47.4% | -46.1% | -46.3% |
| Class F |  |  |  |  |  |  |
| Overall | -35.7% | -33.7% | -33.7% | -34.8% | -32.8% | -32.9% |
|  | -35.7% | -33.7% | -33.7% | -34.8% | -32.8% | -32.9% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 97% |  |  | 96% |  |  |

More detailed coding results can be found in the Excel attachment.

# Conclusion

We have cross-checked TI’s proposal, studied the algorithm and verified their coding simulation results. The cross-check rate-distortion performance results and coded matrix bits for TI’ down-sampling tools (QMatxCompressionOptionx = 8) with asymmetric matrix and symmetric matrix cases match those provided by TI.

# References

[1] M. Zhou, V. Sze, “Further study on compact representation of quantization matrices” in JCTVC-F085, Torino, IT, July. 2011.

[2] K.Sato, “Description of Core Experiment 4 (CE4): Quantization,” JCTVC-F904, Torino, IT, July. 2011.

[3] T.Suzuki, A.Tabatabai, et.al “Proposal to support quantization matrix in HEVC” in JCTVC-F362, Torino, IT, July. 2011.

[4] Frank Bossen, “Common test conditions and software reference configurations,” JCTVC-F900, Torino, IT, July. 2011.