| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, 21-30 Nov, 2011 | Document: JCTVC-G530 WG11 Number: m22094 |
| --- | --- |

| Title: | Non-CE4: Layered quantization matrices compression |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yunfei Wang, Tsinghua Jianhua Zheng, HiSilicon Xiaozhen Zheng, HiSilicon Yun He, Tsinghua Q20, Shi Chuang Ke Ji Shi Fan Yuan No.156 Beiqing Rd.,Hai-Dian District Beijing 100095, P.R. China | Tel: Email: | +86-10-60612141 zhengjianhua@huawei.com |
| Source: | HiSilicon Technologies Tsinghua University |  |  |

_____________________________

# Abstract

In Torino meeting, it is agree to use AVC-like quantization matrix extension to support the larger block transform. However, results in F362[2] show that the AVC-like compression method is not equivalent to encode the larger quantization matrix above 8x8. Several tools are used in [3][4] to compress the quantization matrix. However, the coded bit of matrix is still occupied a large proportion in bitstream. It is important to find a simple and efficient quantization matrix coding method to reduce the matrix bits.

This contribution provides a layered coding structure for quantization matrix compression and reuse some tools in [3] to further improve the compression performance. The compress ratio can be increased to 27x comparing to AVC method and 2x above comparing to [3]. The proposed layered coding structure is a one-pass coding method, encoding time and design complexity can be reduced.

# Description

## Layered Coding structure

In this layered coding structure, different elements in quantization matrix are coded in different matrix layers. A sample layered 8x8 matrix is shown in Fig.1. The numbers in the Fig.1 matrix stand for the layer numbers. As in Fig.1, five layers are predefined and the layer number starts from 0 to 4. And the maximum layer numbers, for example, 4 for 8x8 quantization matrix, are predefined for different size of matrix. In this proposal, 3 for 4x4 matrix, 4 for 8x8 matrix, 5 for 16x16 matrix and 6 for 32x32 matrix, respectively.

The max layer of NxN is predefined as log2N+1. An example is shown in Fig. 1.

Fig. 1, Example of 8x8 matrix layers

Lower layered elements are coded before higher layered elements. The current element can be predicted from coded elements in the previous and current layer and the residue can be coded. Since the predicted residue is coded in matrix bitstream, the matrix element can be reconstructed using the predicted value and residue in lossless way. If compression ratio is required to increase for larger matrix, the residue in higher layer is not coded to reduce the coded bits. Hence the elements in higher layers are reconstructed in a loss way using the predicted value instead of coding in bitstream.

The largest number of layer that should be coded is determined by QMSizeNCodeLevels and configured in encoder. QMSizeNCodeLevels need to be notified to the decoder and coded in bitstream. For an element (i, j) belongs to Layer k,

if k<=QMSizeNCodeLevels, the residue value should be coded, and the reconstructed value equals to the original value;

if k>QMSizeNCodeLevels, residues are not coded and the reconstructed value of element equals to the prediction value of the element.

## Prediction

For an element (i, j) belongs to Layer k (k>0), the current element’s prediction value is interpolated by the reconstruction value of two reference elements R1 and R2. If this element is vertical interpolated, R1 is the nearest coded element which is above it (in a column), and R2 is the nearest coded element which is below it (in a column); if this element is vertical interpolated, R1 is the nearest coded element which is left to it (in a row), and R2 is the nearest coded element which is right to it (in a row),

### Coding Order within a layer

For Layer k (k>1), the coding can be classified into 2 stages:

Stage 1: vertical stage. For each element in Layer k, it can be vertical interpolated (R1 and R2 are coded). The vertical interpolation can be paralleled.

Stage 2: horizontal stage. The rest elements are predicted by horizontal interpolation. Can be paralleled. The horizontal interpolation can be paralleled.

### Interpolation

If the distance of the current element to R1 and the distance of the current element to R2 are the same, the prediction value of the current element is conducted as:

( R1 + R2 +1 ) >>1

Otherwise, the interpolation is weighted by the distance.

## Copy mode

In case the user does not design all the 24 matrices, or the matrices are too much alike, the copy mode can be used. In this mode, only 1 bit is signaled for a matrix, and the matrix is derived directly from its reference matrix.

Down sampling may be required. In this case, the elements with even indexes are extracted from the reference matrix.

## Symmetry mode

One bit is signaled to determine if the matrix is symmetric. In this mode, half of the elements are coded. Same as in [3].

## Residue Coding

For residue from Layer k, if k<=4, the residue is code by signed exp-golomb code; otherwise, the residue is expected to be small, so it is coded by a run-length code.

## Differential mode

Because the difference between two adjacent matrix sets in temporal axis is normally small, the differential mode is employed. If there are coded quantization matrix sets, use the nearest matrix to be the reference matrix. Only the differential is coded. The difference coding method is the same as above. The reconstructed difference matrices are then added to the reference matrices to get the reconstructed matrices. If it is lossy coding, the lossy elements should be interpolated after the previous process is done.

# Simulation Results

The implementation is on HM 4.0. Tests were conducted on all configurations on the specified QPs (22, 27, 32, 37) [5].

## TEST1 results

In test1, only the matrix coding itself is measured. In this simulation, we used the test configuration defined in CE4 Subset3. The matrix error bit measurement and the “dither” method to generate the matrix frames by frames are same as done in CE4 Subset3[4].

The summary for the “dither” case is as follows,

| AVC | Symmetry. bits. 26240. |  |  | Asymmetry. bits. 28232. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| F475 |  | Symmetry |  | AvgError |  |  |  | Asymmetry |  | AvgError |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | bits |  |  |  |  |  | bits |  |  |  |  |
| Cond1: QScalex=1,1,1,1 |  | 3357 |  | 0 |  |  |  | 5719 |  | 0 |  |  |
| Cond2 :QScalex=2,2,2,2 |  | 2409 |  | 0.512132 |  |  |  | 4412 |  | 0.485294 |  |  |
| Cond3:QScalex= 2,2,7,7 |  | 1820 |  | 1.335662 |  |  |  | 3258 |  | 1.204044 |  |  |
| Cond4: :QScalex=6,6,8,8 |  | 725 |  | 2.259926 |  |  |  | 789 |  | 2.179044 |  |  |
| Where, QScalex stands for scale for regions x, (x=0,1,2,3). Cond1 is the lossless coding for matrix, and Cond2/Cond3/Cond4 are selected for matrix loss coding. In this proposal, the number of coded levels for the matrix sizex (x=0,1,2,3) is determined as QMSizexCodeLevels. The coded matrix bit and the average error is measured as following for lossless coding configuration and several loss coding configuration. |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposal | Symmetry |  | AvgError |  |  |  | Asymmetry |  | AvgError |  |  |  |
| QMSizexCodeLevels | bits |  |  |  |  |  | bits |  |  |  |  |  |
| 3,4,5,6 | 3372 |  | 0 |  |  |  | 5614 |  | 0 |  |  |  |
| 3,4,5,5 | 2187 |  | 0.234191 |  |  |  | 3661 |  | 0.188235 |  |  |  |
| 3,4,4,5 | 1740 |  | 0.349632 |  |  |  | 2984 |  | 0.253676 |  |  |  |
| 3,3,4,5 | 1580 |  | 0.419853 |  |  |  | 2690 |  | 0.308456 |  |  |  |
| 3,3,4,4 | 1076 |  | 0.814338 |  |  |  | 1876 |  | 0.559926 |  |  |  |
| 3,3,3,3 | 632 |  | 2.095221 |  |  |  | 1144 |  | 1.474265 |  |  |  |
| 1,2,2,3 | 314 |  | 2.297426 |  |  |  | 522 |  | 1.7625 |  |  |  |
| 1,1,2,3 | 297 |  | 2.332353 |  |  |  | 493 |  | 1.819853 |  |  |  |

The above numbers can be summarized as the following figure,

Illustration of coding error versus coded matrix bit:  dither case

The summary for the “no dither” case is as follows,

AVC

| Symmetry |  |  | Asymmetry |
| --- | --- | --- | --- |
| bits |  |  | bits |
| 26240 |  |  | 28232 |

| F475 | Symmetry | AvgError |  | Asymmetry | AvgError |
| --- | --- | --- | --- | --- | --- |
|  | bits |  |  | bits |  |
| Cond1: QScalex=1,1,1,1 | 3357 | 0 |  | 5719 | 0 |
| Cond2 :QScalex=2,2,2,2 | 2409 | 0.512132 |  | 4412 | 0.485294 |
| Cond3:QScalex= 2,2,7,7 | 1820 | 1.335662 |  | 3258 | 1.204044 |
| Cond4: :QScalex=6,6,8,8 | 725 | 2.259926 |  | 789 | 2.179044 |

| Proposal | Symmetry | AvgError |  | Asymmetry | AvgError |
| --- | --- | --- | --- | --- | --- |
| QMSizexCodeLevels | bits |  |  | bits |  |
| 3,4,5,6 | 3372 | 0 |  | 5614 | 0 |
| 3,4,5,5 | 2187 | 0.234191 |  | 3661 | 0.188235 |
| 3,4,4,5 | 1740 | 0.349632 |  | 2984 | 0.253676 |
| 3,3,4,5 | 1580 | 0.419853 |  | 2690 | 0.308456 |
| 3,3,4,4 | 1076 | 0.814338 |  | 1876 | 0.559926 |
| 3,3,3,3 | 632 | 2.095221 |  | 1144 | 1.474265 |
| 1,2,2,3 | 314 | 2.297426 |  | 522 | 1.7625 |
| 1,1,2,3 | 297 | 2.332353 |  | 493 | 1.819853 |

The above numbers for “no dither” test case can be summarized as the following figure,

Illustration of coding error versus coded matrix bit:  no dither case

It is observed that in the typical average coding error range (0, 2], the proposed layered quantization matrix compression method can further reduce the coded matrix bit to nearly a half.

# Conclusion

A layered compression approach is investigated to improve quantization matrices compression ratio. Simulations show that within same matrix coding error, the proposed approach can further reduce matrix bit to 50% of F475 in average. And it need only one pass to compress the quantization matrix, so the encoding time and design complexity to compress quantization matrix can be reduced.

# References

[1] K.Sato, “Description of Core Experiment 4 (CE4): Quantization,” JCTVC-F904, Torino, IT, July. 2011.

[2] T.Suzuki, A.Tabatabai, et.al “Proposal to support quantization matrix in HEVC” in JCTVC-F362, Torino, IT, July. 2011.

[3] J.Tanaka, Y.Morigami,T.Suzuki, “Enhancement of quantization matrix coding for HEVC” in JCTVC-F475, Torino, IT, July. 2011.

[4] M. Zhou, V. Sze, “Further study on compact representation of quantization matrices” in JCTVC-F085, Torino, IT, July. 2011.

[5] Frank Bossen, “Common test conditions and software reference configurations,” JCTVC-F900, Torino, IT, July. 2011.

# Patent rights declaration(s)

HiSilicon Technologies, Huawei Technologies, Tsinghua University may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).