| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, 21-30 Nov, 2011 | Document: JCTVC-G530 WG11 Number: m22094 |
| --- | --- |

| Title: | Non-CE4: Layered quantization matrices compression |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yunfei Wang, Tsinghua Jianhua Zheng, HiSilicon Xiaozhen Zheng, HiSilicon Yun He, Tsinghua Q20, Shi Chuang Ke Ji Shi Fan Yuan No.156 Beiqing Rd.,Hai-Dian District Beijing 100095, P.R. China | Tel: Email: | +86-10-60612141 zhengjianhua@huawei.com |
| Source: | HiSilicon Technologies Tsinghua University |  |  |

_____________________________

# Abstract

In Torino meeting, it is agree to use AVC-like quantization matrix extension to support the larger block transform. However, results in F362[2] show that the AVC-like compression method is not equivalent to encode the larger quantization matrix above 8x8. Several tools are proposed in [3][4] to compress the quantization matrix. However, the coded bit of matrix still occupied a large proportion in bitstream. It is important to find a simple and efficient quantization matrix coding method to reduce the matrix bits.

This contribution provides a layered coding structure for quantization matrix compression and reuse some tools in F475 [3] to further improve the compression performance. The compress ratio can be increased to 2x above comparing to F475[3]. The proposed layered coding structure is a one-pass coding method, encoding time and design complexity can be reduced. By controlling the number of coded layers, it is easily shift from matrix lossless coding to matrix lossy coding with high compression ratio, which is observed 140x~153x improvements against AVC method to coding the matrices with lossy configuration in CE4 Subset3 test2 condition [1], and obtain 15% BD rate reduction for All Intra, 47% BD rate reduction for Random Access, 61% BD rate reduction for Low Delay B and 60% BD rate reduction for Low Delay P to the overall coding performance compared to AVC method.

# Description

## Layered Coding structure

In this layered coding structure, different elements in quantization matrix are coded in different matrix layers. A sample layered 8x8 matrix is shown in Fig.1. The numbers in the matrix in Fig.1 stand for the layer numbers. As in Fig.1, five layers are predefined and the layer number starts from 0 to 4. And the maximum layer numbers are predefined for different size of matrix. In this proposal, the maximum layer number is predefined as 3 for 4x4 matrix, 4 for 8x8 matrix, 5 for 16x16 matrix and 6 for 32x32 matrix, respectively. Hence the maximum layer number of NxN is determined by log2N+1.

Fig. 1 Example of matrix layers for 8x8 matrix

In the layered coding structure, lower layered elements are coded before higher layered elements. The current element is predicted from coded elements in the previous and current layer and the residue of the predicted value and the original element is coded. Since the residue is coded in matrix bitstream, the matrix element can be reconstructed using the predicted value and residue in lossless way. If compression ratio is required to increase, the residue in higher layer is not coded to reduce the coded bits. Hence the elements in higher layers are reconstructed in a lossy way using the predicted value instead of coding the residues in bitstream.

The largest number of layer to be coded is determined by QMSizeNCodeLevels and configured in encoder. QMSizeNCodeLevels need to be notified to the decoder and coded in bitstream. For an element (i, j) belongs to Layer k,

if k<=QMSizeNCodeLevels, the residue value is coded, so the reconstructed value is obtained by the predicted value and the residue value, and the original element can be reconstructed with lossless;

if k>QMSizeNCodeLevels, residues are not coded, so the reconstructed value of element equals to the prediction value of the element, which is predicted from coded elements in the previous and current layer.

## Prediction

For an element (i, j) belongs to Layer k (k>0), the prediction value of current element is interpolated by the reconstruction value of two reference elements R1 and R2. If this element is vertical interpolated, R1 is the nearest coded element which is above it in the same column, and R2 is the nearest coded element which is below it in the same column; if this element is vertical interpolated, R1 is the nearest coded element which is left to it in the same row, and R2 is the nearest coded element which is right to it in the same row.

### Coding Order within a layer

For Layer k (k>1), the coding can be classified into 2 stages:

Stage 1: vertical stage. For each element in layer k, if it can be vertical interpolated, then element is vertical interpolated by reference elements R1 and R2, (R1 and R2 are the nearest coded elements). The vertical interpolation within this stage can be paralleled.

Stage 2: horizontal stage. The rest elements are predicted by horizontal interpolation. The horizontal interpolation within this stage can be paralleled.

Fig. 2 Illustration of vertical stage and horizontal stage.

### Interpolation

If the distance of the current element to R1 equals to the distance of the current element to R2, the prediction value of the current element is conducted as:

( R1 + R2 +1 ) >>1

Otherwise, the interpolation is weighted by the distance.

## Copy mode

There are totally 24 matrices in the quantization matrices set for one frame. In case the user does not design all the 24 matrices, or the matrices are too much alike, the copy mode can be used. If a matrix is determined as using this mode, only one bit is signaled for the matrix, and the matrix is derived directly from its reference matrix.

Down sampling may be required. In this case, the elements with even indexes are extracted directly from the reference matrix.

## Symmetry mode

One bit is signaled to determine if the matrix is symmetric. In this mode, half of the elements are coded, as same as in [3].

## Residue Coding

For residue from Layer k, if k<=4, the residue is code by signed exp-golomb code; otherwise, the residue is expected to be small, so it is coded by a run-length code.

## Differential coding

Because the difference between two adjacent matrix sets in temporal axis is normally small, the differential coding is employed. If there are coded quantization matrix sets, use the nearest matrix to be the reference matrix. The differential matrix is obtained via reference matrix, and the values in the differential matrix are coded. The coding method is the same as above. The reconstructed differential matrices are then added to the reference matrices to get the reconstructed matrices. If it is lossy coding, the lossy elements should be interpolated after the previous process is done.

# Simulation Results

The implementation is on HM 4.0 software. Tests were conducted on all configurations on the specified QPs (22, 27, 32, 37) [5].

## TEST1 results

In test1[1], only the coding performance of matrix itself is measured. In this simulation, we used the test configuration defined in CE4 Subset3. The method to measure the matrix error and matrix bit and the method to generate small “dither” values on quantization matrix frames by frames are same as in CE4 Subset3[1] and F475[3].

The summary for the “dither” case is as follows,

| AVC | Symmetry. bits. 26252. |  |  | Asymmetry. bits. 28232. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| F475 |  | Symmetry |  | AvgError |  |  |  |  | Asymmetry |  | AvgError |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | bits |  |  |  |  |  |  | bits |  |  |  |
| Cond1: QScalex=1,1,1,1 |  | 3813 |  | 0 |  |  |  |  | 6174 |  | 0 |  |
| Cond2 :QScalex=2,2,2,2 |  | 5704 |  | 0.49326 |  |  |  |  | 9912 |  | 0.496078 |  |
| Cond3:QScalex= 2,2,7,7 |  | 4030 |  | 1.502451 |  |  |  |  | 6956 |  | 1.348407 |  |
| Cond4: :QScalex=6,6,8,8 |  | 1237 |  | 1.938235 |  |  |  |  | 1800 |  | 2.151961 |  |
| Where, QScalex stands for scale for regions x, (x=0,1,2,3). Cond1 is the lossless coding for matrix, and Cond2/Cond3/Cond4 are selected for matrix lossy coding. In this proposal, the number of coded levels for the matrix sizex (x=0,1,2,3) is determined as QMSizexCodeLevels. The coded matrix bits and average coding error are measured for matrix lossless coding configuration and matrix lossy coding configuration. |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposal | Symmetry |  | AvgError |  |  | Asymmetry |  | AvgError |  |  |  |  |
| QMSizexCodeLevels | bits |  |  |  |  | bits |  |  |  |  |  |  |
| 3,4,5,6 | 10090 |  | 0 |  |  | 16814 |  | 0 |  |  |  |  |
| 3,4,5,5 | 6535 |  | 0.234191 |  |  | 10955 |  | 0.188235 |  |  |  |  |
| 3,4,4,5 | 5194 |  | 0.349632 |  |  | 8924 |  | 0.253676 |  |  |  |  |
| 3,3,4,5 | 4714 |  | 0.419853 |  |  | 8042 |  | 0.308456 |  |  |  |  |
| 3,3,4,4 | 3202 |  | 0.814338 |  |  | 5600 |  | 0.559926 |  |  |  |  |
| 3,3,3,3 | 1870 |  | 2.095221 |  |  | 3404 |  | 1.474265 |  |  |  |  |
| 2,2,3,3 | 1452 |  | 2.186642 |  |  | 2506 |  | 1.588603 |  |  |  |  |
| 2,2,2,3 | 1136 |  | 2.347549 |  |  | 1755 |  | 1.886397 |  |  |  |  |
| 1,2,2,3 | 1028 |  | 2.363358 |  |  | 1587 |  | 1.901838 |  |  |  |  |
| 2,2,2,2 | 950 |  | 3.114461 |  |  | 1532 |  | 3.914338 |  |  |  |  |
| 1,2,2,2 | 842 |  | 3.13027 |  |  | 1364 |  | 3.929779 |  |  |  |  |
| 1,1,2,2 | 709 |  | 3.183946 |  |  | 1116 |  | 4.014093 |  |  |  |  |
| 1,1,1,2 | 570 |  | 3.405147 |  |  | 836 |  | 4.347304 |  |  |  |  |

The above numbers for the “dither” case can be summarized as the figure below,

Matrix coding error versus coded bits:  dither case

The summary for the “no dither” case is as follows,

AVC

| Symmetry |  |  | Asymmetry |
| --- | --- | --- | --- |
| bits |  |  | bits |
| 26240 |  |  | 28232 |

| F475 | Symmetry | AvgError |  | Asymmetry | AvgError |
| --- | --- | --- | --- | --- | --- |
|  | bits |  |  | bits |  |
| Cond1: QScalex=1,1,1,1 | 3357 | 0 |  | 5719 | 0 |
| Cond2 :QScalex=2,2,2,2 | 2409 | 0.512132 |  | 4412 | 0.485294 |
| Cond3:QScalex= 2,2,7,7 | 1820 | 1.335662 |  | 3258 | 1.204044 |
| Cond4: :QScalex=6,6,8,8 | 725 | 2.259926 |  | 789 | 2.179044 |

| Proposal | Symmetry | AvgError |  | Asymmetry | AvgError |
| --- | --- | --- | --- | --- | --- |
| QMSizexCodeLevels | bits |  |  | bits |  |
| 3,4,5,6 | 3372 | 0 |  | 5614 | 0 |
| 3,4,5,5 | 2187 | 0.234191 |  | 3661 | 0.188235 |
| 3,4,4,5 | 1740 | 0.349632 |  | 2984 | 0.253676 |
| 3,3,4,5 | 1580 | 0.419853 |  | 2690 | 0.308456 |
| 3,3,4,4 | 1076 | 0.814338 |  | 1876 | 0.559926 |
| 3,3,3,3 | 632 | 2.095221 |  | 1144 | 1.474265 |
| 1,2,2,3 | 314 | 2.297426 |  | 522 | 1.7625 |
| 1,1,2,3 | 297 | 2.332353 |  | 493 | 1.819853 |

The above numbers for “no dither” test case can be summarized as the following figure,

Matrix coding error versus coded bits:  no dither case

It is observed that in the typical average coding error range (0, 2], the proposed layered quantization matrix compression method can further reduce the coded matrix bits to nearly a half.

## TEST2 results

Test2[1] is conducted to evaluate the impact of matrix coding to the overall HM performance. In test2, QMs are changed frame by frame, and are encoded at each frame. The method to generate small “dither” values to change the quantization matrix frames by frames is provided in CE4 subset3 software. The ratio of the generated bits for matrix and the other remaining bits, as well as the BD rate/PSNR are measured and compared with AVC quantization method. Two sets of quantization matrices, symmetric matrices and asymmetric matrices, are provided by CE4 subset3 and can be found in attachment.

In following, three aspects were studied.

### Coding performance impact on the overall HM

This experiment is conducted to evaluate the quantization matrix compression impact to the overall HM coding performance, BD rate/PSNR are measured and the compression configuration uses QMSizexCodeLevels = 2,2,2,3.

- symmetric matrices coding

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1.7% | -3.1% | -2.9% | -3.7% | -8.0% | -7.4% |
| Class B | -6.3% | -6.7% | -6.6% | -7.3% | -10.0% | -9.9% |
| Class C | -15.5% | -15.9% | -16.3% | -15.1% | -16.3% | -16.9% |
| Class D | -34.6% | -34.1% | -34.4% | -33.4% | -33.6% | -33.8% |
| Class E | -19.0% | -19.2% | -18.0% | -18.7% | -21.8% | -21.2% |
| Class F |  |  |  |  |  |  |
| Overall | -14.8% | -15.2% | -15.1% | -15.1% | -17.3% | -17.3% |
|  | -14.8% | -15.2% | -15.2% | -15.1% | -17.4% | -17.4% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 97% |  |  | 100% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -19.4% | -19.0% | -18.6% | -19.0% | -17.7% | -16.3% |
| Class B | -37.6% | -35.0% | -33.9% | -36.1% | -33.4% | -32.4% |
| Class C | -56.8% | -55.1% | -55.3% | -55.8% | -54.2% | -54.4% |
| Class D | -79.2% | -77.8% | -78.0% | -78.6% | -77.5% | -77.7% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | -47.6% | -46.0% | -45.7% | -46.7% | -45.0% | -44.4% |
|  | -47.8% | -46.1% | -45.9% | -46.9% | -45.0% | -44.5% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -39.9% | -36.6% | -36.0% | -38.7% | -35.4% | -35.1% |
| Class C | -57.3% | -54.8% | -55.2% | -56.5% | -54.3% | -55.0% |
| Class D | -79.0% | -77.3% | -77.4% | -78.6% | -77.0% | -77.5% |
| Class E | -82.4% | -80.0% | -80.6% | -81.1% | -78.4% | -79.3% |
| Class F |  |  |  |  |  |  |
| Overall | -62.0% | -59.5% | -59.5% | -61.1% | -58.6% | -59.0% |
|  | -62.0% | -59.6% | -59.6% | -61.1% | -58.6% | -59.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 97% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -38.8% | -35.9% | -35.0% | -37.4% | -34.4% | -33.9% |
| Class C | -56.6% | -54.1% | -54.3% | -55.7% | -53.6% | -54.3% |
| Class D | -78.5% | -76.5% | -77.0% | -78.2% | -76.3% | -76.9% |
| Class E | -81.8% | -79.3% | -80.0% | -80.3% | -77.8% | -78.6% |
| Class F |  |  |  |  |  |  |
| Overall | -61.2% | -58.7% | -58.7% | -60.2% | -57.8% | -58.1% |
|  | -61.2% | -58.8% | -58.9% | -60.2% | -57.9% | -58.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 104% |  |  | 102% |  |  |

- asymmetric matrices coding

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1.5% | -5.2% | -4.4% | -4.3% | -10.7% | -11.5% |
| Class B | -6.0% | -6.2% | -5.9% | -8.0% | -10.5% | -11.8% |
| Class C | -16.6% | -14.3% | -14.3% | -17.0% | -15.1% | -16.3% |
| Class D | -34.5% | -31.3% | -31.5% | -33.8% | -30.9% | -31.4% |
| Class E | -18.7% | -19.8% | -19.1% | -19.0% | -22.3% | -22.9% |
| Class F |  |  |  |  |  |  |
| Overall | -14.8% | -14.7% | -14.4% | -15.9% | -17.3% | -18.2% |
|  | -14.8% | -14.8% | -14.5% | -15.9% | -17.4% | -18.3% |
| Enc Time[%] | 101% |  |  | 97% |  |  |
| Dec Time[%] | 98% |  |  | 96% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -20.2% | -19.9% | -19.3% | -20.0% | -19.9% | -18.9% |
| Class B | -37.2% | -33.9% | -32.8% | -35.8% | -32.9% | -32.0% |
| Class C | -56.6% | -53.7% | -53.8% | -55.5% | -52.7% | -52.9% |
| Class D | -78.6% | -76.8% | -76.8% | -77.9% | -76.2% | -76.3% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | -47.5% | -45.4% | -45.0% | -46.6% | -44.7% | -44.3% |
|  | -47.6% | -45.4% | -45.1% | -46.7% | -44.7% | -44.3% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 98% |  |  | 97% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -39.1% | -35.3% | -34.5% | -38.7% | -35.4% | -33.9% |
| Class C | -56.6% | -53.8% | -53.7% | -55.7% | -53.4% | -53.2% |
| Class D | -78.3% | -76.5% | -76.2% | -77.7% | -76.2% | -76.0% |
| Class E | -82.4% | -79.9% | -79.8% | -81.0% | -78.9% | -79.4% |
| Class F |  |  |  |  |  |  |
| Overall | -61.4% | -58.6% | -58.2% | -60.6% | -58.2% | -57.8% |
|  | -61.4% | -58.7% | -58.4% | -60.6% | -58.3% | -57.9% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -38.0% | -34.5% | -33.7% | -37.3% | -34.4% | -33.0% |
| Class C | -55.7% | -52.9% | -52.9% | -54.9% | -52.4% | -52.3% |
| Class D | -77.6% | -75.9% | -75.8% | -77.1% | -75.4% | -75.2% |
| Class E | -81.8% | -79.3% | -79.3% | -80.2% | -78.2% | -78.7% |
| Class F |  |  |  |  |  |  |
| Overall | -60.5% | -57.9% | -57.6% | -59.7% | -57.4% | -56.9% |
|  | -60.5% | -58.0% | -57.8% | -59.7% | -57.4% | -57.1% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 103% |  |  |

The results show that the encoding time and decoding time is nearly same to the AVC method, while obtain 15% BD rate gain for All Intra, 47% BD rate gain for Random Access, 61% BD rate gain for Low Delay B and 60% BD rate gain for Low Delay P compared to AVC method.

### Ratio of the generated matrix bits in bitstream

The ratio of the generated bits for matrix and the other remaining bits is studied. This is conducted to evaluate the behaviors of quantization matrix methods in practical application. The compression configuration is QMSizexCodeLevels =2,2,2,3.

- symmetric matrices coding

| All Seqs | AVC Method |  |  |  |  | proposed method |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | QP=22 | QP=27 | QP=32 | QP=37 | average | QP=22 | QP=27 | QP=32 | QP=37 | average |
| Intra_LC | 60.1% | 91.0% | 135.3% | 196.1% | 120.7% | 0.5% | 0.8% | 1.3% | 2.3% | 1.2% |
| Intra_HE | 63.2% | 96.5% | 145.0% | 213.4% | 129.5% | 0.5% | 0.8% | 1.4% | 2.5% | 1.3% |
| RA_LC | 208.7% | 313.2% | 420.7% | 520.3% | 365.7% | 2.7% | 5.6% | 11.3% | 21.1% | 10.2% |
| RA_HE | 214.6% | 321.5% | 431.1% | 533.8% | 375.3% | 2.8% | 5.8% | 11.7% | 22.1% | 10.6% |
| LB_LC | 296.2% | 434.9% | 551.9% | 642.3% | 481.3% | 3.9% | 9.1% | 18.7% | 35.5% | 16.8% |
| LB_HE | 305.5% | 444.9% | 561.4% | 651.1% | 490.7% | 4.1% | 9.5% | 19.6% | 37.6% | 17.7% |
| LP_LC | 280.2% | 424.7% | 546.5% | 639.7% | 472.8% | 3.5% | 8.5% | 18.1% | 35.1% | 16.3% |
| LP_HE | 290.2% | 435.4% | 557.1% | 649.6% | 483.1% | 3.7% | 8.9% | 19.1% | 37.3% | 17.2% |

| HDSeqs | AVC Method |  |  |  |  | proposed method |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ClassB | QP=22 | QP=27 | QP=32 | QP=37 | average | QP=22 | QP=27 | QP=32 | QP=37 | average |
| Intra_LC | 18.0% | 32.0% | 53.8% | 89.0% | 48.2% | 0.1% | 0.2% | 0.4% | 0.7% | 0.4% |
| Intra_HE | 19.1% | 34.6% | 60.2% | 102.9% | 54.2% | 0.1% | 0.2% | 0.4% | 0.8% | 0.4% |
| RA_LC | 94.2% | 196.2% | 322.4% | 450.6% | 265.9% | 0.7% | 1.7% | 3.7% | 7.1% | 3.3% |
| RA_HE | 99.7% | 206.9% | 339.2% | 474.7% | 280.1% | 0.8% | 1.9% | 3.9% | 7.9% | 3.6% |
| LB_LC | 94.0% | 203.0% | 343.0% | 483.4% | 280.9% | 0.7% | 1.8% | 4.1% | 8.7% | 3.8% |
| LB_HE | 99.7% | 214.4% | 359.1% | 501.5% | 293.7% | 0.8% | 2.0% | 4.5% | 9.5% | 4.2% |
| LP_LC | 85.8% | 190.9% | 332.1% | 476.7% | 271.4% | 0.6% | 1.7% | 3.9% | 8.3% | 3.6% |
| LP_HE | 92.9% | 204.5% | 351.0% | 497.6% | 286.5% | 0.7% | 1.8% | 4.2% | 9.2% | 4.0% |

- asymmetric matrices coding

| All Seqs | AVC Method |  |  |  |  | proposed method |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | QP=22 | QP=27 | QP=32 | QP=37 | average | QP=22 | QP=27 | QP=32 | QP=37 | average |
| Intra_LC | 53.6% | 83.0% | 126.3% | 188.6% | 112.9% | 0.5% | 0.8% | 1.3% | 2.3% | 1.2% |
| Intra_HE | 58.2% | 91.0% | 140.0% | 209.5% | 124.7% | 0.5% | 0.8% | 1.4% | 2.6% | 1.4% |
| RA_LC | 196.4% | 303.7% | 414.0% | 515.1% | 357.3% | 2.6% | 5.8% | 11.7% | 22.1% | 10.5% |
| RA_HE | 204.3% | 315.1% | 428.0% | 532.4% | 369.9% | 2.8% | 6.1% | 12.3% | 23.6% | 11.2% |
| LB_LC | 281.9% | 426.2% | 547.6% | 641.1% | 474.2% | 3.9% | 9.4% | 19.7% | 37.9% | 17.7% |
| LB_HE | 294.5% | 439.0% | 558.6% | 650.7% | 485.7% | 4.1% | 10.0% | 21.1% | 40.8% | 19.0% |
| LP_LC | 265.9% | 416.0% | 542.4% | 638.8% | 465.8% | 3.5% | 8.8% | 19.1% | 37.5% | 17.2% |
| LP_HE | 278.9% | 429.5% | 554.4% | 649.3% | 478.0% | 3.7% | 9.4% | 20.4% | 40.5% | 18.5% |

| HDSeqs | AVC Method |  |  |  |  | proposed method |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ClassB | QP=22 | QP=27 | QP=32 | QP=37 | average | QP=22 | QP=27 | QP=32 | QP=37 | average |
| Intra_LC | 15.1% | 28.7% | 49.9% | 85.5% | 44.8% | 0.1% | 0.2% | 0.4% | 0.7% | 0.4% |
| Intra_HE | 16.9% | 32.6% | 58.1% | 100.8% | 52.1% | 0.1% | 0.2% | 0.5% | 0.9% | 0.4% |
| RA_LC | 83.2% | 186.6% | 315.4% | 445.7% | 257.7% | 0.7% | 1.8% | 3.8% | 7.5% | 3.5% |
| RA_HE | 89.7% | 200.7% | 337.7% | 475.9% | 276.0% | 0.7% | 1.9% | 4.2% | 8.5% | 3.8% |
| LB_LC | 84.4% | 194.0% | 336.3% | 480.5% | 273.8% | 0.7% | 1.9% | 4.3% | 9.2% | 4.0% |
| LB_HE | 91.2% | 207.6% | 353.7% | 499.4% | 288.0% | 0.7% | 2.0% | 4.7% | 10.1% | 4.4% |
| LP_LC | 77.3% | 182.6% | 326.2% | 474.7% | 265.2% | 0.6% | 1.7% | 4.1% | 8.8% | 3.8% |
| LP_HE | 85.2% | 198.0% | 346.1% | 495.9% | 281.3% | 0.7% | 1.9% | 4.5% | 9.8% | 4.2% |

It is observed that the generated matrix bits have been reduced much more comparing to AVC method. The matrix bits ratio has been decreased from 1-5x to 1.2-18.5%. The generated bits of matrices have reached to an acceptable level and lead light impact on video data, especially for the HD Class B sequences, the generated matrix bits ratio of has been reduced to 4% around. This implied the quantization matrices coded with proposed method can now be used in HD application. While, in SD application, it still need improvement to reduce matrix bits more.

### Compression ratio against AVC method

The compression ratio against AVC method is also investigated. This is conducted to evaluate the quantization matrices compression ratio improvement over than AVC method. The compression ratio against AVC is estimated by the ratio of AVC generated matrix bits and those of the proposed method. The compression configuration is QMSizexCodeLevels =2,2,2,3.

- symmetric matrices coding

|  | proposed method |
| --- | --- |
|  | Average compression ratio against AVC |
| Intra_LC | 152.6 |
| Intra_HE | 152.6 |
| RA_LC | 152.3 |
| RA_HE | 152.3 |
| LB_LC | 153.5 |
| LB_HE | 153.5 |
| LP_LC | 153.5 |
| LP_HE | 153.5 |

- asymmetric matrices coding

|  | proposed method |
| --- | --- |
|  | Average compression ratio against AVC |
| Intra_LC | 139.3 |
| Intra_HE | 139.3 |
| RA_LC | 138.9 |
| RA_HE | 138.9 |
| LB_LC | 140.7 |
| LB_HE | 140.7 |
| LP_LC | 140.7 |
| LP_HE | 140.7 |

It is observed that with configuration of QMSizexCodeLevels =2,2,2,3, the compression ratio of the proposed method reached 153x against AVC for coding symmetric matrices and 140x against AVC for coding asymmetric matrices, respectively. If using configuration of QMSizexCodeLevels =1,1,1,2, the compression ratio of the proposed method can reach over 300x against AVC.

More details results can be found in attachment for lossy configuration QMSizexCodeLevels =1,1,1,2, and lossless configuration QMSizexCodeLevels =3,4,5,6.

# Conclusion

A layered compression approach is proposed to improve the quantization matrices compression. It can be configured as lossless coding or lossy coding with a large range of compression ratio. Simulations show that with similar matrix coding error, the proposed approach can further reduce matrix bit to 50% in average compared to F475. In test2 simulation, the average generated matrix bits have been reduce to 1.2-18.5% of video data for all sequences in average, around 4% for HD sequences. This deduces that the quantization matrices coded with proposed method can be used in practice and lead light impact on video data. In CE4 Subset3 test2 condition [1], the compression ratio is observed 140x~153x improvements against AVC method with similar encoding and decoding time. In addition, only one pass processing need to compress the quantization matrices, so the encoding time and design complexity to compress quantization matrix can be reduced compared to F475. By controlling the number of coded layers, it is easily shift from matrix lossless coding to matrix lossy coding with high compression ratio.

# References

[1] K.Sato, “Description of Core Experiment 4 (CE4): Quantization,” JCTVC-F904, Torino, IT, July. 2011.

[2] T.Suzuki, A.Tabatabai, et.al “Proposal to support quantization matrix in HEVC” in JCTVC-F362, Torino, IT, July. 2011.

[3] J.Tanaka, Y.Morigami,T.Suzuki, “Enhancement of quantization matrix coding for HEVC” in JCTVC-F475, Torino, IT, July. 2011.

[4] M. Zhou, V. Sze, “Further study on compact representation of quantization matrices” in JCTVC-F085, Torino, IT, July. 2011.

[5] Frank Bossen, “Common test conditions and software reference configurations,” JCTVC-F900, Torino, IT, July. 2011.

# Patent rights declaration(s)

HiSilicon Technologies, Huawei Technologies, Tsinghua University may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).