| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G532 |
| --- | --- |

| Title: | CE5 2.1 : Improvement of CAVLC run- coding by prediction mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chanyul Kim Youngo Park Kwang Pyo Choi | Tel: Email: | dionism@samsung.com youngo.park@samsung.com kp5.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

JCTVC-F458 is proposed to run-level coding according to prediction mode of block such as intra and inter at 6th Torino meeting. Simulation shows that -0.1%/0.5%/0.3% AI_LC, 0.0%/0.7%/0.7% RA_LC, and 0.0%/0.2%/0.1% LD_LC. There is loss in chroma.

# Introduction

In the current HM, block type is classified as U, V, inter Luma, intra Luma. U and V coefficients in the run coding are coded using cn = xRunLevelIndInter(lev, run, maxrun,scale)[1].  We propose run- coding for intra or inter regardless of considering Luma or Chroma. In the middle of Y,U,V coding sequentially, they use the same function which increase the cache hit-ratio. It gives a benefit for complexity reduction and simplification.

# Algorithm description

## Luma

The difference of run-coding in both HM4.0 and proposed one is depicted in Figure 1. In the current HM, Intra Luma coefficients are coded using xRunLevelInd(), others are coded using xRunLevelIndInter(). The proposed scheme only checks the block type as inter or intra not to subdivide.

#if  RUN_PRED_MODE

if(isIntra)

#else

if(blockType == 2 || blockType == 5)

#endif

{

#if MOD_INTRA_TABLE

cn = xRunLevelInd(lev, run, maxrun, LumaRunTr1[aiTableTr1[(blSize&4)>>2][tr1]][tmprun]);

#else

cn = xRunLevelInd(lev, run, maxrun, pLumaRunTr1[tr1][tmprun]);

#endif

}

else

{

cn = xRunLevelIndInter(lev, run, maxrun, scale);

}

Figure 1 Comparisons of HM4.0 and Propose method

# Simulation Results

Using HM4.0, simulation is performed under the common test condition [3]. The result shows the gain in Luma in AI_LC and chroma loss without complexity.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | #VALUE! | #VALUE! | #VALUE! | 0.0% | 0.2% | 0.0% |
| Class B | #VALUE! | #VALUE! | #VALUE! | -0.1% | 0.5% | 0.4% |
| Class C | #VALUE! | #VALUE! | #VALUE! | -0.1% | 0.6% | 0.5% |
| Class D | #VALUE! | #VALUE! | #VALUE! | -0.1% | 0.6% | 0.3% |
| Class E | #VALUE! | #VALUE! | #VALUE! | 0.0% | 0.4% | 0.5% |
| Overall | #VALUE! | #VALUE! | #VALUE! | -0.1% | 0.5% | 0.3% |
|  | #VALUE! | #VALUE! | #VALUE! | -0.1% | 0.4% | 0.3% |
| Enc Time[%] | #NUM! |  |  | 100% |  |  |
| Dec Time[%] | #NUM! |  |  | 100% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | #VALUE! | #VALUE! | #VALUE! | 0.0% | 1.0% | 0.8% |
| Class B | #VALUE! | #VALUE! | #VALUE! | 0.0% | 0.6% | 0.7% |
| Class C | #VALUE! | #VALUE! | #VALUE! | 0.0% | 0.6% | 0.7% |
| Class D | #VALUE! | #VALUE! | #VALUE! | 0.0% | 0.8% | 0.6% |
| Class E |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | 0.0% | 0.7% | 0.7% |
|  | #VALUE! | #VALUE! | #VALUE! | 0.0% | 0.7% | 0.7% |
| Enc Time[%] | #NUM! |  |  | 99% |  |  |
| Dec Time[%] | #NUM! |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | 0.0% | 0.3% | 0.4% |
| Class C | #VALUE! | #VALUE! | #VALUE! | 0.0% | 0.3% | 0.1% |
| Class D | #VALUE! | #VALUE! | #VALUE! | 0.0% | 0.2% | -0.2% |
| Class E | #VALUE! | #VALUE! | #VALUE! | -0.1% | -0.1% | 0.2% |
| Overall | #VALUE! | #VALUE! | #VALUE! | 0.0% | 0.2% | 0.1% |
|  | #VALUE! | #VALUE! | #VALUE! | 0.0% | 0.2% | 0.1% |
| Enc Time[%] | #NUM! |  |  | 100% |  |  |
| Dec Time[%] | #NUM! |  |  | 100% |  |  |

# Conclusion

This proposal is the improvement of CAVLC run-coding by prediction mode of block type as inter or intra. Small chroma loss is observed.

# References

- M. Karczewicz, X.Wang, W.-J. Chien, “CE5: coefficient coding with LCEC for large blocks”, JCTVC-E384, Geneva, CH, March. 2011..

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-C500, JCT-VC 3rd Meeting, Guangzhou, October 2010.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex : WD Text change

- The parsing process described in subclause Error: Reference source not found is invoked with vlcNum as input and the variable codeNum as output. If blockType is equal to 2 or blockType is equal to 5, the value of run_level_one is derived as follows.

- The parsing process described in subclause Error: Reference source not found is invoked with vlcNum as input and the variable codeNum as output. If pred mode of block is Intra, the value of run_level_one is derived as follows.