| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G533 |
| --- | --- |

| Title: | On syntax for clean random access (CRA) pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Youngo Park Heechul Yang Chanyul Kim Kwang Pyo Choi JeongHoon Park | Email: | youngo.park@samsung.com h_c.yang@samsung.com dionism@samsung.com kp5.choi@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution has information and proposal. Information is about discrepancy between HM S/W and WD at IDR picture. Proposal is about syntax for clean random access (CRA) picture.

# Information about the discrepancy between HM S/W and WD at IDR picture

## Picture order count in WD

Picture order count (POC) represents picture’s display order. POC is derived from pic_order_cnt_lsb which is signaled in slice header and from PicOrderCntMsb which is derived from previous POC variables. Figure 1 describes the process to get POC when POC type is equal to 0 in AVC.

Figure 1 POC derivation process when POC type is equal to 0

POC is never wraps around and is reset to 0 at IDR picture or memory_management_control_operation is equal to 5. Figure 2 describes POCLsb(pic_order_cnt_lsb), POCMsb and POC.

Figure 2 The example of POC derivation process

## Picture order count in S/W

The current HM S/W does not reset POC to 0 at IDR picture. Figure 3 is the HM S/W example when random access test condition and GOP size is equal to 4. The subscript means the POC of the picture.

Figure 3 Current HM S/W decoding and display order example

If POC is reset to 0 at IDR complying with WD while keeping current display order, abnormal case (negative POC) should be appeared like as Figure 4.

Figure 4 The example when POC is reset to 0 at IDR picture

To comply with WD, it is recommend that IDR picture should be decoded at last order when the GOP includes IDR picture which is explained in Figure 5.

Figure 5 The recommended decoding order to reset POC to 0 at IDR picture

## Experimental Result

Figure 6 is experimental result when decoding order is reordered like as Figure 5.

Figure 6 Experimental Results

# Syntax for clean random access picture

## The problem of CRA picture at random access

The CRA picture can cause decoder crash or generate corrupted picture at random access due to leading pictures. Figure 7 describes the example of CRA picture decoding process at normal play and random access, respectively.

Figure 7 CRA picture decoding process at normal play and random access

## IDR picture vs. CRA picture

IDR picture has two kinds of functionality.

- Decoder Refresh Functionality

- Random Access (Trick Modes, Seek) Functionality

Decoder refresh functionality is for when error is spread out in decoded picture, DPB buffer refresh can stop error propagation. Random access functionality supports a lot of applications such as trick play and video editing.

It is though that current clean random access (CRA) picture has decoder refresh functionality but has not random access functionality. The reason is that CRA picture is meaningful when it has leading picture, but leading picture can case decoder refresh or generate corrupted pictures. Figure 8 describes TV channel zapping example at CRA picture. When TV channel is switched, decoder crash or corrupted pictures can be displayed. This is not proper situation.

Figure 8 TV channel zapping example at CRA picture

## Proposal

The system always can know the current decoding status, i.e., normal play or random access. So if we can have a certain room for signaling the status in system level then decoder can handle the leading pictures appropriately. To achieve this, we propose inserting one flag for current CRA picture’s status in NAL unit header such as random_access_flag. Figure 9 is related syntax for proposed method.

Figure 9 Proposed syntax

## Experiments

It is implemented that when encoder encodes bitstream, encoder generates a truncated bitstream to simulate random access situation that is described in Figure 10. The random_access_flag is set to one when encoder generates a truncated bitstream. This process (set random_access_flag to one) originally should be done in system but encoder do this.

Figure 10 The example of truncated bitstream

The decoder skip leading picture when random_access_flag is set to 1. On the other hand if random_access_flag is set to 0, leading picture is decoded normally. The related S/W is available.

# Conclusion

It is reported that the decoding order of IDR picture in random access condition is implemented wrong in current HM S/W

Regarding CRA picture, it is recommend changing the term from CRA (clean random access) to DDR (deferred decoder refresh) which was an initial term for CRA. The reason is current CRA still supports decoder refresh functionality.

If not, it is noted that considering JCTVC-F604 or JCTVC-G533 to support random access functionality for CRA picture. However regarding JCTVC-G533, it is highly recommended being verified by system experts first because the fact that system level should consider the codec level status looks awkward.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).