| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G535 WG11 Number: m22099 |
| --- | --- |

| Title: | CE3: Experimental Result on Tool 9 (Picture Adaptive 1/8-pel Motion Compensation Method) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Toshiyasu Sugio Takahiro Nishi 1006 Kadoma, Kadoma Osaka, Japan | Tel: Email: | +81-50-3587-2047 { sugio.toshiyasu, nishi.takahiro }@jp.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution is a report of CE3 experiment Tool 9 in JCTVC-F903. Experimental results reportedly showed 2.3% BR saving for HE and 2.0% BR saving for LC on average in the LD P scenarios. It also reportedly showed 0.2% BR saving for HE and 0.3% BR saving for LC in the RA scenarios, and 0.1% BR saving for LC on average in the LD B scenarios relative to the HM4.0.

- Introduction

This contribution reports the test result of CE3 experiment Tool 9 (Picture Adaptive 1/8-pel Motion Compensation Method), which was proposed in JCTVC-F471 [1].

# Technical description on JCTVC-F471

## Concept

In low delay B coding, B slice can refer one single reference picture with two 1/4-pel motion vectors, and thus derive 1/8-pel accuracy prediction samples by bi-linear interpolation using bi- prediction. On the other hand, P slice can refer one single reference picture with only one 1/4-pel motion vector. To improve coding efficiency of P slice, to use 1/8-pel motion vector is expected to be helpful.

In JCTVC-F471, 1-bit flag, which indicates whether 1/8-pel motion compensation is enabled or not, is introduced into PPS. And, two more parameters, which indicate the number of reference pictures to be referred in 1/8-pel precision, are introduced into slice header. Thus an encoder can select slices in which 1/8-pel motion compensation is applied, and reference pictures to be referred in 1/8-pel precision.

## Changes relative to HM4.0

JCTVC-F471 includes the changes written in below relative to HM4.0.

### Syntax

1/8pel_active_flag and num_ref_idx_lx_default_1/8pel_active (x=0, 1) are added to PPS. And num_ref_idx_lx_1/8pel_active (x=0, 1) are added to slice header.

1/8pel_active_flag equal to 1 specifies 1/8-pel motion compensation is enabled.

num_ref_idx_l0_default_1/8pel_active specifies how to be inferred num_ref_idx_l0_1/8pel_active for P and B slices when num_ref_idx_active_override_flag equal to 0. 
The value of num_ref_idx_l0_default_1/8pel_active shall be in the range of 0 to 31, inclusive.

num_ref_idx_l1_default_1/8pel_active specifies how to be inferred num_ref_idx_l1_1/8pel_active for B slices when num_ref_idx_active_override_flag equal to 0. 
The value of num_ref_idx_l0_default_1/8pel_active shall be in the range of 0 to 31, inclusive.

num_ref_idx_l0_1/8pel_active specifies the maximum reference index plus 1 for reference picture list 0 which should be referred by motion vector with 1/8-pel precision.
When the current slice is a P or B slice and num_ref_idx_l0_1/8pel_active is not present, num_ref_idx_l0_1/8pel_active shall be inferred to be equal to num_ref_idx_l0_default_1/8pel_active.

num_ref_idx_l1_1/8pel_active specifies the maximum reference index plus 1 for reference picture list 1 which should be referred by motion vector with 1/8-pel precision.
When the current slice is a B slice and num_ref_idx_l1_1/8pel_active is not present, num_ref_idx_l1_1/8pel_active shall be inferred to be equal to num_ref_idx_l1_default_1/8pel_active.

Table 2 shows the relationship between JCTVC-F471 syntax elements and accuracy of motion parameters. In Table 2, mvLX(ME) means a motion vector to be derived by a motion estimation in encoder, and mvLX(internal) means a motion vector to be used to derive spatial MVP candidate and to determine strength of deblocking filter.

- If 1/8pel_active_flag is equal to 0, all precisions for parameters on motion vector are 1/4-pel.

- If 1/8pel_active_flag is equal to 1 and num_ref_idx_lX_1/8pel_active is equal to 0, precisions for mvdLX, mvLX(ME) and mvCol are 1/4-pel, and precisions for pmvLX and mvLX(internal) are 1/8-pel. It indicates that only precisions for parameters on motion vector, which is used internally, are extended to 1/8-pel, so overhead bits such as mvdLX and computation time for ME are not increased.

- If 1/8pel_active_flag is equal to 1 and num_ref_idx_lX_1/8pel_active is equal to N, precision for mvCol is 1/4-pel, and precisions for pmvLX and mvLX(internal) are 1/8-pel, and precisions for mvdLX and mvLX(ME) is decided as below.

- If refIdxLX < N then, precisions for mvdLX and mvLX(ME) is 1/8-pel

- Otherwise, precisions for mvdLX and mvLX(ME) is 1/4-pel

Table 1: relationship between JCTVC-F471 syntax elements and accuracy of motion parameters

|  | mvdLX | pmvLX | mvLX (ME) | mvLX (internal) | mvCol |
| --- | --- | --- | --- | --- | --- |
| 1/8pel_active_flag=0 | 1/4-pel | 1/4-pel | 1/4-pel | 1/4-pel | 1/4-pel |
| 1/8pel_active_flag=1 num_ref_idx_lX_1/8pel_active=0 | 1/4-pel | 1/8-pel | 1/4-pel | 1/8-pel | 1/4-pel |
| 1/8pel_active_flag=1 num_ref_idx_lX_1/8pel_active=N | if refIdxLX<N 1/8-pel otherwise 1/4-pel | 1/8-pel | if refIdxLX<N 1/8-pel otherwise 1/4-pel | 1/8-pel | 1/4-pel |

### Fractional interpolation filter for 1/8-pel and 1/16-pel

Fractional interpolation filters for 1/8-pel and 1/16-pel are added in HM4.0 to derive 1/8-pel prediction sample value for luminance and 1/16-pel prediction sample value for chrominance.Table3 shows 8-tap interpolation filter coefficient for luminance, and Table4 shows 4-tap interpolation filter coefficient for chrominance. Fractional prediction sample values for luminance and chrominance are derived in the same way as HM4.0. A difference is only filter coefficient for 1/8-pel and 1/16-pel. 8-tap filter coefficient for 1/8-pel sample value is designed based on 12-tap filter which is included in TMuC1.0 software.

Table 2: 8-tap filter coefficient for luminance

| 1/8-pel | -1, 2, -6, 62, 9, -4, 2, 0 |  |
| --- | --- | --- |
| 1/4-pel | -1, 4, -10, 57, 19, -7, 3, -1 | Same as HM4.0 |
| 3/8-pel | -2, 5, -11, 50, 29, -10, 5, -2 |  |
| 1/2-pel | -1, 4, -11, 40, 40, -11, 4, -1 | Same as HM4.0 |
| 5/8-pel | -2, 5, -10, 29, 50, -11, 5, -2 |  |
| 3/4-pel | -1, 3, -7, 19, 57, -10, 4, -1 | Same as HM4.0 |
| 7/8-pel | 0, 2, -4, 9, 62, -6, 2, -1 |  |

Table 3: 4-tap filter coefficient for chrominance

| 1/16-pel | -2, 63, 4, -1 |  |
| --- | --- | --- |
| 1/8-pel | -3, 60, 8, -1 | Same as HM4.0 |
| 3/16-pel | -5, 59, 13, -3 |  |
| 1/4-pel | -4, 54, 16, -2 | Same as HM4.0 |
| 5/16-pel | -6, 52, 23, -5 |  |
| 3/8-pel | -5, 46, 27, -4 | Same as HM4.0 |
| 7/16-pel | -7, 43, 34, -6 |  |
| 1/2-pel | -4, 36, 36, -4 | Same as HM4.0 |
| 9/16-pel | -6, 34, 43, -7 |  |
| 5/8-pel | -4, 27, 46, -5 | Same as HM4.0 |
| 11/16-pel | -5, 23, 52, -6 |  |
| 3/4-pel | -2, 16, 54, -4 | Same as HM4.0 |
| 13/16-pel | -3, 13, 59, -5 |  |
| 7/8-pel | -1, 8, 60, -3 | Same as HM4.0 |
| 15/16-pel | -1, 4, 63, -2 |  |

### Derivation process for motion vector component

Motion vector predictor candidate mvpLX in the mvpListLX is updated such as below before removing duplicated ones.

- If 1/8_active_flag is equal to 1 and refIdxLX >= num_ref_idx_lX_1/8pel_active then, mvpLX[ 0 ] = (mvpLX[ 0 ]/2)*2
mvpLX[ 1 ] = (mvpLX[ 1 ]/2)*2

- Otherwise,
mvpLX[ 0 ] = mvpLX[ 0 ] 
mvpLX[ 1 ] = mvpLX[ 1 ]

### Derivation process for motion vector component

Motion vector component is derived as below.

When predFlagLX is equal to 1, the luma motion vector mvLX is derived as

- If 1/8_active_flag is equal to 1 and refIdxLX >= num_ref_idx_lX_1/8pel_active then, mvLX[ 0 ] = (mvpLX[ 0 ]/2)*2 + mvdLX[ 0 ]*2
mvLX[ 1 ] = (mvpLX[ 1 ]/2)*2 + mvdLX[ 1 ]*2

- Otherwise,
mvLX[ 0 ] = mvpLX[ 0 ] + mvdLX[ 0 ]
mvLX[ 1 ] = mvpLX[ 1 ] + mvdLX[ 1 ]

### Derivation process of boundary filtering strength for deblocking filter

A condition, in which the variable bS is set to 1, is changed as below.

- if any of the following conditions are true, the variable bS is set equal to 1.

- The prediction unit containing sample p0 has different reference pictures or a different number of motion vectors with the prediction unit containing the sample q0.

- One motion vector is used to predict the prediction unit containing sample p0, one motion vector is used to predict the prediction unit containing sample q0, and the absolute difference between the horizontal or vertical component of the motion vector used is greater than or equal to (4<<1/8pel_active_flag) in units of quarter luma samples.

- Two motion vectors are used to predict the prediction unit containing sample p0, two motion vectors are used to predict the prediction unit containing sample q0, and at least one of the motion vector pairs corresponding the same reference pictures and the different boundary samples p0 and q0 satisfies the following condition:

- The absolute difference between the horizontal or vertical component of a motion vector used in the prediction of the two prediction units is greater than or equal to (4<<1/8pel_active_flag) in units of quarter luma samples.

### Motion storage compression

To avoid increasing memory bandwidth and memory capacity for co-located picture, motion vector mvLX is scaled down to 1/4-pel if 1/8pel_active_flag is equal to 1.

- mvLX = mvLX >> 1/8pel_active_flag

# Experimental results

A proposed method was implemented on HM4.0 software and its performance relative to HM4.0 was evaluated under the common test condition described in JCTVC-F900 [2] and Table 4 test setting. The computing platform is Linux 64 bits and executables were compiled on 64-bit Linux with gcc 4.4.3.

Table 4: Test setting for CE3 Tool 9

|  | mvdLX | pmvLX | mvLX (ME) | mvLX (internal) | mvCol |
| --- | --- | --- | --- | --- | --- |
| HM4.0 (reference) | 1/4-pel | 1/4-pel | 1/4-pel | 1/4-pel | 1/4-pel |
| Test1) 1/8pel_active_flag=1 num_ref_idx_l0_1/8pel_active=0 num_ref_idx_l1_1/8pel_active=0 | 1/4-pel | 1/8-pel | 1/4-pel | 1/8-pel | 1/4-pel |
| Test2-a) 1/8pel_active_flag=1 Bslice: num_ref_idx_l0_1/8pel_active=0 num_ref_idx_l1_1/8pel_active=2 Pslice: num_ref_idx_l0_1/8pel_active=2 | if refIdxLX< num_ref_idx_lX_1/8pel_active 1/8-pel otherwise 1/4-pel | 1/8-pel | if refIdxLX< num_ref_idx_lX 1/8-pel otherwise 1/4-pel | 1/8-pel | 1/4-pel |
| Test2-b) 1/8pel_active_flag=1 Bslice: num_ref_idx_l0_1/8pel_active=0 num_ref_idx_l1_1/8pel_active=4 Pslice: num_ref_idx_l0_1/8pel_active=4 | if refIdxLX< num_ref_idx_lX_1/8pel_active 1/8-pel otherwise 1/4-pel | 1/8-pel | if refIdxLX< num_ref_idx_lX 1/8-pel otherwise 1/4-pel | 1/8-pel | 1/4-pel |

R-D performance and execution time of each experiment result are summarized in Table 5 to 8. Detailed results are included in the attached excel sheet.

Test1 setting means that all reference pictures are referred with 1/4-pel motion vectors same as HM4.0, but 1/8-pel motion compensation is enabled. In this setting, internal motion vector, such as motion vector predictor, is extended to 1/8-pel precision, and when motion vector predictor is directly used to derive prediction sample values in Merge/Skip mode, 1/8-pel motion compensation is conducted. A precision of motion vector difference is always to be 1/4-pel, thus the overhead of motion vectors is not increased.

Table 5 shows the experimental result of Test1 setting. In Random Access case, Test1 setting saves 0.2% bits for high efficiency and 0.3% bits for low complexity on average. In low delay B case, Test1 setting saves 0.1% bits for low complexity on average. In low delay P case, Test1 setting saves 1.0% bits for high efficiency and 0.9% bits for low complexity on average. The difference in encoding and decoding time is not obvious.

Table 5: Experimental result of Test1

Test2 setting means that 1/8-pel motion compensation is enabled and motion vector precision is 1/8-pel for reference pictures which reference index is less than num_ref_idx_lX_1/8pel_active. In Test2-a setting, num_ref_idx_l0_1/8pel_active is set to 0 and num_ref_idx_l1_1/8pel_active is set to 2 for B slice, and num_ref_idx_l0_1/8pel_active is set to 2 for P slice. In Test2-b setting, num_ref_idx_l0_1/8pel_active is set to 0 and num_ref_idx_l1_1/8pel_active is set to 4 for B slice, and num_ref_idx_l0_1/8pel_active is set to 4 for P slice.

Table 7 shows the experimental result of Test2-a setting. In Random Access case, Test2-a setting saves 0.2% bits for high efficiency and 0.3% bits for low complexity on average. In low delay B case, Test2-a setting saves 0.2% bits for high efficiency and 0.3% bits for low complexity on average. In low delay P case, Test2-a setting saves 1.9% bits for high efficiency and 1.6% bits for low complexity on average. The encoding time increases about 5-10%, and the decoding time is not increased.

Table 8 shows the experimental result of Test2-b setting. In Random Access case, Test2-a setting saves 0.2% bits for high efficiency and 0.3% bits for low complexity on average. In low delay B case, Test2-a setting saves 0.3% bits for high efficiency and 0.4% bits for low complexity on average. In low delay P case, Test2-a setting saves 2.3% bits for high efficiency and 2.0% bits for low complexity on average. The encoding time increases about 11-20%, and the decoding time is not increased.

Figure 1 to 6 show the subjective quality of JCTVC-F471 with Test2-b setting compared to HM4.0 such as below table. The obvious difference is not observed among them in terms of ringing noises.

Table 6: Sample decoded picture in order to check ringing noises in CE3

| Configuration | Sequences | QP | Frame no. | Tool |
| --- | --- | --- | --- | --- |
| LC-LDP | PartyScene | 37 | 100 | SAO off |
| LC-LDP | PartyScene | 37 | 100 | (SAO on) |
| HE-RA | PartyScene | 37 | 100 | (SAO on, ALF on) |

Table 7: Experimental result of Test2-a

Table 8: Experimental result of Test2-b

Figure 1: HM4.0, LC-LDP, PartyScene, QP37, Frame no. 100

Figure 2: JCTVC-F471 with Test2-b Setting, LC-LDP, PartyScene, QP37, Frame no. 100

Figure 3: HM4.0, LC-LDP, PartyScene, QP37, Frame no. 100, SAO off

Figure 4: JCTVC-F471 with Test2-b setting, LC-LDP, PartyScene, QP37, Frame no. 100 SAO off

Figure 5: HM4.0, HE-RA, PartyScene, QP37, Frame no. 100

Figure 6: JCTVC-F471 with Test2-b setting, HE-RA, PartyScene, QP37, Frame no. 100

# Conclusion

This contribution reports a result of CE3 Tool 9. An experimental result shows that the proposed method with Test2-b setting saves 2.3% bits for high efficiency on average in low delay P scenario. Encoding time is 16% increased relative to HM4.0, but the difference in decoding time is not obvious. On the other hand, the proposed method can be configurable as Test1 setting to achieve 1.0% bits reduction on average for high efficiency in low delay P scenario without obvious difference in encoding and decoding time.

For the other test scenarios, the proposed method with Test1 setting saves 0.2% bits on average for high efficiency in random access scenario.

Considering the gain especially for P slices, we recommend that proposed configurable 1/8-pel motion compensation be adopted in HM/WD of HEVC.

# References

- Toshiyasu Sugio, “Picture Adaptive 1/8-pel Motion Compensation Method”, JCTVC-F471, Torino, July. 2011.

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, Torino, July. 2011.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix

## Proposed changes on Working Draft 4.0 (to be refined)

## Syntax in tabular form

#### 7.3.2.2	Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| entropy_coding_mode_flag | u(1) |
| num_temporal_layer_switching_point_flags | ue(v) |
| for( i = 0; i < num_temporal_layer_switching_point_flags; i++ ) |  |
| temporal_layer_switching_point_flag[ i ] | u(1) |
| num_ref_idx_l0_default_active_minus1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | ue(v) |
| 1/8pel_active_flag |  |
| if (1/8pel_active_flag) |  |
| num_ref_idx_l0_default_1/8pel_active |  |
| num_ref_idx_l1_default_1/8pel_active |  |
| pic_init_qp_minus26 /* relative to 26 */ | se(v) |
| constrained_intra_pred_flag | u(1) |
| slice_granularity | u(2) |
| shared_pps_info_enabled_flag | u(1) |
| if( shared_pps_info_enabled_flag ) |  |
| if( adaptive_loop_filter_enabled_flag ) |  |
| alf_param( ) |  |
| if( cu_qp_delta_enabled_flag ) |  |
| max_cu_qp_delta_depth | u(4) |
| rbsp_trailing_bits( ) |  |
| } |  |

### 7.3.3	Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| lightweight_slice_flag | u(1) |
| if( !lightweight_slice_flag ) { |  |
| slice_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| frame_num | u(v) |
| if( IdrPicFlag ) |  |
| idr_pic_id | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |
| pic_order_cnt_lsb /* | u(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| num_ref_idx_l0_1/8pel_active | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| num_ref_idx_l1_1/8pel_active | ue(v) |
| } |  |
| } |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| if( nal_ref_idc != 0 ) |  |
| dec_ref_pic_marking( ) |  |
| } |  |
| if( entropy_coding_mode_flag && slice_type != I) |  |
| cabac_init_idc | ue(v) |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag == 0 ) |  |
| slice_address | u(v) |
| if( !lightweight_slice_flag ) { |  |
| slice_qp_delta | se(v) |
| if( sample_adaptive_offset_enabled_flag ) |  |
| sao_param() |  |
| if( deblocking_filter_control_present_flag ) { |  |
| disable_deblocking_filter_idc |  |
| if( disable_deblocking_filter_idc != 1 ) { |  |
| slice_alpha_c0_offset_div2 |  |
| slice_beta_offset_div2 |  |
| } |  |
| } |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( adaptive_loop_filter_enabled_flag ) { |  |
| if( !shared_pps_info_enabled_flag ) |  |
| alf_param( ) |  |
| alf_cu_control_param( ) |  |
| } |  |
| } |  |
| } |  |

## Semantics

#### 7.4.2.2	Picture parameter set RBSP semantics

1/8pel_active_flag equal to 1 specifies 1/8-pel motion compensation is enabled.

num_ref_idx_l0_default_1/8pel_active specifies how num_ref_idx_l0_1/8pel_active is inferred for P and B slices with num_ref_idx_active_override_flag equal to 0. 
The value of num_ref_idx_l0_default_1/8pel_active shall be in the range of 0 to 31, inclusive.

num_ref_idx_l1_default_1/8pel_active specifies how num_ref_idx_l1_1/8pel_active is inferred for B slices with num_ref_idx_active_override_flag equal to 0. 
The value of num_ref_idx_l0_default_1/8pel_active shall be in the range of 0 to 31, inclusive.

### Slice header semantics

num_ref_idx_l0_1/8pel_active specifies the maximum reference index plus 1 for reference picture list 0 to which motion vector referred in 1/8-pel precision.
When the current slice is a P or B slice and num_ref_idx_l0_1/8pel_active is not present, num_ref_idx_l0_1/8pel_active shall be inferred to be equal to num_ref_idx_l0_default_1/8pel_active.

num_ref_idx_l1_1/8pel_active specifies the maximum reference index plus 1 for reference picture list 1 to which motion vector referred in 1/8-pel precision.
When the current slice is a B slice and num_ref_idx_l1_1/8pel_active is not present, num_ref_idx_l1_1/8pel_active shall be inferred to be equal to num_ref_idx_l1_default_1/8pel_active.

## 8.2	Slice decoding process

#### 8.4.2.1	Derivation process for motion vector components and reference indices

Input to this process are

- a luma location ( xC, yC ) of the top-left luma sample of the current coding unit relative to the top-left luma sample of the current picture,

- a luma location ( xB, yB ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current coding unit,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

- 1/8pel_active_flag, num_ref_idx_l0_1/8pel_active and num_refidx_l1_1/8pel_active

Outputs of this process are

- luma motion vectors mvL0 and mvL1 and chroma motion vectors mvCL0 and mvCL1,

- reference indices refIdxL0 and refIdxL1,

- prediction list utilization flags predFlagL0 and predFlagL1.

Let ( xP, yP ) specify the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture where xP = xC + xB and yP = yC + yB.

For the derivation of the variables mvL0 and mvL1, refIdxL0 and refIdxL1 as well as PredFlagL0 and PredFlagL1, the following applies.

- If PredMode is equal to MODE_SKIP, the derivation process for luma motion vectors for merge mode as specified in subclause Error: Reference source not found is invoked with the luma location ( xP, yP ), variables nPSW, nPSH and the partition index PartIdx as inputs and the output being the luma motion vectors mvL0, mvL1, the reference indices refIdxL0, refIdxL1, and the prediction list utilization flags predFlagL0 and predFlagL1.

- Otherwise, if PredMode is equal to MODE_INTER and merge_flag[ xP ][ yP ] is equal to 1,, the derivation process for luma motion vectors for merge mode as specified in subclause Error: Reference source not found is invoked with the luma location ( xP, yP ), variables nPSW and nPSH and the partition index PartIdx as inputs and the outputs being the luma motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, the prediction utilization flags predFlagL0 and predFlagL1.

- Otherwise, for X being replaced by either 0 or 1 in the variables predFlagLX, mvLX, refIdxLX and in Pred_LX and in the syntax elements ref_idx_lX and mvd_lX, the following applies.

- The variables LcToLx, refIdxLX and predFlagLX are derived as follows.

- If inter_pred_flag[ xP ][ yP ] is equal to Pred_LC and PredLCToPredLx[ ref_idx_lc[ xP ][ yP ] ] is equal to Pred_LX,

refIdxLX = RefIdxLCToRefIdxLx[ ref_idx_lc[ xP ][ yP ] ]	(8-1)					predFlagLX = 1		(8-2)					mvd_lX[ xP ][ yP ][ 0 ] = mvd_lc[ xP ][ yP ][ 0 ]	(8-3)
					mvd_lX[ xP ][ yP ][ 1 ] = mvd_lc[ xP ][ yP ][ 1 ]	(8-4)
					mvp_idx_lX[ xP ][ yP ][ 0 ] = mvp_idx_lc[ xP ][ yP ]	(8-5)
					LcToLx = LX		(8-6)

- Otherwise, if inter_pred_flag[ xP ][ yP ] is equal to Pred_LX or Pred_BI,

refIdxLX = ref_idx_lX[ xP ][ yP ]		(8-7)
					predFlagLX = 1		(8-8)

- Otherwise, the variables refIdxLX and predFlagLX are specified by

refIdxLX = -1		(8-9)
					predFlagLX = 0		(8-10)

- The variable mvdLX is derived as follows.

mvdLX[ 0 ] = mvd_lX[ xP ][ yP ][ 0 ]		(8-11)
				mvdLX[ 1 ] = mvd_lX[ xP ][ yP ][ 1 ]		(8-12)

- When predFlagLX is equal to 1, the variable mvpLX is derived as follows.

- The derivation process for luma motion vector prediction in subclause Error: Reference source not found is invoked with the luma location ( xP, yP ), variables nPSW and nPSH and refIdxLX as the inputs and the output being mvpLX.

- When predFlagLX is equal to 1, the luma motion vector mvLX is derived as

- If 1/8_active_flag is equal to 1 and refIdxLX >= num_ref_idx_lX_1/8pel_active then, 
mvLX[ 0 ] = (mvpLX[ 0 ]/2)*2 + mvdLX[ 0 ]*2
mvLX[ 1 ] = (mvpLX[ 1 ]/2)*2 + mvdLX[ 1 ]*2

- Otherwise,

mvLX[ 0 ] = mvpLX[ 0 ] + mvdLX[ 0 ]	(8-13)
				mvLX[ 1 ] = mvpLX[ 1 ] + mvdLX[ 1 ]	(8-14)

When ChromaArrayType is not equal to 0 and predFlagLX (with X being either 0 or 1) is equal to 1, the derivation process for chroma motion vectors in subclause Error: Reference source not found is invoked with mvLX and refIdxLX as inputs and the output being mvCLX.

#### 8.4.2.1.7 Derivation process for luma motion vector prediction

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH.

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

- 1/8pel_active_flag and num_ref_idx_lX_1/8pel_active

Output of this process is

- the prediction mvpLX of the motion vector mvLX (with X being 0 or 1).

The motion vector predictor mvpLX is derived in the following ordered steps.

- The derivation process for motion vector predictor candidates from neighboring prediction unit partitions in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the width and the height of the prediction unit nPSW and nPSH, and refIdxLX (with X being 0 or 1, respectively) as inputs and the availability flags availableFlagLXN and the motion vectors mvLXN with N being replaced by A, B as the output.

- The derivation process for temporal luma motion vector prediction in subclause Error: Reference source not found is invoked with luma location ( xP, yP ) , the width and the height of the prediction unit nPSW and nPSH, and refIdxLX (with X being 0 or 1, respectively) as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector predictor mvLXCol.

- The motion vector predictor candidate list, mvpListLX, is constructed as follows.

- mvLXA, if availableFlagLXA is equal to 1

- mvLXB, if availableFlagLXB is equal to 1

- mvLXCol, if availableFlagLXCol is equal to 1

- The motion vector predictor in the motion vector predictor candidate is updated as follows.

    - If 1/8_active_flag is equal to 1 and refIdxLX >= num_ref_idx_lX_1/8pel_active then, 
mvpListLX[ 0 ] = (mvpListLX[ 0 ]/2)*2
mvpListLX[ 1 ] = (mvpListLX[ 1 ]/2)*2

- When motion vectors have the same value, the motion vectors are removed from the list except the motion vector which has the smallest order in the mvpListLX.

- When mvpListLX is empty, a zero motion vector is added as follows.

mvpListLX[ 0 ][0] = 0	(8-15)

mvpListLX[ 0 ][1] = 0	(8-16)

- The variable numMVPCandLX is set to the number of elements within the mvpListLX and maxNumMVPCand is set to 2.

- The motion vector predictor list is modifed to contain exactly maxNumMVPCand motion vector predictor candidates as follows.

    - If numMVPCandLX is less than maxNumMVPCand, the derivation process for zero motion vector predictor candidates specified in subclause Error: Reference source not found is invoked with mvpListLX and numMVPCandLX given as input and the output is assigned to mvpListLX and numMVPCandLX.

    - Otherwise (numMVPCandLX is equal to or greater than maxNumMVPCand), all motion vector predictor candidates mvpListLX[ idx ] with idx greater than maxNumMVPCand − 1 are removed from the list.

- The motion vector of mvpListLX[ mvp_idx_lX[ xP, yP ] ] is assigned to mvpLX.

###### 8.4.2.2.2.1	Luma sample interpolation process

Inputs to this process are:

–	a luma location in full-sample units ( xIntL, yIntL ),

–	a luma location in fractional-sample units ( xFracL, yFracL ),

–	the luma reference sample array refPicLXL.

–	1/8pel_active_flag

Output of this process is a predicted luma sample value predSampleLXL[ xL, yL ]

Figure 8-7 – Integer samples (shaded blocks with upper-case letters) and fractional sample positions (un-shaded blocks with lower-case letters) for quarter sample luma interpolation

In Error: Reference source not found, the positions labelled with upper-case letters Ai, j within shaded blocks represent luma samples at full-sample locations inside the given two-dimensional array refPicLXL of luma samples. These samples may be used for generating the predicted luma sample value predSampleLXL[ xL, yL ]. The locations ( xAi, j, yAi, j ) for each of the corresponding luma samples Ai, j inside the given array refPicLXL of luma samples are derived as follows:

xAi, j = Clip3( 0, PicWidthInSamplesL – 1, xIntL +i )		(8-17)
yAi, j = Clip3( 0, PicHeightInSamplesL – 1, yIntL +j )		(8-18)

Variables shift1, shift2 and shift3 are derived as follows.

–	The variable shift1 is set equal to BitDepthY – 8, the variable shift2 is set equal to 6, and the variable shift3 is set equal to 14 – BitDepthY.

Given the luma samples Ai, j at full-sample locations ( xAi, j, yAi, j ), the luma samples ‘a0,0’ to ‘r0,0’ at fractional sample positions are derived by the following rules.

–	The samples labelled a0,0, b0,0, c0,0, d0,0, h0,0, and n0,0 shall be derived by applying the 8-tap filter to the nearest integer position samples:

a0,0 = ( −A−3,0 + 4*A−2,0 − 10*A−1,0 + 57*A0,0 +	 19*A1,0 − 7*A2,0 + 3*A3,0 − A4,0 ) >> shift1	(8-19)

b0,0 = ( −A−3,0 + 4*A−2,0 − 11*A−1,0 + 40*A0,0 +	 40*A1,0 − 11*A2,0 + 4*A3,0 − A4,0 ) >> shift1	(8-20)

c0,0 = ( −A−3,0 + 3*A−2,0 − 7*A−1,0 + 19*A0,0 +	 57*A1,0 − 10*A2,0 + 4*A3,0 − A4,0 ) >> shift1	(8-21)

d0,0 = ( −A0,−3 + 4*A0,−2 − 10*A0,−1 + 57*A0,0 +	 19*A0,1 − 7*A0,2 + 3*A0,3 − A0,4 ) >> shift1	(8-22)

h0,0 = ( −A0,−3 + 4*A0,−2 − 11*A0,−1 + 40*A0,0 +	 40*A0,1 − 11*A0,2 + 4*A0,3 − A0,4 ) >> shift1	(8-23)

n0,0 = ( −A0,−3 + 3*A0,−2 − 7*A0,−1 + 19*A0,0 +	 57*A0,1 − 10*A0,2 + 4*A0,3 − A0,4 ) >> shift1	(8-24)

–	If 1/8pel_active_flag is equal to 1, the samples labelled Aa0,0, ab0,0, bc0,0, cA0,0, Ad0,0, dh0,0, hn0,0,and nA0,0 shall be derived by applying the 8-tap filter to the nearest integer position samples and clipping the filtered value:

Aa0,0 = ( -A-3,0 + 2*A-2,0 - 6*A-1,0 + 62*A0,0 +	 9*A1,0 - 4*A2,0 + 2*A3,0 ) >> shift1	(8-25)

ab0,0 = ( -2*A-3,0 + 5*A-2,0 - 11*A-1,0 + 50*A0,0 +	 29*A1,0 - 10*A2,0 + 5*A3,0 – 2*A4,0  ) >> shift1	(8-26)

bc0,0 = ( -2*A-3,0 + 5*A-2,0 - 10*A-1,0 + 29*A0,0 +	 50*A1,0 - 11*A2,0 + 5*A3,0 – 2*A4,0  ) >> shift1	(8-27)

cA0,0 = ( 2*A-2,0 - 4*A-1,0 + 9*A0,0 + 62*A1,0 - 6*A2,0 + 2*A3,0 – A4,0  ) >> shift1	(8-28)

Ad0,0 = ( -A0,-3 + 2*A0,-2 - 6*A0,-1 + 62*A0,0 +	 9*A0,1 - 4*A0,2 + 2*A0,3  ) >> shift1	(8-29)

dh0,0 = ( -2*A0,-3 + 5*A0,-2 - 11*A0,-1 + 50*A0,0 +	 29*A0,1 - 10*A0,2 + 5*A0,3 – 2*A0,4 ) >> shift1	(8-30)

hn0,0 = ( -2*A0,-3 + 5*A0,-2 - 10*A0,-1 + 29*A0,0 +	 50*A0,1 - 11*A0,2 + 5*A0,3 – 2*A0,4 ) >> shift1	(8-31)

nA0,0 = ( 2*A0,-2 - 4*A0,-1 + 9*A0,0 + 62*A0,1 - 6*A0,2 + 2*A0,3 - A0,4 ) >> shift1	(8-32)

–	The samples labelled e0,0, f0,0, g0,0, i0,0, j0,0, k0,0, p0,0, q0,0 and r0,0 shall be derived by applying the 8-tap filter to the samples a0,i, b0,i and c0,i where i = −3..4 in vertical direction:

e0,0 = ( −a0,−3 + 4*a0,−2 − 10*a0,−1 + 57*a0,0 +	 19*a0,1 − 7*a0,2 + 3*a0,3 − a0,4 ) >> shift2	(8-33)

f0,0 = ( −a0,−3 + 4*a0,−2 − 11*a0,−1 + 40*a0,0 +	 40*a0,1 − 11*a0,2 + 4*a0,3 − a0,4 ) >> shift2	(8-34)

g0,0 = ( −a0,−3 + 3*a0,−2 − 7*a0,−1 + 19*a0,0 +	 57*a0,1 − 10*a0,2 + 4*a0,3 − a0,4 ) >> shift2	(8-35)

i0,0 = ( −b0,−3 + 4*b0,−2 − 10*b0,−1 + 57*b0,0 +	 19*b0,1 − 7*b0,2 + 3*b0,3 − b0,4 ) >> shift2	(8-36)

j0,0 = ( −b0,−3 + 4*b0,−2 − 11*b0,−1 + 40*b0,0 +	 40*b0,1 − 11*b0,2 + 4*b0,3 − b0,4 ) >> shift2	(8-37)

k0,0 = ( −b0,−3 + 3*b0,−2 − 7*b0,−1 + 19*b0,0 +	 57*b0,1 − 10*b0,2 + 4*b0,3 − b0,4 ) >> shift2	(8-38)

p0,0 = ( −c0,−3 + 4*c0,−2 − 10*c0,−1 + 57*c0,0 +	 19*c0,1 − 7*c0,2 + 3*c0,3 − c0,4 ) >> shift2	(8-39)

q0,0 = ( −c0,−3 + 4*c0,−2 − 11*c0,−1 + 40*c0,0 +	 40*c0,1 − 11*c0,2 + 4*c0,3 − c0,4 ) >> shift2	(8-40)

r0,0 = ( −c0,−3 + 3*c0,−2 − 7*c0,−1 + 19*c0,0 +	 57*c0,1 − 10*c0,2 + 4*c0,3 − c0,4 ) >> shift2	(8-41)

–	If 1/8pel_active_flag is equal to 1, the final prediction values Aade0,0 to rncA0,0 shall be derived by applying the 8-tap filter to the samples Aa0,i, ab0,i, bc0,i and cA0,i where i = −3..4 in vertical direction:

Aade0,0 = ( -Aa0,-3 + 2*Aa0,-2 - 6*Aa0,-1 + 62*Aa0,0 +	 9*Aa0,1 - 4*Aa0,2 + 2*Aa0,3  ) >> shift2	(8-42)

ae0,0 = ( -a0,-3 + 2*a0,-2 - 6*a0,-1 + 62*a0,0 +	 9*a0,1 - 4*a0,2 + 2*a0,3 ) >> shift2	(8-43)

abef0,0 = ( -ab0,-3 + 2*ab0,-2 - 6*ab0,-1 + 62*ab0,0 +	 9*ab0,1 - 4*ab0,2 + 2*ab0,3  ) >> shift2	(8-44)

bf0,0 = ( -b0,-3 + 2*b0,-2 - 6*b0,-1 + 62*b0,0 +	 9*b0,1 - 4*b0,2 + 2*b0,3   ) >> shift2	(8-45)

bcfg0,0 = ( -bc0,-3 + 2*bc0,-2 - 6*bc0,-1 + 62*bc0,0 +	 9*bc0,1 - 4*bc0,2 + 2*bc0,3 ) >> shift2	(8-46)

cg0,0 = ( -c0,-3 + 2*c0,-2 - 6*c0,-1 + 62*c0,0 +	 9*c0,1 - 4*c0,2 + 2*c0,3   ) >> shift2	(8-47)

cAgd0,0 = ( -cA0,-3 + 2*cA0,-2 - 6*cA0,-1 + 62*cA0,0 +	 9*cA0,1 - 4*cA0,2 + 2*cA0,3  ) >> shift2	(8-48)

de0,0 = ( -Aa0,-3 + 4*Aa0,-2 - 10*Aa0,-1 + 57*Aa0,0 +	 19*Aa0,1 - 7*Aa0,2 + 3*Aa0,3 -Aa0,4  ) >> shift2	(8-49)

ef0,0 = ( -ab0,-3 + 4*ab0,-2 - 10*ab0,-1 + 57*ab0,0 +	 19*ab0,1 - 7*ab0,2 + 3*ab0,3 - ab0,4  ) >> shift2	(8-50)

fg0,0 = ( -bc0,-3 + 4*bc0,-2 - 10*bc0,-1 + 57*bc0,0 +	 19*bc0,1 - 7*bc0,2 + 3*bc0,3 - bc0,3 ) >> shift2	(8-51)

gd0,0 = ( -cA0,-3 + 4*cA0,-2 - 10*cA0,-1 + 57*cA0,0 +	 19*cA0,1 - 7*cA0,2 + 3*cA0,3 - cA0,3 ) >> shift2	(8-52)

dehi0,0 = ( -2*Aa0,-3 + 5*Aa0,-2 - 11*Aa0,-1 + 50*Aa0,0 +	 29*Aa0,1 - 10*Aa0,2 + 5*Aa0,3 -2*Aa0,4  ) >> shift2	(8-53)

ei0,0 = ( -2*a0,-3 + 5*a0,-2 - 11*a0,-1 + 50*a0,0 +	 29*a0,1 - 10*a0,2 + 5*a0,3 - 2*a0,4 ) >> shift2	(8-54)

efij0,0 = ( -2*ab0,-3 + 5*ab0,-2 - 11*ab0,-1 + 50*ab0,0 +	 29*ab0,1 - 10*ab0,2 + 5*ab0,3 - 2*ab0,4  ) >> shift2	(8-55)

fj0,0 = ( -2*b0,-3 + 5*b0,-2 - 11*b0,-1 + 50*b0,0 +	 29*b0,1 - 10*b0,2 + 5*b0,3 - 2*b0,3   ) >> shift2	(8-56)

fgjk0,0 = ( -2*bc0,-3 + 5*bc0,-2 - 11*bc0,-1 + 50*bc0,0 +	 29*bc0,1 - 10*bc0,2 + 5*bc0,3 - 2*bc0,3 ) >> shift2	(8-57)

gk0,0 = ( -2*c0,-3 + 5*c0,-2 - 11*c0,-1 + 50*c0,0 +	 29*c0,1 - 10*c0,2 + 5*c0,3 - 2*c0,3  ) >> shift2	(8-58)

gdkh0,0 = ( -2*cA0,-3 + 5*cA0,-2 - 11*cA0,-1 + 50*cA0,0 +	 29*cA0,1 - 10*cA0,2 + 5*cA0,3 - 2*cA0,3 ) >> shift2	(8-59)

hi0,0 = ( -Aa0,-3 + 4*Aa0,-2 - 11*Aa0,-1 + 40*Aa0,0 +	 40*Aa0,1 - 11*Aa0,2 + 4*Aa0,3 -Aa0,4  ) >> shift2	(8-60)

ij0,0 = ( -ab0,-3 + 4*ab0,-2 - 11*ab0,-1 + 40*ab0,0 +	 40*ab0,1 - 11*ab0,2 + 4*ab0,3 - ab0,4  ) >> shift2	(8-61)

jk0,0 = ( -bc0,-3 + 4*bc0,-2 - 11*bc0,-1 + 40*bc0,0 +	 40*bc0,1 - 11*bc0,2 + 4*bc0,3 - bc0,3 ) >> shift2	(8-62)

kh0,0 = ( -cA0,-3 + 4*cA0,-2 - 11*cA0,-1 + 40*cA0,0 +	 40*cA0,1 - 11*cA0,2 + 4*cA0,3 - cA0,3 ) >> shift2	(8-63)

hinp0,0 = ( -2*Aa0,-3 + 5*Aa0,-2 - 10*Aa0,-1 + 29*Aa0,0 +	 50*Aa0,1 - 11*Aa0,2 + 5*Aa0,3 -2*Aa0,4  ) >> shift2	(8-64)

ip0,0 = ( -2*a0,-3 + 5*a0,-2 - 10*a0,-1 + 29*a0,0 +	 50*a0,1 - 11*a0,2 + 5*a0,3 – 2*a0,4 ) >> shift2	(8-65)

ijpq0,0 = ( -2*ab0,-3 + 5*ab0,-2 - 10*ab0,-1 + 29*ab0,0 +	 50*ab0,1 - 11*ab0,2 + 5*ab0,3 - 2*ab0,4  ) >> shift2	(8-66)

jq0,0 = ( -2*b0,-3 + 5*b0,-2 - 10*b0,-1 + 29*b0,0 +	 50*b0,1 - 11*b0,2 + 5*b0,3 - 2*b0,3   ) >> shift2	(8-67)

jkqr0,0 = ( -2*bc0,-3 + 5*bc0,-2 - 10*bc0,-1 + 29*bc0,0 +	 50*bc0,1 - 11*bc0,2 + 5*bc0,3 -2* bc0,3 ) >> shift2	(8-68)

kr0,0 = ( -2*c0,-3 + 5*c0,-2 - 10*c0,-1 + 29*c0,0 +	 50*c0,1 - 11*c0,2 + 5*c0,3 - 2*c0,3  ) >> shift2	(8-69)

khrn0,0 = ( -2*cA0,-3 + 5*cA0,-2 - 10*cA0,-1 + 29*cA0,0 +	 50*cA0,1 - 11*cA0,2 + 5*cA0,3 - 2*cA0,3 ) >> shift2	(8-70)

np0,0 = (  -Aa0,-3+ 3*Aa0,-2 - 7*Aa0,-1 + 19*Aa0,0 +	 57*Aa0,1 - 10*Aa0,2 + 4*Aa0,3 -Aa0,4  ) >> shift2	(8-71)

pq0,0 = (  -ab0,-3+ 3*ab0,-2 - 7*ab0,-1 + 19*ab0,0 +	 57*ab0,1 - 10*ab0,2 + 4*ab0,3 - ab0,4  ) >> shift2	(8-72)

qr0,0 = ( -bc0,-3+ 3*bc0,-2 - 7*bc0,-1 + 19*bc0,0 +	 57*bc0,1 - 10*bc0,2 + 4*bc0,3 - bc0,3 ) >> shift2	(8-73)

rn0,0 = ( -cA0,-3+ 3*cA0,-2 - 7*cA0,-1 + 19*cA0,0 +	 57*cA0,1 - 10*cA0,2 + 4*cA0,3 - cA0,3 ) >> shift2	(8-74)

npAa0,0 = (  2*Aa0,-2 - 4*Aa0,-1 + 9*Aa0,0 +	 62*Aa0,1 - 6*Aa0,2 + 2Aa0,3 -Aa0,4  ) >> shift2	(8-75)

pa0,0 = ( 2*a0,-2 - 4*a0,-1 + 9*a0,0 + 62*a0,1 - 6*a0,2 + 2*a0,3 –a0,4 ) >> shift2	(8-76)

pqab0,0 = (  2*ab0,-2 - 4*ab0,-1 + 9*ab0,0 +	 62*ab0,1 - 6*ab0,2 + 2*ab0,3 - ab0,4  ) >> shift2	(8-77)

qb0,0 = (  2*b0,-2 - 4*b0,-1 + 9*b0,0 +	 62*b0,1 - 6*b0,2 + 2*b0,3 - b0,3   ) >> shift2	(8-78)

qrbc0,0 = ( 2*bc0,-2 - 4*bc0,-1 + 9*bc0,0 +	 62*bc0,1 - 6*bc0,2 + 2*bc0,3 - bc0,3 ) >> shift2	(8-79)

rc0,0 = (  2*c0,-2 - 4*c0,-1 + 9*c0,0 +	 62*c0,1 - 6*c0,2 + 2*c0,3 - c0,3  ) >> shift2	(8-80)

rncA0,0 = ( 2*cA0,-2 - 4*cA0,-1 + 9*cA0,0 +	 62*cA0,1 - 6*cA0,2 + 2*cA0,3 - cA0,3 ) >> shift2	(8-81)

The positions labelled with lower-case letters within un-shaded blocks represent luma samples at quarter-pel sample fractional locations. The luma location offset in fractional-sample units ( xFracL, yFracL ) specifies which of the generated luma samples at full-sample and fractional-sample locations is assigned to the predicted luma sample value predSampleLXL[ xL, yL ]. This assignment is done according to Error: Reference source not found. The value of predSampleLXL[ xL, yL ] shall be the output.

Table 8-9 – Assignment of the luma prediction sample predSampleLXL[ xL, yL ]

| xFracL | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| yFracL | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 |
| predSampleLXL[ xL, yL ] | A << shift3 | d | h | n | a | e | i | p | b | f | j | q | c | g | k | r |

– If 1/8pel_active_flag is equal to 1, the positions labelled with lower-case letters within un-shaded blocks represent luma samples at eighth-pel sample fractional locations. The luma location offset in fractional-sample units ( xFracL, yFracL ) specifies which of the generated luma samples at full-sample and fractional-sample locations is assigned to the predicted luma sample value predSampleLXL[ xL, yL ]. This assignment is done according to Error: Reference source not found. The value of predSampleLXL[ xL, yL ] shall be the output.

Table 8-x – Assignment of the luma prediction sample predSampleLXL[ xL, yL ]

| xFracL | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| yFracL | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| predSampleLXL[ xL, yL ] | A << shift3 | Ad | d | dh | h | hn | n | nA | Aa | Aade | de | dehi | hi | hinp | np | npAa |

| xFracL | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| yFracL | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| predSampleLXL[ xL, yL ] | a | ae | e | ei | i | ip | p | pa | ab | abef | ef | efij | ij | ijpq | pq | pqab |

| xFracL | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| yFracL | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| predSampleLXL[ xL, yL ] | b | bf | f | fj | j | jq | q | qb | bc | bcfg | fg | fgjk | jk | jkqr | qr | qrbc |

| xFracL | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| yFracL | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| predSampleLXL[ xL, yL ] | c | cg | g | gk | k | kr | r | rc | cA | cAgd | gd | gdkh | kh | khrn | rn | rncA |

###### 8.4.2.2.2.2 Chroma sample interpolation process

Inputs to this process are:

–	a chroma location in full-sample units ( xIntC, yIntC ),

–	a chroma location in fractional-sample units ( xFracC, yFracC ),

–	the chroma reference sample array refPicLXC.

–	1/8pel_active_flag

Output of this process is a predicted chroma sample value predSampleLXC[ xC, yC ]

Figure 8-8 – Integer samples (shaded blocks with upper-case letters) and fractional sample positions (un-shaded blocks with lower-case letters) for eighth sample chroma interpolation

In Error: Reference source not found, the positions labelled with upper-case letters Bi, j within shaded blocks represent chroma samples at full-sample locations inside the given two-dimensional array refPicLXC of chroma samples. These samples may be used for generating the predicted chroma sample value predSampleLXC[ xC, yC ]. The locations ( xBi, j, yBi, j ) for each of the corresponding chroma samples Bi, j inside the given array refPicLXC of chroma samples are derived as follows:

xBi, j = Clip3( 0, PicWidthInSamplesC – 1, xIntC +i )		(8-82)
yBi, j = Clip3( 0, PicHeightInSamplesC – 1, yIntC +j )		(8-83)

Variables shift1, shift2 and shift3 are derived as follows.

–	The variable shift1 is set equal to BitDepthC – 8, the variable shift2 is set equal to BitDepthC – 2, and the variable shift3 is set equal to 14 – BitDepthC.

Given the chroma samples Bi, j at full-sample locations ( xBi, j, yBi, j ), the chroma samples ‘ab0,0’ to ‘hh0,0’ at fractional sample positions are derived by the following rules.

–	The samples labelled ab0,0, ac0,0, ad0,0, ae0,0, af0,0, ag0,0, and ah0,0 shall be derived by applying the 4-tap filter to the nearest integer position samples and clipping the filtered value:

ab0,0 = ( −3*B−1,0 + 60*B0,0 + 8*B1,0 – B2,0 ) >> shift1		(8-84)

ac0,0 = ( −4*B−1,0 + 54*B0,0 + 16*B1,0 – 2*B2,0 ) >> shift1	(8-85)

ad0,0 = ( −5*B−1,0 + 46*B0,0 + 27*B1,0 – 4*B2,0 ) >> shift1	(8-86)

ae0,0 = ( −4*B−1,0 + 36*B0,0 + 36*B1,0 – 4*B2,0 ) >> shift1	(8-87)

af0,0 = ( −4*B−1,0 + 27*B0,0 + 46*B1,0 – 5*B2,0 ) >> shift1	(8-88)

ag0,0 = ( −2*B−1,0 + 16*B0,0 + 54*B1,0 – 4*B2,0 ) >> shift1	(8-89)

ah0,0 = ( −B−1,0 + 8*B0,0 + 60*B1,0 – 3*B2,0 ) >> shift1		(8-90)

–	If 1/8pel_active_flag is equal to 1, the samples labelled Bab0,0 to ahB0,0 shall be derived by applying the 4-tap filter to the nearest integer position samples and clipping the filtered value:

Bab0,0 = ( -2*B-1,0 + 63*B0,0 + 4*B1,0 – B2,0  ) >> shift1	(8-91)

abac0,0 = ( -5*B-1,0 + 59*B0,0 + 13*B1,0 –3*B2,0  ) >> shift1	(8-92)

acad0,0 = ( -6*B-1,0 + 52*B0,0 + 23*B1,0 – 5*B2,0  ) >> shift1	(8-93)

adae0,0 = ( -7*B-1,0 + 43*B0,0 + 34*B1,0 – 6*B2,0  ) >> shift1	(8-94)

aeaf0,0 = ( -6*B-1,0 + 34*B0,0 + 43*B1,0 – 7*B2,0 ) >> shift1	(8-95)

afag0,0 = ( -5*B-1,0 + 23*B0,0 + 52*B1,0 – 6*B2,0  ) >> shift1	(8-96)

agah0,0 = ( -3*B-1,0 + 13*B0,0 + 59*B1,0 – 5*B2,0  ) >> shift1	(8-97)

ahB0,0 = ( -B-1,0 + 4*B0,0 + 63*B1,0 – 2*B2,0  ) >> shift1	(8-98)

–	The samples labelled ba0,0, ca0,0, da0,0, ea0,0, fa0,0, ga0,0, and ha0,0 shall be derived by applying the 4-tap filter to the nearest integer position samples and clipping the filtered value:

ba0,0 = ( −3*B0,−1 + 60*B0,0 + 8*B0,1 – B0,2 ) >> shift1		(8-99)

ca0,0 = ( −4*B0,−1 + 54*B0,0 + 16*B0,1 – 2*B0,2 ) >> shift1	(8-100)

da0,0 = ( −5*B0,−1 + 46*B0,0 + 27*B0,1 – 4*B0,2 ) >> shift1	(8-101)

ea0,0 = ( −4*B0,−1 + 36*B0,0 + 36*B0,1 – 4*B0,2 ) >> shift1	(8-102)

fa0,0 = ( −4*B0,−1 + 27*B0,0 + 46*B0,1 – 5*B0,2 ) >> shift1	(8-103)

ga0,0 = ( −2*B0,−1 + 16*B0,0 + 54*B0,1 – 4*B0,2 ) >> shift1	(8-104)

ha0,0 = ( −B0,−1 + 8*B0,0 + 60*B0,1 – 3*B0,2 ) >> shift1		(8-105)

–	If 1/8pel_active_flag is equal to 1, the samples labelled Bba0,0 to haB0,0 shall be derived by applying the 4-tap filter to the nearest integer position samples and clipping the filtered value:

Bba0,0 = ( -2*B0,-1 + 63*B0,0 + 4*B0,1 – B0,2  ) >> shift1	(8-106)

baca0,0 = ( -5*B0,-1 + 59*B0,0 + 13*B0,1 – 3*B0,2  ) >> shift1	(8-107)

cada0,0 = ( -6*B0,-1 + 52*B0,0 + 23*B0,1 – 5*B0,2  ) >> shift1	(8-108)

daea0,0 = ( -7*B0,-1 + 43*B0,0 + 34*B0,1 – 6*B0,2  ) >> shift1	(8-109)

eafa0,0 = ( -6*B0,-1 + 34*B0,0 + 43*B0,1 – 7*B0,2  ) >> shift1	(8-110)

faga0,0 = ( -5*B0,-1 + 23*B0,0 + 52*B0,1 – 6*B0,2  ) >> shift1	(8-111)

gaha0,0 = ( -3*B0,-1 + 13*B0,0 + 59*B0,1 – 5*B0,2  ) >> shift1	(8-112)

haB0,0 = ( -B0,-1 + 4*B0,0 + 63*B0,1 – 2*B0,2  ) >> shift1	(8-113)

–	The samples labelled bX0,0, cX0,0, dX0,0, eX0,0, fX0,0, gX0,0 and hX0,0 for X being replaced by b, c, d, e, f, g and h, respectively, shall be derived by applying the 4-tap filter to the intermediate values aX0,i where i = −1..2 in vertical direction:

bX0,0 = ( −3*aX0,−1 + 60*aX0,0 + 8*aX0,1 – aX0,2 ) >> shift2	(8-114)

cX0,0 = ( −4*aX0,−1 + 54*aX0,0 + 16*aX0,1 – 2*aX0,2 ) >> shift2	(8-115)

dX0,0 = ( −5*aX0,−1 + 46*aX0,0 + 27*aX0,1 – 4*aX0,2 ) >> shift2	(8-116)

eX0,0 = ( −4*aX0,−1 + 36*aX0,0 + 36*aX0,1 – 4*aX0,2 ) >> shift2	(8-117)

fX0,0 = ( −4*aX0,−1 + 27*aX0,0 + 46*aX0,1 – 5*aX0,2 ) >> shift2	(8-118)

gX0,0 = ( −2*aX0,−1 + 16*aX0,0 + 54*aX0,1 – 4*aX0,2 ) >> shift2	(8-119)

hX0,0 = ( −aX0,−1 + 8*aX0,0 + 60*aX0,1 – 3*aX0,2 ) >> shift2	(8-120)

–	If 1/8pel_active_flag is equal to 1, the final prediction values aXaYbXbY0,0 to hXhYaXaY0,0 for X being replaced by a, b, c, d, e, f, g and h, respectively, and for Y being replaced by b, c, d, e, f, g, h and a, respectively, shall be derived by applying the 4-tap filter to the intermediate values aXaY0,i where i = -1..2 in vertical direction:

aXaYbXbY0,0 = ( -2*aXaY0,-1 + 63* aXaY 0,0 + 4* aXaY 0,1 – aXaY 0,2 ) >> shift2

bXbY0,0 = ( -3*aXaY0,-1 + 60* aXaY 0,0 + 8* aXaY 0,1 – aXaY 0,2 ) >> shift2

bXbYcXcY0,0 = ( -5*aXaY0,-1 + 59* aXaY 0,0 + 13* aXaY 0,1 –3*aXaY 0,2 ) >> shift2

cXcY0,0 = ( -4*aXaY0,-1 + 54* aXaY 0,0 + 16* aXaY 0,1 –2* aXaY 0,2 ) >> shift2

cXcYdXdY0,0 = ( -6*aXaY0,-1 + 52* aXaY 0,0 + 23* aXaY 0,1 –5* aXaY 0,2 ) >> shift2

dXdY0,0 = ( -5*aXaY0,-1 + 46* aXaY 0,0 + 27* aXaY 0,1 –4* aXaY 0,2 ) >> shift2

dXdYeXeY0,0 = ( -7*aXaY0,-1 + 43* aXaY 0,0 + 34* aXaY 0,1 –6* aXaY 0,2 ) >> shift2

eXeY0,0 = ( -4*aXaY0,-1 + 36* aXaY 0,0 + 36* aXaY 0,1 –4* aXaY 0,2 ) >> shift2

eXeYfXfY0,0 = ( -6*aXaY0,-1 + 34* aXaY 0,0 + 43* aXaY 0,1 –7* aXaY 0,2 ) >> shift2

fXfY0,0 = ( -4*aXaY0,-1 + 27* aXaY 0,0 + 46* aXaY 0,1 –5* aXaY 0,2 ) >> shift2

fXfYgXgY0,0 = ( -5*aXaY0,-1 + 23* aXaY 0,0 + 52* aXaY 0,1 –6* aXaY 0,2 ) >> shift2

gXgY0,0 = ( -2*aXaY0,-1 + 16* aXaY 0,0 + 54* aXaY 0,1 –4* aXaY 0,2 ) >> shift2

gXgYhXhY0,0 = ( -3*aXaY0,-1 + 13* aXaY 0,0 + 59* aXaY 0,1 –5* aXaY 0,2 ) >> shift2

hXhY0,0 = ( -aXaY0,-1 + 8* aXaY 0,0 + 60* aXaY 0,1 –3* aXaY 0,2 ) >> shift2

hXhYaXaY0,0 = ( -aXaY0,-1 + 4* aXaY 0,0 + 63* aXaY 0,1 –2* aXaY 0,2 ) >> shift2

The positions labelled with lower-case letters within un-shaded blocks represent chroma samples at eighth-pel sample fractional locations. The chroma location offset in fractional-sample units ( xFracC, yFracC ) specifies which of the generated chroma samples at full-sample and fractional-sample locations is assigned to the predicted chroma sample value predSampleLXC[ xC, yC ]. This assignment is done according to Error: Reference source not found. The value of predSampleLXC[ xC, yC ] shall be the output.

Table 8-10 – Assignment of the chroma prediction sample predSampleLXC[ xC, yC ] for ( X, Y ) being replaced by ( 1, b ), ( 2, c ), ( 3, d ), ( 4, e ), ( 5, f ), ( 6, g ), and ( 7, h ), respectively

| xFracC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | X | X | X | X | X | X | X | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| yFracC | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| predSampleLXC[ xC, yC ] | B << shift3 | ba | ca | da | ea | fa | ga | ha | aY | bY | cY | dY | eY | fY | gY | hY |

– If 1/8pel_active_flag is equal to 1, the positions labelled with lower-case letters within un-shaded blocks represent chroma samples at sixteenth-pel sample fractional locations. The chroma location offset in fractional-sample units ( xFracC, yFracC ) specifies which of the generated chroma samples at full-sample and fractional-sample locations is assigned to the predicted chroma sample value predSampleLXC[ xC, yC ]. This assignment is done according to Error: Reference source not found. The value of predSampleLXC[ xC, yC ] shall be the output.

Table 8-x – Assignment of the chroma prediction sample predSampleLXC[ xC, yC ] for ( X, Y ) being replaced by ( 1, b ), ( 2, c ), ( 3, d ), ( 4, e ), ( 5, f ), ( 6, g ), and ( 7, h ), respectively

| xFracC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| yFracC | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| predSampleLXC[ xC, yC ] | B << shift3 | Bba | ba | baca | ca | cada | da | daea | ea | eafa | fa | faga | ga | gaha | ha | haB |

Table 8-x – Assignment of the chroma prediction sample predSampleLXC[ xC, yC ] for ( X, Y , Z ) being replaced by ( 1, a , b ), ( 3, b , c ), ( 5, c , d ), ( 7, d , e ), ( 9, e , f ), ( 11, f , g ), ( 13, g , h ) and ( 15, h , a ) , respectively

| xFracC | X | X | X | X | X | X | X | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| yFracC | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| predSampleLXC[ xC, yC ] | aYaZ | aYaZbYbZ | bYbZ | bYbZcYcZ | cYcZ | cYcZdYdZ | dYdZ | dYdZeYeZ |

| xFracC | X | X | X | X | X | X | X | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| yFracC | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| predSampleLXC[ xC, yC ] | eYeZ | eYeZfYfZ | fYfZ | fYfZgYgZ | gYgZ | gYgZhYhZ | hYhZ | hYhZaYaZ |

Table 8-x – Assignment of the chroma prediction sample predSampleLXC[ xC, yC ] for ( X, Y ) being replaced by ( 2, b ), ( 4, c ), ( 6, d ), ( 8, e ), ( 10, f ), ( 12, g ) and ( 14, h ) , respectively

| xFracC | X | X | X | X | X | X | X | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| yFracC | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| predSampleLXC[ xC, yC ] | aY | aYbY | bY | bYcY | cY | cYdY | dY | dYeYe |

| xFracC | X | X | X | X | X | X | X | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| yFracC | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| predSampleLXC[ xC, yC ] | eY | eYfY | fY | fYgY | gY | gYhY | hY | hYaY |

#### 8.6.1.3	Derivation process of boundary filtering strength

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top-left luma sample of the current picture,

–	a variable log2CUSize specifying the size of the current coding unit,

–	a two-dimensional arrays of size (nS)x(nS), horEdgeFlags and verEdgeFlags.

Output of this process is an array of size (2)x(nS)x(nS), bS specifying the boundary filtering strength.

Let ( xEk, yEj ) with k = 0..nE-1 and j = 0..nE-1 specify a set of edge sample locations where nE is set equal to ( ( 1 << log2CUSize ) >> 2 ), xE0 = 0, yE0 = 0, xEk+1 = xEk + 4 and yEj+1 = yEj + 4.

For ( xEk, yEj ) with k = 0..nE-1 and j = 0..nE-1, the following applies.

- If horEdgeFlags[ xEk ][ yEj ] is equal to 1,

- Set sample p0 = recPicture[ xC + xEk ][ yC + yEj – 1 ] and q0 = recPicture[ xC + xEk ][ yC + yEj ].

- The variable filterDir is set equal to 1.

- Otherwise, if verEdgeFlags[ xEk ][ yEj ] is equal to 1,

- Set sample p0 = recPicture[ xC + xEk – 1 ][ yC + yEj ] and q0 = recPicture[ xC + xEk ][ yC + yEj ].

- The variable filterDir is set equal to 0.

- Depending on the value of filterDir, the variable bS[ filterDir ][ xEk ][ yEj ] is derived as follows.

- If the block edge is also a coding unit edge and the following condition is true, the variable bS[ 0 ][ xEk ][ yEj ] is set equal to 4.

- The sample p0 or q0 is in a coding unit coded with intra prediction mode

- Otherwise, if the following condition is true, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 3.

- The sample p0 or q0 is in a coding unit coded with intra prediction mode

- Otherwise, if the following condition is true, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 2.

- The sample p0 or q0 is in a transform unit which contains non-zero transform coefficient level.

- Otherwise, if any of the following conditions are true, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 1.

- The prediction unit containing sample p0 has different reference pictures or a different number of motion vectors with the prediction unit containing the sample q0.

NOTE – The determination of whether the reference pictures used for the two prediction are the same or different is based on which pictures are referenced, without regard to whether a prediction is formed using an index into list 0 or an index into list 1, and also without regard to whether or not the index position within a reference picture list is different or not.

- One motion vector is used to predict the prediction unit containing sample p0, one motion vector is used to predict the prediction unit containing sample q0, and the absolute difference between the horizontal or vertical component of the motion vector used is greater than or equal to (4<<1/8pel_active_flag) 4 in units of quarter luma samples.

- [Ed.: (WJ) needs to be checked again whether this condition covers all 2-motion cases] Two motion vectors are used to predict the prediction unit containing sample p0, two motion vectors are used to predict the prediction unit containing sample q0, and at least one of the motion vector pairs corresponding the same reference pictures and the different boundary samples p0 and q0 satisfies the following condition:

- The absolute difference between the horizontal or vertical component of a motion vector used in the prediction of the two prediction units is greater than or equal to (4<<1/8pel_active_flag) 4 in units of quarter luma samples.

- Otherwise, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 0.