| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G537 |
| --- | --- |

| Title: | Table reduction and Improvement of last position coding in CAVLC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chanyul Kim Youngo Park JungHun Park | Tel: Email: | dionism@samsung.com youngo.park@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In this contribution, table reduction and performance improvement of last position coding is presented. Only 136 bytes out of 336 bytes are used for last position coding which leads to 59.5% table reduction as shown in Table 1. In addition, -0.2%/-1.3%/-1.5% for AI_LC, -0.1%/-0.7%/-0.8% for RA_LC, and -0.2%/0.0%/-0.3% for LD_LC coding gain is obtained without additional complexity.

# Introduction

In the 5th meeting, coefficient coding with larger block was presented [1], where last position coding except for 4x4 block size was proposed and adopted. The current HM uses eight global tables for larger than 4x4 blocks, three tables only for 4x4 blocks. In this proposal, we reduce table size simply by introducing size based method for Luma and one context for all Chroma as shown in Figure 1. Based on pre-analysis, the last position is more affected by block size than prediction mode. Therefore, reduction of global table can be achieved and coding gain is also obtained, especially high coding in Chroma as well.

# Algorithm description

The current HM uses eight counters for table adaptation larger than 4x4 blocks, three counters for 4x4 blocks. Chroma has similar property for last position coding, therefore, all Chroma use the same counter and table for improvement of last position coding. In case of Luma, prediction mode does not effect on severely coding performance of last position. The proposed method use separated size based counter such as 8x8, 16x16, and 32x32 as depicted in Figure 1. Moreover, one table is used for Luma of 4x4 blocks case in order to further reduce tables. VLC table is selected by its block size as follows,

#if IMP_LAST

UInt map = g_aucConvertToBit[blSize];

if(blockType <2)

map =0;

vlc = g_auiLastPosVlcNum[map][min(16u,m_uiLastPosVlcIndex[map])];

#else

vlc = g_auiLastPosVlcNum[blockType][min(16u,m_uiLastPosVlcIndex[blockType])];

#endif

In case of 4x4 blocks, we don’t require separation of Chroma as follows, which give a benefit for complexity.

#if IMP_LAST

int nTab = blockType-2;

#else

int nTab = max(0,blockType-2);

#endif

All the remaining procedure is the same as current last position coding in HM4.0. (See 7.Annex)

Figure 1. Required table and procedure comparison between HM4.0 and the proposed method

Table 1. Table reduction ration between HM4.0 and the proposed method

# Simulation Results

Using HM4.0, simulation is performed under the common test condition [2]. The result shows coding gain for all configurations. Moreover, high Chroma gain is obtained by combining all Chroma components, especially for class E in AI_LC (-4.2%/-3.2%).

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | #VALUE! | #VALUE! | #VALUE! | -0.1% | -1.0% | -1.1% |
| Class B | #VALUE! | #VALUE! | #VALUE! | -0.2% | -0.8% | -1.5% |
| Class C | #VALUE! | #VALUE! | #VALUE! | -0.4% | -0.5% | -1.1% |
| Class D | #VALUE! | #VALUE! | #VALUE! | -0.3% | -1.0% | -1.1% |
| Class E | #VALUE! | #VALUE! | #VALUE! | 0.0% | -4.2% | -3.2% |
| Overall | #VALUE! | #VALUE! | #VALUE! | -0.2% | -1.3% | -1.5% |
|  | #VALUE! | #VALUE! | #VALUE! | -0.2% | -1.3% | -1.4% |
| Enc Time[%] | #NUM! |  |  | 100% |  |  |
| Dec Time[%] | #NUM! |  |  | 100% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | #VALUE! | #VALUE! | #VALUE! | -0.1% | -1.1% | -0.5% |
| Class B | #VALUE! | #VALUE! | #VALUE! | -0.1% | -0.6% | -1.2% |
| Class C | #VALUE! | #VALUE! | #VALUE! | -0.1% | -0.4% | -0.8% |
| Class D | #VALUE! | #VALUE! | #VALUE! | -0.1% | -0.9% | -0.5% |
| Class E |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | -0.1% | -0.7% | -0.8% |
|  | #VALUE! | #VALUE! | #VALUE! | -0.1% | -0.7% | -0.7% |
| Enc Time[%] | #NUM! |  |  | 99% |  |  |
| Dec Time[%] | #NUM! |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | -0.2% | 0.3% | -0.1% |
| Class C | #VALUE! | #VALUE! | #VALUE! | -0.2% | 0.2% | -0.6% |
| Class D | #VALUE! | #VALUE! | #VALUE! | -0.1% | 0.9% | 0.4% |
| Class E | #VALUE! | #VALUE! | #VALUE! | -0.2% | -1.9% | -1.3% |
| Overall | #VALUE! | #VALUE! | #VALUE! | -0.2% | 0.0% | -0.3% |
|  | #VALUE! | #VALUE! | #VALUE! | -0.2% | 0.0% | -0.2% |
| Enc Time[%] | #NUM! |  |  | 100% |  |  |
| Dec Time[%] | #NUM! |  |  | 100% |  |  |

# Conclusion

This proposal has target for reducing table size of last position coding. The experimental results show that the proposed method improves coding performance for all test sequences consistently without additional complexity even though it reduces the table size 59.4% (from 336 bytes  136 bytes). We anticipate that the proposed method be adopted into next HM.

# References

- K.Marth, etal., “CE5: coefficient coding with LCEC for large blocks,” JCT-VC document JCTVC-E700, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 5th meeting, Geneva, March 2011.

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-C500, JCT-VC 3rd Meeting, Guangzhou, October 2010.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex : WD Text change

Based on JCTVC-F-803_d5.doc

9.2.2.2 Initialisation process for lastPosVlcNumIndex

Outputs of this process are initial values of the variable array lastPosVlcNumIndex.

The variable array lastPosVlcIndex is initialized as follows.

sizeMap = log2TrafoSize -2

If(cIdx >0)  sizeMap = 0;

lastPosVlcNumIndex [blockType sizeMap ]  = 0,  for blockType = 0...7 sizeMap = 0…4

#### 9.2.2.3 Initialisation process for lastPosTable

Outputs of this process are initial values of the variable array lastPosTable.

The variable array lastPosTable is initialized as specified in Error: Reference source not found.

Table 9-1 – Specification of initial values of lastPosTable[tableNum][codeNum]

|  | - codeNum |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - tableNum | - 0 | - 1 | - 2 | - 3 | - 4 | - 5 | - 6 | - 7 | - 8 | - 9 | - 10 | - 11 | - 12 | - 13 | - 14 | - 15 |
| - 0 | - 0 | - 1 | - 2 | - 3 | - 5 | - 4 | - 7 | - 6 | - 11 | - 8 | - 14 | - 9 | - 16 | - 15 | - 10 | - 13 |
| - 1 | - 0 | - 1 | - 2 | - 5 | - 7 | - 4 | - 6 | - 3 | - 9 | - 12 | - 11 | - 8 | - 10 | - 14 | - 13 | - 15 |
| - 2 | - 0 | - 1 | - 2 | - 5 | - 7 | - 4 | - 6 | - 3 | - 9 | - 12 | - 11 | - 8 | - 10 | - 14 | - 13 | - 15 |

|  | - codeNum |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - tableNum | - 16 | - 17 | - 18 | - 19 | - 20 | - 21 | - 22 | - 23 | - 24 | - 25 | - 26 | - 27 | - 28 | - 29 | - 30 | - 31 |
| - 0 | - 12 | - 17 | - 18 | - 19 | - 22 | - 31 | - 23 | - 21 | - 27 | - 20 | - 25 | - 30 | - 24 | - 26 | - 28 | - 29 |
| - 1 | - 16 | - 17 | - 21 | - 22 | - 25 | - 18 | - 28 | - 23 | - 30 | - 26 | - 20 | - 19 | - 27 | - 24 | - 29 | - 31 |
| - 2 | - 16 | - 17 | - 21 | - 22 | - 25 | - 18 | - 28 | - 23 | - 30 | - 26 | - 20 | - 19 | - 27 | - 24 | - 29 | - 31 |

#### 9.2.4.1 Parsing process for last_pos_level_one

Inputs to this process are bits from RBSP and the variable arrays lastPosVlcNumIndex and lastPosTable.

Output of this process is the syntax element last_pos_level_one, the variables trOne and vlcNumLevel, and the variable arrays lastPosVlcNumIndex and lastPosTable.

The parsing process for last_pos_level_one is specified as follows.

- The variable N is set equal to (1 << log2TrafoSize) >> (cIdx > 0 ? 2 : 0).

- If N is greater than 4 and cIdx >0, last_pos_level_one is derived in the following ordered steps:

- The variable blockType is derived as

blockType = (cIdx == 0 ? (PredMode==MODE_INTRA ? 0 : slice_type + 1) + (N > 8 ? 5 : 2) : cIdx – 1)											(9-1)

- The variable vlcNumIndex is derived as

vlcNumIdx = Min(16, lastPosVlcNumIndex[blockType sizeMap]) 	(9-2)

- The variable vlcNum is derived as

vlcNum = lastPosVlcNumTable[blockType sizeMap][vlcNumIdx] 	(9-3)

The values of the array lastPosVlcNumTable are shown in Error: Reference source not found.

- The parsing process described in subclause Error: Reference source not found is invoked with vlcNum as input and the variable codeNum as output.

- The variable lastPos is derived as follows

if (codeNum <= N+N*N)
			if (codeNum != 0 && (codeNum & (N-1)) = = 0)
				lastPos = (codeNum >> Log2(N)) – 1	(9-4)
			else
				lastPos = codeNum – (codeNum >> Log2(N))
		else
			lastPos = codeNum – N*N

- The variable levelGreaterThanOneFlag is derived as

levelGreaterThanOneFlag = (codeNum > (N + N*N)) | | (codeNum != 0 && (codeNum & (N-1))) 											(9-5)

- The syntax element last_pos_level_one is derived as

last_pos_level_one = levelGreaterThanOneFlag ? codeNum + N : codeNum	(9-6)

- The variable array lastPosVlcNumIndex[blockType sizeMap] is updated as follows.

if ((N = = 8 ? codeNum : codeNum>>2) < lastPosVlcNumIndex[blockType sizeMap])
			lastPosVlcNumIndex[blockType sizeMap] -=  1	(9-7)
		else if ((N = = 8 ? codeNum : codeNum>>2)  > lastPosVlcNumIndex[blockType sizeMap])
			lastPosVlcNumIndex[blockType sizeMap] +=  1

- Otherwise, (N equal to 4 and cIdx = = 0), last_pos_level_one is derived in the following ordered steps:

- The parsing process described in subclause Error: Reference source not found is invoked with 2 as input and the variable codeNum as output.

- The variable tableNum is derived as

tableNum = (PredMode==MODE_INTRA | | cIdx > 0) ? 0 : slice_type + 1	(9-8)

- The syntax element last_pos_level_one is derived as

last_pos_level_one = lastPosTable[tableNum][codeNum] 	(9-9)

- The variable array lastPosTable[tableNum] is updated by invoking process Error: Reference source not found with lastPosTable[tableNum], codeNum and 0 as inputs.

- The variable trOne is derived as

trOne = ! levelGreaterThanOneFlag		(9-10)

- The variable vlcNumLevel is set equal to 0

Table 9-2 – Specification of lastPosVlcNumTable[blockType sizeMap][vlcNumIdx]

|  | vlcNumIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| blockType | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| 0 | 10 | 10 | 10 | 10 | 2 | 2 | 2 | 7 | 9 | 9 | 9 | 9 | 9 | 4 | 4 | 4 | 4 |
| 1 | 10 | 10 | 10 | 10 | 10 | 2 | 9 | 9 | 9 | 9 | 9 | 9 | 9 | 4 | 4 | 4 | 4 |
| 2 | 2 | 2 | 2 | 2 | 2 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 4 | 4 | 13 |
| 3 | 2 | 2 | 2 | 2 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 13 |
| 4 | 2 | 2 | 2 | 2 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 13 |
| 5 | 10 | 10 | 10 | 4 | 4 | 4 | 4 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| 6 | 10 | 10 | 10 | 10 | 4 | 4 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| 7 | 10 | 10 | 10 | 10 | 4 | 4 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |

|  | vlcNumIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| sizeMap | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| 0 | 10 | 10 | 10 | 10 | 2 | 2 | 2 | 7 | 9 | 9 | 9 | 9 | 9 | 4 | 4 | 4 | 4 |
| 1 | 2 | 2 | 2 | 2 | 2 | 7 | 7 | 7 | 7 | 7 | 7 | 9 | 7 | 7 | 4 | 4 | 13 |
| 2 | 10 | 10 | 10 | 4 | 4 | 4 | 4 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| 3 | 10 | 10 | 10 | 10 | 4 | 4 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |