| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 Nov., 2011 | Document: JCTVC-G540 WG11 Number: m22104 |
| --- | --- |

| Title: | CE13: Cross-check report of subset 3.2 series by Panasonic |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Toshiyasu Sugio Takahiro Nishi | Tel: Email: | +81 6 6900 9689 sugio.toshiyasu@jp.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution is a report of cross-check result of CE13 subset 3.2 series in JCTVC-F913. The experimental results perfectly match with the ones provided by TI in the rate-distortion performance.

- Experimental Condition

Table 1 to 3 shows summary of the subset 3.2.1, 3.2.2 and 3.2.3 in CE13 [1]. The software to be tested was provided by TI and its performance relative to the HM4.0 software was checked under the common test condition described in JCTVC-F900 [2]. The computing platform is Linux 64bits and the executables were compiled on 64-bit Linux with gcc 4.4.3.

### 1) Subset 3.2.1 Coding efficiency study with different maximum truncated unary table sizes signaled in slice header

### 2) Subset 3.2.2 Reduction of the number of candidates for the 2nd pruning process

### 3) Subset 3.2.3 Combined tests for 1) and 2)

Table 1: Summary of the subset 3.2.1 Test2a, 3a, 4a, 1b, 2b, 3b and 4b

|  | maxNumMerge Cand | Truncated unary table for merge_idx coding | Encoder restriction |
| --- | --- | --- | --- |
| HM4.0 Reference | 5 | 0 / 10 / 110 / 1110 / 1111 | No |
| Test 2a | 3 | 0 / 10 / 11 | No |
| Test 3a | 2 | 0 / 1 | No |
| Test 4a | 1 | N/A | No |
| Test 1b | 5 | 0 / 10 / 110 / 1110 / 1111 | Yes, merge_idx restricted to [0:3] in ME |
| Test 2b | 5 | 0 / 10 / 110 / 1110 / 1111 | Yes, merge_idx restricted to [0:2] in ME |
| Test 3b | 5 | 0 / 10 / 110 / 1110 / 1111 | Yes, merge_idx restricted to [0:1] in ME |
| Test 4b | 5 | 0 / 10 / 110 / 1110 / 1111 | Yes, merge_idx restricted to 0 in ME |

Table 2: Summary of the subset 3.2.2 test1 and 2

|  | Maximum number of MVP candidates after the 1st pruning process | Maximum number of combined bi-predictive MVP candidates | Maximum number of non-scaled bi-predictive MVP candidates | Maximum number of zero MVP candidates |
| --- | --- | --- | --- | --- |
| HM4.0 reference | 4 | 5 | 1 | 2 |
| Test 1 | 4 | 4 – n - z | 1 (n) | 2 (z) |
| Test 2 | 4 | 5 – n - z | 1 (n) | 2 (z) |

Table 3: Summary of the subset 3.2.3 test1b, 2b, 3b, 4b

|  | maxNumMergeCand | Truncated unary table for merge_idx coding | Maximum number of combined bi-predictive MVP candidates |
| --- | --- | --- | --- |
| HM4.0 Reference | 5 | 0 / 10 / 110 / 1110 / 1111 | 5 |
| Test 1b | 4 | 0 / 10 / 110 /111 | 5 – n – z |
| Test 2b | 3 | 0 / 10 / 11 |  |
| Test 3b | 2 | 0 / 1 |  |
| Test 4b | 1 | N/A |  |

- Results

R-D performance and execution time of each experimental result are summarized in Table 4 to 16. Detailed results are included in the attached excel sheet. It was confirmed that these results perfectly match with the one provided by TI.

Table 4: Experimental results of subset 3.2.1 test2a

Table 5: Experimental results of subset 3.2.1 test3a

Table 6: Experimental results of subset 3.2.1 test4a

Table 7: Experimental results of subset 3.2.1 test1b

Table 8: Experimental results of subset 3.2.1 test2b

Table 9: Experimental results of subset 3.2.1 test3b

Table 10: Experimental results of subset 3.2.1 test4b

Table 11: Experimental results of subset 3.2.2 test1

Table 12: Experimental results of subset 3.2.2 test2

Table 13: Experimental results of subset 3.2.3 test1b

Table 14: Experimental results of subset 3.2.3 test2b

Table 15: Experimental results of subset 3.2.3 test3b

Table 16: Experimental results of subset 3.2.3 test4b

- Conclusion

The results of CE13 subset 3.2 series were confirmed.

- References

- Joel Jung et al., “Description of Core Experiment CE13: Motion data parsing robustness and throughput”, JCTVC-F913, Torino, July 2011.

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, Torino, July 2011.