| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G541 WG11 Number: m22105 |
| --- | --- |

| Title: | Non-CE9: Simplified scaling calculation method for temporal/spatial MVP of AMVP/Merge |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Toshiyasu Sugio Takahiro Nishi 1006 Kadoma, Kadoma Osaka, Japan | Tel: Email: | +81-50-3587-2047 { sugio.toshiyasu, nishi.takahiro }@jp.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

In this contribution, a simplified scaling calculation for AMVP/Merge was proposed. In the proposed method, the scaling factor is quantized to a power of 2 based on the distance among current picture, reference picture and co-located picture. Experimental results reportedly showed BD rate loss in the range of 0.0-0.1% with reducing the number of multiplier relative to the HM4.0 by applying the proposed method to temporal MVP in AMVP.

# Introduction

The scaling calculation is conducted to create motion vector predictor for 3 different modes in HM4.0 (temporal MVP for AMVP, spatial MVP for AMVP and temporal MVP for Merge). However, it was reported in JCTVC-F068[1] that the scaling calculation was expensive not only in terms of cycles but also in terms of area. In this proposal, a simplified scaling method, which is constructed of comparator and bit shift, is applied to each 3 modes, and the impact to coding efficiency is reported.

# Simplified Scaling Method

A sample architecture diagram of scaling calculation in the HM4.0, which was reported in JCTVC-F068,   is shown in Figure 1. In this sample, two multipliers are needed to calculate one motion vector component in parallel. It is considered that hardware impact is quite bigger than AVC design.

Figure 1: sample architecture diagram of scaling calculation in HM4.0 (JCTVC-F068)

To reduce hardware impact on scaling calculation, proposed scaling calculation uses only comparison and bit shift operations for deriving motion vector predictor. Figure 2 shows the concept of simplified scaling calculation. The scaling factor is quantized to a power of 2 based on the two distances, one is that between currPic and refIdxLX and another is that between colPic and refIdxCol. The flowchart of proposed method is shown in Figure 3.

Figure 2: the amount of bit shift in simplified scaling method

Figure 3: flowchart of simplified scaling method

# Experimental results

A proposed method was implemented on HM4.0 software and applied to 3 different modes (temporal MVP in AMVP, spatial MVP in AMVP and temporal MVP in Merge). Table 1 shows its RD performance and execution time of the software relative to HM4.0 in the case of applying the simplified scaling method to temporal MVP in AMVP. In random access cases, proposed method loses 0.1% BD rate for high efficiency and no loss for low complexity on average. In low delay case, proposed method loses 0.1% BD rate for high efficiency and low complexity on average. The difference in the encoding and decoding time is not obvious.

Table 1: Experimental result of simplified scaling for temporal MVP in AMVP

Table 2 shows its RD performance and execution time of the software relative to HM4.0 in the case of applying the simplified scaling method to spatial MVP in AMVP. In random access cases, proposed method loses 0.2% BD rate for high efficiency and low complexity on average. In low delay case, proposed method loses 0.1% BD rate for high efficiency and low complexity on average. The difference in the encoding and decoding time is not obvious.

Table 2: Experimental result of simplified scaling for spatial MVP in AMVP

Table 3 shows its RD performance and execution time of the software relative to HM4.0 in the case of applying the simplified scaling method to temporal MVP in Merge. In random access cases, proposed method loses 0.1% BD rate for high efficiency and low complexity on average. In low delay case, proposed method loses 0.1% BD rate for high efficiency and low complexity on average. The difference in the encoding and decoding time is not obvious.

Table 3: : Experimental result of simplified scaling for temporal MVP in Merge

# Conclusion

In this contribution, a simplified scaling calculation method is proposed. In the case of applying it to temporal MVP in AMVP, the proposed method can reduce the number of multiplier significantly while keeping BD rate loss in the range of 0.0-0.1% as shown in the experimental result. Based on this minor BD rate loss and significant complexity reduction, we suggest JCT-VC to consider the future adoption of the proposed method based on the study in a CE.

# References

- Minhua Zhou, Vivienne Sze, Yasutomo Matsuba, ” A study on HM3.0 parsing throughput issue”, JCTVC-F068, Torino, July. 2011

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).