| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G541 WG11 Number: m22105 |
| --- | --- |

| Title: | Non-CE9: Simplified scaling calculation method for temporal/spatial MVP of AMVP/Merge |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Toshiyasu Sugio Takahiro Nishi 1006 Kadoma, Kadoma Osaka, Japan | Tel: Email: | +81-50-3587-2047 { sugio.toshiyasu, nishi.takahiro }@jp.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

In this contribution, a simplified scaling calculation for AMVP/Merge was proposed. In the proposed method, the scaling factor is quantized to a power of 2 based on the distance among current picture, reference picture and co-located picture. Experimental results reportedly showed BD rate loss in the range of 0.0-0.1% with reducing the number of multiplier relative to the HM4.0 by applying the proposed method to temporal MVP in AMVP.

# Introduction

The scaling calculation is conducted to create motion vector predictor for 3 different modes in HM4.0 (temporal MVP for AMVP, spatial MVP for AMVP and temporal MVP for Merge). However, it was reported in JCTVC-F068[1] that the scaling calculation was expensive not only in terms of cycles but also in terms of area. In this proposal, a simplified scaling method, which is constructed of comparator and bit shift, is applied to each 3 modes, and the impact to coding efficiency is reported.

# Simplified Scaling Method

A sample architecture diagram of scaling calculation in the HM4.0, which was reported in JCTVC-F068,   is shown in Figure 1. In this sample, two multipliers are needed to calculate one motion vector component in parallel. It is considered that hardware impact is quite bigger than AVC design.

Figure 1: sample architecture diagram of scaling calculation in HM4.0 (JCTVC-F068)

To reduce hardware impact on scaling calculation, proposed scaling calculation uses only comparison and bit shift operations for deriving motion vector predictor. Figure 2 shows the concept of simplified scaling calculation. The scaling factor is quantized to a power of 2 based on the two distances, one is that between currPic and refIdxLX and another is that between colPic and refIdxCol. The flowchart of proposed method is shown in Figure 3.

Figure 2: the amount of bit shift in simplified scaling method

Figure 3: flowchart of simplified scaling method

# Experimental results

A proposed method was implemented on HM4.0 software and applied to 3 different modes (temporal MVP in AMVP, spatial MVP in AMVP and temporal MVP in Merge). Table 1 shows its RD performance and execution time of the software relative to HM4.0 in the case of applying the simplified scaling method to temporal MVP in AMVP. In random access cases, proposed method loses 0.1% BD rate for high efficiency and no loss for low complexity on average. In low delay case, proposed method loses 0.1% BD rate for high efficiency and low complexity on average. The difference in the encoding and decoding time is not obvious.

Table 1: Experimental result of simplified scaling for temporal MVP in AMVP

Table 2 shows its RD performance and execution time of the software relative to HM4.0 in the case of applying the simplified scaling method to spatial MVP in AMVP. In random access cases, proposed method loses 0.2% BD rate for high efficiency and low complexity on average. In low delay case, proposed method loses 0.1% BD rate for high efficiency and low complexity on average. The difference in the encoding and decoding time is not obvious.

Table 2: Experimental result of simplified scaling for spatial MVP in AMVP

Table 3 shows its RD performance and execution time of the software relative to HM4.0 in the case of applying the simplified scaling method to temporal MVP in Merge. In random access cases, proposed method loses 0.1% BD rate for high efficiency and low complexity on average. In low delay case, proposed method loses 0.1% BD rate for high efficiency and low complexity on average. The difference in the encoding and decoding time is not obvious.

Table 3: : Experimental result of simplified scaling for temporal MVP in Merge

# Conclusion

In this contribution, a simplified scaling calculation method is proposed. In the case of applying it to temporal MVP in AMVP, the proposed method can reduce the number of multiplier significantly while keeping BD rate loss in the range of 0.0-0.1% as shown in the experimental result. Based on this minor BD rate loss and significant complexity reduction, we suggest JCT-VC to consider the future adoption of the proposed method based on the study in a CE.

# References

- Minhua Zhou, Vivienne Sze, Yasutomo Matsuba, ” A study on HM3.0 parsing throughput issue”, JCTVC-F068, Torino, July. 2011

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix

## Proposed changes on Working Draft 4.0

## 8.2 Slice decoding process

##### 8.4.2.1.9 Derivation process for temporal luma motion vector prediction

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Outputs of this process are

- the motion vector prediction mvLXCol,

- the availability flag availableFlagLXCol.

The function RefPicOrderCnt( pic, refidx, LX ) is specified by the value of PicOrderCnt of the picture that is the reference picture RefPicListX[ refidx ] of pic with X being 0 or 1. PicOrderCnt of the reference picture shall be maintained until the picture is marked as “non-existing”.

[Ed. (BB): PicOrderCnt(.) needs to be defined. Currently the Decoding process for picture order count is not described]

Depending on the values of slice_type and collocated_from_l0_flag, the variable colPic, specifying the picture that contains the co-located partition, is derived as follows.

- If slice_type is equal to B and collocated_from_l0_flag is equal to 0, the variable colPic specifies the picture that contains the co-located partition as specified by RefPicList1[ 0 ].

- Otherwise (slice_type is equal to B and collocated_from_l0_flag is equal to 1 or slice_type is equal to P) , the variable colPic specifies the picture that contains the co-located partition as specified by RefPicList0[ 0 ].

Variable colPu and its position ( xPCol, yPCol ) are derived in the following ordered steps:

- Right-bottom luma position of the current prediction unit is defined by

xPRb = xP + nPSW	(8-1)

yPRb = yP + nPSH	(8-2)

- The variable colPu is set as the prediction unit covering the modified position given by ( ( xPRb >> 4 ) << 4, ( yPRb >> 4 ) << 4 ) inside the colPic.

- If colPu is coded in an intra prediction mode or colPu is unavailable, the following applies.

  - Central luma position of the current prediction unit is defined by

xPCtr = ( xP + ( nPSW >> 1 ) – 1	(8-3)

yPCtr = ( yP + ( nPSH >> 1 ) – 1	(8-4)

  - The variable colPu is set as the prediction unit covering the modified position given by ( ( xPCtr >> 4 ) << 4, ( yPCtr >> 4 ) << 4 ) inside the colPic.

- ( xPCol, yPCol ) is set equal to the top-left luma sample of the colPu relative to the top-left luma sample of the colPic.

The variables mvLXCol and availableFlagLXCol are derived as follows.

- If colPu is coded in an intra prediction mode or colPu is unavailable, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

- Otherwise (colPu is not coded in an intra prediction mode and colPu is available), the variables mvCol and refIdxCol are derived as follows,

  - If PredFlagL0[ xPCol ][ yPCol ] is equal to 0, the motion vector mvCol and the reference index refIdxCol are set equal to MvL1[ xPCol ][ yPCol ] and RefIdxL1[ xPCol ][ yPCol ], respectively.

  - Otherwise (PredFlagL0[ xPCol ][ yPCol ] is equal to 1), the following applies.

  - If PredFlagL1[ xPCol ][ yPCol ] is equal to 0, the motion vector mvCol and the reference index refIdxCol are set equal to MvL0[ xPCol ][ yPCol ] and RefIdxL0[ xPCol ][ yPCol ], respectively.

  - Otherwise (PredFlagL1[ xPCol ][ yPCol ] is equal to 1), the following assignments are made.

      - If all PicOrderCnts of pictures in all reference picture lists are smaller than PicOrderCnt( currPic ), the motion vector mvCol, the reference index refIdxCol and ListCol are set equal to MvLN[ xPCol ][ yPCol ], RefIdxColLN and LN, respectively, with N being value of RefIdxLX.

      - Otherwise (one of PicOrderCnt of pictures in all reference picture lists are larger than PicOrderCnt( currPic ) ), the motion vector mvCol, the reference index refIdxCol and ListCol are set equal to MvLN[ xPCol ][ yPCol ], RefIdxColLN and LN, respectively, with N being value of collocated_from_l0_flag.

and the variable availableFlagLXCol is set equal to 1 and the following applies.

  - If PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, ListCol ) is equal to PicOrderCnt( currPic ) - RefPicOrderCnt( currPic, refIdxLX, LX ),

mvLXCol = mvCol	(8-5)

  - Otherwise, mvLXCol is derived as scaled version of the motion vector mvCol as specified below

tx = ( 16384 + Abs( td / 2 ) ) / td	(8-6)

DistScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 )	(8-7)

mvLXCol = ClipMv( Sign( DistScaleFactor * mvLXA ) * 			
				( (Abs( DistScaleFactor * mvLXA ) + 127 ) >> 8 ) )	(8-8)

[Ed. (WJ): software has a clip function for scaled motion vector. Do we need this? AVC does not have it]

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, ListCol ) )	(8-9)

tb = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxLX, LX ) )	(8-10)

The variable diff_Curr and diff_Col are derived as

      - diff Curr = PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxLX, LX )

      - diff_Col = PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, ListCol )

The variable shift is derived as

      - If abs( diff_Curr ) is equal or larger than ( abs( diff_Col ) << 2 ), the variable shift is set to 2

      - Otherwise if abs( diff_Cur ) is equal or larger than ( abs( diff_Col ) << 1 ), the variable shift is set to 1

      - Otherwise if ( abs( diff_Cur ) << 2 ) is equal or larger than abs( diff_Col ), the variable shift is set to -2

      - Otherwise if ( abs( diff_Cur ) << 1 ) is equal or larger than abs( diff_Col ), the variable shift is set to -1

      - Otherwise the variable shift is set to 0

If either diff_Cur is larger than 0 and diff_Col is less than 0 or diff_Cur is less than 0 and diff_Col is larger than 0, mvLXCol is derived as

      - mvLXCol = - ( mvLXA << shift )

Otherwise, mvLXCol is derived as

      - mvLXCol = (mvLXA << shift)