| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G542 WG11 Number: m22106 |
| --- | --- |

| Title: | Non-CE9/Non-CE13: Simplification on AMVP/Merge |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Toshiyasu Sugio Takahiro Nishi 1006 Kadoma, Kadoma Osaka, Japan | Tel: Email: | +81-50-3587-2047 { sugio.toshiyasu, nishi.takahiro }@jp.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

In this contribution, three simplification methods on AMVP/Merge were proposed. 1) On avoiding merging candidate decision 2) On AMVP spatial scaling candidate 3) On zero motion vector merging candidate. Experimental results reportedly showed no loss on average relative to the HM4.0 with reducing complexity on AMVP/Merge.

# Proposed Method

This proposal includes the following three simplifications on AMVP/Merge.

- On avoiding merging candidate decision [simplification1]

- On AMVP spatial scaling candidate [simplification2]

- On zero motion vector merging candidate [simplification3]

## On avoiding merging candidate decision

In HM4.0, when merging candidate list for PU1 of PART_2NxN, PART_Nx2N, PART_2NxnU, PART_2NxnD, PART_nLx2N and PART_nRx2N is derived, the motion parameters of the neighboring PU_N (N=A1, B1, B0, A0, B2) is checked whether it’s identical to that of PU0, and if it’s true, that neighboring PU_N is not added into merging candidate list (Figure1). On the other hands, when merging candidate list for PU3 of PART_NxN is derived, if PU0 and PU1 have the identical motion parameters, the motion parameters of neighboring PU_N is checked whether it’s identical to that of PU2, and if it’s true, that neighboring PU_N is not added into merging candidate list.

Figure 1: Current avoiding process for merging candidate in HM4.0

To reduce the complexity of this process, proposed method decides the avoided neighboring PU by using only PartMode of the current PU. Figure 2 shows the avoid neighboring PU position according to PartMode of the current PU. In proposed method, for the case of PU1 of PART_2NxN, PART_2NxnU, PART_2NxnD, neighboring PU_B1, which has certainly the identical motion parameter to PU0, is not added into merging candidate list. For the case of PU1 of PART_Nx2N, PART_nLx2N and PART_nRx2N, neighboring PU_A1, which has certainly the identical motion parameter to PU0, is not added into merging candidate list. On the other hands, for the case of PU3 of PART_NxN, this avoiding decision is not applied. The flowchart of proposed method is shown in Figure 3.

Figure 2: Simplified avoiding process for merging candidate

Figure 3: flowchart of simplified avoiding process for merging candidate

## On AMVP spatial scaling candidate

In HM4.0, when AMVP candidate list is derived, it’s allowed to use scaling calculation for upper positions (B0, B1 and B2 in Figure4) to derive the spatial candidate, only if left positions (A0 and A1) were not available or the scaling calculation was not used to derive spatial candidate of left position. If a parallel derivation of the spatial candidates is assumed, this dependency would produce some waiting time to derive the spatial candidate of upper position (Figure 5).

Figure 4: Spatial motion vector neighbors

Figure 5: Derivation process of spatial MVP in AMVP

To remove the dependency, the parameter isScaledFlagLX, which indicates whether the scaling calculation is allowed to use for upper position, is set according to PredMode of left position. In detail, if the PU covering A0 position is available and PredMode of that PU is not MODE_INTRA, the parameter isScaledFlagLX is set to 1. Or, if the PU covering A1 position is available and PredMode of that PU is not MODE_INTRA, the parameter isScaledFlagLX is also set to 1.That is, only if both PUs are not available or both PredMode are MODE_INTRA, the parameter isScaledFlagLX is set to 0 and the scaling calculation is allowed to use for upper position.

Figure 6: Proposed derivation process of spatial MVP in AMVP

## On zero motion vector merging candidate

In HM4.0, zero motion vector merging candidate is added if the number elements of mergeCandList is less than maxNumCand. Figure 8 shows the flowchart of derivation process for zero motion vector merging candidate. In this process, before adding new zero motion vector merging candidate, it’s checked whether that new candidate has already been included in the mergeCandList. This checking process would cause some complexity issues.

Figure 7: Flowchart of derivation process for zero motion vector merging candidate

To reduce the number of duplicate checking process in HM4.0, proposed method removes it from the derivation process of zero vector merging candidate. And instead of that, the parameter zeroCnt, which indicates the maximum number of the duplicate checking, is removed, and all available zeroCand are added to the mergeCandList in order not to produce empty space in it. Figure 9 shows the flowchart of propose method.

Figure 8: Proposed derivation process for zero motion vector merging candidate

# Experimental results

A proposed method was implemented on HM4.0 software and its performance relative to the HM4.0 software was evaluated under the common test condition described in JCTVC-F900 [1]. Table 1 shows its RD performance and execution time of the software relative to HM4.0 in the case of applying only simplification1 method. In random access cases, proposed method shows no loss for high efficiency and for low complexity on average. On the other hand, in low delay case, proposed method loses 0.1% BD rate for high efficiency and no loss for low complexity on average. The difference in the encoding and decoding time is not obvious. Table 2 shows its RD performance and execution time of the software relative to HM4.0+inter4x4enable in the case of applying only simplification1 method. Proposed method shows negligible impact to coding efficiency for all test condition.

Table 1: Experimental result of simplification1

Table 2: Experimental result of simplification1 (anchor: inter_4x4 enable)

Table 2 shows its RD performance and execution time of the software relative to HM4.0 in the case of applying only simplification2 method. Proposed method shows negligible impact to coding efficiency for all test condition.

Table 3: Experimental result of simplification2

Table 4 shows its RD performance and execution time of the software relative to HM4.0 in the case of applying only simplification3 method. In random access cases, proposed method shows no loss for high efficiency and low complexity on average. On the other hand, in low delay case, proposed method saves 0.1% BD rate for high efficiency and low complexity on average. The difference in the encoding and decoding time is not obvious.

Table 4: Experimental result of simplification3

Table 5 shows its RD performance and execution time of the software relative to HM4.0 in the case of applying all simplification method. In random access cases, proposed method shows no loss for high efficiency and low complexity on average. On the other hand, in low delay case, proposed method saves 0.1% BD rate for low complexity on average. The difference in the encoding and decoding time is not obvious.

Table 5: Experimental result of combination

# Conclusion

In this contribution, three simplification methods on AMVP/Merge are proposed. An experimental result shows that proposed methods show no loss for all test condition with reducing complexity in HM4.0. Based on the complexity reduction without BD rate loss, we suggest the proposed method be adopted in the HM/WD of HEVC.

# References

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, Torino, July 2011.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).