| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G542 WG11 Number: m22106 |
| --- | --- |

| Title: | Non-CE9/Non-CE13: Simplification on AMVP/Merge |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Toshiyasu Sugio Takahiro Nishi 1006 Kadoma, Kadoma Osaka, Japan | Tel: Email: | +81-50-3587-2047 { sugio.toshiyasu, nishi.takahiro }@jp.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

In this contribution, three simplification methods on AMVP/Merge were proposed. 1) On avoiding merging candidate decision 2) On AMVP spatial scaling candidate 3) On zero motion vector merging candidate. Experimental results reportedly showed no loss on average relative to the HM4.0 with reducing complexity on AMVP/Merge.

# Proposed Method

This proposal includes the following three simplifications on AMVP/Merge.

- On avoiding merging candidate decision [simplification1]

- On AMVP spatial scaling candidate [simplification2]

- On zero motion vector merging candidate [simplification3]

## On avoiding merging candidate decision

In HM4.0, when merging candidate list for PU1 of PART_2NxN, PART_Nx2N, PART_2NxnU, PART_2NxnD, PART_nLx2N and PART_nRx2N is derived, the motion parameters of the neighboring PU_N (N=A1, B1, B0, A0, B2) is checked whether it’s identical to that of PU0, and if it’s true, that neighboring PU_N is not added into merging candidate list (Figure1). On the other hands, when merging candidate list for PU3 of PART_NxN is derived, if PU0 and PU1 have the identical motion parameters, the motion parameters of neighboring PU_N is checked whether it’s identical to that of PU2, and if it’s true, that neighboring PU_N is not added into merging candidate list.

Figure 1: Current avoiding process for merging candidate in HM4.0

To reduce the complexity of this process, proposed method decides the avoided neighboring PU by using only PartMode of the current PU. Figure 2 shows the avoid neighboring PU position according to PartMode of the current PU. In proposed method, for the case of PU1 of PART_2NxN, PART_2NxnU, PART_2NxnD, neighboring PU_B1, which has certainly the identical motion parameter to PU0, is not added into merging candidate list. For the case of PU1 of PART_Nx2N, PART_nLx2N and PART_nRx2N, neighboring PU_A1, which has certainly the identical motion parameter to PU0, is not added into merging candidate list. On the other hands, for the case of PU3 of PART_NxN, this avoiding decision is not applied. The flowchart of proposed method is shown in Figure 3.

Figure 2: Simplified avoiding process for merging candidate

Figure 3: flowchart of simplified avoiding process for merging candidate

## On AMVP spatial scaling candidate

In HM4.0, when AMVP candidate list is derived, it’s allowed to use scaling calculation for upper positions (B0, B1 and B2 in Figure4) to derive the spatial candidate, only if left positions (A0 and A1) were not available or the scaling calculation was not used to derive spatial candidate of left position. If a parallel derivation of the spatial candidates is assumed, this dependency would produce some waiting time to derive the spatial candidate of upper position (Figure 5).

Figure 4: Spatial motion vector neighbors

Figure 5: Derivation process of spatial MVP in AMVP

To remove the dependency, the parameter isScaledFlagLX, which indicates whether the scaling calculation is allowed to use for upper position, is set according to PredMode of left position. In detail, if the PU covering A0 position is available and PredMode of that PU is not MODE_INTRA, the parameter isScaledFlagLX is set to 1. Or, if the PU covering A1 position is available and PredMode of that PU is not MODE_INTRA, the parameter isScaledFlagLX is also set to 1.That is, only if both PUs are not available or both PredMode are MODE_INTRA, the parameter isScaledFlagLX is set to 0 and the scaling calculation is allowed to use for upper position.

Figure 6: Proposed derivation process of spatial MVP in AMVP

## On zero motion vector merging candidate

In HM4.0, zero motion vector merging candidate is added if the number elements of mergeCandList is less than maxNumCand. Figure 8 shows the flowchart of derivation process for zero motion vector merging candidate. In this process, before adding new zero motion vector merging candidate, it’s checked whether that new candidate has already been included in the mergeCandList. This checking process would cause some complexity issues.

Figure 7: Flowchart of derivation process for zero motion vector merging candidate

To reduce the number of duplicate checking process in HM4.0, proposed method removes it from the derivation process of zero vector merging candidate. And instead of that, the parameter zeroCnt, which indicates the maximum number of the duplicate checking, is removed, and all available zeroCand are added to the mergeCandList in order not to produce empty space in it. Figure 9 shows the flowchart of propose method.

Figure 8: Proposed derivation process for zero motion vector merging candidate

# Experimental results

A proposed method was implemented on HM4.0 software and its performance relative to the HM4.0 software was evaluated under the common test condition described in JCTVC-F900 [1]. Table 1 shows its RD performance and execution time of the software relative to HM4.0 in the case of applying only simplification1 method. In random access cases, proposed method shows no loss for high efficiency and for low complexity on average. On the other hand, in low delay case, proposed method loses 0.1% BD rate for high efficiency and no loss for low complexity on average. The difference in the encoding and decoding time is not obvious. Table 2 shows its RD performance and execution time of the software relative to HM4.0+inter4x4enable in the case of applying only simplification1 method. Proposed method shows negligible impact to coding efficiency for all test condition.

Table 1: Experimental result of simplification1

Table 2: Experimental result of simplification1 (anchor: inter_4x4 enable)

Table 2 shows its RD performance and execution time of the software relative to HM4.0 in the case of applying only simplification2 method. Proposed method shows negligible impact to coding efficiency for all test condition.

Table 3: Experimental result of simplification2

Table 4 shows its RD performance and execution time of the software relative to HM4.0 in the case of applying only simplification3 method. In random access cases, proposed method shows no loss for high efficiency and low complexity on average. On the other hand, in low delay case, proposed method saves 0.1% BD rate for high efficiency and low complexity on average. The difference in the encoding and decoding time is not obvious.

Table 4: Experimental result of simplification3

Table 5 shows its RD performance and execution time of the software relative to HM4.0 in the case of applying all simplification method. In random access cases, proposed method shows no loss for high efficiency and low complexity on average. On the other hand, in low delay case, proposed method saves 0.1% BD rate for low complexity on average. The difference in the encoding and decoding time is not obvious.

Table 5: Experimental result of combination

# Conclusion

In this contribution, three simplification methods on AMVP/Merge are proposed. An experimental result shows that proposed methods show no loss for all test condition with reducing complexity in HM4.0. Based on the complexity reduction without BD rate loss, we suggest the proposed method be adopted in the HM/WD of HEVC.

# References

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, Torino, July 2011.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix

## Proposed changes on Working Draft 4.0

## Slice decoding process

##### 8.4.2.1.2 Derivation process for spatial merging candidates

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are (with N being replaced by A0, A1, B0, B1 or B2 and with X being replaced by 0 or 1)

- the availability flags availableFlagN of the neighbouring prediction units,

- the reference indices refIdxLXN of the neighbouring prediction units,

- the prediction list utilization flags predFlagLXN of the neighbouring prediction units,

- the motion vectors mvLXN of the neighbouring prediction units.

For the derivation of availableFlagN, with N being A0, A1, B0, B1 or B2 and ( xN, yN ) being ( xP – 1,  yP + nPSH ), ( xP − 1,  yP + nPSH − 1 ), ( xP + nPSW,  yP – 1 ), ( xP + nPSW − 1,  yP – 1 ) or ( xP – 1,  yP – 1 ), the following applies.

–	If one of the following conditions is true, the availableFlagN is set equal to 0, both components mvLXN are set equal to 0, refIdxLXN and predFlagLX[ xN, yN ] of the prediction unit covering luma location ( xN, yN ) are assigned respectively to mvLXN, refIdxLXN and predFlagLXN.

  - N is equal to B2 and availableFlagA0 + availableFlagA1 + availableFlagB0 + availableFlagB1 is equal to 4.

  - The prediction unit covering luma location ( xN, yN ) is not available or PredMode is MODE_INTRA.

- PartMode of the current prediction unit is PART_2NxN or PART_2NxnU or PART_2NxnD and PartIdx is equal to 1 and N is equal to B1. the prediction units covering luma location ( xP,  yP – 1 ) ( PartIdx = 0 ) and luma location ( xN, yN ) (Cand. N) have identical motion parameters:

  - mvLX[ xP,  yP – 1 ]  = =  mvLX[ xN, yN ]

  - refIdxLX[ xP,  yP – 1 ]  = =  refIdxLX[ xN, yN ]

    - predFlagLX[ xP,  yP – 1 ]  = =  predFlagLX[ xN, yN ]

- PartMode of the current prediction unit is PART_Nx2N or PART_nLx2N or PART_nRx2N and PartIdx is equal to 1 and N is equal to A1. the prediction units covering luma location ( xP – 1,  yP ) ( PartIdx = 0 ) and luma location ( xN, yN ) (Cand. N) have identical motion parameters:

  - mvLX[ xP – 1,  yP ]  = =  mvLX[ xN, yN ]

  - refIdxLX[ xP – 1,  yP ]  = =  refIdxLX[ xN, yN ]

  - predFlagLX[ xP – 1,  yP ]  = =  predFlagLX[ xN, yN ]

- PartMode of the current prediction unit is PART_NxN and PartIdx is equal to 3 and the prediction units covering luma location ( xP – 1,  yP ) ( PartIdx = 2 ) and luma location ( xP – 1,  yP – 1 ) ( PartIdx = 0 )  have identical motion parameters:

  - mvLX[ xP – 1,  yP ]  = =  mvLX[ xP – 1,  yP – 1 ]

  - refIdxLX[ xP – 1,  yP ]  = =  refIdxLX[ xP – 1,  yP – 1 ]

  - predFlagLX[ xP – 1,  yP ]  = =  predFlagLX[ xP – 1,  yP – 1 ]

and the prediction units covering luma location ( xP,  yP – 1 ) ( PartIdx = 1 ) and luma location ( xN, yN ) (Cand. N) have identical motion parameters:

  - mvLX[ xP,  yP – 1 ]  = =  mvLX[ xN, yN ]

  - refIdxLX[ xP,  yP – 1 ]  = =  refIdxLX[ xN, yN ]

  - predFlagLX[ xP,  yP – 1 ]  = =  predFlagLX[ xN, yN ]

- PartMode of the current prediction unit is PART_NxN and PartIdx is equal to 3 and the prediction units covering luma location ( xP,  yP – 1 ) ( PartIdx = 1 ) and luma location ( xP – 1,  yP – 1 ) ( PartIdx = 0 ) have identical motion parameters:

  - mvLX[ xP,  yP – 1 ]  = =  mvLX[ xP – 1,  yP – 1 ]

  - refIdxLX[ xP,  yP – 1 ]  = =  refIdxLX[ xP – 1,  yP – 1 ]

  - predFlagLX[ xP,  yP – 1 ]  = =  predFlagLX[ xP – 1,  yP – 1 ]

and the prediction units covering luma location ( xP – 1,  yP ) ( PartIdx = 2 ) and luma location ( xN, yN ) (Cand. N) have identical motion parameters:

  - mvLX[ xP – 1,  yP ]  = =  mvLX[ xN, yN ]

  - refIdxLX[ xP – 1,  yP ]  = =  refIdxLX[ xN, yN ]

  - predFlagLX[ xP – 1,  yP ]  = =  predFlagLX[ xN, yN ]

–	Otherwise, availableFlagN is set equal to 1 and the variables mvLX[ xN, yN ], refIdxLX[ xN, yN ] and predFlagLX[ xN, yN ] of the prediction unit covering luma location ( xN, yN ) are assigned respectively to mvLXN, refIdxLXN and predFlagLXN.

##### 8.4.2.1.7 Derivation process for zero motion vector merging candidates

Inputs of this process are

- a merging candidate list mergeCandList,

- reference indices refIdxL0N and refIdxL1N of every candidate N being in mergeCandList,

- prediction list utilization flags predFlagL0N and predFlagL1N of every candidate N being in mergeCandList,

- motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList,

- the number of elements numMergeCand within mergeCandList,

- the maximum number of elements maxNumMergeCand within the mergeCandList.

Outputs of this process are

- the merging candidate list mergeCandList,

- the number of elements numMergeCand within mergeCandList.

- reference indices refIdxL0zeroCandm and refIdxL10zeroCandm of every new candidate zeroCandm being added in mergeCandList during the invokation of this process,

- prediction list utilization flags predFlagL0zeroCandm and predFlagL10zeroCandm of every new candidate zeroCandm being added in mergeCandList during the invokation of this process,

- motion vectors mvL0zeroCandm and mvL10zeroCandm of every new candidate zeroCandm being added in mergeCandList during the invokation of this process,

The variable numRefIdx is derived as follows.

- If slice_type is equal to P, numRefIdx is set to num_ref_idx_l0_active_minus1 + 1

- Otherwise (slice_type is equal to B), numRefIdx is set to min(num_ref_idx_l0_active_minus1 + 1, num_ref_idx_l1_active_minus1 + 1)

When numMergeCand is less than maxNumMergeCand, the variable numInputMergeCand is set to numMergeCand, the variables zeroIdx isand zeroCnt are set to 0, the variable zeroStop is set to false and the following steps are repeated until zeroStop is equal to true.

- For the derivation of the reference indices, the prediction list utilization flags and the motion vectors of the zero motion vector merging candidate, the following applies.

    - If all of the following conditions are true,

      - slice_type is equal to P

      - no candidate in mergeCandList has a list0 motion vector equal to (0,0) and reference index equal to zeroIdx or (zeroCnt is equal to 2 and zeroIdx is equal to 0),

the candidate zeroCandm with m equal to ( numMergeCand − numInputMergeCand ) is added at the end of mergeCandList ( mergeCandList[ numMergeCand ] = zeroCandm ) and the reference indices, the prediction list utilization flags and the motion vectors of zeroCandm are dervied as follows and numMergeCand is incremented by 1.

refIdxL0zeroCandm = zeroIdx	(8-1)

refIdxL1zeroCandm = −1	(8-2)

predFlagL0zeroCandm = 1	(8-3)

predFlagL1zeroCandm = 0	(8-4)

mvL0zeroCandm[ 0 ] = 0	(8-5)

mvL0zeroCandm[ 1 ] = 0	(8-6)

mvL1zeroCandm[ 0 ] = 0	(8-7)

mvL1zeroCandm[ 1 ] = 0	(8-8)

numMergeCand = numMergeCand + 1	(8-9)

    - Otherwise if all of the following conditions are true,

      - slice_type is equal to B

      - no candidate in mergeCandList has a list0 motion vector equal to (0,0), a list0 reference index equal to zeroIdx, a list1 motion vector equal to (0,0) and a list1 reference index equal to zeroIdx or (zeroCnt is equal to 2 and zeroIdx is equal to 0)

the candidate zeroCandm with m equal to ( numMergeCand − numInputMergeCand ) is added at the end of mergeCandList ( mergeCandList[ numMergeCand ] = zeroCandm ) and the reference indices, the prediction list utilization flags and the motion vectors of zeroCandm are dervied as follows and numMergeCand is incremented by 1.

refIdxL0zeroCandm = zeroIdx	(8-10)

refIdxL1zeroCandm = zeroIdx	(8-11)

predFlagL0zeroCandm = 1	(8-12)

predFlagL1zeroCandm = 1	(8-13)

mvL0zeroCandm[ 0 ] = 0	(8-14)

mvL0zeroCandm[ 1 ] = 0	(8-15)

mvL1zeroCandm[ 0 ] = 0	(8-16)

mvL1zeroCandm[ 1 ] = 0	(8-17)

numMergeCand = numMergeCand + 1	(8-18)

- The variables zeroCnt and zeroIdx isare modifed as follows.

    - If zeroIdx is equal to numRefIdx or zeroCnt is equal to 2, zeroIdx is set to 0.

    - Otherwise (zeroIdx is less than numRefIdxand zeroCnt is less than 2), zeroIdx isand zeroCnt are incremented by 1.

- When numMergeCand is equal to maxNumMergeCand, zeroStop is set to true.

##### 8.4.2.1.9 Derivation process for motion vector predictor candidates

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Outputs of this process are (with N being replaced by A, or B)

- the motion vectors mvLXN of the neighbouring prediction units,

- the availability flags availableFlagLXN of the neighbouring prediction units.

Figure 8-9 – Spatial motion vector neighbours

The function RefPicOrderCnt( pic, refidx, LX ) is specified by the value of PicOrderCnt of the picture that is the reference picture RefPicListX[ refidx ] of pic with X being 0 or 1. PicOrderCnt of the reference picture shall be maintained until the picture is marked as “non-existing”

[Ed. (BB): PicOrderCnt(.) needs to be defined. Currently the Decoding process for picture order count is not described]

The variable isScaledFlagLX with X being 0 or 1 is set equal to 0.

The motion vector mvLXA and the availability flag availableFlagLXA are derived in the following ordered steps:

- Let a set of two sample locations be (xAk, yAk), with k = 0, 1, specifies sample locations with xAk = xP – 1, yA0 = yP + nPSH and yA1 = yA0 - MinPuSize. The set of sample locations ( xAk, yAk ) represent the sample locations immediately to the left side of the left partition boundary and it’s extended line. [Ed.: (WJ) MinPuSize should be defined somewhere]

- Let the availability flag availableFlagLXA be initially set equal to 0 and the both components of mvLXA are set equal to 0.

- If one of the following conditions is true, the variable isScaledFlagLX is set equal to 1.

- the prediction unit covering luma location ( xA0, yA0 ) is available and PredMode is not MODE_INTRA.

- the prediction unit covering luma location ( xA1, yA1 ) is available and PredMode is not MODE_INTRA.

- For ( xAk, yAk ) from ( xA0, yA0 ) to ( xA1, yA1 ) where yA1 = yA0 - MinPuSize, the following applies repeatedly until availableFlagLXA is equal to 1:

- If the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1 and the reference index refIdxLX[ xAk ][ yAk ] is equal to the reference index of the current prediction unit refIdxLX, availableFlagLXA is set equal to 1 and the motion vector mvLXA is set equal to the motion vector mvLX[ xAk ][ yAk ], refIdxA is set equal to refIdxLX[ xAk ][ yAk ] and ListA is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1 and RefPicOrderCnt( currPic, refIdxLY[ xAk ][ yAk ], LY) is equal to RefPicOrderCnt( currPic, refIdxLX, LX), availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLY[ xAk ][ yAk ], refIdxA is set equal to refIdxLY[ xAk ][ yAk ] ,  ListA is set equal to LY and mvLXA is set equal to mvLXA.

- When availableFlagLXA is equal to 0, for ( xAk, yAk ) from ( xA0, yA0 ) to ( xA1, yA1 ) where yA1 = yA0 - MinPuSize, the following applies repeatedly until availableFlagLXA is equal to 1:

- If the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1, availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLX[ xAk ][ yAk ], refIdxA is set equal to refIdxLX[ xAk ][ yAk ], ListA is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1, availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLY[ xAk ][ yAk ], refIdxA is set equal to refIdxLY[ xAk ][ yAk ], ListA is set equal to LY.

- When availableFlagLXA is equal to 1, isScaledFlagLX is set equal to 1 and mvLXA is derived as specified below.

tx = ( 16384 + Abs( td / 2 ) ) / td	(8-19)

DistScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 )	(8-20)

mvLXA = ClipMv( Sign( DistScaleFactor * mvLXA ) * 			
			( (Abs( DistScaleFactor * mvLXA ) + 127 ) >> 8 ) )	(8-21)

[Ed. (WJ): software has a clip function for scaled motion vector. Do we need this? AVC does not have it]

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic , refIdxA, ListA ) )	(8-22)

tb = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxLX, LX ) )	(8-23)

The motion vector mvLXB and the availability flag availableFlagLXB are derived in the following ordered steps:

- Let a set of three sample location (xBk, yBk), with k = 0,1,2, specifies sample locations with xB0 = xP + nPSW, xB1 = xB0 MinPuSize , xB2 = xP - MinPuSize and yBk = yP – 1. The set of sample locations ( xBk, yBk ) represent the sample locations immediately to the upper side of the above partition boundary and it’s extended line. [Ed.: (WJ) MinPuSize should be defined somewhere]

- Let the availability flag availableFlagLXB be initially set equal to 0 and the both components of mvLXB are set equal to 0.

- For ( xBk, yBk ) from ( xB0, yB0 ) to ( xB2, yB2 ) where xB0 = xP +nPSW, xB1 = xB0 - MinPuSize , and xB2 =  xP - MinPuSize, the following applies repeatedly until availableFlagLXB is equal to 1:

- If the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1, and the reference index refIdxLX[ xBk ][ yBk ] is equal to the reference index of the current prediction unit refIdxLX, availableFlagLXB is set equal to 1 and the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ], refIdxB is set equal to refIdxLX[ xBk ][ yBk ] and ListB is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xBk ][ yBk ] (with Y = !X) is equal to 1 and RefPicOrderCnt( currPic , refIdxLY[ xBk ][ yBk ], LY) is equal to RefPicOrderCnt( currPic, refIdxLX, LX ),  availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLY[ xBk ][ yBk ],  refIdxB is set equal to refIdxLY[ xBk ][ yBk ],  and ListB is set equal to LY.

- When availableFlagLXA or availableFlagLXB is equal to 0 and isScaledFlagLX is equal to 0, availableFlagLXB is set equal to 0 and for ( xBk, yBk ) from ( xB0, yB0 ) to ( xB2, yB2 ) where xB0 = xP +nPSW, xB1 = xB0 - MinPuSize , and xB2 =  xP - MinPuSize, the following applies repeatedly until availableFlagLXB is equal to 1:

- If the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1, availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ], refIdxB is set equal to refIdxLX[ xBk ][ yBk ], ListB is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xBk ][ yBk ] (with Y = !X) is equal to 1, availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLY[ xBk ][ yBk ], refIdxB is set equal to refIdxLY[ xBk ][ yBk ], ListB is set equal to LY.

- When availableFlagLXB is equal to 1 and RefPicOrderCnt( currPic , refIdxB, ListB ) is not equal to RefPicOrderCnt( currPic, refIdxLX, LX ),, mvLXB is derived as specified below.

tx = ( 16384 + Abs( td / 2 ) ) / td	(8-24)

DistScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 )	(8-25)

mvLXB = ClipMv( Sign( DistScaleFactor * mvLXA ) * 			
			( (Abs( DistScaleFactor * mvLXA ) + 127 ) >> 8 ) ) 	(8-26)

[Ed. (WJ): software has a clip function for scaled motion vector. Do we need this? AVC does not have it]

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic , refIdxB, ListB ) )	(8-27)

tb = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxLX, LX ) )	(8-28)