| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G546 |
| --- | --- |

| Title: | On high-level syntax for maximum DPB size and frame latency |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Youngo Park Chanyul Kim Kwang Pyo Choi JeongHoon Park | Email: | youngo.park@samsung.com dionism@samsung.com kp5.choi@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

The JCTVC-E339 proposed to move max_dec_frame_buffering and num_reorder_frames from the optional VUI to mandatory SPS. The JCTVC-F541 proposed to add max_latency_frames_plus1 or max_latency_increase_plus1. We propose move max_dec_frame_buffering and add max_latency_frames_plus1 in SPS. We propose the num_reorder_frames to be left in VUI without change.

# Introduction

## Regarding MaxDpbSize

The MaxDpbSize specifies the required size of HDR decoded picture buffer (DPB) and it is determined as below:

If (max_dec_frame_buffering)

MaxDpbSize = max_dec_frame_buffering

else

MaxDpbSize = Min(1024*MaxDPB / (PicWidthInMbs*FrameHeightInMbs*384),16)

Since the max_dec_frame_buffering is defined optional VUI, sometimes it is absent. The level will decide the MaxDpbSize for current bitstream if VUI is not sent. The Figure 1 is the example when the MaxDpbSize is fixed by level value. Although the display resolution is WQVGA (400x240p) at 30Hz, the MaxDpbSize should be larger than 13 frames because minimum level is equal to 1.3.

Figure 1 The example of MaxDpbSize when is determined by level

When decoder needs only two reference frames and VUI is not sent, decoder should allocate minimum 13 frames. This situation can cause a lot of unnecessary memory use. Figure 2 is popular coding structure used in AVC which uses one or two MaxDpbSize.

Figure 2 The example when MaxDpbSize is one or two

Moreover, when a device exchanges the decoder capability negotiation checking whether it can play the bitstream or not with server, the negotiation could be failed even the decoder inside a device can play the bitstream. Figure 3 describes decoder capability negotiation scenario.

Figure 3 The example when device negotiates its capability with server

## Regarding MaxLatencyFrames

MaxLatencyFrames is required for decoder to display the decoded frame in time.

In Figure 4, if MaxLatencyFrames is not defined and MaxDpbSize is equal to 4, decoder cannot know when the decoded frame is displayed out until bumping process is happened.

However as in Figure 5, if MaxLatencyFrames is set to decoder can display out the decoded frame without waiting for bumping process.

Figure 4 The example when MaxDpbSize is set to 4 and MaxLatencyFrames is not defined

Figure 4 The example when MaxDpbSize is set to 4 and MaxLatencyFrames is set to 0

# Proposal

- Move max_dec_frame_buffering to SPS for mandatory

- Add max_latency_frame_plus1 to SPS for mandatory

- Leave num_reorder_frames to VUI as same as current WD

It is though that num_reorder_frames only affects the size of DPB. So if max_dec_frame_buffering can be signaled mandatory, num_reorder_frames needs not to be mandatory

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).