| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G547 |
| --- | --- |

| Title: | Non-CE1:Counter-based probability model update with adapted arithmetic coding engine |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jan Stegemann Heiner Kirchhoffer Detlev Marpe Thomas Wiegand Fraunhofer HHI Einsteinufer 37 D-10587 Berlin | Tel: Email: | +49 30 31002 183 jan.stegemann@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution combines the counter based probability model update algorithm of JCTVC-F254[1] with an adapted version of the M Coder of HEVC. The proposed method is closely related to the Multi-Parameter probability update proposal[1] but features a concise design of the arithmetic coding engine and retains the R-D performance improvements of employing sophisticated probability estimation. Gains in coding efficiency amount from 0.6 to 0.9%.

# Problem statement and analysis of the state of the art

Probability model update is a key feature of efficient arithmetic entropy coders by offering the possibility to adapt the internal state of the coder to the underlying source statistics. Such adaptivitiy capacitates high levels of compression efficiency and is utilized in schemes like the M Coder and PIPE. In the current working draft of HEVC probability model update is done using LPS-/MPS-transition tables as originally specified in [7]. Superpositions of counter based probability estimators according to the virtual sliding window approach[3][4] can achieve higher degrees of coding efficiency by combining different window sizes. Implementations of such an approach are studdied in [1][5]. For counter based probability estimation every probability model is equipped with a counter variable which is used to keep track of the local statistics of the bin sequence that is assigned to the probability model. After the encoding or decoding of a bin the count is updated and provides a probability for encoding or decoding the next bin. The update is done after every bin, thus a tradeoff between computational complexity, memory requirements and accurate probability estimation is targeted. In this proposal a high resolution counter based probability update - almost identical to [1] - and its usage with a concisely designed arithmetic coding engine is presented.

# Counter based probability model update

The first core element of the proposed idea is replacing the M Coders MPS-/LPS-Transition tables with a counter based probability estimator as it is done in [1]. The estimator is based on two counter variables that keep track of the source statistics of the underlying bin sequence. Each counter is fitted with a window size w=2n that determines to which extent the past number of bins influences the count. After a bin with the value xt is decoded or encoded the update of the counts at and bt is done according to Equation (1) with q = a, b:

qt+1=qt+((215−qt)>>nq),        if xt=1			(1)

This corresponds to the probability model update as proposed in [1]. However, it requires a distinction whether xt equals 0 or 1 which is not a problem if the used arithmetic coding engine already has separate code execution branches for xt equal 0 and 1. In the case of the M coder, there are separate code execution branches for MPS and LPS instead. For this case, the implementation of the virtual sliding window according to Equation (2) may be more suitable.

qt+1=qt+(xt<<(15−nq))−(qt>>nq),			(2)

To prevent overflow in Equation (2) a further clipping operation is needed for one of the two counters:

at=at+(at>>15),						(3)

In Equations (1) and (2) BITSresnqnb denotes the window size. These two ways of implementing the virtual sliding window approach do not deliver identical bit streams due to the right-shift operation. In the proposed design the BITSres=15two counters use the identical window sizes as in [1](wa=2na=16 and wb=2nb=256).

## Weighting counts of two window sizes

The counts at and bt presented in the previous section are used to derive the probability for encoding or decoding the next bin. They are weighted as follows:

c~t=(at+bt)>>1.					(4)

c~tis a scaled value that indicates the probability for the next symbol with the value xt=1 which is used for the interval subdivision in the arithmetic coding engine. However there is one exception which defines the alternative configuration of this proposal and is explained in the following section.

## Small window size for first 50 symbols

Based on the analysis of probability model update in [1] and [2] the estimation of probabilities in the beginning of a sequence can achieve an improved RD performance if the count with the smaller window size at is used for the first 50 bins of each probability model. In case the predefined initialization value does not match the statistics of the underlying bin sequence the smaller windows size will enable a faster adaption. After the first 50 bins, the probability is derived as shown in equation (4). The effects on the RD performance and runtime are compared and presented in section 4.

# Adjustments to the arithmetic coding engine

The second core element of this proposal is the adjustments to the arithmetic coding engine to allow a seamless operation of the counter based probability model update. In contrast to [1] the layout of the coding engine is designed in the same dimensions as the M Coder in the current version of HEVC. Furthermore the concept of the distinction between the MPS and LPS remains in the coding engine.

## Uniform quantization for probability interval subdivision

As mentioned in the previous chapter the value c~t is derived from the scaled counts that operate in the interval (0, 32768). Hence the same interval has to be mapped to the probabilities that are actually used by the coding engine. In the M Coder these probabilities are found in the 64x4 table of the rangeTabLPS as pre calculated values of the interval subdivision. They are quantized in a way that probabilities close to equal probability are represented in coarser steps and probabilities close to the minimum LPS probability are represented in a higher accuracy. Though, the counts of the counter based probability estimator are proportional to the associated probability values. As a consequence the rangeTabLPS table values of the coding engine have been replaced by values that are pre calculated based on probability values proportional to pStateIdx, i.e. the table row index. Thus, an index to access the rows of the rangeTabLPS table can easily be computed. Equations (5) show this computation for a table with 32 rows using complement-2 arithmetic.

valMPS=ct>>14pStateIdx=((ct−214)XOR(valMPS−1))>>9			(5)

The probabilities used to compute the rangeTabLPS table values start with the minimum probability pmin=0.0078125 for the leftmost interval (pStateIdx=31, valMPS=0). For each further interval, pminΔp=0.015625 is added. This corresponds to a uniform quantization without offset and the center of the intervals used as representative probability values (for the probability interval (0.0, 0.5) being quantized to 32 sub-intervals). Furthermore the table containing the entries is dimensioned with 32x8 elements and is thus equally sized as the M Coder’s 64x4 table. Computation of qCodIRangeIdx, i.e. the column index is the same as in the current working draft except for the operands:

qCodIRangeIdx =( codIRange  >>  5 ) & 7				(5)

# Results

The binaries were compiled on a 64 bit Ubuntu 10.10 Linux using the included make files and gcc 4.4.5. The coding conditions used for the simulations are specified in [6]. This tested proposal affects the arithmetic coding engine only and all low-complexity configurations are skipped. The design was implemented in the software of CE1 Subtest C Test 17[5], which uses table based bitrate estimation during mode decision in the encoder. This also includes the use of the context model initialization values as present in [1].

## Hardware

The type of computer that has been used to run the simulations  all running a 64 bit Ubuntu 10.10 Linux operating system are listed in Table 4-1.

Table 4-1	 Computers used to run the simulations.

| Type | Purpose | Cores | CPU | RAM |
| --- | --- | --- | --- | --- |
| 1 | Encoding and Decoding | 2x4 | 2 x 4 x Intel(R) Xeon(R) CPU X5570 @ 2.93GHz | 32 GB |

## Results

The results show the performance of the default configuration using Equation (1) on the one hand and with the alternative configuration as explained in 2.2 on the other hand.

### Results of the default configuration of the proposal

Table 2 Results of the proposed design with default configuration

### Results of the alternative configuration

In this section the results of the alternative configuration according to section 2.2 are presented.

Table 3 Results of the proposed design with the alternative configuration

# Conclusion

In this document an alternative probability model update in combination with a clearly laid out coding engine is presented. The original M Coder has been fitted to the multiplication free probability model update but retaining its original dimensions. Improvements in coding efficiency average 0.6-0.9% compared to HM 4.0. The alternative configuration outperforms the default configuration especially for lower bitrates with a slight increase in encoding and decoding runtimes.

# References

- A. Alshin, E. Alshina, “Multi-parameter probability update for CABAC”, JCTVC-F254, Torino, July, 2011.

- J. Stegemann, “CE1: Cross-check of Samsung’s Multi-parameter probability model up-date for CABAC”, JCTVC-G553, Geneva, November, 2011.

- E.Meron, M. Feder, “Finite-memory universal prediction of individual sequences”, IEEE Transactions on Information Theory, Vol. 50, No. 7, pp. 1506-1523, July 2004.

- E. Belyaev, M. Gilmutdinov, A. Turlikov, “Binary Arithmetic Coding System with Adaptive Probability Estimation by "Virtual Sliding Window"”, IEEE Tenth International Symposium on Consumer Electronics, ISCE 2006.

- Rajan Joshi, Elena Alshina, Hisao Sasai, Heiner Kirchhoffer, Jani Leinema, “Description of Core Experiment 1: Entropy Coding”, JCTVC-F901, Torino, July, 2011.

- JCT-VC, “HEVC Test Model Software 4.0”, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-4.0

- D. Marpe, H. Schwarz, and T. Wiegand: Context-Based Adaptive Binary Arithmetic Coding in the H.264 / AVC Video Compression Standard, IEEE Transactions on Circuits and Systems for Video Technology, Vol. 13, No. 7, pp. 620-636, July 2003.

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, Torino, July, 2011.

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e. V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).