| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G549 |
| --- | --- |

| Title: | Syntax rearrangement for list combination |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Youngo Park Seungsoo Jeong Chanyul Kim Kwang Pyo Choi JeongHoon Park | Email: | youngo.park@samsung.com ss00.jeong@samsung.com dionism@samsung.com kp5.choi@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

The list combination (LC) is proposed for uni-prediction at B-slices to improve coding efficiency. However current syntax for list combination has redundancy so we propose to remove the redundancy and rearrange the syntax.

# Background

The list combination is always used for reference list of uni-prediction in B-slices. The number of LC is determined by the number of List0 and List1. The range of number of LC is from 0 to the number of List0 multiplies the number of List1. Figure 1 describes POC, List0, List1 and LC.

Figure 1 The example of POC, List0, List1 and LC

The current WD (JCTVC-F803) describes the syntax for list combination in slice header as below Figure 2.

Figure 2 The syntax for list combination in slice header

The semantics for ref_pic_list_combination_flag is as below:

# Proposal

The ref_pic_list_combination_flag is equal to 0 when List0 and List1 are identical and it is equal to 1 when List0 and List1 are different. If ref_pic_list_combination_flag is signaled as 0, List0 will be used as LC. If ref_pic_list_combination_flag is equal to 1, LC will be constructed by the status of List0 and List1. However the status of List0 and List1 whether they are identical or not can be inferred in the decoder automatically without signaling. So it looks like this syntax is a kind of redundancy.

Furthermore if encoder wants to restrict the number of LC to specific number, the number of LC should be signaled in every slice header explicitly. By signaling num_ref_idx_lc_default_active in PPS like as nu m_ref_idx_lx_default_active_minus1, the default number of LC also could be sent. If num_ref_idx_lc_default_active is equal to 0, it means the number of LC shall be generated based on the status List0 and List1 automatically. The Figure 2 describes the example when num_ref_idx_lc_defualt_active is equal to 0 and 2.

Figure 3 The LC status when num_ref_idx_lc_default_active is equal to 0 and 2

The num_ref_idx_lc_active_minus1 syntax in ref_pic_list_combination() can be move to slice header.

The LC reordering syntax can be merged to ref_pic_list_modification().

# Syntax

# Experimental Results

# Conclusions

- Remove ref_pic_list_combination_flag syntax

- Move num_ref_idx_lc_active_minus1 syntax to slice_header()

- Merge LC reordering syntax to ref_pic_lisc_modification()

- Define num_ref_idx_lc_default_active in PPS

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).