| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G550 |
| --- | --- |

| Title: | TU size based dQP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chanyul Kim Youngo Park Kwangpyo Choi JungHun Park | Tel: Email: | dionism@samsung.com youngo.park@samsung.com kp5.choi@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In this proposal, implicit signaling of Delta-QP (dQP) based on transform size is presented. The current HM do not consider QP variation on TU level but CU level. This scheme apply for inter coding case only. 0.0%/0.2%/0.1% for RA_HE, -0.1%/0.1%/0.0% for RA_LC, -0.3%/0.4%/0.4% for LD_HE

and -0.5%/0.4%/0.4% for LD_LC coding gain is obtained without additional complexity.

# Introduction

dQP signaling by the CU level is a method for rate control which is adopted in the current HM. Lack of rate control algorithm implementation on the HM leads to make question “Is it enough for rate control only dQP in CU level?”. Based on knowledge of rate control algorithm, the static areas have more possibility to be reference area rather than moving region. The current HM uses different size of CU and TU. Therefore, CU based dQP is only not sufficient method for those targets, especially for large CU. We does not hurt the current dQP on CU level, but extend it to TU level without additional information transmitting to the decoder side.

We proposed TU size based implicit signal which does not require any complexity and additional bits for signaling.

# Algorithm description

The idea of the proposed algorithm is as follows,

- The area which uses the large transform has more possibility to be static region.

- The low QP is apply for large transform blocks, on the contrary, relatively high QP is apply for small transform block size.

- The static region has more probability to be reference frame than moving area that is why the QP decreases in static region.

Moreover, the proposed method is only applied for inter frame (RA or LD cases) because static region or moving region terms are only used in the inter picture.

The example of TU size based dQP is depicted in Figure 1. Assume that CU size is 64, and CU QP is QP_CU, the partitioned transform blocks are applied for different QP with QP_CU. Table 1 shows the implicit dQP based on TU size.

Table 1. dQP relative alloction TU size

|  | 4x4 | 8x8 | 16x16 | 32x32 |
| --- | --- | --- | --- | --- |
| Implicit dQP | 2x∆ | ∆ | 0 | -∆ |

Figure 1. Example of TU size based dQP implicit signaling for the case of 64x64 CU size

# Simulation Results

Using HM4.0, simulation is performed under the common test condition [1]. We tested various ∆, the performance does not much difference, so the test result is shown when ∆ = 1. The bit difference ratio between HM4.0 bit-stream and bit-stream generated by the proposed method is less than 3% for all configuration as shown in Table 3. The average absolute bit difference is

Bit_avg_diff  =  Avg(100*|bit_HM4.0-bit_proposed|/bit_HM4.0) for all sequences and QP.

Table 2. Result of TU size based dQP when ∆ =1

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 1.5% | 1.3% | 0.2% | 1.6% | 1.5% |
| Class B | 0.1% | 0.5% | 0.2% | 0.1% | 0.4% | 0.2% |
| Class C | -0.2% | -0.4% | -0.3% | -0.4% | -0.6% | -0.8% |
| Class D | -0.1% | -0.8% | -1.0% | -0.3% | -0.9% | -0.9% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.2% | 0.1% | -0.1% | 0.1% | 0.0% |
|  | 0.1% | 0.4% | 0.2% | -0.2% | 0.2% | 0.1% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.9% | 1.2% | -0.1% | 1.2% | 1.5% |
| Class C | -0.5% | -0.3% | -0.2% | -0.9% | -0.2% | -0.5% |
| Class D | -0.5% | -0.1% | -0.6% | -0.8% | 0.4% | -0.1% |
| Class E | -0.1% | 1.4% | 1.2% | -0.2% | -0.4% | -0.3% |
| Overall | -0.3% | 0.4% | 0.4% | -0.5% | 0.4% | 0.3% |
|  | -0.3% | 0.4% | 0.4% | -0.5% | 0.5% | 0.5% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

Table 3. Average absolute bit difference between HM4.0 and the proposed one

|  | RA_HE | RA_LC | LD_HE | LD_LC |
| --- | --- | --- | --- | --- |
| Bit_avg_diff (%) | 2.3 | 2.6 | 2.2 | 2.8 |

# Conclusion

The implicit dQP signaling based on TU size is presented. We consider the large transform block as static region in the inter frame. It gives high coding gain especially in LD configuration. Moreover, no additional complexity is observed for all configurations.

# References

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-C500, JCT-VC 3rd Meeting, Guangzhou, October 2010.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex : WD Text change

8.5.1. Scaling and transformation process

Inputs to this process are:

–	a luma location ( xT, yT ) specifying the top-left luma sample of the current transform unit relative to the top-left luma sample of the current picture,

–	a variable trafoDepth specifying the hierarchy depth of the current block relative to the coding unit,

–	a variable cIdx specifying the chroma component of the current block,

–	a variable nW specifying the width of the current transform block.

–	a variable nH specifying the height of the current transform block.

Output of this process is:

–	a modified version of the (nW)x(nH) array of residual samples r with elements rij.

The quantization parameter qP is derived as follows.

–  If slice_type = = I

If cIdx is equal to 0,

qP = QP’Y			(8-1)

Otherwise,

qP = QP’C			(8-2)

–  Otherwise,

If cIdx is equal to 0,

qP = QP’Y + offset_tbl[log2TrafoSize-2]

Otherwise,

qP = QP’C + offset_tbl[log2TrafoSize -2]

Table – Specification of offset_tbl[log2TrafoSize -2]

| Log2TrafoSize -2 | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| offset_tbl | 2 | 1 | 0 | -1 |