| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G552 |
| --- | --- |

| Title: | Simplification of temporal motion vector (TMVP) candidate derivation for Merge and AMVP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Il-Koo Kim Nikolay Shlyakhov JeongHoon Park | Email: | ilkoo.kim@samsung.com n.shlyakhov@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In this contribution, two simplification methods are proposed to simplify TMVP derivation process of Merge and AMVP. The first item is utilization of upper-left position instead of center position. The second one is utilization of zero reference index instead of the value which has majority of occurrence among neighbours. By utilizing upper-left position, no additional calculation is required to get the specific position since it is easy to find upper-left position which is starting point of PU. In addition, utilizing zero reference index for Merge TMVP candidate saves the steps for checking and calculating the majority of occurrence. Checking which one is majority is complex process which involves several comparisons and conditional branches. When the both simplification is combined, the average loss is 0.0% with 100% and 99% encoding and decoding time, respectively, compared to HM-4.0. Based on these experimental results, it is recommended to adopt these two simplifications on Merge and AMVP.

# Introduction

In HM-4.0, two separate positions are used to get a temporal motion vector predictor (TMVP) candidate both for Merge and AMVP. To derive a TMVP candidate, right-bottom (RB) position is examined first. If Prediction Unit (PU) of RB position is available, motion information of the PU is used to get TMVP candidate. When the PU of RB position is not available, center position is examined next for TMVP candidate selection. Offset to get the center position is calculated as follows.

ruiPartIdxCenter = g_auiRasterToZscan[ g_auiZscanToRaster[ ruiPartIdxCenter ]

+ ( iPartHeight/m_pcPic->getMinCUHeight() -1 )/2*m_pcPic->getNumPartInWidth()

+ ( iPartWidth/m_pcPic->getMinCUWidth()  -1 )/2];

Te get the offset, the calculation requires 2 divisions, 1 multiplication, 2 additions and 2 subtractions. In addition, mappings should be done two times with two separate memory arrays, g_auiRasterToZscan and g_auiZscanToRaster.

Figure 1 Positions used for derivation of reference index of TMVP candidate

Besides, in order to get reference index of TMVP candidate of Merge, statistics about reference index of neighbouring PU is needed. The reference index of TMVP is set as the value which has majority occurrence among neighbouring PU A, B and C (Figure 1). If PU C is not available, PU D or E is used. For example, when 2 positions (A and B) have 0 as reference index and the other (C) has 1 as reference index among 3 candidate positions, reference index of TMVP is determined to 1. It is noted that the positions used for reference index derivations are different from the positions used for other prediction candidates of Merge and AMVP. Thus, separate and additional process is required to do this reference index selection process.

# Simplification

Two simplification methods are proposed in this contribution to simplify TMVP derivation process of Merge and AMVP. The first item is utilization of upper-left position instead of center position. The second one is utilization of zero reference index instead of the value which has majority of occurrence among neighbours. By utilizing upper-left position, no additional calculation is required to get the specific position since it is easy to find upper-left position which is starting point of PU (staring position is always specified in WD and HM). In addition, utilizing zero reference index for Merge TMVP candidate saves the steps for checking and calculating the majority of occurrence. Checking which one is majority is complex process which involves several comparisons and conditional branches.

# Implementation

In order to investigate the impact of the proposed simplifications, HM-4.0 reference software is modified as follows.

1) Utilization of upper-left position instead of center position for Merge and AMVP. TCTR_AMVP_MRG_OFF macro is added in HM-4.0. By setting TCTR_AMVP_MRG_OFF 1, zero offset is returned instead of the offset for center position.

Void TComDataCU::xDeriveCenterIdx( PartSize eCUMode, UInt uiPartIdx, UInt& ruiPartIdxCenter )

{

UInt uiPartAddr;

Int  iPartWidth;

Int  iPartHeight;

getPartIndexAndSize( uiPartIdx, uiPartAddr, iPartWidth, iPartHeight);

ruiPartIdxCenter = m_uiAbsIdxInLCU+uiPartAddr; // partition origin.

#if !TCTR_AMVP_MRG_OFF

ruiPartIdxCenter = g_auiRasterToZscan[ g_auiZscanToRaster[ ruiPartIdxCenter ]

+ ( iPartHeight/m_pcPic->getMinCUHeight() -1 )/2*m_pcPic->getNumPartInWidth()

+ ( iPartWidth/m_pcPic->getMinCUWidth()  -1 )/2];

#endif

}

2) Utilization of zero reference index for TMVP candidate of Merge: MRG_TMVP_REFIDX macro in HM-4.0 is set 0.

# Experimental results

Three experiments were conducted under common test condition [1]: 1) upper-left position for TMVP of Merge and AMVP, 2) zero reference index for TMVP of Merge, 3) combination of 1) and 2). Brief summarization is given by Table 1. No loss is observed in all three experiments. Encoding time is same with HM-4.0 and decoding time fluctuates within the range of +-1%.

Table 1 Summarized results of three experiments

|  | Y BD-rate | Encoding time | Decoding time |
| --- | --- | --- | --- |
| Upper-left position for TMVP of Merge and AMVP | 0.0% | 100% | 101% |
| Zero reference index for TMVP of Merge | 0.0% | 100% | 101% |
| Combination of upper-left position and zero reference index for TMVP | 0.0% | 100% | 99% |

Table 2 summarizes the results when upper-left position is used instead of center position as one of TMVP candidate position. Average loss is 0.0% with 100% and 101% encoding and decoding time, respectively, compared to HM-4.0 anchor.

Table 2 Experimental result when upper-left position is used instead of center position as one of TMVP candidate position

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.2% |
| Class C | 0.0% | -0.1% | -0.1% | 0.0% | 0.2% | -0.2% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class E | 0.0% | 0.4% | 0.5% | 0.1% | -0.1% | 0.3% |
| Overall | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

Table 3 summarizes the results when reference index of TMVP of Merge set by zero. Average loss is 0.0% with 100% and 101% encoding and decoding time, respectively, compared to HM-4.0 anchor.

Table 3 Experimental result when reference index of TMVP of Merge set by zero

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class C | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.3% | 0.6% | 0.1% | 0.7% | 0.8% |
| Class C | 0.0% | 0.1% | 0.2% | 0.0% | 0.4% | 0.3% |
| Class D | 0.3% | 0.8% | 0.4% | 0.2% | 0.8% | 0.3% |
| Class E | 0.0% | 0.1% | 0.5% | 0.1% | 1.0% | 0.6% |
| Overall | 0.1% | 0.3% | 0.4% | 0.1% | 0.7% | 0.5% |
|  | 0.1% | 0.3% | 0.4% | 0.1% | 0.7% | 0.6% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 101% |  |  |

Table 4 summarizes the results when reference index of TMVP of Merge set by zero and upper-left position is used instead of center position as one of TMVP candidate. Average loss is 0.0% with 100% and 99% encoding and decoding time, respectively, compared to HM-4.0 anchor.

Table 4 Experimental result when reference index of TMVP of Merge set by zero and upper-left position is used instead of center position as one of TMVP candidate

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class D | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.2% | 0.4% | 0.1% | 0.6% | 1.0% |
| Class C | 0.0% | 0.1% | 0.0% | 0.0% | 0.3% | 0.1% |
| Class D | 0.2% | 0.7% | 0.6% | 0.3% | 0.5% | 0.8% |
| Class E | 0.0% | 0.0% | 0.6% | 0.0% | 0.9% | 0.9% |
| Overall | 0.1% | 0.3% | 0.4% | 0.1% | 0.6% | 0.7% |
|  | 0.1% | 0.2% | 0.3% | 0.1% | 0.6% | 0.7% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 96% |  |  |

# Conclusions

It is proposed to simplify the derivation process of TMVP candidate for Merge and AMVP. The simplification consists of two parts: 1) upper-left position is used instead of center position as one of TMVP candidate position, 2) reference index of TMVP of Merge set by zero. When the both simplification is combined, the average loss is 0.0% with 100% and 99% encoding and decoding time, respectively. Based on these experimental results, it is recommended to adopt these two simplifications on Merge and AMVP.

# Suggested Working Draft (WD) modification

Suggested WD text is enclosed with contribution document. Modified texts are mark as green color.

# References

- F. Bossen, “Common test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-F900, Jul. 2011.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).