| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G553 |
| --- | --- |

| Title: | CE1: Cross-check of Samsung’s Multi-parameter probability model up-date for CABAC ( JCTVC-F254 ) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Core Experiment Result Report |  |  |
| Author(s) or Contact(s): | Jan Stegemann Fraunhofer HHI Einsteinufer 37 D-10587 Berlin | Tel: Email: | +49 30 31002 183 jan.stegemann@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document reports the subtest B results of Samsung’s Multi-parameter probability model up-date within core experiment 1.In this subtest improvements of the probability model update of the arithmetic coding engine are explored. Samsung’s proposal suggests the replacement of the LPS-/MPS-transition tables of the M Coder by using counter-based probability estimation. Furthermore the resolution of the arithmetic coding engine is greatly increased and its probability interval is extended superseding the concept of LPS/ MPS of the M Coder. The proposed code has been checked, compiled and the results reported by Samsung can be confirmed.

# Tested Proposal

The tested proposal has been integrated into HEVC HM4.0 [2] and is activated by defining the macro as follows:

#define MULTI_PAR_CABAC_PROB_UP_DATE          1

Activating this macro replaces the probability model update and the arithmetic coding engine of the M Coder as specified in [3].

## Probability model update

Samsung proposal offers two modes of operation for updating the probability model. Both modes are based on symbol counters with a finite memory. After a bin with a value xt is decoded the counter ct+1 of the assigned probability model is updated as specified in the following formula:

ct+1 = ct - ctw+xtw.						(1)

Here wdenotes the windows size of the counter. The choice of an adequate window size is a tradeoff between the desire for fast adaptivity to the underlying source statistics of the bin sequence and the need of a solid and accurate probability estimation that is prone to random events and noise. In the tested proposal two different solutions to this problem are presented.

### One Model

The One Model operating mode uses a single counter variable for each probability model. After decoding a bin the counter update of the assigned probability model is performed similar to the equation (1). For this mode a window size of w = 64 symbols is suggested. Scaling the range of the counters and using a window size that is a power of 2 (w=2n) the multiplication can be replaced by bit shifts and the counts can be represented with integer arithmetic:

ct+1=ct+((xt<<BITSres)−ct>>n),      if xt=1.				(2)

When using One Model the current value ctis used for the encoding and decoding the next bin xt+1 during the interval division of the arithmetic coding engine as specified in more detail in section 1.2.

### Multi-Parameter Model

The Multi-Parameter Model operation is closely related to the One Model but uses two independent counter variables. Both counter variables coexist for each probability model and are updated simultaneously using the equation in (2) with two different window sizes. In this proposal countera operates with the window size wa=2na=16 and counter b operates with the window size wb=2nb=256. For the encoding or decoding of the next bin xt+1both counts are weighted as follows:

c~t=(at+bt+1)>>1.						(3)

The weighted count is then used for the interval division of the arithmetic coding engine as explained in the following section.

## Arithmetic coding engine

In this proposal the arithmetic coding engine is replaced by an extended version compared to the state of the art M Coder as specified in [3]. The table containing the approximated interval subdivision results RLPS is extended from a 64x4 table of the M Coder to a 512x64table. It represents the quantization of the probability interval at a much higher resolution on the one hand and because the concept of the LPS and MPS distinction is omitted it includes the pre calculated range values the pre-calculated range values for the operation of probabilitiesp>0.5, too. In contrast to the probabilities used for the interval subdivision of the M Coder the arithmetic coding engine of the proposed design is based on a uniform quantization with an increment of Δq=0.00197 resulting in 256 probabilities 0.5 and 256 probabilities0.5. The probabilities used to access the interval subdivision table are derived from the count ct - in case of the One Model mode – or from the weighted count ct~ - in case of the Multi-Parameter mode. Both of the proposed probability estimation modes operate with integer arithmetic and output counts in the interval (0, 32768). These counts are directly mapped to the probability indices used for the interval subdivision of the arithmetic coding engine by shifting them 6 bit to the right.

# Software

The binaries were compiled on a 64 bit Ubuntu 10.10 Linux using the included make files and gcc 4.4.5. The coding conditions used for the simulations are specified in [2]. This tested proposal affects the arithmetic coding engine only and all low-complexity configurations are skipped.

## Hardware

The type of computer that has been used to run the simulations  all running a 64 bit Ubuntu 10.10 Linux operating system are listed in Table 2-1.

Table 2-1	 Computers used to run the simulations.

| Type | Purpose | Cores | CPU | RAM |
| --- | --- | --- | --- | --- |
| 1 | Encoding and Decoding | 2x6 | 2 x 6 x Intel(R) Xeon(R) CPU X5670 @ 2.93GHz | 48 GB |

# References

- Rajan Joshi, Elena Alshina, Hisao Sasai, Heiner Kirchhoffer, Jani Leinema, “Description of Core Experiment 1: Entropy Coding”, JCTVC-F901, Torino, July, 2011.

- JCT-VC, “HEVC Test Model Software 4.0”, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-4.0

- D. Marpe, H. Schwarz, and T. Wiegand: Context-Based Adaptive Binary Arithmetic Coding in the H.264 / AVC Video Compression Standard, IEEE Transactions on Circuits and Systems for Video Technology, Vol. 13, No. 7, pp. 620-636, July 2003.

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, Torino, July, 2011.