| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G554 |
| --- | --- |

| Title: | Grouping of bypass bins for last position coding of transform coefficients |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Il-Koo Kim JeongHoon Park | Email: | ilkoo.kim@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In this contribution, grouping of bypass bins for last position coding of transform coefficients is presented for CABAC entropy coding. In HM-4.0, bypass bins for last position coding are interleaved between context modelled bins because each component is encoded alternately. It is proposed that bypass bins are group together and encoded after all context modelled bins are encoded. By grouping bypass bins, several bins are processed at a time since probability is fixed for bypass bins. Exactly same results with HM-4.0 anchor are observed in the experiment. Encoding time is same with HM-4.0 and decoding time fluctuates within the range of +-1% due to the usage of heterogeneous clustered systems.

# Introduction

In the last JCTVC meeting, grouping bypass bins was proposed and adopted for coding of transform coefficient level and motion vector difference to increase throughput of CABAC. In this contribution, grouping of bypass bins for last position coding of transform coefficients is presented for the same purpose.

# Grouping of bypass bins for last position coding

In HM-4.0, the last position of transform coefficients in transform unit (TU) is signaled to decoder explicitly. Each component of the last position, x, y, is signaled separately. For CABAC entropy coder, binarization of each component is depicted as Figure 1.

Figure 1. Binarisation process for the last position (CABAC case)

First part of binary representation of x and y, i.e. first width/2 bins of x and first height/2 bins of y are coded with conventional method with context modelling. If x >= width/2 or y >=height/2, then x = x – width/2, y = y – height /2. And, new x and y are represented with fixed binary with log2(width) – 1 and log2(height) – 1 bins length and coded with bypass mode.

Figure 2. Illustration of last position coding in HM-4.0 and proposed last position coding by bypass bin grouping

Since each component is encoded alternately, bypass bins are interleaved between context modelled bins depicted in Figure 2 (a). In this contribution, bypass bins are group together and encoded after all context modelled bins are encoded like Figure 2 (b). By grouping bypass bins, several bins are processed at a time since probability is fixed for bypass bins. This leads increase of throughput.

# Experimental results

An experiment was conducted under common test condition [1]. Exactly same results with HM-4.0 anchor are observed in the experiment. Encoding time is same with HM-4.0 and decoding time fluctuates within the range of +-1% due to the usage of heterogeneous clustered systems.

Table 1. Experimental result for grouping of bypass bins for last position coding of transform coefficients

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

# Conclusions

In this contribution, grouping of bypass bins for last position coding of transform coefficients was proposed for CABAC entropy coding. By grouping bypass bins, several bins are processed at a time since probability is fixed for bypass bins. Exactly same results with HM-4.0 anchor are observed in the experiment. Encoding time is same with HM-4.0 and decoding time fluctuates within the range of +-1% due to the usage of heterogeneous clustered systems. It is recommended to adopt this grouping to increase throughput of CABAC entropy coding.

# Suggested Working Draft (WD) modification

Suggested WD text is enclosed with contribution document. Modified texts are mark as green color.

# References

- F. Bossen, “Common test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-F900, Jul. 2011.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).