| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, 21-30 November, 2011 | Document: JCTVC-G556_r2 |
| --- | --- |

| Title: | CE6.c Report on Simplification of Short Distance Intra Prediction Method |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoran Cao, Tsinghua Xiulian Peng, USTC Changcai Lai, HiSilicon Haitao Yang, Huawei Jizheng Xu, Microsoft Lingzhi Liu, HiSilicon Jianhua Zheng, HiSilicon Yun He, Tsinghua Haoping Yu, Huawei | Email: | laichangcai@huawei.com jzxu@microsoft.com haoping.yu@huawei.com |
| Source: | Tsinghua University, University of Science and Technology of China, HiSilicon, Microsoft, Huawei |  |  |

_____________________________

# Abstract

This report presents the experiment results of sub-test 4.3.1, CE6.C, on HM4.0-based encoding simplification for SDIP (short distance intra prediction). These tests were conducted around three simplification scenarios, Case 1, 2, and 3, representing three different trade-off points between coding efficiency and encoding time. Basically, Case 1 is an implementation of F532 on HM4.0, Case 2 is similar to Case 1 and further simplifies the encoding process of SDIP, and Case 3 shows a simplification solution to the SDIP algorithm itself. In summary, the trade-off for Case 1 was about 1.4%/2.3% (AIHE/AILC) coding gain with 127%/134% encoding time increase for Class A and E sequences, and 2.1%/3.0% gain for all test sequences including Class F. For Case 2, the trade-off changed to 1.0%/1.6% (AIHE/AILC) gain with 113%/118% encoding time increase for Class A and E sequences, and 1.7%/2.3% for all test sequences including Class F. For Case 3, the trade-off became to 0.96%/1.43% (AIHE/AILC) gain with 114%/114% encoding time increase without Class F and 1.39%/1.82% gain for all sequences from Class A to F. Detailed descriptions of these 3 methods as well as the complete test results of AI, RA, and LD are given in this report.

# Introduction

SDIP was adopted into HM3.0-sdip branch in the last Geneva meeting in March of this year. The harmonization between SDIP and other intra tools in HM software was carried out and its performance was evaluated and considered satisfactory at the Turin meeting in July. This CE subset is to conduct further tests, based on the comments received in the discussions in Turin, focusing on the trade-off between coding efficiency and encoding/decoding running time of the harmonized SDIP on the HM4.0 software platform.

# SDIP simplification descriptions and test results

In the last meeting harmonized SDIP introduced two new Intra partition types 2NxhN and hNx2N for CUs smaller than 64x64, where h means half or 0.5. These new partitions were called as SDIP PUs and could have 32x8, 16x4, and 8x2 sizes for the 2NxhN case and, similarly, 8x32, 4x16, and 2x8 for the hNx2N case. Various rectangular transform units were also used at different depth of RQT. For example, 1x16/16x1 and 2x32/32x2 were the second depth of transform unit of 4x16/16x4 PU and 8x32/32x8 PU, respectively. According to the suggestions and comments made in the last meeting, the 1x16 and 16x1 TUs were removed from the TU tree this time and they were not used at all. For convenience, all candidate non-square PUs are summarized in Table 1. For a PU in intra slice, one flag is needed to first identify the PU shape. If it’s a rectangular PU, then another flag is needed to further identify this PU as  2NxhN or hNx2N.

The existing intra prediction algorithms are all used for non-square prediction blocks. In particular, 18 prediction modes are supported in 2x8/8x2 PU, 35 modes are supported in 4x16/16x4 and 8x32/32x8 PU, planar mode and DC mode are harmonized as described in JCTVC-F518[1] and JCTVC-F584[2], MDIS is used to generate the reference pixels according to the PU size and prediction modes [3]. The residuals of a non-square prediction unit are transformed by a pre-defined size of non-square transform unit as NSQT did for inter non-square prediction units in HM4 with the harmonized non-square RQT structure  [4], except a 2x2 DCT transform core is used for 2x8/8x2 in SDIP. For 4x16/16x4 TUs, mode dependant 4x4 DCT/DST is used as JCTVC-F506[5]. The harmonized MDCS is used to scan the quantized transform coefficients as described in JCTVC-F197[6], and these coefficients are mapped to the relevant square TUs for the subsequent entropy coding process. For 8x32/32x8 and 4x16/16x4 PU, the deblocking filtering in HM4 is also used for the edges belong to 8x8 blocks [7]. For chroma prediction, a harmonization with HM4 square block coding is done where the candidate mode set contains six modes. For SDIP blocks, the first six available modes from a maximum of ten candidate modes, i.e. four luma intra prediction modes, LM, PLANAR, VER, HOR, DC and VER+8, compose the chroma intra prediction mode set. All the WD text changes were described in the attached file.

Table 1. Non square PU/TU in SDIP                 Figure 1. Non square PU early skip

| CU size | Non-square PUs | Non-square TUs | Intra Mode number |
| --- | --- | --- | --- |
| 64x64 | - |  |  |
| 32x32 | 32x8/8x32 | 32x8->32x2 /8x32->2x32 | 35 |
| 16x16 | 16x4/4x16 | 16x4/4x16 | 35 |
| 8x8 | 8x2/2x8 | 8x2/2x8 | 18 |

## Simplification of SDIP and results

Based on the above described harmonized SDIP, three cases of simplification were made to find the best trade-off between coding efficiency and encoding complexity, both Case 1 and Case 2 are encoder only simplifications while Case 3 also simplifies the decoding process of SDIP as well.

### Case 1: early skip based simplification

In this case, the non square PU type maybe skipped according to the coding results of square PU, as illustrated in Figure 1. The coding information of 2Nx2N PU type is used to guide the coding of non square PUs. For a size of CU in Intra slice, totally four candidate partition units may be selected to code a CU, which includes square PU (2Nx2N, or NxN) and non-square PU (hNx2N, 2NxhN). Square PU is coded as the same as HM4.0, and the bits of 2Nx2N PU type was saved and compared with a threshold before encoding the non square PU types. If the bits of 2Nx2N PU is smaller than the threshold, this CU is generally belonging to a smooth texture, which is not the favorite texture for non square PU types, so hNx2N and 2NxhN PU types will be skipped and not tested in the RDO loop. In addition, since 8x32 and 32x8 PUs are favorite in the regions without too much textural information, the 8x32 and 32x8 PUs will also be skipped if the bit count of 2Nx2N is larger than a threshold. The threshold T is calculated as formula (1) when QP is not equal to 0:

T = a *(64.0 / ((QP < 32)? QP: 64))                                        (1)

where variable “a” is set to 5, 10 and 300 for 8x8, 16x16 and 32x32 CU respectively.

For each individual PU, the intra prediction mode is decided by two steps in HM4. First, a set of M candidate modes were selected from total N modes (M<= N) by minimize the HAD based RD cost, where N is the intra mode number in Table 1, then the final mode was selected from the M candidate modes and the most probable modes by calculating the actual rate distortion cost. In Case 1 of SDIP, we set the M as 2 for non square Pus, which is the same for square PUs in HM4. In addition, at the first step of non square PUs in a CU, not all of the N modes will be tested but only the following modes will be tested: the M candidate modes selected in 2Nx2N and the modes whose number is smaller than N>>1. Table 2 & 3 are the test results with and without Class F sequences for Case 1.

Table 2 Results of Case 1 without Class F

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.5% | -1.5% | -1.9% | -1.2% | -1.9% | -2.5% |
| Class B | -1.2% | -2.4% | -2.6% | -2.3% | -2.4% | -3.0% |
| Class C | -1.5% | -1.4% | -1.7% | -2.3% | -2.4% | -2.9% |
| Class D | -1.5% | -1.2% | -1.1% | -2.1% | -2.1% | -2.1% |
| Class E | -2.2% | -5.3% | -4.9% | -3.8% | -7.1% | -5.6% |
| Overall | -1.35% | -2.22% | -2.34% | -2.28% | -2.95% | -3.10% |
|  | -1.35% | -2.21% | -2.32% | -2.28% | -2.93% | -3.07% |
| Enc Time[%] | 127% |  |  | 135% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.5% | -0.7% | -1.2% | -0.9% | -0.7% | -1.2% |
| Class B | -0.7% | -1.2% | -1.3% | -1.2% | -1.4% | -1.4% |
| Class C | -0.8% | -0.8% | -1.0% | -1.2% | -1.3% | -1.4% |
| Class D | -0.6% | -0.6% | -0.6% | -0.9% | -1.0% | -1.0% |
| Class E |  |  |  |  |  |  |
| Overall | -0.63% | -0.86% | -1.05% | -1.05% | -1.13% | -1.24% |
|  | -0.63% | -0.90% | -1.08% | -1.06% | -1.12% | -1.22% |
| Enc Time[%] | 105% |  |  | 104% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | -0.3% | -0.8% | -0.3% | -0.4% | -0.5% |
| Class C | -0.3% | -0.5% | -0.6% | -0.4% | -0.3% | -0.8% |
| Class D | -0.1% | -0.1% | -0.4% | -0.3% | -0.4% | -1.2% |
| Class E | -0.6% | -2.5% | -2.3% | -0.9% | -2.3% | -2.0% |
| Overall | -0.27% | -0.69% | -0.93% | -0.46% | -0.73% | -1.04% |
|  | -0.27% | -0.72% | -0.89% | -0.46% | -0.73% | -0.99% |
| Enc Time[%] | 104% |  |  | 103% |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  |

Table 3 Results of Case 1 with Class F.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.5% | -1.5% | -1.9% | -1.2% | -1.9% | -2.5% |
| Class B | -1.2% | -2.4% | -2.6% | -2.3% | -2.4% | -3.0% |
| Class C | -1.5% | -1.4% | -1.7% | -2.3% | -2.4% | -2.9% |
| Class D | -1.5% | -1.2% | -1.1% | -2.1% | -2.1% | -2.1% |
| Class E | -2.2% | -5.3% | -4.9% | -3.8% | -7.1% | -5.6% |
| Class F | -6.1% | -5.0% | -5.1% | -6.7% | -5.1% | -5.2% |
| Overall | -2.1% | -2.7% | -2.8% | -3.0% | -3.3% | -3.4% |
|  | -2.1% | -2.7% | -2.8% | -3.0% | -3.3% | -3.4% |
| Enc Time[%] | 127% |  |  | 134% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.5% | -0.7% | -1.2% | -0.9% | -0.7% | -1.2% |
| Class B | -0.7% | -1.2% | -1.3% | -1.2% | -1.4% | -1.4% |
| Class C | -0.8% | -0.8% | -1.0% | -1.2% | -1.3% | -1.4% |
| Class D | -0.6% | -0.6% | -0.6% | -0.9% | -1.0% | -1.0% |
| Class E |  |  |  |  |  |  |
| Class F | -4.4% | -3.7% | -4.0% | -4.9% | -3.9% | -4.1% |
| Overall | -1.4% | -1.4% | -1.6% | -1.8% | -1.7% | -1.8% |
|  | -1.3% | -1.4% | -1.6% | -1.8% | -1.7% | -1.8% |
| Enc Time[%] | 105% |  |  | 104% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | -0.3% | -0.8% | -0.3% | -0.4% | -0.5% |
| Class C | -0.3% | -0.5% | -0.6% | -0.4% | -0.3% | -0.8% |
| Class D | -0.1% | -0.1% | -0.4% | -0.3% | -0.4% | -1.2% |
| Class E | -0.6% | -2.5% | -2.3% | -0.9% | -2.3% | -2.0% |
| Class F | -2.9% | -2.9% | -2.6% | -2.9% | -2.5% | -2.7% |
| Overall | -0.8% | -1.1% | -1.3% | -0.9% | -1.1% | -1.4% |
|  | -0.8% | -1.1% | -1.2% | -1.0% | -1.1% | -1.3% |
| Enc Time[%] | 104% |  |  | 103% |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  |

### Case 2: fast non-square PU selection

In addition to the early skip method described in case_1, a fast non-square PU selection method was used to further reduce the encoder complexity in this case. At the stage of 2Nx2N encoding, the best intra mode for 2Nx2N and its RD cost were saved, if non square PU type should be tested, which is determined by early skip method in Case 1, each PU with hNx2N and 2NxhN type will be encoded with the best mode of the 2Nx2N partition. The best non square PU type will be selected to compare with the RD cost of 2Nx2N. If the RD cost of the best non square PU (i.e. 2NxhN) is smaller than that of 2Nx2N, 2NxhN will do a real RDO and choose the best intra mode for each 2NxhN PUs from N Modes. Therefore only one non-square PU together with its prediction mode needs to be tested in RDO for each CU. Table 4&5 are the results with and without Class F sequences for Case_2.

Table 4 Results of Case 2 without Class F

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.3% | -1.1% | -1.4% | -0.8% | -1.3% | -1.8% |
| Class B | -0.9% | -2.0% | -2.1% | -1.6% | -1.6% | -2.1% |
| Class C | -1.0% | -1.0% | -1.3% | -1.6% | -1.8% | -2.2% |
| Class D | -1.1% | -0.9% | -0.9% | -1.6% | -1.6% | -1.7% |
| Class E | -1.6% | -4.3% | -3.9% | -2.7% | -5.7% | -4.4% |
| Overall | -1.0% | -1.7% | -1.8% | -1.6% | -2.2% | -2.4% |
|  | -1.0% | -1.7% | -1.8% | -1.6% | -2.2% | -2.3% |
| Enc Time[%] | 113% |  |  | 118% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -0.3% | -0.6% | -0.6% | -0.4% | -0.6% |
| Class B | -0.4% | -0.9% | -1.2% | -0.8% | -0.9% | -1.1% |
| Class C | -0.5% | -0.8% | -0.8% | -0.8% | -1.0% | -1.1% |
| Class D | -0.5% | -0.5% | -0.8% | -0.7% | -1.0% | -0.8% |
| Class E |  |  |  |  |  |  |
| Overall | -0.41% | -0.67% | -0.87% | -0.71% | -0.84% | -0.90% |
|  | -0.41% | -0.67% | -0.89% | -0.71% | -0.81% | -0.89% |
| Enc Time[%] | 105% |  |  | 104% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -0.4% | -0.3% | -0.2% | -0.2% | -0.6% |
| Class C | -0.2% | -0.2% | -0.4% | -0.3% | -0.3% | -0.8% |
| Class D | -0.1% | -0.2% | -0.7% | -0.2% | -0.2% | -0.7% |
| Class E | -0.4% | -1.1% | -1.4% | -0.7% | -3.2% | -2.3% |
| Overall | -0.18% | -0.43% | -0.66% | -0.32% | -0.77% | -1.00% |
|  | -0.18% | -0.41% | -0.66% | -0.32% | -0.82% | -0.99% |
| Enc Time[%] | 104% |  |  | 103% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |

Table 5 Results of Case 2 with Class F

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.3% | -1.1% | -1.4% | -0.8% | -1.3% | -1.8% |
| Class B | -0.9% | -2.0% | -2.1% | -1.6% | -1.6% | -2.1% |
| Class C | -1.0% | -1.0% | -1.3% | -1.6% | -1.8% | -2.2% |
| Class D | -1.1% | -0.9% | -0.9% | -1.6% | -1.6% | -1.7% |
| Class E | -1.6% | -4.3% | -3.9% | -2.7% | -5.7% | -4.4% |
| Class F | -5.3% | -4.2% | -4.3% | -5.5% | -4.1% | -4.1% |
| Overall | -1.7% | -2.1% | -2.2% | -2.3% | -2.5% | -2.6% |
|  | -1.7% | -2.1% | -2.2% | -2.3% | -2.5% | -2.6% |
| Enc Time[%] | 113% |  |  | 118% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -0.3% | -0.6% | -0.6% | -0.4% | -0.6% |
| Class B | -0.4% | -0.9% | -1.2% | -0.8% | -0.9% | -1.1% |
| Class C | -0.5% | -0.8% | -0.8% | -0.8% | -1.0% | -1.1% |
| Class D | -0.5% | -0.5% | -0.8% | -0.7% | -1.0% | -0.8% |
| Class E |  |  |  |  |  |  |
| Class F | -3.8% | -3.1% | -3.4% | -4.0% | -3.0% | -3.1% |
| Overall | -1.1% | -1.1% | -1.3% | -1.3% | -1.2% | -1.3% |
|  | -1.1% | -1.1% | -1.4% | -1.3% | -1.2% | -1.3% |
| Enc Time[%] | 105% |  |  | 105% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -0.4% | -0.3% | -0.2% | -0.2% | -0.6% |
| Class C | -0.2% | -0.2% | -0.4% | -0.3% | -0.3% | -0.8% |
| Class D | -0.1% | -0.2% | -0.7% | -0.2% | -0.2% | -0.7% |
| Class E | -0.4% | -1.1% | -1.4% | -0.7% | -3.2% | -2.3% |
| Class F | -2.3% | -2.2% | -2.2% | -2.3% | -2.0% | -2.0% |
| Overall | -0.6% | -0.8% | -1.0% | -0.7% | -1.0% | -1.2% |
|  | -0.6% | -0.8% | -1.0% | -0.7% | -1.0% | -1.2% |
| Enc Time[%] | 104% |  |  | 103% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |

### Case 3: NSQT for intra

In this case, the simplification was done by constraining all of the rectangular PUs within a CU to share the same prediction mode and the prediction residuals are further processed and encoded with rectangular TUs. As a result, the rectangular PU partitions are not necessary any more in this scheme. In fact, this method can also been viewed as an extension of NSQT for Intra CUs, NSQT_INTRA, which simply introduces non-square quad-tree transforms (NSQT) to the existing intra CU. Most of the modules in the implementation of SDIP can be re-used for NSQT_INTRA. The major changes done in the implementation are, in brief, 1) remove non-square PU and introduce non-square TU instead; 2) move SDIP side info from PU level to TU level; 3) remove modifications introduced to chroma components. Details are provided below.

All non-square partitions are “TUs” in NSQT_INTRA, so partition PART_2NxhN and PART_hNx2N are not used in intra CU. Instead, a set of NSQT partitions are introduced to complement the existing square TUs for the Intra coding case. For different CU size and different trafoDepth, the candidate TU partitioning methods (including both SQT partitions and NSQT partitions) are listed in Table 6. In some cases, there are more than one candidate TU partitioning method. For example, the candidates are 8x8, 16x4, and 4x16 in case of 16x16 CU and trafoDepth=1. The selection of the optimal TU partitioning method is signaled to the decoder by syntax element non_square_transform_flag and non_square_transform_direction, which is described below in details. Note that the proposed INTRA_NSQT is not applied to 64x64 CU, which is consistent with Case 1 and Case 2.

Table 6 Candidate TU partitioning methods (including both SQT and NSQT partitions) for different CU size and different trafoDepth

|  | 64x64 CU (SIZE_2Nx2N) |  | 32x32 CU (SIZE_2Nx2N) |  | 16x16 CU (SIZE_2Nx2N) |  | 8x8 CU (SIZE_2Nx2N) |  | 8x8 CU (SIZE_NxN) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SQT | NSQT | SQT | NSQT | SQT | NSQT | SQT | NSQT | SQT | NSQT |
| trafoDepth=0 | N/A | N/A | 32x32 | N/A | 16x16 | N/A | 8x8 | N/A | N/A | N/A |
| trafoDepth=1 | 32x32 | 32x32 | 16x16 | 32x8 8x32 | 8x8 | 16x4 4x16 | 4x4 | 8x2 2x8 | 4x4 | N/A |
| trafoDepth=2 | 16x16 | 16x16 | 8x8 | 32x2 2x32 | 4x4 | N/A | N/A | N/A | N/A | N/A |

A flag non_square_transform_flag is added to signal the decoder whether the original square quad-tree partition (SQT) or the non-square quad-tree (NSQT) partition is used for each intra CU. To distinguish between horizontal and vertical partition, an additional flag non_square_transform_direction is also sent to the decoder when non_square_transform_flag is true. The above described side info is written into bitstream if a CU contains more than one TUs as indicated by split_transform_flag.

The intra chroma prediction method in the current SDIP design composes the candidate prediction mode list based on the mode statistics of all non-square PUs. Since there is no non-square PUs in NSQT_INTRA, the intra chroma prediction method introduced to SDIP can not be applied to NSQT_INTRA. Therefore, the original HM scheme is maintained, with restriction of only 1-level quad-tree TU. In other words, the chroma TU would not be split when NSQT_INTRA is used.

As to the encoder implementation, the NSQT_INTRA firstly choose the best prediction mode for a CU, and then try different quad-tree TU partitions, including square and non-square TU partitions at different depth, to decide the RQT structure. No encoder speedup is used in the current implementation. Obviously, the encoder complexity of NSQT_INTRA is essentially much less than the current SDIP implementation, especially a naive one where all prediction modes are tried for each non-square PU for mode decision. Table 7&8 are the results with and without class F sequences for case_3, and Table 9&10 summarized the average results with and without class F for each case.

Table 7 Results of Case 3 without Class F

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.3% | -0.9% | -1.0% | -0.7% | -1.3% | -1.5% |
| Class B | -1.0% | -1.7% | -1.7% | -1.6% | -1.8% | -2.1% |
| Class C | -1.0% | -0.8% | -0.9% | -1.3% | -1.3% | -1.5% |
| Class D | -0.9% | -0.6% | -0.5% | -1.1% | -1.2% | -1.1% |
| Class E | -1.9% | -3.1% | -2.8% | -2.7% | -5.0% | -3.9% |
| Overall | -0.96% | -1.34% | -1.33% | -1.43% | -1.95% | -1.94% |
|  | -0.97% | -1.35% | -1.32% | -1.43% | -1.96% | -1.92% |
| Enc Time[%] | 114% |  |  | 114% |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.3% | -0.5% | -0.5% | -0.5% | -0.6% | -0.9% |
| Class B | -0.5% | -0.7% | -0.8% | -0.8% | -1.2% | -1.1% |
| Class C | -0.4% | -0.5% | -0.4% | -0.6% | -0.7% | -0.7% |
| Class D | -0.3% | -0.3% | -0.6% | -0.4% | -0.6% | -0.2% |
| Class E |  |  |  |  |  |  |
| Overall | -0.38% | -0.52% | -0.60% | -0.57% | -0.81% | -0.76% |
|  | -0.37% | -0.56% | -0.59% | -0.57% | -0.80% | -0.76% |
| Enc Time[%] | 102% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -0.3% | -0.6% | -0.3% | -0.5% | -0.4% |
| Class C | -0.1% | 0.0% | -0.3% | -0.2% | 0.0% | -0.5% |
| Class D | -0.1% | -0.3% | 0.1% | -0.1% | 0.0% | -0.6% |
| Class E | -0.5% | -1.5% | -1.6% | -0.9% | -1.5% | -1.8% |
| Overall | -0.19% | -0.47% | -0.55% | -0.34% | -0.44% | -0.72% |
|  | -0.19% | -0.45% | -0.56% | -0.34% | -0.46% | -0.71% |
| Enc Time[%] | 102% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

Table 8 Results of Case 3 with Class F

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.3% | -0.9% | -1.0% | -0.7% | -1.3% | -1.5% |
| Class B | -1.0% | -1.7% | -1.7% | -1.6% | -1.8% | -2.1% |
| Class C | -1.0% | -0.8% | -0.9% | -1.3% | -1.3% | -1.5% |
| Class D | -0.9% | -0.6% | -0.5% | -1.1% | -1.2% | -1.1% |
| Class E | -1.9% | -3.1% | -2.8% | -2.7% | -5.0% | -3.9% |
| Class F | -3.5% | -2.7% | -2.4% | -3.8% | -2.8% | -2.7% |
| Overall | -1.39% | -1.58% | -1.51% | -1.82% | -2.09% | -2.07% |
|  | -1.39% | -1.57% | -1.50% | -1.82% | -2.10% | -2.07% |
| Enc Time[%] | 114% |  |  | 115% |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.3% | -0.5% | -0.5% | -0.5% | -0.6% | -0.9% |
| Class B | -0.5% | -0.7% | -0.8% | -0.8% | -1.2% | -1.1% |
| Class C | -0.4% | -0.5% | -0.4% | -0.6% | -0.7% | -0.7% |
| Class D | -0.3% | -0.3% | -0.6% | -0.4% | -0.6% | -0.2% |
| Class E |  |  |  |  |  |  |
| Class F | -2.4% | -1.9% | -2.0% | -2.6% | -2.0% | -2.2% |
| Overall | -0.76% | -0.78% | -0.88% | -0.96% | -1.04% | -1.04% |
|  | -0.76% | -0.81% | -0.87% | -0.96% | -1.03% | -1.04% |
| Enc Time[%] | 102% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -0.3% | -0.6% | -0.3% | -0.5% | -0.4% |
| Class C | -0.1% | 0.0% | -0.3% | -0.2% | 0.0% | -0.5% |
| Class D | -0.1% | -0.3% | 0.1% | -0.1% | 0.0% | -0.6% |
| Class E | -0.5% | -1.5% | -1.6% | -0.9% | -1.5% | -1.8% |
| Class F | -1.6% | -1.6% | -1.2% | -1.6% | -1.2% | -1.3% |
| Overall | -0.48% | -0.70% | -0.69% | -0.59% | -0.58% | -0.83% |
|  | -0.48% | -0.69% | -0.66% | -0.59% | -0.59% | -0.82% |
| Enc Time[%] | 102% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

Table 9 Summary of the average results without Class F for each case

| Test case |  | BD-RATE |  |  |  |  |  | Enc/Dec Time |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | HE |  |  | LC |  |  | HE |  | LC |  |
|  |  | Y | U | V | Y | U | V | Enc time | Dec time | Enc time | Dec time |
| case_1 | Intra | -1.35% | -2.22% | -2.34% | -2.28% | -2.95% | -3.10% | 127% | 101% | 135% | 102% |
|  | RA | -0.63% | -0.86% | -1.05% | -1.05% | -1.13% | -1.24% | 105% | 100% | 104% | 99% |
|  | LD | -0.27% | -0.69% | -0.93% | -0.46% | -0.73% | -1.04% | 104% | 98% | 103% | 100% |
| case_2 | Intra | -0.96% | -1.74% | -1.82% | -1.60% | -2.20% | -2.35% | 113% | 102% | 118% | 101% |
|  | RA | -0.41% | -0.67% | -0.87% | -0.71% | -0.84% | -0.90% | 105% | 99% | 104% | 99% |
|  | LD | -0.18% | -0.43% | -0.66% | -0.32% | -0.77% | -1.00% | 104% | 98% | 103% | 99% |
| case_3 | Intra | -0.96% | -1.34% | -1.33% | -1.43% | -1.95% | -1.94% | 114% | 100% | 114% | 98% |
|  | RA | -0.38% | -0.52% | -0.60% | -0.57% | -0.81% | -0.76% | 102% | 100% | 101% | 102% |
|  | LD | -0.19% | -0.47% | -0.55% | -0.34% | -0.44% | -0.72% | 102% | 100% | 100% | 101% |

Table 10 Summary of the average results with Class F for each case

| Test case |  | BD-RATE |  |  |  |  |  | Enc/Dec Time |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | HE |  |  | LC |  |  | HE |  | LC |  |
|  |  | Y | U | V | Y | U | V | Enc time | Dec time | Enc time | Dec time |
| case_1 | Intra | -2.14% | -2.69% | -2.80% | -3.01% | -3.31% | -3.44% | 127% | 101% | 134% | 101% |
|  | RA | -1.40% | -1.40% | -1.60% | -1.80% | -1.70% | -1.80% | 105% | 100% | 104% | 99% |
|  | LD | -0.80% | -1.10% | -1.30% | -0.90% | -1.10% | -1.40% | 104% | 98% | 103% | 100% |
| case_2 | Intra | -1.70% | -2.10% | -2.20% | -2.30% | -2.50% | -2.60% | 113% | 101% | 118% | 101% |
|  | RA | -1.10% | -1.10% | -1.30% | -1.30% | -1.20% | -1.30% | 105% | 99% | 105% | 98% |
|  | LD | -0.60% | -0.80% | -1.00% | -0.70% | -1.00% | -1.20% | 104% | 98% | 103% | 99% |
| case_3 | Intra | -1.39% | -1.58% | -1.51% | -1.82% | -2.09% | -2.07% | 114% | 100% | 115% | 98% |
|  | RA | -0.76% | -0.78% | -0.88% | -0.96% | -1.04% | -1.04% | 102% | 100% | 100% | 101% |
|  | LD | -0.48% | -0.70% | -0.69% | -0.59% | -0.58% | -0.83% | 102% | 100% | 100% | 102% |

Figure 2 is the summary of these three cases of SDIP for all intra configurations without Class F.

Figure 2 Summary of SDIP without Class F

Figure 3 is the summary of these three cases of SDIP for all intra configurations with Class F.

Figure 3 Summary of SDIP with Class F

# Conclusion

Compared with Turin meeting, the encoding time increase was greatly reduced while the gain was retained. Based on the significant gain and drastically reduced complexity, we recommend adopt SDIP into HM.

# References

- Yongbing Lin, Changcai Lai, Xiaoran, Cao, etc. “CE6.b Test6: Report of harmonization on Planar prediction in SDIP”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F518, 6th Meeting: Torino, IT, 14-22 July, 2011.

- Yongbing Lin, Changcai Lai, “Modification to DC prediction in SDIP”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F584, 6th Meeting: Torino, IT, 14-22 July, 2011.

- Guichun Li, Lingzhi Liu, Changcai Lai, etc. “CE6.b.5 Report: Harmonization of SDIP and MDIS”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC- F336, 6th Meeting: Torino, IT, 14-22 July, 2011.

- Xiaoran Cao, Xiulian Peng, Changcai Lai, etc. “CE6.b Report on test1 Harmonization of SDIP and RQT”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC- F533, 6th Meeting: Torino, IT, 14-22 July, 2011.

- Haitao Yang, Jiantong Zhou, Haoping Yu, etc. “CE6.b: Harmonization of SDIP and MDDT”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC- F506, 6th Meeting: Torino, IT, 14-22 July, 2011.

- Xiulian Peng, Jizheng Xu, “CE6.b Report of Test 3: Interaction between SDIP and MDCS”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC- F197, 6th Meeting: Torino, IT, 14-22 July, 2011.

- Xiaoran Cao, Xiulian Peng, Changcai Lai, etc. “CE6.b Report on test7 Harmonization of SDIP and deblocking filter”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC- F536, 6th Meeting: Torino, IT, 14-22 July, 2011.

# Syntax and Semantics changes to WD text

The related syntax and semantics changes are listed below, the detailed changes to WD can be found in the attached document of this proposal.

## Syntax and Semantics changes for Case1 and Case2 of SDIP

#### Sequence parameter set RBSP syntax

| …… |  |
| --- | --- |
| cu_qp_delta_enabled_flag | u(1) |
| intra_sdip_enabled_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| ...... |  |
| } |  |

### Coding unit syntax

| coding_unit( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if( entropy_coding_mode_flag && slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| else { |  |
| sdipEnabledFlag = intra_sdip_enabled_flag && log2CUSize < 6 |  |
| if( !entropy_coding_mode_flag ) { |  |
| if( slice_type == I && log2CUSize == Log2MinCUSize ) |  |
| intra_part_mode | u(1) |
| } else if( slice_type != I ¦ ¦ log2CUSize = = Log2MinCUSize ) |  |
| pred_type | u(v) ¦ ae(v) |
| if ( sdipEnabledFlag && PredMode == MODE_INTRA && (log2CUSize > Log2MinCUSize ¦ ¦ PartMode == PART_NxN) ) { |  |
| sdip_flag | ae(v) |
| if ( sdip_flag ) |  |
| sdip_direction | ae(v) |
| } |  |
| x1 = x0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| x2 = x1 − ( ( 1 << log2CUSize ) >> 2 ) |  |
| y2 = y1 − ( ( 1 << log2CUSize ) >> 2 ) |  |
| x3 = x1 + ( ( 1 << log2CUSize ) >> 2 ) |  |
| y3 = y1 + ( ( 1 << log2CUSize ) >> 2 ) |  |
| if( PartMode == PART_2Nx2N ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| } else if( PartMode == PART_2NxN ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x0, y1 , log2CUSize ) |  |
| } else if( PartMode == PART_Nx2N ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x1, y0 , log2CUSize ) |  |
| } else if( PartMode == PART_2NxnU ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x0, y2 , log2CUSize ) |  |
| } else if( PartMode == PART_2NxnD ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x0, y3 , log2CUSize ) |  |
| } else if( PartMode == PART_nLx2N ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x2, y0 , log2CUSize ) |  |
| } else if( PartMode == PART_nRx2N ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x3, y0 , log2CUSize ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x1, y0 , log2CUSize ) |  |
| prediction_unit( x0, y1 , log2CUSize ) |  |
| prediction_unit( x1, y1 , log2CUSize ) |  |
| } else if ( PartMode == PART_2NxhN ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x0, y2 , log2CUSize ) |  |
| prediction_unit( x0, y1 , log2CUSize ) |  |
| prediction_unit( x0, y3 , log2CUSize ) |  |
| } else if( PartMode == PART_hNx2N ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x2, y0 , log2CUSize ) |  |
| prediction_unit( x1, y0 , log2CUSize ) |  |
| prediction_unit( x3, y0 , log2CUSize ) |  |
| } |  |
| } |  |
| if( !pcm_flag ) { |  |
| transform_tree( x0, y0, log2CUSize, log2CUSize, 0, 0 ) |  |
| transform_coeff( x0, y0, log2CUSize, log2CUSize, 0, 0 ) |  |
| transform_coeff( x0, y0, log2CUSize, log2CUSize, 0, 1 ) |  |
| transform_coeff( x0, y0, log2CUSize, log2CUSize, 0, 2 ) |  |
| } |  |
| } |  |

### Transform tree syntax

| transform_tree( x0, y0, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| if( entropy_coding_mode_flag && trafoDepth = = 0 && IntraSplitFlag = = 0) { |  |
| if( PredMode != MODE_INTRA ) |  |
| no_residual_data_flag | ae(v) |
| residualDataPresentFlag = !no_residual_data_flag |  |
| } else { |  |
| residualDataPresentFlag = TRUE |  |
| } |  |
| if( residualDataPresentFlag) { |  |
| log2TrafoSize = ( log2TrafoWidth + log2TrafoHeight ) >> 1 |  |
| intraSdipFlag = (PartMode == PART_2NxhN ¦ ¦ PartMode == PART_hNx2N ) |  |
| intraSplitFlag = ( IntraSplitFlag && trafoDepth = = 0 ? 1 : 0 ) |  |
| maxDepth = ( PredMode = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| if(PredMode == MODE_INTRA && intraSdipFlag && maxDepth > 1) |  |
| maxDepth = (1 << log2TrafoSize ) <= 16 ? 1 : 2 |  |
| if( PartMode == PART_2NxhN ){ |  |
| xBase = x0 |  |
| yBase = y0 − ( y0 & ( ( 1 << (log2TrafoHeight + 2)) − 1 ) ) |  |
| }else if( PartMode == PART_hNx2N ){ |  |
| xBase = x0 − ( x0 & ( ( 1 << (log2TrafoWidth + 2)) − 1 ) ) |  |
| yBase = y0 |  |
| }else{ |  |
| xBase = x0 − ( x0 & ( 1 << log2TrafoWidth ) ) |  |
| yBase = y0 − ( y0 & ( 1 << log2TrafoHeight ) ) |  |
| } |  |
| if( !entropy_coding_mode_flag ) { |  |
| if( trafoDepth != 0 && !cbf_luma[ xBase ][ yBase ][ trafoDepth − 1 ] ) { |  |
| if(! intraSdipFlag && cbf_cb[ xBase ][ yBase ][ trafoDepth − 1 ] && log2TrafoSize > Log2MinTrafoSize ) |  |
| cbf_cb[ x0 ][ y0 ] [ trafoDepth ] | u(1) |
| if(! intraSdipFlag && cbf_cr[ xBase ][ yBase ][ trafoDepth − 1 ] && log2TrafoSize > Log2MinTrafoSize ) |  |
| cbf_cr[ x0 ][ y0 ] [ trafoDepth ] | u(1) |
| if(log2TrafoSize <= Log2MaxTrafoSize && !intraSplitFlag && log2TrafoSize > Log2MinTrafoSize && trafoDepth < maxDepth ) |  |
| split_transform_flag[ x0 ][ y0 ] [ trafoDepth ] | u(1) |
| } else |  |
| cbp_and_split_transform | vlc(n,v) |
| } |  |
| if( log2TrafoSize <= Log2MaxTrafoSize && log2TrafoSize > Log2MinTrafoSize && trafoDepth < maxDepth && !intraSplitFlag && entropy_coding_mode_flag ) |  |
| split_transform_flag[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| if( PredMode != MODE_INTRA && log2TrafoSize <= Log2MaxTrafoSize && entropy_coding_mode_flag ) { |  |
| firstChromaCbf = ( log2TrafoSize = = Log2MaxTrafoSize ¦ ¦ trafoDepth = = 0 ? 1 : 0 ) |  |
| if( firstChromaCbf ¦ ¦ log2TrafoSize > Log2MinTrafoSize ) { |  |
| if( firstChromaCbf ¦ ¦ cbf_cb[ xBase ][ yBase ][ trafoDepth − 1 ] ) { |  |
| readCbf = true |  |
| if( blkIdx = = 3 && log2TrafoSize < Log2MaxTrafoSize ) |  |
| readCbf = cbf_cb[ xBase ][ yBase ][ trafoDepth ] ¦ ¦ cbf_cb[ xBase + ( 1 << log2TrafoWidth ) ][ yBase ][ trafoDepth ] ¦ ¦ cbf_cb[ xBase ][ yBase + ( 1 << log2TrafoHeight ) ][ trafoDepth ] |  |
| if ( !readCbf ) |  |
| cbf_cb[ x0 ][ y0 ][ trafoDepth ] = 1 |  |
| else |  |
| cbf_cb[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| } |  |
| if( firstChromaCbf ¦ ¦ cbf_cr[ xBase ][ yBase ][ trafoDepth − 1 ] ) { |  |
| readCbf = true |  |
| if( blkIdx = = 3 && log2TrafoSize < Log2MaxTrafoSize ) |  |
| readCbf = cbf_cr[ xBase ][ yBase ][ trafoDepth ] ¦ ¦ cbf_cr[ xBase + ( 1 << log2TrafoWidth ) ][ yBase ][ trafoDepth ] ¦ ¦ cbf_cr[ xBase ][ yBase + ( 1 << log2TrafoHeight ) ][ trafoDepth ] |  |
| if ( !readCbf ) |  |
| cbf_cr[ x0 ][ y0 ][ trafoDepth ] = 1 |  |
| else |  |
| cbf_cr[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| } |  |
| } |  |
| } |  |
| if( split_transform_flag[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| if( PartMode == PART_2NxhN){ |  |
| x1 = x0 |  |
| y1 = y0 + ( ( 1 << log2TrafoHeight) >> 2 ) |  |
| x2 = x0 |  |
| y2 = y1 + ( ( 1 << log2TrafoHeight) >> 2 ) |  |
| x3 = x0 |  |
| y3 = y2 + ( ( 1 << log2TrafoHeight) >> 2 ) |  |
| log2TrafoWidth = log2TrafoWidth + 1 |  |
| log2TrafoHeight = log2TrafoHeight − 1 |  |
| } else if ( PartMode == PART_hNx2N){ |  |
| x1 = x0 + ( ( 1 << log2TrafoWidth) >> 2 ) |  |
| y1 = y0 |  |
| x2 = x1 + ( ( 1 << log2TrafoWidth) >> 2 ) |  |
| y2 = y0 |  |
| x3 = x2 + ( ( 1 << log2TrafoWidth) >> 2 ) |  |
| y3 = y0 |  |
| log2TrafoWidth = log2TrafoWidth − 1 |  |
| log2TrafoHeight = log2TrafoHeight + 1 |  |
| } |  |
| else if( InterTUSplitDirection = = 2 ) { |  |
| x1 = x0 + ( ( 1 << log2TrafoWidth ) >> 1 ) |  |
| y1 = y0 |  |
| x2 = x0 |  |
| y2 = y0 + ( ( 1 << log2TrafoHeight ) >> 1 ) |  |
| x3 = x1 |  |
| y3 = y2 |  |
| } else { |  |
| x1 = x0 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y1 = y0 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| x2 = x1 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y2 = y1 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| x3 = x2 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y3 = y2 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| log2TrafoHeight = log2TrafoHeight + 2 * InterTUSplitDirection − 1 |  |
| log2TrafoWidth = log2TrafoWidth − 2 * InterTUSplitDirection + 1 |  |
| } |  |
| transform_tree( x0, y0, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 0 ) |  |
| transform_tree( x1, y1, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 1 ) |  |
| transform_tree( x2, y2, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 2 ) |  |
| transform_tree( x3, y3, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 3 ) |  |
| } else if( entropy_coding_mode_flag ) { |  |
| if( PredMode = = MODE_INTRA ¦ ¦ trafoDepth != 0 ¦ ¦ cbf_cb[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| readCbf = true |  |
| if( blkIdx = = 3 && log2TrafoSize < Log2MaxTrafoSize && PredMode != MODE_INTRA ) |  |
| readCbf = cbf_luma[ xBase ][ yBase ][ trafoDepth ] ¦ ¦ cbf_luma[ xBase + ( 1 << log2TrafoWidth ) ][ yBase ][ trafoDepth ] ¦ ¦ cbf_luma[ xBase ][ yBase + ( 1 << log2TrafoHeight ) ][ trafoDepth ] ¦ ¦ cbf_cb[ xBase ][ yBase ][ trafoDepth – 1] ¦ ¦ cbf_cr[ xBase ][ yBase ][ trafoDepth – 1] |  |
| if ( !readCbf ) |  |
| cbf_luma[ x0 ][ y0 ][ trafoDepth ] = 1 |  |
| else |  |
| cbf_luma[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| } |  |
| if( PredMode = = MODE_INTRA && ( ! intraSdipFlag ¦ ¦ trafoDepth == 0)) |  |
| if( log2TrafoSize > Log2MinTrafoSize ) { |  |
| cbf_cb[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| cbf_cr[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| } else if( blkIdx = = 0 ) { |  |
| cbf_cb[ x0 ][ y0 ][ trafoDepth − 1 ] | ae(v) |
| cbf_cr[ x0 ][ y0 ][ trafoDepth − 1 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

### Transform coefficient syntax

| transform_coeff( x0, y0, log2TrafoWidth, log2TrafoHeight, trafoDepth, cIdx ) { | Descriptor |
| --- | --- |
| if( ( ( cIdx = = 0 && cbf_luma[ x0 ][ y0 ][ trafoDepth ] ) ¦ ¦ ( cIdx = = 1 && cbf_cb[ x0 ][ y0 ][ trafoDepth ] ) ¦ ¦ ( cIdx = = 2 && cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) ) ) { |  |
| if( cu_qp_delta_enabled_flag && !IsCuQpDeltaCoded ) { |  |
| cu_qp_delta | se(v) ¦ ae(v) |
| IsCuQpDeltaCoded = 1 |  |
| } |  |
| log2TrafoSize = (log2TrafoWidth + log2TrafoHeight) >> 1 |  |
| if( split_transform_flag[ x0 ][ y0 ][ trafoDepth ] && ( cIdx = = 0 ¦ ¦ log2TrafoSize > Log2MinTrafoSize + 1 ) ) { |  |
| if( PartMode == PART_2NxhN){ |  |
| x1 = x0 |  |
| y1 = y0 + ( ( 1 << log2TrafoHeight) >> 2 ) |  |
| x2 = x0 |  |
| y2 = y1 + ( ( 1 << log2TrafoHeight) >> 2 ) |  |
| x3 = x0 |  |
| y3 = y2 + ( ( 1 << log2TrafoHeight) >> 2 ) |  |
| log2TrafoWidth = log2TrafoWidth + 1 |  |
| log2TrafoHeight = log2TrafoHeight − 1 |  |
| } else if ( PartMode == PART_hNx2N){ |  |
| x1 = x0 + ( ( 1 << log2TrafoWidth) >> 2 ) |  |
| y1 = y0 |  |
| x2 = x1 + ( ( 1 << log2TrafoWidth) >> 2 ) |  |
| y2 = y0 |  |
| x3 = x2 + ( ( 1 << log2TrafoWidth) >> 2 ) |  |
| y3 = y0 |  |
| log2TrafoWidth = log2TrafoWidth − 1 |  |
| log2TrafoHeight = log2TrafoHeight + 1 |  |
| } |  |
| else if( InterTUSplitDirection = = 2 ) { |  |
| x1 = x0 + ( ( 1 << log2TrafoWidth ) >> 1 ) |  |
| y1 = y0 |  |
| x2 = x0 |  |
| y2 = y0 + ( ( 1 << log2TrafoHeight ) >> 1 ) |  |
| x3 = x1 |  |
| y3 = y2 |  |
| } else { |  |
| x1 = x0 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y1 = y0 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| x2 = x1 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y2 = y1 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| x3 = x2 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y3 = y2 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| log2TrafoHeight = log2TrafoHeight + 2 * InterTUSplitDirection − 1 |  |
| log2TrafoWidth = log2TrafoWidth − 2 * InterTUSplitDirection + 1 |  |
| } |  |
| transform_coeff( x0, y0, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, cIdx ) |  |
| transform_coeff( x1, y1, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, cIdx ) |  |
| transform_coeff( x2, y2, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, cIdx ) |  |
| transform_coeff( x3, y3, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, cIdx ) |  |
| } else { |  |
| log2TrafoSize = ( ( log2TrafoWidth + log2TrafoHeight ) >> 1 ) − ( cIdx > 0 ? 1 : 2 ) |  |
| if ( PredMode = = MODE_INTRA ) { |  |
| if (cIdx = = 0) |  |
| scanIdx = ScanType[ log2TrafoSize – 2 ][ IntraPredMode ] |  |
| else |  |
| scanIdx = ScanType[ log2TrafoSize – 2 ][ IntraPredModeC ] |  |
| } else { |  |
| scanIdx = 0 |  |
| } |  |
| if ( entropy_coding_mode_flag ) |  |
| if ( scanIdx = = 0 ) |  |
| scanIdx = 3 |  |
| residual_coding_cabac( x0, y0, log2TrafoSize, trafoDepth, scanIdx, cIdx ) |  |
| else if ( !entropy_coding_mode_flag ) |  |
| residual_coding_cavlc( x0, y0, log2TrafoSize, trafoDepth, cIdx ) |  |
| } |  |
| } |  |
| } |  |

intra_sdip_enabled_flag equal to 1 specifies that the short distance intra prediction process is applied according to the intra luma prediction mode.

sdip_flag specifies whether short distance intra prediction is applied to the current coding unit. sdip_flag equal to 1 specifies that the short distance intra prediction is applied to the current coding unit, and shall be split into four prediction units with size of 2NxhN or hNx2N.

When sdip_flag is not present, it shall be inferred to be equal to 0.

sdip_direction specifies the partition mode of the current coding unit when sdip_direction is equal to 1. PartMode shall be set to PART_2NxhN when sdip_direction is equal to 1, and PART_hNx2N when sdip_direction is equal to 0.

## Syntax and Semantics changes for Case3 of SDIP

| transform_tree( x0, y0, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| … |  |
| if( residualDataPresentFlag) { |  |
| log2TrafoSize = ( log2TrafoWidth + log2TrafoHeight ) >> 1 |  |
| intraSplitFlag = ( IntraSplitFlag && trafoDepth = = 0 ? 1 : 0 ) |  |
| maxDepth = ( PredMode = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| if( PredMode == MODE_INTRA && non_square_transform_flag && max_transform_hierarchy_depth_intra > 1 ) { |  |
| maxDepth = ( (trafoDepth + log2TrafoSize) == 5 ? 2 : 1 ) |  |
| } |  |
| if( non_square_transform_flag ) { |  |
| xBase = ( non_square_transform_direction ? x0 : x0 − ( x0 & ( ( 1 << ( log2TrafoWidth + 2 ) – 1 ) ) ) |  |
| yBase = ( non_square_transform_direction ? y0 − ( y0 & ( ( 1 << ( log2TrafoHeight + 2 ) – 1) ) : y0 ) |  |
| } else { |  |
| xBase = x0 − ( x0 & ( 1 << log2TrafoWidth ) ) |  |
| yBase = y0 − ( y0 & ( 1 << log2TrafoHeight ) ) |  |
| } |  |
| if( !entropy_coding_mode_flag ) { |  |
| if( trafoDepth != 0 && !cbf_luma[ xBase ][ yBase ][ trafoDepth − 1 ] ) { |  |
| if( !non_square_transform_flag && cbf_cb[ xBase ][ yBase ][ trafoDepth − 1 ] && log2TrafoSize > Log2MinTrafoSize ) |  |
| cbf_cb[ x0 ][ y0 ] [ trafoDepth ] | u(1) |
| if( !non_square_transform_flag && cbf_cr[ xBase ][ yBase ][ trafoDepth − 1 ] && log2TrafoSize > Log2MinTrafoSize ) |  |
| cbf_cr[ x0 ][ y0 ] [ trafoDepth ] | u(1) |
| if(log2TrafoSize <= Log2MaxTrafoSize && !intraSplitFlag && log2TrafoSize > Log2MinTrafoSize && trafoDepth < maxDepth ) |  |
| split_transform_flag[ x0 ][ y0 ] [ trafoDepth ] | u(1) |
| } else |  |
| cbp_and_split_transform | vlc(n,v) |
| } |  |
| … |  |
| if( intra_non_square_transform_enabled_flag && PredMode == MODE_INTRA && trafoDepth == 0 && log2TrafoSize > 2 && log2TrafoSize < 6 && split_transform_flag[ x0 ][ y0 ][ trafoDepth ] && PartMode == PART_2Nx2N ) { |  |
| non_square_transform_flag | u(1) ¦ ae(v) |
| if( non_square_transform_flag ) |  |
| non_square_transform_direction | u(1) ¦ ae(v) |
| } |  |
| if( split_transform_flag[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| if( PredMode == MODE_INTRA ) { |  |
| TUSplitDirection = non_square_transform_flag ? (!non_square_transform_direction):2 |  |
| } else { |  |
| TUSplitDirection = InterTUSplitDirection |  |
| } |  |
| if( TUSplitDirection = = 2 ) { |  |
| … |  |
| } else { |  |
| x1 = x0 + ( ( 1 << log2TrafoWidth ) >> 2 ) * TUSplitDirection |  |
| y1 = y0 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − TUSplitDirection ) |  |
| x2 = x1 + ( ( 1 << log2TrafoWidth ) >> 2 ) * TUSplitDirection |  |
| y2 = y1 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − TUSplitDirection ) |  |
| x3 = x2 + ( ( 1 << log2TrafoWidth ) >> 2 ) * TUSplitDirection |  |
| y3 = y2 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − TUSplitDirection ) |  |
| log2TrafoHeight = log2TrafoHeight + 2 * TUSplitDirection – 1 |  |
| log2TrafoWidth = log2TrafoWidth − 2 * TUSplitDirection + 1 |  |
| } |  |
| … |  |
| } else if( entropy_coding_mode_flag ) { |  |
| … |  |
| if( PredMode = = MODE_INTRA && ( !non_square_transform_flag ¦ ¦ trafoDepth == 0)) |  |
| … |  |
| } |  |
| } |  |
| } |  |

7.4.8 Transform tree semantics

non_square_transform_flag specifies a block is split into whether non-square blocks or square blocks when the block is split into blocks for the purpose of transform coding. non_square_transform_flag equal to 1 specifies a block is split into non-square blocks. non_square_transform_flag equal to 0 specifies a block is split into square blocks. When sdip_flag is not present, it shall be inferred to be equal to 0.

Note that non_square_transform_flag shall be initialized to 0 outside transform_tree().

non_square_transform_direction specifies a block is split along whether the horizontal side or the vertical side when the block is split into non-square blocks. When non_square_transform_direction is equal to 1, the width of each block after splitting is the same as the width of the block before splitting, while the height of the each block after splitting is one-fourth of the height of the block before splitting. When non_square_transform_direction is equal to 0, the width of each block after splitting is one-fourth of the width of the block before splitting, while the height of the each block after splitting is the same as the height of the block before splitting.

# Patent rights declaration(s)

Huawei, Microsoft, HiSilicon, Tsinghua and USTC may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).