| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, 21-30 November, 2011 | Document: JCTVC-G556 |
| --- | --- |

| Title: | CE6.c Report on Simplification of Short Distance Intra Prediction Method |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoran Cao, Tsinghua Xiulian Peng, USTC Changcai Lai, HiSilicon Haitao Yang, Huawei Jizheng Xu, Microsoft Lingzhi Liu, HiSilicon Jianhua Zheng, HiSilicon Yun He, Tsinghua Haoping Yu, Huawei | Email: | laichangcai@huawei.com jzxu@microsoft.com haoping.yu@huawei.com |
| Source: | Tsinghua University, University of Science and Technology of China, HiSilicon, Microsoft, Huawei |  |  |

_____________________________

# Abstract

This document reports the experimental results of CE6.c subset for simplification of SDIP (short distance intra prediction) in the common test condition test sequences. Totally three cases of results were reported for achieving different trade-off between coding efficiency and encoding/decoding running time by respective simplification method. 1.4%/2.3% (AIHE/AILC) gain with 127%/134% encoding time and 101%/101% decoding time can be achieved for the first case, when with classF, the gain is 2.1%/3.0%. For the second case, 1.0%/1.6% (AIHE/AILC) gain with 113%/118% encoding time and 101%/101% decoding time can be achieved, when with classF, the gain is 1.7%/2.3. For the third case, without classF, 0.8%/1.1% (AIHE/AILC) gain with 114%/115% encoding time and 101%/102% decoding time can be achieved, when with classF, the gain is 1.1%/1.5%. Compared with Turin meeting, the encoding time increase was greatly reduced while the gain was retained.

# Introduction

SDIP was adopted into HM3.0-sdip branch at Geneva in March of this year. Harmonization between SDIP and other intra tools in HM software were investigated and the harmonization effort was judged to be satisfactory at Turin meeting in July. This CE subset is to conduct further tests, based on the comments received in the discussions at Turin meeting, on the tradeoff of coding efficiency and encoding/decoding running time of the harmonized SDIP integrated on the HM4.0 software platform.

# SDIP description and results in CE6.c

The harmonized SDIP in Turin meeting introduced two types of PUs which were named as 2NxhN and hNx2N for CUs whose size is smaller than 64x64, where h means half or 0.5. 2NxhN and hNx2N can also be called as SDIP PUs. 2NxhN includes 32x8, 16x4 and 8x2 PU, and similarly, hNx2N includes 8x32, 4x16 and 2x8 PU for 32x32 to 8x8 CU. 1x16 and 16x1 are the second depth of transform unit of 4x16 and 16x4 PU, 2x32 and 32x2 are the second depth of transform unit of 8x32 and 32x8 PU. In this test, 1x16/16x1 TU were removed according to the expert’s suggestion by considering of the complexity. The candidate non-square PUs are summarized as Table 1. For a PU in intra slice, one flag is needed to identify the PU is square PU or rectangle PU, if it’s a rectangle PU, then another flag is needed to identify this PU is a 2NxhN or hNx2N PU.

The same intra prediction is used for non-square prediction blocks, 18 modes were supported in 2x8/8x2 PU, 35 modes were supported in 4x16/16x4 and 8x32/32x8 PU, planar mode and DC mode were harmonized as described in JCTVC-F518[1] and JCTVC-F584[2], MDIS is used to generate the reference pixels according to the PU size and prediction modes [3], the residuals of a non-square predicted unit were transformed by a related size of non-square transform unit as NSQT did for inter non-square prediction units in HM4 in the structure of harmonized non-square RQT structure [4], except for introducing a 2x2 DCT transform core for 2x8/8x2 in SDIP. For 4x16/16x4 TU, mode dependant 4x4 DCT/DST is used as JCTVC-F506 [5]. The harmonized MDCS was used to scan the quantized transform coefficients as described in JCTVC-F197[6], and then entropy coded by reusing the square contexts and entropy coding engine. For 8x32/32x8 and 4x16/16x4 PU, the deblocking filtering in HM4 is also used for the edges belong to 8x8 blocks [7], all the WD text changes were described in attached file.

Table 1. Non square PU in SDIP           Figure 1. Non square PU early skip

| CU size | Non-square PUs | Intra Mode number |
| --- | --- | --- |
| 64x64 | - |  |
| 32x32 | 32x8/8x32 | 35 |
| 16x16 | 16x4/4x16 | 35 |
| 8x8 | 8x2/2x8 | 18 |

## Simplification of SDIP and results

Based on the above described harmonized SDIP, three cases of simplification were made for SDIP to find the best trade-off between coding efficiency and encoding complexity, both case_1 and case_2 are encoder only simplification and case_3 need some change at decoder based on SDIP.

### case_1: early skip based simplification

In this case, the non square PU type maybe skipped according to the coding results of square PU, as illustrated in Figure 1. The coding information of 2Nx2N PU type is used to guide the coding of non square PUs. For a size of CU in Intra slice, totally four candidate partition units may be selected to code a CU, which include square PU (2Nx2N, NxN) and non-square PU (hNx2N, 2NxhN). Square PU is coded as the same as HM4.0, and the bits of 2Nx2N PU type was saved and compared with a threshold before encoding the non square PU types. If the bits of 2Nx2N PU is smaller than the threshold, this CU is generally belongs to a smooth texture, which is not the favorite texture for non square PU types, so hNx2N and 2NxhN PU types will be skipped and not tested in the RDO loop. Since 8x32/32x8 PU are favorite of the regions without too much textures, for 8x32/32x8 PU, the skip condition is that the bits of 2Nx2N is larger than a threshold. The threshold T is calculated as formula (1) when QP is not equal to 0:

T = a *(64.0 / ((QP < 32)? QP: 64))                                        (1)

Where variable a set as 5, 10 and 300 for 8x8, 16x16 and 32x32 CU respectively.

For a kind of PU, an intra prediction mode should be decided by two steps in HM4, at the first step, a set of M candidate modes were selected from total N modes (M<= N) by minimize the HAD based RD cost, where N is the intra mode number in Table 1, then the final mode was selected from the M candidate modes and the most probable modes by calculate the real rate distortion cost. In case_1 of SDIP, we set the M as 2 for non square PUs and the same for square PUs as HM4. In addition, at the first step of non square PUs in a CU, not all of the N modes will be tested but only the following modes will be tested: the M candidate modes selected in 2Nx2N and the modes whose number is smaller than N>>1. Table 2&3 are the results with and without classF sequences for case_1.

Table 2 Results of case_1 without classF

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.5% | -1.5% | -1.9% | -1.2% | -1.9% | -2.5% |
| Class B | -1.2% | -2.4% | -2.6% | -2.3% | -2.4% | -3.0% |
| Class C | -1.5% | -1.4% | -1.7% | -2.3% | -2.4% | -2.9% |
| Class D | -1.5% | -1.2% | -1.1% | -2.1% | -2.1% | -2.1% |
| Class E | -2.2% | -5.3% | -4.9% | -3.8% | -7.1% | -5.6% |
| Overall | -1.35% | -2.22% | -2.34% | -2.28% | -2.95% | -3.10% |
|  | -1.35% | -2.21% | -2.32% | -2.28% | -2.93% | -3.07% |
| Enc Time[%] | 127% |  |  | 135% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.5% | -0.7% | -1.2% | -0.9% | -0.7% | -1.2% |
| Class B | -0.7% | -1.2% | -1.3% | -1.2% | -1.4% | -1.4% |
| Class C | -0.8% | -0.8% | -1.0% | -1.2% | -1.3% | -1.4% |
| Class D | -0.6% | -0.6% | -0.6% | -0.9% | -1.0% | -1.0% |
| Class E |  |  |  |  |  |  |
| Overall | -0.63% | -0.86% | -1.05% | -1.05% | -1.13% | -1.24% |
|  | -0.63% | -0.90% | -1.08% | -1.06% | -1.12% | -1.22% |
| Enc Time[%] | 105% |  |  | 104% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | -0.3% | -0.8% | -0.3% | -0.4% | -0.5% |
| Class C | -0.3% | -0.5% | -0.6% | -0.4% | -0.3% | -0.8% |
| Class D | -0.1% | -0.1% | -0.4% | -0.3% | -0.4% | -1.2% |
| Class E | -0.6% | -2.5% | -2.3% | -0.9% | -2.3% | -2.0% |
| Overall | -0.27% | -0.69% | -0.93% | -0.46% | -0.73% | -1.04% |
|  | -0.27% | -0.72% | -0.89% | -0.46% | -0.73% | -0.99% |
| Enc Time[%] | 104% |  |  | 103% |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  |

Table 3 Results of case_1 with classF.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.5% | -1.5% | -1.9% | -1.2% | -1.9% | -2.5% |
| Class B | -1.2% | -2.4% | -2.6% | -2.3% | -2.4% | -3.0% |
| Class C | -1.5% | -1.4% | -1.7% | -2.3% | -2.4% | -2.9% |
| Class D | -1.5% | -1.2% | -1.1% | -2.1% | -2.1% | -2.1% |
| Class E | -2.2% | -5.3% | -4.9% | -3.8% | -7.1% | -5.6% |
| Class F | -6.1% | -5.0% | -5.1% | -6.7% | -5.1% | -5.2% |
| Overall | -2.1% | -2.7% | -2.8% | -3.0% | -3.3% | -3.4% |
|  | -2.1% | -2.7% | -2.8% | -3.0% | -3.3% | -3.4% |
| Enc Time[%] | 127% |  |  | 134% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.5% | -0.7% | -1.2% | -0.9% | -0.7% | -1.2% |
| Class B | -0.7% | -1.2% | -1.3% | -1.2% | -1.4% | -1.4% |
| Class C | -0.8% | -0.8% | -1.0% | -1.2% | -1.3% | -1.4% |
| Class D | -0.6% | -0.6% | -0.6% | -0.9% | -1.0% | -1.0% |
| Class E |  |  |  |  |  |  |
| Class F | -4.4% | -3.7% | -4.0% | -4.9% | -3.9% | -4.1% |
| Overall | -1.4% | -1.4% | -1.6% | -1.8% | -1.7% | -1.8% |
|  | -1.3% | -1.4% | -1.6% | -1.8% | -1.7% | -1.8% |
| Enc Time[%] | 105% |  |  | 104% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | -0.3% | -0.8% | -0.3% | -0.4% | -0.5% |
| Class C | -0.3% | -0.5% | -0.6% | -0.4% | -0.3% | -0.8% |
| Class D | -0.1% | -0.1% | -0.4% | -0.3% | -0.4% | -1.2% |
| Class E | -0.6% | -2.5% | -2.3% | -0.9% | -2.3% | -2.0% |
| Class F | -2.9% | -2.9% | -2.6% | -2.9% | -2.5% | -2.7% |
| Overall | -0.8% | -1.1% | -1.3% | -0.9% | -1.1% | -1.4% |
|  | -0.8% | -1.1% | -1.2% | -1.0% | -1.1% | -1.3% |
| Enc Time[%] | 104% |  |  | 103% |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  |

### case_2: fast non-square PU selection

In addition to the early skip method described in case_1, a fast non-square PU selection method was used to further reduce the encoder complexity in this case. At the stage of 2Nx2N encoding, the best intra mode for 2Nx2N and its RD cost were saved, if non square PU type should be tested judged by early skip method in case_1, then each PU with hNx2N and 2NxhN type will be encoded with the best mode of 2Nx2N type. The best non square PU type will be selected to compare with the RD cost of 2Nx2N, if the RD cost of the best non square PU (ie.2NxhN) is smaller than that of 2Nx2N, 2NxhN will do a real RDO and choose the intra mode for each PU of 2NxhN type from N Modes, so only one non square PU type need to be tested in RDO for each CU. Table 4&5 are the results with and without classF sequences for case_2.

Table 4 Results of case_2 without classF

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.3% | -1.1% | -1.4% | -0.8% | -1.3% | -1.8% |
| Class B | -0.9% | -2.0% | -2.1% | -1.6% | -1.6% | -2.1% |
| Class C | -1.0% | -1.0% | -1.3% | -1.6% | -1.8% | -2.2% |
| Class D | -1.1% | -0.9% | -0.9% | -1.6% | -1.6% | -1.7% |
| Class E | -1.6% | -4.3% | -3.9% | -2.7% | -5.7% | -4.4% |
| Overall | -1.0% | -1.7% | -1.8% | -1.6% | -2.2% | -2.4% |
|  | -1.0% | -1.7% | -1.8% | -1.6% | -2.2% | -2.3% |
| Enc Time[%] | 113% |  |  | 118% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -0.3% | -0.6% | -0.6% | -0.4% | -0.6% |
| Class B | -0.4% | -0.9% | -1.2% | -0.8% | -0.9% | -1.1% |
| Class C | -0.5% | -0.8% | -0.8% | -0.8% | -1.0% | -1.1% |
| Class D | -0.5% | -0.5% | -0.8% | -0.7% | -1.0% | -0.8% |
| Class E |  |  |  |  |  |  |
| Overall | -0.41% | -0.67% | -0.87% | -0.71% | -0.84% | -0.90% |
|  | -0.41% | -0.67% | -0.89% | -0.71% | -0.81% | -0.89% |
| Enc Time[%] | 105% |  |  | 104% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -0.4% | -0.3% | -0.2% | -0.2% | -0.6% |
| Class C | -0.2% | -0.2% | -0.4% | -0.3% | -0.3% | -0.8% |
| Class D | -0.1% | -0.2% | -0.7% | -0.2% | -0.2% | -0.7% |
| Class E | -0.4% | -1.1% | -1.4% | -0.7% | -3.2% | -2.3% |
| Overall | -0.18% | -0.43% | -0.66% | -0.32% | -0.77% | -1.00% |
|  | -0.18% | -0.41% | -0.66% | -0.32% | -0.82% | -0.99% |
| Enc Time[%] | 104% |  |  | 103% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |

Table 5 Results of case_2 with classF

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.3% | -1.1% | -1.4% | -0.8% | -1.3% | -1.8% |
| Class B | -0.9% | -2.0% | -2.1% | -1.6% | -1.6% | -2.1% |
| Class C | -1.0% | -1.0% | -1.3% | -1.6% | -1.8% | -2.2% |
| Class D | -1.1% | -0.9% | -0.9% | -1.6% | -1.6% | -1.7% |
| Class E | -1.6% | -4.3% | -3.9% | -2.7% | -5.7% | -4.4% |
| Class F | -5.3% | -4.2% | -4.3% | -5.5% | -4.1% | -4.1% |
| Overall | -1.7% | -2.1% | -2.2% | -2.3% | -2.5% | -2.6% |
|  | -1.7% | -2.1% | -2.2% | -2.3% | -2.5% | -2.6% |
| Enc Time[%] | 113% |  |  | 118% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -0.3% | -0.6% | -0.6% | -0.4% | -0.6% |
| Class B | -0.4% | -0.9% | -1.2% | -0.8% | -0.9% | -1.1% |
| Class C | -0.5% | -0.8% | -0.8% | -0.8% | -1.0% | -1.1% |
| Class D | -0.5% | -0.5% | -0.8% | -0.7% | -1.0% | -0.8% |
| Class E |  |  |  |  |  |  |
| Class F | -3.8% | -3.1% | -3.4% | -4.0% | -3.0% | -3.1% |
| Overall | -1.1% | -1.1% | -1.3% | -1.3% | -1.2% | -1.3% |
|  | -1.1% | -1.1% | -1.4% | -1.3% | -1.2% | -1.3% |
| Enc Time[%] | 105% |  |  | 105% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -0.4% | -0.3% | -0.2% | -0.2% | -0.6% |
| Class C | -0.2% | -0.2% | -0.4% | -0.3% | -0.3% | -0.8% |
| Class D | -0.1% | -0.2% | -0.7% | -0.2% | -0.2% | -0.7% |
| Class E | -0.4% | -1.1% | -1.4% | -0.7% | -3.2% | -2.3% |
| Class F | -2.3% | -2.2% | -2.2% | -2.3% | -2.0% | -2.0% |
| Overall | -0.6% | -0.8% | -1.0% | -0.7% | -1.0% | -1.2% |
|  | -0.6% | -0.8% | -1.0% | -0.7% | -1.0% | -1.2% |
| Enc Time[%] | 104% |  |  | 103% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |

### case_3: NSQT for intra

In this case, the NSQT_INTRA introduces non-square quad-tree transform (NSQT) to intra CU. On one hand, it can be seen as an extension of NSQT from inter CU to intra CU. On the other hand, it can be seen as a simplification of the SDIP technique since all non-square partitions within a CU are TUs sharing one prediction mode, while each non-square partition is a PU and can have individual prediction mode in the original SDIP design. Therefore most of the modules in the implementation of SDIP can be re-used for NSQT_INTRA. The major changes from SDIP to NSQT_INTRA in the implementation are, in brief, 1) remove non-square PU and introduce non-square TU instead; 2) move SDIP side info from PU level to TU level; 3) remove modifications introduced to chroma components. Details are provided below.

All non-square partitions are “TUs” in NSQT_INTRA rather than “PUs” in SDIP. So partition PART_2NxhN and PART_hNx2N are not used in intra CU. Instead, a set of NSQT partitions are introduced to complement the existing square partitions. For different CU sizes, the NSQT partitions added by NSQT_INTRA are listed in Table 6. The candidate non-square partitions are the same as those in the current SDIP scheme.

Table 6 NSQT partitions supported by NSQT_INTRA

| CU size | NSQT partitions |
| --- | --- |
| 64x64 | - |
| 32x32 | 32x8/8x32, 32x2/2x32 |
| 16x16 | 16x4/4x16 |
| 8x8 | 8x2/2x8 |

A flag non_square_transform_flag is added to signal the decoder whether the original square quad-tree partition or the non-square quad-tree partition is used for each intra CU. To distinguish between horizontal and vertical partition, an additional flag non_square_transform_direction is also sent to the decoder when non_square_transform_flag is true. The above described side info is written into bitstream if a CU contains more than one TUs as indicated by split_transform_flag or IntraSplitFlag.

The intra chroma prediction method in the current SDIP design composes the candidate prediction mode list based on the mode statistics of all non-square PUs. Since there is no non-square PUs in NSQT_INTRA, the intra chroma prediction method introduced to SDIP can not be applied to NSQT_INTRA. Therefore, the original HM scheme is maintained, with restriction of only 1-level quad-tree TU. In other words, the chroma TU would not be split when NSQT_INTRA is used.

As to the encoder implementation, the NSQT_INTRA firstly choose the best prediction mode for a CU, and then try different quad-tree TU partition, including square and non-square TU partitions at different depth, to decide the RQT structure. No encoder speedup is used in the current implementation. Obviously, the encoder complexity of NSQT_INTRA is essentially much less than the SDIP implementation, especially a naive one where all prediction modes are tried for each non-square PU for mode decision. Table 7&8 are the results with and without classF sequences for case_3, and Table 9&10 summarized the average results with and without classF for each case.

Table 7 Results of case_3 without classF

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -0.8% | -0.9% | -0.5% | -1.1% | -1.4% |
| Class B | -0.8% | -1.6% | -1.7% | -1.4% | -1.7% | -1.9% |
| Class C | -0.7% | -0.7% | -0.8% | -0.8% | -1.0% | -1.2% |
| Class D | -0.6% | -0.5% | -0.5% | -0.7% | -0.8% | -0.8% |
| Class E | -1.6% | -3.1% | -2.8% | -2.4% | -4.9% | -3.7% |
| Overall | -0.76% | -1.28% | -1.29% | -1.09% | -1.72% | -1.71% |
|  | -0.76% | -1.27% | -1.27% | -1.09% | -1.74% | -1.70% |
| Enc Time[%] | 114% |  |  | 115% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -1.0% | -0.4% | -0.3% | -0.8% | -0.9% |
| Class B | -0.4% | -0.8% | -0.9% | -0.7% | -1.2% | -1.2% |
| Class C | -0.3% | -0.5% | -0.4% | -0.3% | -0.6% | -0.6% |
| Class D | -0.2% | -0.5% | -0.5% | -0.2% | -0.7% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | -0.27% | -0.68% | -0.57% | -0.40% | -0.82% | -0.71% |
|  | -0.27% | -0.70% | -0.58% | -0.40% | -0.81% | -0.68% |
| Enc Time[%] | 102% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -0.4% | -0.4% | -0.2% | -0.5% | -0.3% |
| Class C | -0.1% | -0.3% | -0.6% | -0.1% | -0.1% | -0.3% |
| Class D | -0.1% | -0.1% | 0.0% | 0.0% | 0.3% | -0.2% |
| Class E | -0.5% | -2.4% | -1.5% | -0.7% | -2.8% | -2.1% |
| Overall | -0.17% | -0.66% | -0.54% | -0.23% | -0.62% | -0.64% |
|  | -0.17% | -0.61% | -0.50% | -0.23% | -0.68% | -0.57% |
| Enc Time[%] | 102% |  |  | 101% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |

Table 8 Results of case_3 without classF

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -0.8% | -0.9% | -0.5% | -1.1% | -1.4% |
| Class B | -0.8% | -1.6% | -1.7% | -1.4% | -1.7% | -1.9% |
| Class C | -0.7% | -0.7% | -0.8% | -0.8% | -1.0% | -1.2% |
| Class D | -0.6% | -0.5% | -0.5% | -0.7% | -0.8% | -0.8% |
| Class E | -1.6% | -3.1% | -2.8% | -2.4% | -4.9% | -3.7% |
| Class F | -3.1% | -2.5% | -2.2% | -3.4% | -2.5% | -2.5% |
| Overall | -1.1% | -1.5% | -1.4% | -1.5% | -1.9% | -1.8% |
|  | -1.1% | -1.5% | -1.4% | -1.5% | -1.9% | -1.8% |
| Enc Time[%] | 114% |  |  | 115% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -1.0% | -0.4% | -0.3% | -0.8% | -0.9% |
| Class B | -0.4% | -0.8% | -0.9% | -0.7% | -1.2% | -1.2% |
| Class C | -0.3% | -0.5% | -0.4% | -0.3% | -0.6% | -0.6% |
| Class D | -0.2% | -0.5% | -0.5% | -0.2% | -0.7% | -0.1% |
| Class E |  |  |  |  |  |  |
| Class F | -2.1% | -1.7% | -1.8% | -2.4% | -1.9% | -1.9% |
| Overall | -0.6% | -0.9% | -0.8% | -0.8% | -1.0% | -0.9% |
|  | -0.6% | -0.9% | -0.8% | -0.8% | -1.0% | -0.9% |
| Enc Time[%] | 102% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -0.4% | -0.4% | -0.2% | -0.5% | -0.3% |
| Class C | -0.1% | -0.3% | -0.6% | -0.1% | -0.1% | -0.3% |
| Class D | -0.1% | -0.1% | 0.0% | 0.0% | 0.3% | -0.2% |
| Class E | -0.5% | -2.4% | -1.5% | -0.7% | -2.8% | -2.1% |
| Class F | -1.4% | -1.1% | -1.1% | -1.4% | -1.0% | -1.0% |
| Overall | -0.4% | -0.7% | -0.6% | -0.5% | -0.7% | -0.7% |
|  | -0.4% | -0.7% | -0.6% | -0.5% | -0.7% | -0.7% |
| Enc Time[%] | 102% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |

Table 9 Summary of the average results without classF for each case

| Test case |  | BD-RATE |  |  |  |  |  | Enc/Dec Time |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | HE |  |  | LC |  |  | HE |  | LC |  |
|  |  | Y | U | V | Y | U | V | Enc time | Dec time | Enc time | Dec time |
| case_1 | Intra | -1.35% | -2.22% | -2.34% | -2.28% | -2.95% | -3.10% | 127% | 101% | 135% | 102% |
|  | RA | -0.63% | -0.86% | -1.05% | -1.05% | -1.13% | -1.24% | 105% | 100% | 104% | 99% |
|  | LD | -0.27% | -0.69% | -0.93% | -0.46% | -0.73% | -1.04% | 104% | 98% | 103% | 100% |
| case_2 | Intra | -0.96% | -1.74% | -1.82% | -1.60% | -2.20% | -2.35% | 113% | 102% | 118% | 101% |
|  | RA | -0.41% | -0.67% | -0.87% | -0.71% | -0.84% | -0.90% | 105% | 99% | 104% | 99% |
|  | LD | -0.18% | -0.43% | -0.66% | -0.32% | -0.77% | -1.00% | 104% | 98% | 103% | 99% |
| case_3 | Intra | -0.76% | -1.28% | -1.29% | -1.09% | -1.72% | -1.71% | 114% | 102% | 115% | 102% |
|  | RA | -0.27% | -0.68% | -0.57% | -0.40% | -0.82% | -0.71% | 102% | 99% | 100% | 99% |
|  | LD | -0.17% | -0.66% | -0.54% | -0.23% | -0.62% | -0.64% | 102% | 98% | 101% | 99% |

Table 10 Summary of the average results with classF for each case

| Test case |  | BD-RATE |  |  |  |  |  | Enc/Dec Time |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | HE |  |  | LC |  |  | HE |  | LC |  |
|  |  | Y | U | V | Y | U | V | Enc time | Dec time | Enc time | Dec time |
| case_1 | Intra | -2.14% | -2.69% | -2.80% | -3.01% | -3.31% | -3.44% | 127% | 101% | 134% | 101% |
|  | RA | -1.40% | -1.40% | -1.60% | -1.80% | -1.70% | -1.80% | 105% | 100% | 104% | 99% |
|  | LD | -0.80% | -1.10% | -1.30% | -0.90% | -1.10% | -1.40% | 104% | 98% | 103% | 100% |
| case_2 | Intra | -1.70% | -2.10% | -2.20% | -2.30% | -2.50% | -2.60% | 113% | 101% | 118% | 101% |
|  | RA | -1.10% | -1.10% | -1.30% | -1.30% | -1.20% | -1.30% | 105% | 99% | 105% | 98% |
|  | LD | -0.60% | -0.80% | -1.00% | -0.70% | -1.00% | -1.20% | 104% | 98% | 103% | 99% |
| case_3 | Intra | -1.10% | -1.50% | -1.40% | -1.50% | -1.90% | -1.80% | 114% | 101% | 115% | 102% |
|  | RA | -0.60% | -0.90% | -0.80% | -0.80% | -1.00% | -0.90% | 102% | 99% | 100% | 98% |
|  | LD | -0.40% | -0.70% | -0.60% | -0.50% | -0.70% | -0.70% | 102% | 98% | 100% | 99% |

# Conclusion

Compared with Turin meeting, the encoding time increase was greatly reduced while the gain was retained. Based on the significant gain and drastically reduced complexity, we recommend adopt SDIP into HM.

# References

- Yongbing Lin, Changcai Lai, Xiaoran, Cao, etc. “CE6.b Test6: Report of harmonization on Planar prediction in SDIP”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F518, 6th Meeting: Torino, IT, 14-22 July, 2011.

- Yongbing Lin, Changcai Lai, “Modification to DC prediction in SDIP”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F584, 6th Meeting: Torino, IT, 14-22 July, 2011.

- Guichun Li, Lingzhi Liu, Changcai Lai, etc. “CE6.b.5 Report: Harmonization of SDIP and MDIS”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC- F336, 6th Meeting: Torino, IT, 14-22 July, 2011.

- Xiaoran Cao, Xiulian Peng, Changcai Lai, etc. “CE6.b Report on test1 Harmonization of SDIP and RQT”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC- F533, 6th Meeting: Torino, IT, 14-22 July, 2011.

- Haitao Yang, Jiantong Zhou, Haoping Yu, etc. “CE6.b: Harmonization of SDIP and MDDT”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC- F506, 6th Meeting: Torino, IT, 14-22 July, 2011.

- Xiulian Peng, Jizheng Xu, “CE6.b Report of Test 3: Interaction between SDIP and MDCS”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC- F197, 6th Meeting: Torino, IT, 14-22 July, 2011.

- Xiaoran Cao, Xiulian Peng, Changcai Lai, etc. “CE6.b Report on test7 Harmonization of SDIP and deblocking filter”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC- F536, 6th Meeting: Torino, IT, 14-22 July, 2011.

# Syntax and Semantics changes to WD text

Syntax and Semantics changes are listed in this document, the full changes of WD can be found in the attached document of this proposal.

## Syntax and Semantics changes for case1 and case2 of SDIP

#### Sequence parameter set RBSP syntax

| …… |  |
| --- | --- |
| cu_qp_delta_enabled_flag | u(1) |
| intra_sdip_enabled_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| ...... |  |
| } |  |

### Coding unit syntax

| coding_unit( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if( entropy_coding_mode_flag && slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| else { |  |
| sdipEnabledFlag = intra_sdip_enabled_flag && log2CUSize < 6 |  |
| if( !entropy_coding_mode_flag ) { |  |
| if( slice_type == I && log2CUSize == Log2MinCUSize ) |  |
| intra_part_mode | u(1) |
| } else if( slice_type != I ¦ ¦ log2CUSize = = Log2MinCUSize ) |  |
| pred_type | u(v) ¦ ae(v) |
| if ( sdipEnabledFlag && PredMode == MODE_INTRA && (log2CUSize > Log2MinCUSize ¦ ¦ PartMode == PART_NxN) ) { |  |
| sdip_flag | ae(v) |
| if ( sdip_flag ) |  |
| sdip_direction | ae(v) |
| } |  |
| x1 = x0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| x2 = x1 − ( ( 1 << log2CUSize ) >> 2 ) |  |
| y2 = y1 − ( ( 1 << log2CUSize ) >> 2 ) |  |
| x3 = x1 + ( ( 1 << log2CUSize ) >> 2 ) |  |
| y3 = y1 + ( ( 1 << log2CUSize ) >> 2 ) |  |
| if( PartMode == PART_2Nx2N ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| } else if( PartMode == PART_2NxN ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x0, y1 , log2CUSize ) |  |
| } else if( PartMode == PART_Nx2N ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x1, y0 , log2CUSize ) |  |
| } else if( PartMode == PART_2NxnU ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x0, y2 , log2CUSize ) |  |
| } else if( PartMode == PART_2NxnD ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x0, y3 , log2CUSize ) |  |
| } else if( PartMode == PART_nLx2N ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x2, y0 , log2CUSize ) |  |
| } else if( PartMode == PART_nRx2N ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x3, y0 , log2CUSize ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x1, y0 , log2CUSize ) |  |
| prediction_unit( x0, y1 , log2CUSize ) |  |
| prediction_unit( x1, y1 , log2CUSize ) |  |
| } else if ( PartMode == PART_2NxhN ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x0, y2 , log2CUSize ) |  |
| prediction_unit( x0, y1 , log2CUSize ) |  |
| prediction_unit( x0, y3 , log2CUSize ) |  |
| } else if( PartMode == PART_hNx2N ) { |  |
| prediction_unit( x0, y0 , log2CUSize ) |  |
| prediction_unit( x2, y0 , log2CUSize ) |  |
| prediction_unit( x1, y0 , log2CUSize ) |  |
| prediction_unit( x3, y0 , log2CUSize ) |  |
| } |  |
| } |  |
| if( !pcm_flag ) { |  |
| transform_tree( x0, y0, log2CUSize, log2CUSize, 0, 0 ) |  |
| transform_coeff( x0, y0, log2CUSize, log2CUSize, 0, 0 ) |  |
| transform_coeff( x0, y0, log2CUSize, log2CUSize, 0, 1 ) |  |
| transform_coeff( x0, y0, log2CUSize, log2CUSize, 0, 2 ) |  |
| } |  |
| } |  |

### Transform tree syntax

| transform_tree( x0, y0, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| if( entropy_coding_mode_flag && trafoDepth = = 0 && IntraSplitFlag = = 0) { |  |
| if( PredMode != MODE_INTRA ) |  |
| no_residual_data_flag | ae(v) |
| residualDataPresentFlag = !no_residual_data_flag |  |
| } else { |  |
| residualDataPresentFlag = TRUE |  |
| } |  |
| if( residualDataPresentFlag) { |  |
| log2TrafoSize = ( log2TrafoWidth + log2TrafoHeight ) >> 1 |  |
| intraSdipFlag = (PartMode == PART_2NxhN ¦ ¦ PartMode == PART_hNx2N ) |  |
| intraSplitFlag = ( IntraSplitFlag && trafoDepth = = 0 ? 1 : 0 ) |  |
| maxDepth = ( PredMode = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| if(PredMode == MODE_INTRA && intraSdipFlag && maxDepth > 2) |  |
| maxDepth = (1 << log2TrafoSize ) <= 16 ? 1 : 2 |  |
| if( PartMode == PART_2NxhN ){ |  |
| xBase = x0 |  |
| yBase = y0 − ( y0 & ( ( 1 << (log2TrafoHeight + 2)) − 1 ) ) |  |
| }else if( PartMode == PART_hNx2N ){ |  |
| xBase = x0 − ( x0 & ( ( 1 << (log2TrafoWidth + 2)) − 1 ) ) |  |
| yBase = y0 |  |
| }else{ |  |
| xBase = x0 − ( x0 & ( 1 << log2TrafoWidth ) ) |  |
| yBase = y0 − ( y0 & ( 1 << log2TrafoHeight ) ) |  |
| } |  |
| if( !entropy_coding_mode_flag ) { |  |
| if( trafoDepth != 0 && !cbf_luma[ xBase ][ yBase ][ trafoDepth − 1 ] ) { |  |
| if(! intraSdipFlag && cbf_cb[ xBase ][ yBase ][ trafoDepth − 1 ] && log2TrafoSize > Log2MinTrafoSize ) |  |
| cbf_cb[ x0 ][ y0 ] [ trafoDepth ] | u(1) |
| if(! intraSdipFlag && cbf_cr[ xBase ][ yBase ][ trafoDepth − 1 ] && log2TrafoSize > Log2MinTrafoSize ) |  |
| cbf_cr[ x0 ][ y0 ] [ trafoDepth ] | u(1) |
| if(log2TrafoSize <= Log2MaxTrafoSize && !intraSplitFlag && log2TrafoSize > Log2MinTrafoSize && trafoDepth < maxDepth ) |  |
| split_transform_flag[ x0 ][ y0 ] [ trafoDepth ] | u(1) |
| } else |  |
| cbp_and_split_transform | vlc(n,v) |
| } |  |
| if( log2TrafoSize <= Log2MaxTrafoSize && log2TrafoSize > Log2MinTrafoSize && trafoDepth < maxDepth && !intraSplitFlag && entropy_coding_mode_flag ) |  |
| split_transform_flag[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| if( PredMode != MODE_INTRA && log2TrafoSize <= Log2MaxTrafoSize && entropy_coding_mode_flag ) { |  |
| firstChromaCbf = ( log2TrafoSize = = Log2MaxTrafoSize ¦ ¦ trafoDepth = = 0 ? 1 : 0 ) |  |
| if( firstChromaCbf ¦ ¦ log2TrafoSize > Log2MinTrafoSize ) { |  |
| if( firstChromaCbf ¦ ¦ cbf_cb[ xBase ][ yBase ][ trafoDepth − 1 ] ) { |  |
| readCbf = true |  |
| if( blkIdx = = 3 && log2TrafoSize < Log2MaxTrafoSize ) |  |
| readCbf = cbf_cb[ xBase ][ yBase ][ trafoDepth ] ¦ ¦ cbf_cb[ xBase + ( 1 << log2TrafoWidth ) ][ yBase ][ trafoDepth ] ¦ ¦ cbf_cb[ xBase ][ yBase + ( 1 << log2TrafoHeight ) ][ trafoDepth ] |  |
| if ( !readCbf ) |  |
| cbf_cb[ x0 ][ y0 ][ trafoDepth ] = 1 |  |
| else |  |
| cbf_cb[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| } |  |
| if( firstChromaCbf ¦ ¦ cbf_cr[ xBase ][ yBase ][ trafoDepth − 1 ] ) { |  |
| readCbf = true |  |
| if( blkIdx = = 3 && log2TrafoSize < Log2MaxTrafoSize ) |  |
| readCbf = cbf_cr[ xBase ][ yBase ][ trafoDepth ] ¦ ¦ cbf_cr[ xBase + ( 1 << log2TrafoWidth ) ][ yBase ][ trafoDepth ] ¦ ¦ cbf_cr[ xBase ][ yBase + ( 1 << log2TrafoHeight ) ][ trafoDepth ] |  |
| if ( !readCbf ) |  |
| cbf_cr[ x0 ][ y0 ][ trafoDepth ] = 1 |  |
| else |  |
| cbf_cr[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| } |  |
| } |  |
| } |  |
| if( split_transform_flag[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| if( PartMode == PART_2NxhN){ |  |
| x1 = x0 |  |
| y1 = y0 + ( ( 1 << log2TrafoHeight) >> 2 ) |  |
| x2 = x0 |  |
| y2 = y1 + ( ( 1 << log2TrafoHeight) >> 2 ) |  |
| x3 = x0 |  |
| y3 = y2 + ( ( 1 << log2TrafoHeight) >> 2 ) |  |
| log2TrafoWidth = log2TrafoWidth + 1 |  |
| log2TrafoHeight = log2TrafoHeight − 1 |  |
| } else if ( PartMode == PART_hNx2N){ |  |
| x1 = x0 + ( ( 1 << log2TrafoWidth) >> 2 ) |  |
| y1 = y0 |  |
| x2 = x1 + ( ( 1 << log2TrafoWidth) >> 2 ) |  |
| y2 = y0 |  |
| x3 = x2 + ( ( 1 << log2TrafoWidth) >> 2 ) |  |
| y3 = y0 |  |
| log2TrafoWidth = log2TrafoWidth − 1 |  |
| log2TrafoHeight = log2TrafoHeight + 1 |  |
| } |  |
| else if( InterTUSplitDirection = = 2 ) { |  |
| x1 = x0 + ( ( 1 << log2TrafoWidth ) >> 1 ) |  |
| y1 = y0 |  |
| x2 = x0 |  |
| y2 = y0 + ( ( 1 << log2TrafoHeight ) >> 1 ) |  |
| x3 = x1 |  |
| y3 = y2 |  |
| } else { |  |
| x1 = x0 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y1 = y0 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| x2 = x1 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y2 = y1 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| x3 = x2 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y3 = y2 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| log2TrafoHeight = log2TrafoHeight + 2 * InterTUSplitDirection − 1 |  |
| log2TrafoWidth = log2TrafoWidth − 2 * InterTUSplitDirection + 1 |  |
| } |  |
| transform_tree( x0, y0, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 0 ) |  |
| transform_tree( x1, y1, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 1 ) |  |
| transform_tree( x2, y2, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 2 ) |  |
| transform_tree( x3, y3, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 3 ) |  |
| } else if( entropy_coding_mode_flag ) { |  |
| if( PredMode = = MODE_INTRA ¦ ¦ trafoDepth != 0 ¦ ¦ cbf_cb[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| readCbf = true |  |
| if( blkIdx = = 3 && log2TrafoSize < Log2MaxTrafoSize && PredMode != MODE_INTRA ) |  |
| readCbf = cbf_luma[ xBase ][ yBase ][ trafoDepth ] ¦ ¦ cbf_luma[ xBase + ( 1 << log2TrafoWidth ) ][ yBase ][ trafoDepth ] ¦ ¦ cbf_luma[ xBase ][ yBase + ( 1 << log2TrafoHeight ) ][ trafoDepth ] ¦ ¦ cbf_cb[ xBase ][ yBase ][ trafoDepth – 1] ¦ ¦ cbf_cr[ xBase ][ yBase ][ trafoDepth – 1] |  |
| if ( !readCbf ) |  |
| cbf_luma[ x0 ][ y0 ][ trafoDepth ] = 1 |  |
| else |  |
| cbf_luma[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| } |  |
| if( PredMode = = MODE_INTRA && ( ! intraSdipFlag ¦ ¦ trafoDepth == 0)) |  |
| if( log2TrafoSize > Log2MinTrafoSize ) { |  |
| cbf_cb[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| cbf_cr[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| } else if( blkIdx = = 0 ) { |  |
| cbf_cb[ x0 ][ y0 ][ trafoDepth − 1 ] | ae(v) |
| cbf_cr[ x0 ][ y0 ][ trafoDepth − 1 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

### Transform coefficient syntax

| transform_coeff( x0, y0, log2TrafoWidth, log2TrafoHeight, trafoDepth, cIdx ) { | Descriptor |
| --- | --- |
| if( ( ( cIdx = = 0 && cbf_luma[ x0 ][ y0 ][ trafoDepth ] ) ¦ ¦ ( cIdx = = 1 && cbf_cb[ x0 ][ y0 ][ trafoDepth ] ) ¦ ¦ ( cIdx = = 2 && cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) ) ) { |  |
| if( cu_qp_delta_enabled_flag && !IsCuQpDeltaCoded ) { |  |
| cu_qp_delta | se(v) ¦ ae(v) |
| IsCuQpDeltaCoded = 1 |  |
| } |  |
| log2TrafoSize = (log2TrafoWidth + log2TrafoHeight) >> 1 |  |
| if( split_transform_flag[ x0 ][ y0 ][ trafoDepth ] && ( cIdx = = 0 ¦ ¦ log2TrafoSize > Log2MinTrafoSize + 1 ) ) { |  |
| if( PartMode == PART_2NxhN){ |  |
| x1 = x0 |  |
| y1 = y0 + ( ( 1 << log2TrafoHeight) >> 2 ) |  |
| x2 = x0 |  |
| y2 = y1 + ( ( 1 << log2TrafoHeight) >> 2 ) |  |
| x3 = x0 |  |
| y3 = y2 + ( ( 1 << log2TrafoHeight) >> 2 ) |  |
| log2TrafoWidth = log2TrafoWidth + 1 |  |
| log2TrafoHeight = log2TrafoHeight − 1 |  |
| } else if ( PartMode == PART_hNx2N){ |  |
| x1 = x0 + ( ( 1 << log2TrafoWidth) >> 2 ) |  |
| y1 = y0 |  |
| x2 = x1 + ( ( 1 << log2TrafoWidth) >> 2 ) |  |
| y2 = y0 |  |
| x3 = x2 + ( ( 1 << log2TrafoWidth) >> 2 ) |  |
| y3 = y0 |  |
| log2TrafoWidth = log2TrafoWidth − 1 |  |
| log2TrafoHeight = log2TrafoHeight + 1 |  |
| } |  |
| else if( InterTUSplitDirection = = 2 ) { |  |
| x1 = x0 + ( ( 1 << log2TrafoWidth ) >> 1 ) |  |
| y1 = y0 |  |
| x2 = x0 |  |
| y2 = y0 + ( ( 1 << log2TrafoHeight ) >> 1 ) |  |
| x3 = x1 |  |
| y3 = y2 |  |
| } else { |  |
| x1 = x0 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y1 = y0 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| x2 = x1 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y2 = y1 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| x3 = x2 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y3 = y2 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| log2TrafoHeight = log2TrafoHeight + 2 * InterTUSplitDirection − 1 |  |
| log2TrafoWidth = log2TrafoWidth − 2 * InterTUSplitDirection + 1 |  |
| } |  |
| transform_coeff( x0, y0, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, cIdx ) |  |
| transform_coeff( x1, y1, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, cIdx ) |  |
| transform_coeff( x2, y2, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, cIdx ) |  |
| transform_coeff( x3, y3, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, cIdx ) |  |
| } else { |  |
| log2TrafoSize = ( ( log2TrafoWidth + log2TrafoHeight ) >> 1 ) − ( cIdx > 0 ? 1 : 2 ) |  |
| if ( PredMode = = MODE_INTRA ) { |  |
| if (cIdx = = 0) |  |
| scanIdx = ScanType[ log2TrafoSize – 2 ][ IntraPredMode ] |  |
| else |  |
| scanIdx = ScanType[ log2TrafoSize – 2 ][ IntraPredModeC ] |  |
| } else { |  |
| scanIdx = 0 |  |
| } |  |
| if ( entropy_coding_mode_flag ) |  |
| if ( scanIdx = = 0 ) |  |
| scanIdx = 3 |  |
| residual_coding_cabac( x0, y0, log2TrafoSize, trafoDepth, scanIdx, cIdx ) |  |
| else if ( !entropy_coding_mode_flag ) |  |
| residual_coding_cavlc( x0, y0, log2TrafoSize, trafoDepth, cIdx ) |  |
| } |  |
| } |  |
| } |  |

intra_sdip_enabled_flag equal to 1 specifies that the short distance intra prediction process is applied according to the intra luma prediction mode.

sdip_flag specifies whether short distance intra prediction is applied to the current coding unit. sdip_flag equal to 1 specifies that the short distance intra prediction is applied to the current coding unit, and shall be split into four prediction units with size of 2NxhN or hNx2N.

When sdip_flag is not present, it shall be inferred to be equal to 0.

sdip_direction specifies the partition mode of the current coding unit when sdip_direction is equal to 1. PartMode shall be set to PART_2NxhN when sdip_direction is equal to 1, and PART_hNx2N when sdip_direction is equal to 0.

## Syntax and Semantics changes for case3 of SDIP

| transform_tree( x0, y0, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| … |  |
| if( residualDataPresentFlag) { |  |
| log2TrafoSize = ( log2TrafoWidth + log2TrafoHeight ) >> 1 |  |
| intraSplitFlag = ( IntraSplitFlag && trafoDepth = = 0 ? 1 : 0 ) |  |
| maxDepth = ( PredMode = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| if( PredMode == MODE_INTRA && non_square_transform_flag && max_transform_hierarchy_depth_intra > 1 ) { |  |
| maxDepth = ( (trafoDepth + log2TrafoSize) == 5 ? 2 : 1 ) |  |
| } |  |
| if( non_square_transform_flag ) { |  |
| xBase = ( non_square_transform_direction ? x0 : x0 − ( x0 & ( ( 1 << ( log2TrafoWidth + 2 ) – 1 ) ) ) |  |
| yBase = ( non_square_transform_direction ? y0 − ( y0 & ( ( 1 << ( log2TrafoHeight + 2 ) – 1) ) : y0 ) |  |
| } else { |  |
| xBase = x0 − ( x0 & ( 1 << log2TrafoWidth ) ) |  |
| yBase = y0 − ( y0 & ( 1 << log2TrafoHeight ) ) |  |
| } |  |
| if( !entropy_coding_mode_flag ) { |  |
| if( trafoDepth != 0 && !cbf_luma[ xBase ][ yBase ][ trafoDepth − 1 ] ) { |  |
| if( !non_square_transform_flag && cbf_cb[ xBase ][ yBase ][ trafoDepth − 1 ] && log2TrafoSize > Log2MinTrafoSize ) |  |
| cbf_cb[ x0 ][ y0 ] [ trafoDepth ] | u(1) |
| if( !non_square_transform_flag && cbf_cr[ xBase ][ yBase ][ trafoDepth − 1 ] && log2TrafoSize > Log2MinTrafoSize ) |  |
| cbf_cr[ x0 ][ y0 ] [ trafoDepth ] | u(1) |
| if(log2TrafoSize <= Log2MaxTrafoSize && !intraSplitFlag && log2TrafoSize > Log2MinTrafoSize && trafoDepth < maxDepth ) |  |
| split_transform_flag[ x0 ][ y0 ] [ trafoDepth ] | u(1) |
| } else |  |
| cbp_and_split_transform | vlc(n,v) |
| } |  |
| … |  |
| if( intra_non_square_transform_enabled_flag && PredMode == MODE_INTRA && trafoDepth == 0 && log2TrafoSize > 2 && log2TrafoSize < 6 && ( split_transform_flag[ x0 ][ y0 ][ trafoDepth ] ) ) { |  |
| non_square_transform_flag | u(1) ¦ ae(v) |
| if( non_square_transform_flag ) |  |
| non_square_transform_direction | u(1) ¦ ae(v) |
| } |  |
| if( split_transform_flag[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| if( PredMode == MODE_INTRA ) { |  |
| TUSplitDirection = non_square_transform_flag ? (!non_square_transform_direction):2 |  |
| } else { |  |
| TUSplitDirection = InterTUSplitDirection |  |
| } |  |
| if( TUSplitDirection = = 2 ) { |  |
| … |  |
| } else { |  |
| x1 = x0 + ( ( 1 << log2TrafoWidth ) >> 2 ) * TUSplitDirection |  |
| y1 = y0 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − TUSplitDirection ) |  |
| x2 = x1 + ( ( 1 << log2TrafoWidth ) >> 2 ) * TUSplitDirection |  |
| y2 = y1 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − TUSplitDirection ) |  |
| x3 = x2 + ( ( 1 << log2TrafoWidth ) >> 2 ) * TUSplitDirection |  |
| y3 = y2 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − TUSplitDirection ) |  |
| log2TrafoHeight = log2TrafoHeight + 2 * TUSplitDirection – 1 |  |
| log2TrafoWidth = log2TrafoWidth − 2 * TUSplitDirection + 1 |  |
| } |  |
| … |  |
| } else if( entropy_coding_mode_flag ) { |  |
| … |  |
| if( PredMode = = MODE_INTRA && ( !non_square_transform_flag ¦ ¦ trafoDepth == 0)) |  |
| … |  |
| } |  |
| } |  |
| } |  |

7.4.8 Transform tree semantics

non_square_transform_flag specifies a block is split into whether non-square blocks or square blocks when the block is split into blocks for the purpose of transform coding. non_square_transform_flag equal to 1 specifies a block is split into non-square blocks. non_square_transform_flag equal to 0 specifies a block is split into square blocks. When sdip_flag is not present, it shall be inferred to be equal to 0.

Note that non_square_transform_flag shall be initialized to 0 outside transform_tree().

non_square_transform_direction specifies a block is split along whether the horizontal side or the vertical side when the block is split into non-square blocks. When non_square_transform_direction is equal to 1, the width of each block after splitting is the same as the width of the block before splitting, while the height of the each block after splitting is one-fourth of the height of the block before splitting. When non_square_transform_direction is equal to 0, the width of each block after splitting is one-fourth of the width of the block before splitting, while the height of the each block after splitting is the same as the height of the block before splitting.

# Patent rights declaration(s)

Huawei, Microsoft, HiSilicon, Tsinghua and USTC may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).