| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, 21-30 Nov, 2011 | Document: JCTVC-G562 |
| --- | --- |

| Title: | Unification for motion compensation filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chanyul Kim I.-K. Kim Minseok Choi | Email: dionism@samsung.com ilkoo.kim@samsung.com minseok48.choi@samsung.com |  |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

- Abstract

This contribution presents an unification scheme for motion compensation filters. In HM4.0, bit-depths for output of uni-prediction and bi-prediction are different, and it makes filters complicated and inconsistent. In this proposal, results of both filters have the same resolutions by leaving the output bit-depth of uni-prediction to that of bi-prediction. Using this proposal, the filters for bi- and uni-directional prediction can be unified and simplified without any coding loss.

Proposal

Currently, the filters for bi-prediction and uni-prediction are separated in HM4.0. In case of bi-prediction, the rounding offset isn’t signaled but keeps each of the interpolated signals at high precision. The rounding is performed at the last step after both prediction signals are added. The calculations are performed in 16-bit intermediate buffer. In case uni-prediction, each interpolated signals are rounded at the filter stage. So the calculations are performed in input-bit depth. This is illustrated in Figure 1 where the red N and M number represent each bit depth and blue number in parenthesis is the example when input bit depth is equal to eight.

Figure 1. Current MC prediction in HM4.0

In order to unify filters for bi- and uni- prediction, we propose the scheme that the uni-prediction keeps the interpolated signals at high precision. The calculations are performed in 16-bit. The rounding is performed when prediction results are added with residual coefficients. According to this proposal, the uni-prediction filter has high accuracy with 16 bit depth as same with bi-prediction. So finally bi- and uni- prediction can be used with the same filter in high precision. Figure 2 illustrates the proposal.

Figure 2. Proposed unified MC prediction method

Experimental Results

Experimental Results will be updated soon.

Conclusion

This contribution proposed a minor modification on the uni-prediction. Using this proposal, uni-prediction performs high accuracy calculation using 16 bit-depth same as bi-prediction. So the two directional prediction filters can be unified with no coding loss and the increase of complexity. Finally the encoder and decoder need not to use separated prediction filters any more for motion compensation. The test result of modifying the size of intermediate buffer is similar to the result of our own proposal.

Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).