| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G563 |
| --- | --- |

| Title: | CE5 2.2 : Handling for exception cases longer than 32bit code-word in CAVLC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chanyul Kim Youngo Park Kwang Pyo Choi Marta Karczewicz Xianglin Wang Wei-Jung Chien Liwei Guo | Tel: Email: | dionism@samsung.com youngo.park@samsung.com kp5.choi@samsung.com martak@qualcomm.com xianglin@qualcomm.com wchien@qualcomm.com liweig@qualcomm.com |
| Source: | Samsung Electronics Co., Ltd., Qualcomm Inc. |  |  |

_____________________________

# Abstract

In this contribution, the result of handling longer than 32bit code-word for CE5 2.2 is presented. Any error for longer than 32 bit code-word both Luma and Chroma does not occur using the modified VLC tables which covers the maximum level of quantized coefficients in CAVLC without performance degradation.

# Introduction

In the 6th meeting, two proposals for handling longer than 32bit code-word were presented both Luma and Chroma [1][2]. This proposal presents the uninfected solution for assertion fail due to longer than 32bit code-word both Luma and Chroma. The quantized coefficient level is represented as

Where N is block size, offset is 26214. The maximum level of quantized coefficient would be 15-bit (14-bit level +sign). In the CAVLC coefficient coding, the required bit (RB) for code-word corresponding to the maximum level of quantized coefficients can be obtained as Equation. (2).

Where n is VLC table number and k is the leading zeroes in the Exp-golomb. The possible maximum level (PML) of quantized coefficient represented with a 32bit does not cover the maximum range of level denoted in Equation. (3). Moreover, there is error if the code word is longer than 32-bit due to current VLC tables do not fully support for maximum RB of quantized coefficients as follows.

# Algorithm description

## Luma

Modified VLC tables for supporting maximum RB of quantized coefficient are proposed. The current HM uses the concatenated unary and Exp- golomb like code. Not more than five of VLC table number is considered for Luma. The VLC tables are combined with unary and Exp-golomb like code as shown in Figure 2. The maximum representable coefficients are smaller than maximum quantized coefficient in case of less than VLC table number 2, where more than 32-bit code words are generated. Therefore, the proposed VLC tables could cover the full range of maximum quantized coefficients within 32-bits code word. The VLC tables based on unary, Exp-golomb mixed with fixed length code are presented as shown in Figure 3.

Figure 2. VLC Table structure and the maximum coverage quantized coefficient

Figure 3. Modified VLC Table structure and the maximum coverage of quantized coefficient

## Chroma

When coefficient coding for chroma blocks is extended to 16x16 blocks codeword longer than 32 bits can be generated. To avoid it two modifications are proposed.

The index of the VLC table, vlcNum,  used to code syntax element last_pos_level_one for chroma  is derived as

vlcNum = lastPosVlcNumTable[blockType][vlcNumIdx].

It is proposed to set the value of the array lastPosVlcNumTable for blockType=0  and vlcNumIdx=7 to 2 insted of 7.

We further propose to modify VLC table for vlcNum=9, by concatenating Golomb codes with two different parameters k. Currently Golomb code with k=4 is used, we propose to concatenate it with Golomb code k=7. The new decoding process for vlcNum=9 is:

–	If leadingZeroBits is equal to 0,

b =  read_bits(1)

if (b) {

b =  read_bits(1)

if (b) {

codeNum = 3 + read_bits(3)

}

else{

b =  read_bits(1)

codeNum = b ? 1 + read_bits(1) : 0

}

}

–	Otherwise (leadingZeroBits is less than than 16),

codeNum = (leadingZeroBits << 4) +  read_bits(4) + 11

–	Otherwise,

codeNum = ((leadingZeroBits-16) << 7) +  read_bits(7) + 267

# Simulation Results

Using HM4.0, simulation is performed under the common test condition [3]. The result shows the minor Chroma gain without any assertion error and complexity.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | #VALUE! | #VALUE! | #VALUE! | 0.0% | 0.0% | 0.0% |
| Class B | #VALUE! | #VALUE! | #VALUE! | 0.0% | 0.0% | 0.0% |
| Class C | #VALUE! | #VALUE! | #VALUE! | 0.0% | 0.0% | 0.0% |
| Class D | #VALUE! | #VALUE! | #VALUE! | 0.0% | 0.0% | 0.0% |
| Class E | #VALUE! | #VALUE! | #VALUE! | 0.0% | 0.0% | 0.0% |
| Overall | #VALUE! | #VALUE! | #VALUE! | 0.0% | 0.0% | 0.0% |
|  | #VALUE! | #VALUE! | #VALUE! | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | #NUM! |  |  | 100% |  |  |
| Dec Time[%] | #NUM! |  |  | 101% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | #VALUE! | #VALUE! | #VALUE! | 0.0% | -0.1% | -0.1% |
| Class B | #VALUE! | #VALUE! | #VALUE! | 0.0% | 0.0% | 0.1% |
| Class C | #VALUE! | #VALUE! | #VALUE! | 0.0% | 0.0% | 0.0% |
| Class D | #VALUE! | #VALUE! | #VALUE! | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | 0.0% | 0.0% | 0.0% |
|  | #VALUE! | #VALUE! | #VALUE! | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | #NUM! |  |  | 100% |  |  |
| Dec Time[%] | #NUM! |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | 0.0% | -0.1% | -0.2% |
| Class C | #VALUE! | #VALUE! | #VALUE! | 0.0% | 0.0% | 0.0% |
| Class D | #VALUE! | #VALUE! | #VALUE! | 0.0% | 0.0% | 0.0% |
| Class E | #VALUE! | #VALUE! | #VALUE! | 0.0% | 0.0% | -0.1% |
| Overall | #VALUE! | #VALUE! | #VALUE! | 0.0% | 0.0% | -0.1% |
|  | #VALUE! | #VALUE! | #VALUE! | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | #NUM! |  |  | 100% |  |  |
| Dec Time[%] | #NUM! |  |  | 101% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | 0.0% | 0.0% | -0.1% |
| Class C | #VALUE! | #VALUE! | #VALUE! | 0.0% | -0.1% | 0.0% |
| Class D | #VALUE! | #VALUE! | #VALUE! | 0.0% | 0.0% | -0.1% |
| Class E | #VALUE! | #VALUE! | #VALUE! | 0.0% | 0.0% | 0.0% |
| Overall | #VALUE! | #VALUE! | #VALUE! | 0.0% | 0.0% | -0.1% |
|  | #VALUE! | #VALUE! | #VALUE! | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | #NUM! |  |  | 100% |  |  |
| Dec Time[%] | #NUM! |  |  | 99% |  |  |

# Conclusion

This proposal has target for handling excerption cases regarding to longer than 32bit code-word in both Luma and Chroma. The exception occurs due to large block size. The current VLC tables do not fully support for coding maximum quantized coefficients. The propose scheme is to handle exception case of longer than 32-bit code word in CAVLC without any performance changing.

# References

- C.Y.Kim, etal., “Handling for exception cases regarding to code-word larger than 32bit in CAVLC,” JCT-VC document JCTVC-E700, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 6th meeting, Torino, July 2011.

- K. Marth, et al., “Removing Chroma Zonal Coding in CAVLC,” JCT-VC document JCTVC-E704, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/ WG11  6th meeting, Torino, July 2011.

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-E700, JCT-VC 5th Meeting: Geneva, March, 2011.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. and Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex : WD Text change

Depending on the value of vlcNum, the value of codeNum is assigned as follows.

- Thresh = (1<<(11+vlcNum)) + (5 * (1 << )) - 1

- If vlcNum is equal to 0,

- If leadingZeroBits is less than or equal to 6,

codeNum = leadingZeroBits	(9-1)

- Otherwise (leadingZeroBits is greater than 6),

numBits =  leadingZeroBits – 6 	(9-2)
	codeNum = 5 + (1<< numBits) + read_bits( numBits )

- Else If (leadingZeroBits is greater than 6 and less than 17)

numBits =  leadingZeroBits – 6 	(9-3)
	codeNum = 5 + (1<< numBits) + read_bits( numBits )

- Else(leadingZeroBits is equal 17)

numBits = 17

codeNum = Thresh + read_bits(numBits)

- Otherwise, if vlcNum is less than 5,

If vlcNum is less than 5,

- If leadingZeroBits is less than or equal to 6,

codeNum = (leadingZeroBits << vlcNum) +  read_bits( vlcNum ) 	(9-4)

- Otherwise (leadingZeroBits is greater than 6),

numBits =  leadingZeroBits – 6 + vlcNum	(9-5)
	codeNum = 5*(1<<vlcNum) + (1<< numBits) + read_bits( numBits)

- Else If (leadingZeroBits is greater than 6 and less than 17)

numBits = leadingZeroBits -6 +vlcNum	(9-6)
	codeNum = 5*(1<<vlcNum) +(1<<numBits) +read_bits(numBits)

- Else(leadingZeroBits is equal 17)

numBits = 15

codeNum = Thresh + read_bits(numBits)

Inputs to this process are bits from the RBSP and the parameter vlcNum specifying the VLC code. If vlcNum is equal to 14, inputs to this process additionally include a parameter cMax.

…

- Otherwise, if vlcNum is equal to 9,

- If leadingZeroBits is equal to 0,

b =  read_bits(1)
	if (b) {
		b =  read_bits(1)
		if (b) {
		     codeNum = 3 + read_bits(3) 	(9-7)
		}
		else{
		     b =  read_bits(1)
		     codeNum = b ? 1 + read_bits(1) : 0
		}
	}

- Otherwise (leadingZeroBits is greater than 0),

codeNum = (leadingZeroBits << 4) +  read_bits( 4) + 11

- Otherwise , if leadingZeroBits is less than 16,

codeNum = (leadingZeroBits << 4) +  read_bits( 4) + 11

- Otherwise

codeNum = ((leadingZeroBits – 16) << 7) +  read_bits( 7) + 267