| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G564 |
| --- | --- |

| Title: | CE8 subtest c tool 1: Line memory reduction for ALF and SAO decoding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Semih Esenlik, Matthias Narroschke, Thomas Wedi Panasonic R&D Center Germany Monzastr. 4c, 63225 Langen, Germany | Tel: Email: | +49 6103 766 1306 Semih.Esenlik@eu.panasonic.com +49 6103 766 204 Matthias.Narroschke@eu.panasonic.com +49 6103 766 1194 Thomas.Wedi@eu.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution is a part of CE8 on in-loop filtering. Proposed is a method to reduce the line memory which is required by consecutive filtering operations in the decoder. In the current HM 4.0, Deblocking Filter (DF), Sample Adaptive Offset (SAO) and Adaptive Loop Filter (ALF) all pose difficulties related to storage requirements in the block-based decoding procedure. Namely, for the purpose of filtering across the boundaries of LCUs (Largest Coding Unit), horizontal and vertical line memory need to be employed which is increases the implementation complexity of decoder chips. This contribution focuses on the reduction of the line memory for LCU-based decoding. The main focus is the reduction in the so called horizontal line memory, whose size is directly proportional to the width of the decoded picture. With the help of the proposed technique the horizontal line memory that needs to be employed is reduced from 9 lines to 5 lines for the luminance component and from 7 lines to 4 lines for the chrominance components.

# Introduction

To reduce external frame buffer access, hardware implementations of decoders usually store data that is used multiple times in the decoding process in an on-chip memory. A specific type of on-chip memory is called “line memory”. It is depicted in Figure 1. During the decoding process the coded blocks are decoded one by one in a raster scan order. Since the blocks neighboring to the currently decoded block to the bottom and to the right are not yet decoded, the filter operations Deblocking Filter (DF), Sample Adaptive Offset (SAO) and Adaptive Loop Filter (ALF) have to be delayed at the coding block borders as they require decoded samples of the not yet decoded neighboring blocks. For a quick access, decoders typically store those samples that are required for delayed filtering in the line memory.

The line memory can be further divided into horizontal and vertical line memory which is also shown in Figure 1. The size of the horizontal line memory is proportional to the frame width, whereas vertical line memory size is proportional to Largest Coding Unit (LCU) height. Since the horizontal line memory is considerably larger in size than the vertical line memory the goal of the proposal is set to achieve a reduction of the horizontal line memory. In the remaining part of the contribution, the term line memory is used synonymously with horizontal line memory.

Block that is currently decoded

Horizontal Line Memory

Frame Width

Vertical Line Memory

9 Lines

Figure 1: Horizontal and vertical line memory.

# Line memory requirements for HM4.0

In-loop filtering in decoders can be carried out in either frame-based or LCU-based fashion. In frame-based filtering each filtering stage is performed on the whole video frame and the intermediate output is written to the external memory. Although no line memory implementation is required for frame based decoding, external memory access is formidable since the whole reconstructed image has to be retrieved from and written back to the external memory. In LCU-based filtering, the three filtering operations are carried out on an LCU one after the other. Therefore all DF, SAO and ALF can be applied on a current LCU before the output is written to the reference frame memory. Since the frame based filtering requires formidable amount of external memory access, LCU-based filtering is considered preferable in hardware implementations.

In H.264/MPEG-4 AVC only a single in-loop filter is employed which is the deblocking filter. The line memory requirement of H.264/MPEG-4 AVC was therefore just 4 lines. However in HM4.0 two more in-loop filters are introduced, namely Sample Adaptive Offset and Adaptive Loop Filter, that are applied in a sequential order subsequent to the deblocking filter. As a result the line memory requirement is increased from 4 lines to 9 lines in total.

## Line memory requirements for the Deblocking Filter of HM4.0

In Figure 2 the deblocking operation on an LCU is depicted according to HM4.0. First, the horizontal deblocking of the vertical edges are performed and then vertical deblocking of the horizontal edges. The last 4 rows of samples however cannot be deblocked at the moment due to the absence of the decoded neighboring LCU to the bottom. Therefore, 4 lines (shown in a pink rectangle) have to be stored in the line memory for delayed processing.

Figure 2: Vertical and horizontal deblocking filtering.  Red region cannot be processed by deblocking filter.

The focus of this proposal is the reduction of the horizontal line memory. For a convenient description, the neighboring LCU to the right is assumed to be present through the remaining part of the paper (absence of which leads to the requirement for the vertical line memory). In Figure 2 the deblocking at the right LCU border is considered as not delayed due this assumption.

## Line memory requirements for SAO of HM4.0

Sample Adaptive Offset [1] is applied on the output of the deblocking filter. The deblocking filter has to delay the modification of the last 3 lines of pixels at the bottom LCU border. This delay leads to a further delay by SAO. The problem is depicted in the figure below.

Figure 3: Samples that can be used by SAO during the filtering of the LCU.

In the Figure 3, the red cross marks the samples used for classifications of SAO according to [1]. It can be seen that the SAO processing of the last 4 lines from the bottom of the LCU needs to be delayed as a consequence of the fact that 3 lines from the bottom LCU border are delayed for deblocking. As a result the SAO process requires 1 additional line of line memory (5 lines in total when only DF and SAO are considered).

## Line memory requirements for ALF of HM4.0

In Figure 4, application of ALF mask is depicted. ALF uses the output signal of the SAO process as input signal. Since the last 4 lines in an LCU are delayed for processing of the deblocking filter and SAO, the ALF application needs to be delayed as well. As it is shown in the Figure below, the filtering by ALF needs to be delayed by 2 additional lines from the bottom of the LCU.

The delayed processing of 6 lines from the bottom LCU border requires 4 lines of line memory in addition to the 5 lines required by SAO and DF. Therefore in total 9 lines of line memory is required for the filtering with 3 consecutive in-loop filters.

Figure 4: Blue, green and yellow samples represent vertically deblocked, fully deblocked and SAO processed samples respectively.

# Proposed Method

In HM4.0 the problem of high amount of line memory requirement stems from the strict input-output relations between the 3 consecutive filters. A filtering stage requires that all the samples have to be already processed by the preceding filtering stage. As a result each filtering stage delays the following stage at the LCU borders.

To overcome the problem the filter application procedure is slightly modified at the LCU borders in order to reduce the line memory requirement substantially. The principle applied throughout the whole proposal is “substituting input samples that are not available in the instant of processing with available counterparts in the memory”. This principle is applied to both SAO and ALF. As a result, all 3 filters (DF, SAO and ALF) are able process all the samples in an LCU till the 3rd line from the bottom border of LCU.

An important remark about the proposal is that it does not change the order of the 3 consecutive filtering stages as the central tap of the ALF filter mask and of the SAO classification pattern always uses the SAO-processed and deblocked samples respectively. Moreover, there is no change in the details of Deblocking Filtering, SAO and ALF processes. According to the proposal only input substitution is performed at the bottom LCU border.

In the remaining part of Section Error: Reference source not found, the proposed method is explained in detail.

## Proposed modification for SAO

During the decoding of an LCU, last 3 rows of pixels are inaccessible to SAO since they are going to be modified by Deblocking Filter later on. Therefore SAO processing terminated at the 4th line above the LCU border.

In this proposal, the termination point is moved 1 line down as shown in Figure 5. The proposed SAO process uses 1 line of partially deblocked samples which are already available in the memory. As a result the LCU is processed by SAO until the 3rd line from the bottom LCU border.

Figure 5: Termination points in HM4.0 and proposed method.

## Proposed modification for the block-based pixel classification method of ALF

In the Figure 6 the 5x5 filter computation block (shown in solid black) and the 4x4 filter selection block (shown in red) are shown for HM4.0 on the left hand side. The filter decision can not be performed for the last 4 lines in HM4.0 due to unavailable samples.

In the proposed method the filter selection block is extended by 1 line at the bottom LCU border. As a result the filter decision for the 4th line from the bottom of the LCU is also made available.

Figure 6: Filter decision is not available for last 4 lines in HM4.0 and for 3 lines in the proposed method.

According to the proposal there is no change in the function that is used for filter decision.

## Proposed modification for the ALF filter application

During the filter application, 2 rows of partially deblocked pixels at the bottom are used instead of delaying the filtering operation by 2 rows. In Figure 7 the ALF filter mask is shown in black. Filter taps that are below the central filter tap are allowed to use partially deblocked pixels that are already available in the memory.

Figure 7: Filtering by ALF is not possible for 6 lines in HM4.0 and for 3 lines in the proposed method.

In Figure 7 the line memory requirements for DF, SAO and ALF are shown. In the proposed method line memory requirement is reduced to 5 lines in total for the luminance component.

## Proposed change for Chroma SAO and ALF filter application

The proposed method that is explained in Section 3.3 is applied also to the chroma components. For the chrominance components the total line memory requirement is reduced from 7 lines to 4 lines by the proposed method.

# Experimental Results

As a reference, the official tagged 4.0 version of the HM software [2] is used. The simulations are performed according to the common conditions [3]. The results are presented in Table 1.

The proposed method has almost no impact on the coding gain. The minor increase in the encoding and decoding times result from the clean and easy-to-read implementation of the proposed idea.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,1% |
| Class B | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,1% |
| Class C | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class D | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class E | 0,0% | 0,1% | 0,1% | 0,0% | 0,1% | 0,1% |
| Overall | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
|  | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Enc Time[%] | 102% |  |  | 98% |  |  |
| Dec Time[%] | 103% |  |  | 102% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,0% | -0,2% | -0,1% | -0,1% | 0,1% | 0,0% |
| Class B | 0,0% | 0,0% | 0,1% | 0,0% | 0,0% | 0,0% |
| Class C | 0,0% | -0,1% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class D | 0,0% | -0,1% | 0,1% | 0,0% | 0,0% | 0,0% |
| Class E |  |  |  |  |  |  |
| Overall | 0,0% | -0,1% | 0,0% | 0,0% | 0,0% | 0,0% |
|  | 0,0% | -0,1% | 0,0% | 0,0% | 0,1% | 0,0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 106% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0,0% | -0,2% | -0,2% | 0,0% | 0,1% | -0,1% |
| Class C | 0,0% | -0,1% | -0,1% | 0,0% | 0,2% | -0,2% |
| Class D | 0,0% | 0,0% | -0,1% | 0,0% | 0,3% | 0,0% |
| Class E | 0,0% | 0,0% | 0,4% | 0,0% | 0,2% | -0,1% |
| Overall | 0,0% | -0,1% | 0,0% | 0,0% | 0,2% | -0,1% |
|  | 0,0% | -0,1% | -0,1% | 0,0% | 0,1% | -0,1% |
| Enc Time[%] | 104% |  |  | 99% |  |  |
| Dec Time[%] | 102% |  |  | 103% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0,0% | -0,1% | 0,5% | 0,0% | 0,1% | -0,1% |
| Class C | 0,0% | -0,1% | 0,0% | 0,0% | -0,1% | 0,0% |
| Class D | 0,1% | 0,2% | 0,2% | 0,0% | -0,3% | -0,4% |
| Class E | -0,1% | 0,3% | 0,1% | 0,1% | -0,4% | 0,0% |
| Overall | 0,0% | 0,1% | 0,2% | 0,0% | -0,1% | -0,1% |
|  | 0,0% | 0,0% | 0,1% | 0,0% | -0,1% | -0,1% |
| Enc Time[%] | 107% |  |  | 98% |  |  |
| Dec Time[%] | 104% |  |  | 103% |  |  |

Table 1: Simulation results according to [3].

# Conclusion

With this contribution we propose a method for reducing the line memory requirement for the LCU-based filtering procedure. The proposed method provides 4 lines reduction (from 9 in HM4.0 to 5) in the horizontal line memory for the luminance component and 3 lines reduction (from 7 in HM4.0 to 4) for each chrominance components. Results of experiments following the common test conditions show practically no change in coding gain. A slight increase in the runtimes is observed which is the result of a clean implementation of the line memory concept in the reference software.

Since the amount of line memory that is required to be implemented in decoder chips is significantly reduced, the proposed method is thought to be very beneficial for LCU-based decoding approach. We propose to include this technique in the next version of the HM.

# References

- Chih-Ming Fu et al., “CE13: Sample Adaptive Offset with LCU-Independent Decoding”, JCTVC-E049, Geneva, CH, March 2011.

- HM 4.0 Software, http://hevc.kw.bbc.co.uk/trac/browser/tags/HM-4.0

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, Torino, July 2011.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).