##### Filtering process for luma block edge

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top left luma sample of the current coding unit,

–	a variable verticalEdgeFlag,

–	a variable bS specifying the boundary filtering strength,

–	variables dE, dEp1 and dEq1 containing decisions,

–	one-dimensional array of size (8), dS containing decisions,

–	a variable bSL,

–	a variable tCL,

Output of this process is:

–	modified reconstruction of the picture.

Let s’ represent the luma sample array recPictureL of the current picture.

Depending on pcm_flag, a variable β is specified as follows:

–	If pcm_flag is equal to 1, the variables β is specified as Error: Reference source not found with luma quantization parameter 0 as input.

–	Otherwise, the variables β is specified as Error: Reference source not found with luma quantization parameter qPL as input.

A variable tC is specified as follows:

–	If bS is greater than 2, the variable tC is specified as Error: Reference source not found with luma quantization parameter Clip3(0, 55, qPL + 2 ) as input,

–	Otherwise (bS is equal or less than 2), the variable tC is specified as Error: Reference source not found with luma quantization parameter qPL as input.

Depending on verticalEdgeFlag, the following applies:

–	If verticalEdgeFlag is equal to 1, the following ordered steps apply:

- The sample values pi,k and qi,k with i = 0..3 and k = 0..7 are derived as follows:

qi,k = s’[ xC + xB +i, yC + yB + k ]		(8-1)

pi,k = s’[ xC + xB – i – 1, yC + yB + k ]		(8-2)

- If dE is not equal to 0, for each sample location ( xC + xB, yC + yB + k ), k = 0..7, the following ordered steps apply:

- The filtering process for a luma sample specified in subclause Error: Reference source not found is invoked with sample values pi,k, qi,k with i = 0..3, the decision dS[k], variables dEp1 and dEq1, the boundary filtering strength bS and the variable tC as inputs and the number of filtered samples nDp and nDq from each side of the block boundary, and the filtered sample values pi’ and qj’ as outputs.

- The filtered sample values pi’ and qj’ with i = 0..nDp – 1, j = 0..nDq – 1 replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB +j, yC + yB + k ] = qj’		(8-3)

s’[ xC + xB – i – 1, yC + yB + k ] = pi’		(8-4)

- The output of step 2 is denoted as s° as follows:

s°[ xC + xB +i, yC + yB + k ] = s’[ xC + xB +i, yC + yB + k ]  with i = -4..3, k = 0..7

–	Otherwise (verticalEdgeFlag is equal to 0), the following ordered steps apply:

- If xPOS is equal to 1, the parameters ks and ke are set to -3 and 4 respectively. If xD is equal to 2, the parameters ks and ke are set to 0 and 4 respectively. Otherwise ks and ke are set to 0 and 7 respectively.

- The sample values pi,k and qi,k with i = 0..3 and k = ks..ke are derived as follows:

qi,k = s’[ xC + xB +k, yC + yB + i ]		(8-5)

pi,k = s’[ xC + xB +k, yC + yB – i – 1 ]		(8-6)

- If xPOS is less than 0 and dEL is not equal to 0, for each sample location ( xC + xB + k, yC + yB ), k = -3..-1, the following ordered steps apply:

- The filtering process for a luma sample specified in subclause Error: Reference source not found is invoked with sample values pi,k, qi,k with i = 0..3, decision dSL[k+3], variables dEp1 and dEq1, the boundary filtering strength bSL and the variable tCL as inputs and the number of filtered samples nDp and nDq from each side of the block boundary and the filtered sample values pi’ and qj’ as outputs.

- The filtered sample values pi’ and qj’ with i = 0..nDp – 1, j = 0..nDq – 1 replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB +k, yC + yB + j ] = qj’		(8-7)

s’[ xC + xB +k, yC + yB – i – 1 ] = pi’		(8-8)

- If dE is not equal to 0, for each sample location ( xC + xB + k, yC + yB ), k = 0.. ke , the following ordered steps apply:

- The filtering process for a luma sample specified in subclause Error: Reference source not found is invoked with sample values pi,k, qi,k with i = 0..3, decision dS[k], variables dEp1 and dEq1, the boundary filtering strength bS and the variable tC as inputs and the number of filtered samples nDp and nDq from each side of the block boundary and the filtered sample values pi’ and qj’ as outputs.

- The filtered sample values pi’ and qj’ with i = 0..nDp – 1, j = 0..nDq – 1 replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB +k, yC + yB + j ] = qj’		(8-9)

s’[ xC + xB +k, yC + yB – i – 1 ] = pi’

##### Filtering process for chroma block edge

[Ed.: (WJ) cIdx cannot be 0 here]

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left chroma sample of the current coding unit relative to the top left chroma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left chroma sample of the current block relative to the top left chroma sample of the current coding unit,

–	a variable verticalEdgeFlag,

–	a variable bS specifying the boundary filtering strength,

–	a variable cIdx specifying the chroma component index.

–	a variable xPOS,

–	a variable bSL,

–	a variable tCL

Output of this process is:

–	modified reconstruction of the picture.

Let s’ be a variable specifying chroma sample array which is derived as follows.

–	If cIdx is equal to 1, s’ represents the chroma sample array recPictureCb of the current picture.

–	Otherwise (cIdx is equal to 2), s’ represents the chroma sample array recPictureCr of the current picture.

A variable tC is specified as follows:

–	If bS is greater than 2, the variable tC is specified as Error: Reference source not found with luma quantization parameter Clip3(0, 55, qPL + 2 ) as input,

–	Otherwise (bS is equal or less than 2), the variable tC is specified as Error: Reference source not found with luma quantization parameter qPL as input.

Depending on verticalEdgeFlag, the following applies:

–	If verticalEdgeFlag is equal to 1, for each sample location ( xC + xB, yC + yB + k ), k = 0..3, the following ordered steps apply:

- The sample values pi and qi with i = 0..1 are derived as follows:

qi = s’[ xC + xB +i, yC + yB + k ]		(8-10)

pi = s’[ xC + xB – i – 1, yC + yB + k ]		(8-11)

- If bS is greater than 2, the following ordered steps apply:

- The filtering process for a sample specified in subclause Error: Reference source not found is invoked with sample values pi, qi, with i = 0..1, the boundary filtering strength bS and the variable tC as inputs and the filtered sample values p0’ and q0’ as outputs.

- The filtered sample values p0’ and q0’ replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB , yC + yB + k ] = q0’		(8-12)

s’[ xC + xB – 1, yC + yB + k ] = p0’		(8-13)

- The output of step 2 is denoted as s° as follows:

s°[ xC + xB +i, yC + yB + k ] = s’[ xC + xB +i, yC + yB + k ]  with i = -2...1 and k = 0..3

–	Otherwise (verticalEdgeFlag is equal to 0), the following ordered steps apply:

- If xPOS is equal to 1, the parameters ks and ke are set to -1 and 2 respectively. If xD is equal to 2, the parameters ks and ke are set to 0 and 2 respectively. Otherwise ks and ke are set to 0 and 3 respectively.

- The sample values pi and qi with i = 0..1 and k = ks..ke are derived as follows:

qi = s’[ xC + xB +k, yC + yB + i ]		(8-14)

pi = s’[ xC + xB +k, yC + yB – i – 1 ]		(8-15)

- If xPOS is less than 0, and if bSL, is greater than 2, for each sample location ( xC + xB - 1, yC + yB ), the following ordered steps apply:

- The filtering process for a sample specified in subclause Error: Reference source not found is invoked with sample values pi, qi, with i = 0..1, the boundary filtering strength bSL and the variable tCL as inputs and the filtered sample values p0’ and q0’ as outputs.

- The filtered sample values p0’ and q0’ replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB +k, yC + yB ] = q0’		(8-16)

s’[ xC + xB +k, yC + yB – i – 1 ] = pi’		(8-17)

- If bS is greater than 2, for each sample location ( xC + xB + k, yC + yB ), k = 0.. ke, the following ordered steps apply:

- The filtering process for a sample specified in subclause Error: Reference source not found is invoked with sample values pi, qi, with i = 0..1, the boundary filtering strength bS and the variable tC as inputs and the filtered sample values p0’ and q0’ as outputs.

- The filtered sample values p0’ and q0’ replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB +k, yC + yB ] = q0’		(8-18)

s’[ xC + xB +k, yC + yB – 1 ] = p0’

#### 8.6.3.3   Derivation process for filter index array for luma samples

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a variable log2CUSize specifying the size of the current coding unit.

Output of this process is the two-dimensional filter index array of (nS)x(nS), fIdx.

A variable nS is set equal to ( 1 << log2CUSize ).

The boundary padding process specified in subclause Error: Reference source not found is invoked with the luma location ( xC, yC ), the size of coding unit log2CUSize and the chroma component index cIdx set equal to 0, and the output is assigned to the luma sample array s’’. [Ed. (WJ): s’’ is now a picture-size array, but actually CU size + appropriate margin is enough]

The filter index array fIdx is specified in the follows:

- When alf_region_adaptation_flag is equal to 1, the following ordered steps apply.

- The variables xIdx and yIdx are derived as

regionTab[16] = { 0, 1, 4, 5, 15, 2, 3, 6, 14, 11, 10, 7, 13, 12, 9, 8 }	(8-19)
	offset = 1 << (Log2MaxCUSize – 1) 		(8-20)
	xInterval = ( ( ( PicWidthInSamplesL + offset ) >> Log2MaxCUSize ) + 1 ) >> 2	(8-21)
	yInterval = ( ( ( PicHeightInSamplesL + offset ) >> Log2MaxCUSize ) + 1 ) >> 2	(8-22)
	xIdx = Min( 3, Floor( ( xC + x ) / ( xInterval << Log2MaxCUSize ) ) )	(8-23)
	yIdx = Min( 3, Floor( ( yC + y ) / ( yInterval << Log2MaxCUSize ) ) )	(8-24)

- The filter index fIdx[ x, y ] with x, y = 0..(nS)-1 is derived as

fIdx[ x ][ y ] = regionTab[ ( yIdx << 2 ) + xIdx ]		(8-25)

- Otherwise (alf_region_flag is equal to 0), the following ordered steps apply.

- The variables varTempH[ x ][ y ], varTempV[ x ][ y ] and varTemp1[ x ][ y ] with x, y = 0, 2, 4,...(nS) is derived as

s’’’[xC + x,yC + y] = (y == (nS-4))?s°[ xC + x,yC + y]:s’’[xC + x,yC + y]  with x, y = 0..(nS)

varTempH[ x ][ y ] = | ( s’’’[ xC+x, yC+y ] << 1 ) – s’’’[ xC+x-1, yC+y ] – s’’’[ xC+x+1, yC+y ] |	      (8-xxx)
   varTempV[ x ][ y ] = | ( s’’’[ xC+x, yC+y ] << 1 ) – s’’’[ xC+x, yC+y-1 ] – s’’’[ xC+x, yC+y+1 ] |	      (8-xxx)

- The variables varTempH1[ x, y ], varTempV1[ x, y ], and varTemp3[ x, y ] with x, y = 0..( (nS) – 1 )>>2 are derived as

varTempH1[ x ][ y ] = ij varTempH[ (x << 2 ) + i ][ (y << 2) + j ] with i, j = 0, 2	(8-26)
	varTempV1[ x ][ y ] = ij varTempV[ (x << 2 ) + i ][ (y << 2) + j ] with i, j = 0, 2	(8-27)
	varTemp3[ x ][ y ] = ( varTempH1[ x ][ y ] + varTempV1[ x ][ y ] ) >> 2	(8-28)

- The variable direction is derived as

- If varTempV1[ x >> 2 ][ y >> 2 ] is greater than varTempH1[ x >> 2 ][ y >> 2 ] << 1,

direction = 1

- Otherwise, if varTempH1[ x >> 2 ][ y >> 2 ] is greater than varTempV1[ x >> 2 ][ y >> 2 ] << 1,

direction = 2

- Otherwise,

direction = 0

- The variable avgvar is derived as

varTab[16] = { 0, 1, 2, 2, 2, 3, 3, 3, 3, 3, 4, 4, 4, 4, 4, 4 }	(8-29)
	avgVar = Clip3( 0, 15, (varTemp3[ x >> 2 ][ y >> 2 ] * 1024 ) >> (3 + BitDepthY) )	(8-30)

- The filter index fIdx[ x, y ] with x, y = 0..(nS)-1 is derived as

fIdx[ x ][ y ] = Clip3( 0, 4, varTab[avgVar] ) + 5 * direction	(8-31)

- The filter index fIdx[x,nS-4] with x = 0..(nS)-1 is derived as

fIdx[ x ][ nS-4 ] = fIdx[ x ][ nS-5 ]		(8-xxx)

#### 8.6.3.4   Filtering process for luma samples

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a variable log2CUSize specifying the size of the current coding unit,

–	a filter index array of (nS)x(nS), fIdx.

Output of this process is the filtered reconstruction of luma picture.

The boundary padding process specified in subclause Error: Reference source not found is invoked with the luma location ( xC, yC ), the size of coding unit log2CUSize and the chroma component index cIdx set equal to 0, and the output is assigned to the luma sample array s’’. [Ed. (WJ): s’’ is now a picture-size array, but actually CU size + appropriate margin is enough]

A variable nS is set equal to ( 1 << log2CUSize ) and a variable alfTap is set equal to ( alf_length_luma_minus_5_div2 << 1 ) + 5.

Each sample of luma picture recFiltPictureL[ xC + x ][ yC + y ] with x, y = 0..(nS)-1, is derived as follows.

- If y is smaller than (nS–3)

s’’’[xC + x,yC + y]= (y > (nS-4))?s°[ xC + x,yC + y]:s’’[xC + x,yC + y]  with x, y = -4..(nS)+3

- otherwise

s’’’[xC + x,yC + y]= (y == (nS-4))?s°[ xC + x,yC + y]:s’’[xC + x,yC + y]  with x, y = -4..(nS)+3

recFiltPictureL[xC+x][yC+y]=             cL[fIdx[x][y]][N]+i=0N−1s'''[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y]][i]             (8-32)

where N is set equal to AlfNumCoeffLuma-1 and horPos[i] and verPos[i] are specified in Table 8-1 and Table 8-2, respectively.

Table 8-1 – Specification of horPos[ i ] according to alfTap for adaptive loop filter process of luma samples

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| alfTap = 5 | 0 | -1 | 0 | 1 | -2 | -1 | 0 | 1 | 2 | -1 | 0 | 1 | 0 | - | - | - | - | - | - | - |
| alfTap = 7 | 0 | -1 | 0 | 1 | -2 | -1 | 0 | 1 | 2 | -3 | -2 | -1 | 0 | 1 | 2 | 3 | -2 | -1 | 0 | 1 |
| alfTap = 9 | -1 | 0 | 1 | -2 | -1 | 0 | 1 | 2 | -3 | -2 | -1 | 0 | 1 | 2 | 3 | -4 | -3 | -2 | -1 | 0 |

| i | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| alfTap = 5 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| alfTap = 7 | 2 | -1 | 0 | 1 | 0 | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| alfTap = 9 | 1 | 2 | 3 | 4 | -3 | -2 | -1 | 0 | 1 | 2 | 3 | -2 | -1 | 0 | 1 | 2 | -1 | 0 | 1 |

Table 8-2 – Specification of verPos[ i ] according to alfTap for adaptive loop filter process of luma samples

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| alfTap = 5 | -2 | -1 | -1 | -1 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 2 |  |  |  |  |  |  |  |
| alfTap = 7 | -3 | -2 | -2 | -2 | -1 | -1 | -1 | -1 | -1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
| alfTap = 9 | -3 | -3 | -3 | -2 | -2 | -2 | -2 | -2 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | 0 | 0 | 0 | 0 | 0 |

| i | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| alfTap = 5 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| alfTap = 7 | 1 | 2 | 2 | 2 | 3 | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| alfTap = 9 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 2 | 3 | 3 | 3 |

Figure 8-1 Mapping between geometric position and luma adaptive loop filter index according to alfTap (informative)

#### 8.6.3.5   Filtering process for chroma samples

Inputs of this process are:

–	a chroma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left chroma sample of the current picture,

–	a variable log2CUSize specifying the size of the current coding unit.

–	a variable cIdx specifying the chroma component index.

Output of this process is the filtered reconstruction of chroma picture.

The boundary padding process specified in subclause Error: Reference source not found is invoked with the chroma location ( xC, yC ), the size of coding unit log2CUSize and the chroma component index cIdx, and the output is assigned to the luma sample array s’’. [Ed. (WJ): s’’ is now a picture-size array, but actually CU size + appropriate margin is enough]

A variable nS is set equal to ( 1 << log2CUSize ) and a variable alfTapChroma is set equal to ( alf_length_chroma_minus_5_div2 << 1 ) + 5.

Filtered samples of chroma picture recFiltPicture[ xC + x ][ yC + y ] with x, y = 0..(nS)-1, are derived as follows.

- If y is equal to (nS– (alfTapChroma+1)/2)

s’’’[xC + x,yC + y]= (y ==nS-1)?s°[xC + x,yC + y]:s’’[xC + x,yC + y]  with x, y = -4..(nS)+3

- otherwise

s’’’[xC + x,yC + y]= s’’[xC + x,yC + y]  with x, y = -4..(nS)+3

recFiltPicture[xC+x][yC+y]=cC[N]+i=0N−1s'''[xC+x+horPos[i],yC+y+verPos[i]]∗cC[i]	      (8-33)

where

N = AlfNumCoeffChroma – 1,		(8-34)

horPos[ i ] = ( i %  alfTapChroma ) – ( alfTapChroma >> 1 ), and	(8-35)

verPos[ i ] = ( i /  alfTapChroma ) – ( alfTapChroma >> 1 )	(8-36)