| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G565 |
| --- | --- |

| Title: | CE6.d: Nokia report on intra prediction with secondary boundary |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jani Lainema Kemal Ugur | Email: | jani.lainema@nokia.com kemal.ugur@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution presents Nokia’s report on different approaches tested in CE6.d (intra prediction with secondary boundary). The document provides simulation results for each test together with assessment of aspects contributing to implementation and computational complexity of each approach.

# Introduction

At Turin meeting there was a number of contributions with similar intent to improve intra prediction quality with utilization of the secondary block boundary (the boundary which is not in the major direction of the directional intra prediction). As a result, core experiment CE6.d was set up to study different aspects of the proposals and identify the best complexity – coding efficiency trade-off available.

# Candidate technologies

The candidate technologies could be grouped roughly into four groups:

- Gradient based methods to horizontal and vertical prediction (intraPredAngle 0)

- Methods to improve small angle prediction (intraPredAngles 2, 5, 9)

- Directional methods to improve large angle prediction (intraPredAngles +/- 13 - 32)

- A method to improve planar intra prediction

Within different categories there were further variants of proposals as follows:

- Gradient based methods to horizontal and vertical prediction (intraPredAngle 0)

  - F122: Gradient based smoothing (variants for 1, 2 and 4 line processing)

  - F172: Gradient based smoothing, single line processing

- Methods to improve small angle prediction (intraPredAngles 2, 5, 9)

  - F122: Combination of gradient based and directional smoothing using fractional pixel accuracy, singe line processing (variants for doing angles 2, 5, 9 or 2, 5)

  - F358: diagonal smoothing with full pixel accuracy, single line processing

- Directional methods to improve large angle prediction (intraPredAngles +/- 13 - 32)

  - F122: directional smoothing for positive angles with fractional pixel accuracy (variants for 1 or 4 line processing for angle 32)

  - F358: diagonal smoothing for positive angles with full pixel accuracy, single line processing

  - F456: diagonal smoothing for positive and negative angles with full pixel accuracy, single line processing

- Method to improve planar intra prediction

  - F358: Smoothing for the planar mode (identical to HM 4.0 DC smoothing)

When it comes to implementation aspects of the proposals, the following could be considered:

Gradient based technologies:

- F122: adding a pixel specific gradient term to sample value and clipping the result

- F172: adding a pixel specific gradient term to sample value and clipping the result

Directional smoothing:

- F122: calculation of weighted average of the prediction value and two reference sample values (with angle dependent locations and weights)

- F358 & F456: calculation of a weighted average of two samples with fixed weight

Methods utilizing both gradient and directional techniques for the same prediction direction:

- F122: calculation of a weighted average of two reference sample values (with angle dependent locations and weights) and in addition the operations described for gradient based F122.

Planar smoothing:

- F358: calculation of a weighted average of two samples with fixed weight

Variants with multiple line processing:

- F122: variants with multi-line processing have some additional complexity

# Summary of the objective performance

The tables in this section summarize objective performance of each of the CE6.d test performed. Tests 1 – 8 measure performance of different candidate technologies separately. Whereas, tests 9 – 13 provide results for the identified most promising combinations of different candidates. Tool IDs in the results tables are described in Table 1.

Table 1: Summary of the different methods tested

| Tool ID | Proponent | Turin doc. | Description |
| --- | --- | --- | --- |
| F122(G1) | MediaTek | F122 | Gradient based method for vertical and horizontal modes (intraPredAngle== 0), single line processing |
| F122(GD1) | MediaTek | F122 | Combination of gradient based and directional smoothing for directional modes (intraPredAngle 2, 5, 9), single line processing using fractional pixel accuracy |
| F122(D1) | MediaTek | F122 | Directional smoothing for directional modes (intraPredAngle>= 13), single line processing using fractional pixel accuracy |
| F122(G4) | MediaTek | F122 | Gradient based method for vertical and horizontal modes (intraPredAngle== 0), processing 4 lines |
| F122(D4) | MediaTek | F122 | Directional smoothing for directional modes (intraPredAngle== 32), processing 4 lines using full pixel accuracy |
| F172 | Mitsubishi | F172 | Gradient based smoothing for horizontal and vertical modes (intraPredAngle == 0), single line processing |
| F358 | ETRI | F358 | Diagonal smoothing for modes with intraPredAngle > 0, using full pixel accuracy , single line processing |
| F358(P) | ETRI | F358 | Smoothing for the planar mode, single line processing |
| F456 | Nokia | F456 | Diagonal smoothing for modes with abs(intraPredAngle) > 9, using full pixel accuracy, single line processing |
| Harm(G2) | MTK&Mits | F122& F172 | Harmonized design for 2-line gradient based approach for vertical and horizontal modes |
| Harm(GD1) | MediaTek | F122 | Identical to F122(GD1), but only applied to directions 2 and 5 |

Table 2: Summary of the objective performance of different methods tested

|  | Tools enabled for each directionality (intraPredAngles) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Hor/Ver | Low | High | High | -High | Planar | Intra HE |  |  | Intra LC |  |  |
| Test | (0) | (2,5,9) | (13,...,26) | (32) | -(13,...,32) |  | Y | U | V | Y | U | V |
| 1 | F172 |  |  |  |  |  | -0.3 % | -0.2 % | -0.2 % | -0.3 % | -0.3 % | -0.3 % |
| 2 | F122(G1) | F122(GD1) |  |  |  |  | -0.4 % | -0.3 % | -0.3 % | -0.4 % | -0.4 % | -0.4 % |
| 3 | F122(G4) | F122(GD1) |  |  |  |  | -0.4 % | -0.4 % | -0.5 % | -0.4 % | -0.5 % | -0.5 % |
| 4 |  |  | F122(D1) | F122(D1) |  |  | -0.2 % | -0.2 % | -0.2 % | -0.2 % | -0.2 % | -0.2 % |
| 5 |  |  | F122(D1) | F122(D4) |  |  | -0.3 % | -0.3 % | -0.2 % | -0.3 % | -0.3 % | -0.3 % |
| 6 |  | F358 | F358 | F358 |  |  | -0.2 % | -0.2 % | -0.2 % | -0.2 % | -0.3 % | -0.3 % |
| 7 |  |  | F456 | F456 | F456 |  | -0.3 % | -0.2 % | -0.2 % | -0.2 % | -0.3 % | -0.3 % |
| 8 |  |  |  |  |  | F358(P) | -0.1 % | -0.1 % | -0.1 % | -0.1 % | -0.1 % | -0.1 % |
| 9 | F172 | F358 | F358 | F358 |  | F358(P) | -0.5 % | -0.5 % | -0.5 % | -0.5 % | -0.6 % | -0.6 % |
| 10 | F172 |  | F456 | F456 | F456 | F358(P) | -0.6 % | -0.5 % | -0.5 % | -0.5 % | -0.6 % | -0.6 % |
| 11 | F122(G1) | F122(GD1) | F122(D1) | F122(D1) |  | F358(P) | -0.6 % | -0.5 % | -0.6 % | -0.5 % | -0.6 % | -0.7 % |
| 12 | F122(G4) | F122(GD1) | F122(D1) | F122(D4) |  | F358(P) | -0.7 % | -0.7 % | -0.7 % | -0.7 % | -0.8 % | -0.9 % |
| 13 | Harm(G2) | Harm(GD1) | F358/F456 | F122(D4) | F456 | F358(P) | -0.7 % | -0.7 % | -0.7 % | -0.7 % | -0.8 % | -0.8 % |

Table 3: Summary of the run times measured

|  | Tools enabled for each directionality (intraPredAngles) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Hor/Ver | Low | High | High | -High | Planar | Intra HE |  | Intra LC |  |
| Test | (0) | (2,5,9) | (13,...,26) | (32) | -(13,...,32) |  | EncTi | DecTi | EncTi | DecTi |
| 1 | F172 |  |  |  |  |  | 100 % | 101 % | 99 % | 102 % |
| 2 | F122(G1) | F122(GD1) |  |  |  |  | 100 % | 101 % | 100 % | 100 % |
| 3 | F122(G4) | F122(GD1) |  |  |  |  | 100 % | 100 % | 102 % | 101 % |
| 4 |  |  | F122(D1) | F122(D1) |  |  | 100 % | 100 % | 100 % | 100 % |
| 5 |  |  | F122(D1) | F122(D4) |  |  | 100 % | 100 % | 100 % | 101 % |
| 6 |  | F358 | F358 | F358 |  |  | 100 % | 102 % | 100 % | 101 % |
| 7 |  |  | F456 | F456 | F456 |  | 100 % | 100 % | 100 % | 100 % |
| 8 |  |  |  |  |  | F358(P) | 100 % | 100 % | 100 % | 100 % |
| 9 | F172 | F358 | F358 | F358 |  | F358(P) | 101 % | 101 % | 100 % | 101 % |
| 10 | F172 |  | F456 | F456 | F456 | F358(P) | 101 % | 101 % | 100 % | 101 % |
| 11 | F122(G1) | F122(GD1) | F122(D1) | F122(D1) |  | F358(P) | 101 % | 102 % | 101 % | 102 % |
| 12 | F122(G4) | F122(GD1) | F122(D1) | F122(D4) |  | F358(P) | 101 % | 101 % | 101 % | 102 % |
| 13 | Harm(G2) | Harm(GD1) | F358/F456 | F122(D4) | F456 | F358(P) | 101 % | 102 % | 100 % | 102 % |

# Additional results

In order to assess interactions between secondary intra prediction boundary smoothing and SDIP, we integrated the combination of Test 10 into the CE6 SDIP software (using the “HM4.0 SDIP 4.3.1 INTER HE bugfix” software distributed to CE6 participants). The table below summarizes the improvement provided by the Test 10 configuration in SDIP environment (both anchor and Test 10 has SDIP on, full results provided in HM4.0nt-SDIP anchor vs Test10 with SDIP.xls):

Table 4: Test 10 results with SDIP (SDIP enabled for both anchor and Test 10)

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.68% | -0.54% | -0.62% | -0.59% | -0.81% | -0.87% |
| Class B | -0.45% | -0.43% | -0.47% | -0.40% | -0.61% | -0.63% |
| Class C | -0.44% | -0.54% | -0.51% | -0.50% | -0.61% | -0.55% |
| Class D | -0.48% | -0.51% | -0.58% | -0.53% | -0.60% | -0.57% |
| Class E | -0.55% | -0.58% | -0.59% | -0.60% | -0.68% | -0.71% |
| Overall | -0.52% | -0.51% | -0.55% | -0.51% | -0.66% | -0.66% |
|  | -0.52% | -0.52% | -0.55% | -0.52% | -0.66% | -0.66% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

Comparing to CE6.d Test 10 results with HM 4.0 (table below), we can see that SDIP and secondary boundary smoothing seem to have only very minimal interaction: 0.05% for AI_HE and 0.03% for AI_LC.

Table 5: CE6.d Test 10 results

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.67% | -0.58% | -0.57% | -0.55% | -0.70% | -0.76% |
| Class B | -0.48% | -0.42% | -0.42% | -0.41% | -0.57% | -0.59% |
| Class C | -0.55% | -0.50% | -0.49% | -0.59% | -0.66% | -0.59% |
| Class D | -0.60% | -0.51% | -0.62% | -0.63% | -0.58% | -0.61% |
| Class E | -0.59% | -0.45% | -0.52% | -0.56% | -0.68% | -0.62% |
| Overall | -0.57% | -0.49% | -0.52% | -0.54% | -0.63% | -0.63% |
|  | -0.57% | -0.50% | -0.53% | -0.54% | -0.63% | -0.64% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

The table 5 below contains the BD-rate results for the Test 10 including Class F sequences. Class F has smaller gains than other classes, but unlike Test 13, Test 10 has coding efficiency improvements also for that category.

Table 6: Test 10 results with Class F

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.7% | -0.6% | -0.6% | -0.5% | -0.7% | -0.8% |
| Class B | -0.5% | -0.4% | -0.4% | -0.4% | -0.6% | -0.6% |
| Class C | -0.6% | -0.5% | -0.5% | -0.6% | -0.7% | -0.6% |
| Class D | -0.6% | -0.5% | -0.6% | -0.6% | -0.6% | -0.6% |
| Class E | -0.6% | -0.4% | -0.5% | -0.6% | -0.7% | -0.6% |
| Class F | -0.2% | -0.5% | -0.3% | -0.3% | -0.4% | -0.6% |
| Overall | -0.51% | -0.49% | -0.49% | -0.50% | -0.60% | -0.62% |
|  | -0.52% | -0.50% | -0.51% | -0.50% | -0.61% | -0.63% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | N/A |  |  | N/A |  |  |

# Conclusions

Results indicate that combined methods in test category 4 provide good trade-off between coding efficiency improvements and implementation complexity. The two most promising methods appear to be the combinations tested in tests 10 and 13. If JCT-VC values coherent design and minimizing the implementation complexity, the combination of test 10 appears desirable. Similarly, if maximum performance while allowing somewhat more complexity in the design is desirable, the combination of test 13 is to be preferred.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).