| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G566 |
| --- | --- |

| Title: | Syntax Refinements for SAO and ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Semih Esenlik, Matthias Narroschke Thomas Wedi Panasonic R&D Center Germany Monzastr. 4c, 63225 Langen, Germany | Tel: Email: | +49 6103 766 1306 Semih.Esenlik@eu.panasonic.com +49 6103 766 204 Matthias.Narroschke@eu.panasonic.com +49 6103 766 1194 Thomas.Wedi@eu.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

The Adaptation Parameter Set (APS) syntax structure has been adopted during the 6th JCT-VC meeting in Torino to be used in the conveyance of parameter sets of Adaptive Loop Filter (ALF) and Sample Adaptive Offset (SAO). However APS is a very recent syntax structure whose performance and usage needs to be improved and refined. In the current proposal 3 issues related to APS and ALF/SAO syntax are addressed. The proposed solutions provide on average 0.2% coding gain for low-bitrate applications, improve parsing robustness and reduce latency of HEVC.

# Introduction

The Adaptation Parameter Set (APS) [1] syntax structure has been adopted during the 6th JCT-VC meeting in Torino to be used in the conveyance of parameter sets of Adaptive Loop Filter (ALF) and Sample Adaptive Offset (SAO). The benefits provided by the APS can be summarized under two items:

- A suitable means of transportation for SAO and ALF parameter sets.

- Reduction in the ALF and SAO overhead, by the facilitation of information sharing between multiple slices.

However APS is a very recent syntax structure whose performance and usage needs to be improved and refined. In the current proposal 3 issues related to APS addressed.

# Problem Description

The proposal addresses three issues related to ALF and SAO syntax. Following subsections present the issues individually.

## Parsing dependency between APS and Slice Header

The Adaptation Parameter Set is designed to convey parameter set information associated with ALF and SAO filters (and other possible candidate tools). APS inherits the activation and deactivation procedure that is defined for other parameter sets like SPS and PPS. However APS has properties different from the PPS and SPS as follows:

- APS is designed to carry information that is usually large in size and that change more frequently: The ALF and SAO parameter sets have sizes that can theoretically be as large as 1000 to 4000 Bytes respectively. Moreover usually an APS is required to be transmitted corresponding to every frame in a video sequence. Therefore the size of the data that needs to be transmitted by the APS medium contradicts with the assumptions that hold for the SPS and PPS, which are expected to be used much less often and to carry much less information.

- ALF and SAO parameter sets carry information that are less significant for the decoding process compared to the syntax elements conveyed by PPS and SPS: The PPS and SPS carry crucial information regarding the parsing and decoding processes, that make them suitable to be transmitted in guaranteed channels. However the APS carries information only related to the correct application of ALF and SAO processes. Therefore the damage that a lost APS would cause is limited to hindrance in SAO and ALF processes only.

- The APS data has to be conveyed synchronously with the slice data, as opposed to PPS and SPS: The ALF and SAO parameter sets are usually generated for the current frame and required in by the decoder in a timely fashion. Therefore as opposed to SPS or PPS, the APS transmission should be synchronous with the slice data.

Due to the properties of the APS that are listed above, it is expected that the APS is conveyed not through guaranteed mediums together with PPS and SPS, but through regular mediums together with the slice data. As a result the APS is expected to be subject to the packet losses and latency issues that apply to slice data.

Table 1 below shows parts of the slice header syntax in HM4.0. Numbers are assigned to the lines for easy reference and are shown on the left-hand side of the table. Please note the parsing dependency between APS and slice header that is introduced at line 27 by the aps_adaptive_loop_filter_flag. Due to this dependency parsing errors and latencies in the decoding process are expected.

| Line No | slice_header( ) { | Descriptor |
| --- | --- | --- |
| 1 | lightweight_slice_flag | u(1) |
| 2 | if( !lightweight_slice_flag ) { |  |
| 3 | slice_type | ue(v) |
| 4 | pic_parameter_set_id | ue(v) |
| 5 | if( sample_adaptive_offset_enabled_flag ¦¦ adaptive_loop_filter_enabled_flag ) |  |
| 6 | aps_id |  |
| 7 | frame_num | u(v) |
| 8 | …….. | … |
| 9 | } |  |
| 10 | if( entropy_coding_mode_flag && slice_type != I) |  |
| 11 | cabac_init_idc | ue(v) |
| 12 | first_slice_in_pic_flag | u(1) |
| 13 | if( first_slice_in_pic_flag == 0 ) |  |
| 14 | slice_address | u(v) |
| 15 | if( !lightweight_slice_flag ) { |  |
| 16 | slice_qp_delta | se(v) |
| 17 | if( deblocking_filter_control_present_flag ) { |  |
| 18 | disable_deblocking_filter_idc |  |
| 19 | if( disable_deblocking_filter_idc != 1 ) { |  |
| 20 | slice_alpha_c0_offset_div2 |  |
| 21 | slice_beta_offset_div2 |  |
| 22 | } |  |
| 23 | } |  |
| 24 | if( slice_type = = B ) |  |
| 25 | collocated_from_l0_flag | u(1) |
| 26 | byte_align() |  |
| 27 | if( adaptive_loop_filter_enabled_flag && aps_adaptive_loop_filter_flag ) { |  |
| 28 | alf_cu_control_param( ) |  |
| 29 | byte_align() |  |
| 30 | } |  |
| 31 | } |  |

Table 1: Slice header syntax

## APS signalization overhead

In current HM4.0 the on/off flags of ALF and SAO are transmitted in the APS only. Apart from the parsing dependency that is discussed in Section 2.1 three other problems are is expected to occur due to this syntax structure organization:

- Unnecessary latency is introduced in the cases where either one of the SAO or ALF is off: Although these tools can be switched off in the slice/picture level, since the on/off flag is transmitted inside the APS, the decoding process has to be paused till the corresponding APS is available. In other words decoding of a slice can only be completed as soon as the APS that is refers to is available.

- The slice header and APS cannot be parsed in parallel: Since the aps_adaptive_loop_filter_flag is transmitted in the APS, the parsing of the slice is delayed until the corresponding APS is available.

- Superfluous APS NAL unit transmission: Even in the cases where ALF and SAO are both off for a current frame/slice, an APS NAL unit has to be transmitted. The overhead of the NAL unit is especially high in the byte stream format, since the NAL unit also includes a 3-byte start code.

## CU-based adaptation flag coding

In current HM4.0 the alf_cu_control_flag is coded inside the alf_cu_control_param() syntax structure which is in turn encoded with CABAC and placed between the slice header and the slice data. Please note that a CABAC flush operation is performed at position 29 in Table 1 according to F399 of JCT-VC [3]. As a result, placement of alf_cu_control_param() after the slice header requires one additional byte alignment and CABAC termination bits beside the actual CABAC codeword.

Table 2 shows the syntax structure of the alf_cu_control_param(). Please note that if the CU-based adaptation is not used, the syntax structure is composed only of a single bit, which is used to represent the value of alf_cu_control_flag. However additional CABAC termination bits as well as the following byte alignment operation have to be performed in all cases irrespective of the state of CU-based adaptation. On the average 2 additional Bytes are required for CABAC termination bits and byte alignment operation, even if the CU-based adaptation of ALF is not utilized in a current frame/slice.

| alf_cu_control_param() { | Descriptor |
| --- | --- |
| if( adaptive_loop_filter_flag ) { |  |
| alf_cu_control_flag | u(1) |
| if( alf_cu_control_flag ) { |  |
| alf_cu_control_max_depth | ue(v) |
| alf_length_cu_control_info | se(v) |
| for( i = 0; i < NumAlfCuFlag; i++ ) |  |
| alf_cu_flag[ i ] | u(1) ¦ ae(v) |
| } |  |
| } |  |
| } |  |

Table 2: ALF CU-based adaptation syntax structure

# Proposed Solutions

The problems stated in the subsections of the Section 2 are resolved by three minor modifications in the slice header syntax structure and one minor modification in the alf_cu_control_param syntax structure. The modification points are shown highlighted in Table 3 and Table 4. The explanations follow the same organization of subsections of Section 2.

| Line No | slice_header( ) { | Descriptor |
| --- | --- | --- |
| 1 | lightweight_slice_flag | u(1) |
| 2 | if( !lightweight_slice_flag ) { |  |
| 3 | slice_type | ue(v) |
| 4 | pic_parameter_set_id | ue(v) |
| 5 | if( sample_adaptive_offset_enabled_flag ¦¦ adaptive_loop_filter_enabled_flag ){ |  |
|  | if( sample_adaptive_offset_enabled_flag ) |  |
|  | slice_sample_adaptive_offset_flag |  |
|  | if(adaptive_loop_filter_enabled_flag) |  |
|  | slice_adaptive_loop_filter_flag |  |
|  | if(slice_sample_adaptive_offset_flag ¦¦ slice_adaptive_loop_filter_flag) |  |
| 6 | aps_id |  |
|  | } |  |
| 7 | frame_num | u(v) |
| 8 | …….. | … |
| 9 | } |  |
| 10 | if( entropy_coding_mode_flag && slice_type != I) |  |
| 11 | cabac_init_idc | ue(v) |
| 12 | first_slice_in_pic_flag | u(1) |
| 13 | if( first_slice_in_pic_flag == 0 ) |  |
| 14 | slice_address | u(v) |
| 15 | if( !lightweight_slice_flag ) { |  |
| 16 | slice_qp_delta | se(v) |
| 17 | if( deblocking_filter_control_present_flag ) { |  |
| 18 | disable_deblocking_filter_idc |  |
| 19 | if( disable_deblocking_filter_idc != 1 ) { |  |
| 20 | slice_alpha_c0_offset_div2 |  |
| 21 | slice_beta_offset_div2 |  |
| 22 | } |  |
| 23 | } |  |
| 24 | if( slice_type = = B ) |  |
| 25 | collocated_from_l0_flag | u(1) |
|  | if( adaptive_loop_filter_enabled_flag && slice_adaptive_loop_filter_flag ) |  |
|  | alf_cu_adaptation_flag |  |
| 26 | byte_align() |  |
| 27 | if( adaptive_loop_filter_enabled_flag && slice_adaptive_loop_filter_flag && alf_cu_adaptation_flag) { |  |
| 27 | if( adaptive_loop_filter_enabled_flag && aps_adaptive_loop_filter_flag) { |  |
| 28 | alf_cu_control_param( ) |  |
| 29 | byte_align() |  |
| 30 | } |  |
| 31 | } |  |

Table 3: Proposed changes in the slice header

| alf_cu_control_param() { | Descriptor |
| --- | --- |
| if( adaptive_loop_filter_flag ) { |  |
| alf_cu_control_flag | u(1) |
| if( alf_cu_control_flag ) { |  |
| alf_cu_control_max_depth | ue(v) |
| alf_length_cu_control_info | se(v) |
| for( i = 0; i < NumAlfCuFlag; i++ ) |  |
| alf_cu_flag[ i ] | u(1) ¦ ae(v) |
| } |  |
| } |  |
| } |  |

Table 4: Proposed changes in the alf_cu_control_param syntax structure

Note: If the slice_sample_adaptive_offset_flag, slice_adaptive_loop_filter_flag or alf_cu_control_flag are not present in the slice header, their values are inferred to be false.

## Parsing dependency between APS and Slice Header

As explained earlier in Section 2.1, the parsing dependency results from the reference to aps_adaptive_loop_filter_flag in the slice header which is transmitted inside the APS. In order to resolve the dependency, one additional flag (slice_adaptive_loop_filter_flag) is introduced in the slice header which is transmitted in the slice header and replaces aps_adaptive_loop_filter_flag at line 27 of Table 3. As a result no parsing dependency exists between the slice and the APS. The benefits of the proposed modification can be summarized as follows:

- Parsing robustness against losses and latency in the transmission of APS.

- Parallel parsing capability for APS and slice data.

## APS signalization overhead

After line 5 of Table 3 a new flag is introduced (slice_sample_adaptive_offset_flag) in order to convey the state of the SAO operation in the current slice. The slice_sample_adaptive_offset_flag can either be the duplicate of its counterpart in the APS (aps_sample_adaptive_offset_flag) or it can be used to overwrite the SAO decision in the slice level, offering localized adaptation. The benefits of the new slice_sample_adaptive_offset_flag in the slice header can be summarized as follows:

- Together with the slice_adaptive_loop_filter_flag, the slice_sample_adaptive_offset_flag can reduce the latency in the application of SAO and ALF operations in the decoder. In the cases where SAO and ALF are not used in the current slice, the APS is not required to be present in order to decode a slice. Therefore the slice can immediately be outputted thereby reducing the latency.

- Superfluous APS NAL unit transmission is eliminated. In the case where both SAO and ALF are off for a current frame/slice, no APS NAL unit needs to be transmitted. As a result coding gain is achieved especially in low-bitrate applications.

## CU-based adaptation flag coding

In order to reduce the overhead of CABAC termination bits and byte alignment, the coding method of alf_cu_control_flag is changed as in Table 3 and Table 4. The benefits of the proposed modification can be summarized as follows:

- Coding gain especially for low bitrate applications since the additional CABAC termination bits and byte alignment is not necessary when the CU-based adaptation is not used in a current frame/slice.

- Savings in clock cycles since the CABAC engine is not used in the cases when the CU-based adaptation is not used in a current frame/slice.

# Simulation Results

During the simulations the Revision 1423 branch of the HM4.0-dev-misc [2] software has been used due to the fact that the HM4.1 software was not delivered in a timely fashion. Moreover 3 sets of simulation results are provided as follows:

- Simulation results according to [4], with slice mode equal to 2.

- Simulation results with data points equal to QP 31, 34, 37, 40 and with slice mode equal to 2.

- Simulation results according to Annex B of [5], and with data points QP 31, 34, 37, 40 and with slice mode equal to 2.

During the first set of simulations maximum slice size of 1500Bytes is used. The other test conditions are used from [4]. The simulation results of this test case are presented in Table 5. Please note that the encoding times are not reliable, more precise encoding time measurements are going to be provided as soon as they are available.

The second set of simulations is used to demonstrate low-bitrate applications. Compared to the first set of simulations the data points are changed to QP 31, 34, 37 and 40.

Please note that the proposal reduces the amount of APS NAL unit transmission since the states of ALF and SAO are now known in the slice header. Therefore if the SAO and ALF are off for a slice, there is no need to transmit an APS NAL unit anymore. In order to show the benefit of the reduction in NAL unit transmission a third set of simulation results are provided. In Table 7 the bitrates are measured according to Annex B of [5], where the bitrate of the byte-stream format is measured. This means that the overhead resulting from NAL unit start codes are also taken into consideration.

Table 5: Simulation results according to [4] with 1500Byte slices

Table 6: Simulation results with data points: QP 31,34,37,40 and 1500Byte slices

Table 7: Simulation results according to Annex B of [5], with data points: QP 31,34,37,40 and 1500Byte slices

# Finally the following table shows the simulation results for the first two parts (inclusion of slice_sample_adaptive_offset_flag and slice_adaptive_loop_filter_flag in the slice header) of the proposal. In the simulations 1500Byte slices are used.

Table 8: Simulation results for inclusion of slice_adaptive_loop_filter_flag and slice_sample_adaptive_offset_flag, with 1500Byte slices

# Conclusions

In the proposal 3 solutions are provided as refinements to the APS and ALF/SAO syntax for HM4.0 The proposed solutions improve the parsing robustness and reduce the latency in the decoding process. Moreover on average 0.2% coding gain is achieved in the low-bitrate test conditions where data points are changed to QP 31, 34, 37 and 40. The three solutions are proposed to be included in the next version of HM.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- S. Wenger et al., “Adaptation Parameter Set (APS)”, JCTVC-F747, Torino, IT, August 2011

- HM 4.0-dev-brances Software, Revision: 1423, http://hevc.kw.bbc.co.uk/trac/browser#jctvc-hm/branches/HM-4.0-dev-misc

- M. Narroschke et al., “Restart of CABAC after coding of ALF and SAO slice header data”, JCTVC-F399, Torino, July 2011

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, Torino, July 2011

- B. Bross et al, “WD4: Working Draft 4 of High-Efficiency Video Coding”, JCTVC-F803_d5, Torino, July 2011