| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G567 |
| --- | --- |

| Title: | Simplified DC prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jani Lainema Kemal Ugur | Email: | jani.lainema@nokia.com kemal.ugur@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes to apply subsampling to the intra prediction reference samples when calculating DC prediction for Intra mode PUs. It is reported that the number of reference samples generating the DC prediction value in HM 4.0 can be reduced to a quarter of the original number without affecting compression efficiency of the draft HEVC standard.

# Introduction

HM 4.0 generates the DC prediction value by averaging all the Intra prediction reference samples directly above and directly to the left of the PU. It was found that the number of reference samples that need to be averaged is unnecessarily large and a simplified design with averaging quarter of the samples provides the same compression efficiency.

Figure 1 below illustrates the reference samples averaged in order to calculate the DC value for a DC predicted PU of size 16x16 pixels in HM 4.0.

Figure 1: Reference samples averaged to obtain DC prediction value for a 16x16 PU

# Proposed method

It is proposed to simplify the process for generating the DC prediction value and average only every 4th sample on the reference row above and the reference column to the left of the blocks. Figure 2 below illustrates locations of the reference samples used for generating the DC prediction value in the proposed approach.

Figure 2: Reference samples generating the DC prediction value in the proposed algorithm for a PU of size 16x16 pixels.

# Proposed specification text and source code

Only proposed change to WD 4 draft 5 is to revise Equation (8-32) to:

DCVal =

The source code implementing the proposal in HM 4.0 environment is provided as an attachment to this contribution.

# Objective performance

The table below summarizes objective performance of the proposed change with respect to the HM 4.0 anchors. Detailed data is provided as an attachment to this contribution.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.00% | -0.07% | -0.04% | 0.00% | -0.05% | -0.04% |
| Class B | 0.03% | -0.04% | -0.04% | 0.01% | -0.06% | -0.05% |
| Class C | 0.05% | -0.01% | -0.01% | 0.03% | -0.03% | -0.07% |
| Class D | 0.01% | -0.04% | -0.07% | 0.01% | 0.00% | -0.06% |
| Class E | 0.00% | -0.06% | -0.07% | -0.02% | -0.18% | -0.07% |
| Overall | 0.02% | -0.04% | -0.05% | 0.01% | -0.06% | -0.06% |
|  | 0.02% | -0.05% | -0.05% | 0.01% | -0.06% | -0.06% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Conclusions

As the proposed method simplifies HEVC design without affecting coding efficiency, we propose including it into WD5 of the HEVC standard.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).