| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G570 |
| --- | --- |

| Title: | Byte alignment overhead reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Semih Esenlik, Matthias Narroschke, Thomas Wedi Panasonic R&D Center Germany Monzastr. 4c, 63225 Langen, Germany | Tel: Email: | +49 6103 766 1306 Semih.Esenlik@eu.panasonic.com +49 6103 766 204 Matthias.Narroschke@eu.panasonic.com +49 6103 766 1194 Thomas.Wedi@eu.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

In the current HM4.0 byte alignment is used in various places in order to facilitate several purposes such as Emulation Prevention Byte (EPB) insertion and alignment of the bitstream in the encoder and decoder. The proposal introduces a coding method for the coding of syntax elements such that byte alignment condition is achieved automatically and the need for an explicit byte alignment function is eliminated. As a result the 0.1% bitrate reduction is achieved on average.

# Introduction

In the current HM4.0 byte alignment is used in various places in order to facilitate;

- Emulation Prevention Byte (EPB) insertion,

- Alignment of the bitstream in the encoder and decoder.

However byte alignment operation consists of adding padding bits into the bitstream and increases the bitrate.

# Byte alignment in HM4.0

The byte alignment function is provided below in Table 1.

| byte_align( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| bit_equal_to_one | f(1) |
| } |  |

Table 1: Byte alignment function

The byte alignment operation is basically a padding operation that is used at the beginning of each CABAC codeword that needs to be inserted in a NAL unit. Usage of byte alignment before the CABAC codeword facilitates EPB insertion, by rendering the EPB insertion possible on-the-fly during the CABAC coding. As a result the CABAC codeword that is generated by the CABAC engine can be inserted in a specified byte aligned position in a NAL unit without introducing any accidental start codes in the total bitstream.

However the cost of each byte alignment operation is 3.5 bits on average. The mandatory byte alignment operations cause considerable amount of losses especially in the low-bitrate scenarios. The Table 2 shows the APS syntax structure. The syntax structure has been taken from [1] and modified in order to reflect the revision 1423 of the HM software [2]. Please note that in the case when both aps_sample_adaptive_offset_flag and aps_adaptive_loop_filter_flag are positive, meaning that the APS carries information about both ALF and SAO, 4 byte alignment operations have to be performed resulting in 14 padding bits on average.

| Line No | aps_rbsp( ) { | Descriptor |
| --- | --- | --- |
| 1 | aps_id | ue(v) |
| 2 | aps_sample_adaptive_offset_flag | u(1) |
| 3 | aps_adaptive_loop_filter_flag | u(1) |
| 4 | if( aps_sample_adaptive_offset_flag ¦¦ aps_adaptive_loop_filter_flag ) { |  |
| 5 | aps_cabac_use_flag | u(1) |
| 6 | if( aps_cabac_use_flag ) { |  |
| 7 | aps_cabac_init_idc | ue(v) |
| 8 | aps_cabac_init_qp_minus26 | se(v) |
| 9 | } |  |
| 10 | } |  |
| 11 | if(aps_sample_adaptive_offset_flag ) { |  |
| 12 | sao_data_byte_count | u(12) |
| 13 | byte_align() |  |
| 14 | sao_param() | ae(v) |
| 15 | if(aps_adaptive_loop_filter_flag) |  |
| 16 | byte_align() |  |
| 17 | } |  |
| 18 | if(aps_adaptive_loop_filter_flag) { |  |
| 19 | alf_data_byte_count | u(8) |
| 20 | byte_align () |  |
| 22 | alf_param() | ae(v) |
| 23 | } |  |
| 24 | rbsp_trailing_bits( ) |  |
| 25 | } |  |

Table 2: APS syntax

# Byte alignment overhead reduction

In order to reduce the impact of byte alignment padding bits, a new coding method is proposed which is called “byte-aligned variable-length code”. The features of the coding method are as follows:

- Variable length code words are generated that can be extended without limit.

- The last bit of each codeword lies at the end of a byte. In other words lengths of the codes can be varied in 8-bit increments; therefore the byte alignment condition is automatically achieved without an explicit byte alignment operation.

The construction of the code is explained in Section 4 and the parsing process in the decoder is explained in section 10.2.2.

The proposed byte-aligned variable-length codes are used to code syntax elements that precede a byte alignment operation. Since the coded syntax elements achieve byte alignment condition automatically, an explicit byte alignment operation is rendered unnecessary.

In the current proposal 3 syntax elements are proposed to be coded with byte-aligned variable length codes which are alf_data_byte_count, sao_data_byte_count and slice_qp_delta. The modifications in the syntax are presented in Section 10.1.

In the current HM, the two syntax elements alf_data_byte_count and sao_data_byte_count are coded with fixed length codes. As a result the encoder has to make sure that the sizes of the data pertaining to alf_param() and sao_param() do not exceed the representation range. In other words the alf_data_byte_count and sao_data_byte_count have upper limits of 28 and 212 respectively, and if they are exceeded re-encoding of the ALF and SAO parameter sets are required.

Another benefit provided by the proposal is the variable length coding of alf_data_byte_count and sao_data_byte_count syntax elements. Since the codewords can be extended indefinitely in the theory, no encoding checks and/or reencoding mechanisms are required for the coding of ALF and SAO parameter sets anymore.

# Construction of byte-aligned variable-length codes

The proposed coding method can be described under two items:

- The syntax elements are coded in a right bit first (least significant bit first) way.

- The variable length property is achieved in the codeword by the use of termination bits at byte end positions (the last bit of a byte is the termination bit).

Below is an example for coding of a syntax element that has the unsigned integer value of “91” (Binary: 1011011). The bits of the syntax element are written in the bitstream (starting at the current position of the bitstream pointer) in the order of increasing significance. The last bit of each byte is reserved for the “termination bit”.

The following figure demonstrates the example. The coding of the previous syntax element (represented by “xxxx”) ends at the 4th bit of Byte 1. Starting at this position in the bitstream, the least significant 3 bits of “1011011” are written in the order of increasing significance.

After the 3 least significant bits of the syntax element are written in “Byte 1”, the termination bit is appended whose value is set to 0 in “Byte 1” (Note that at least 7 bits are required to represent the value 91). The bits of the syntax element are resumed to be written starting at the beginning of “Byte 2”. And a second termination bit is appended at the end of “Byte 2” whose value is set to 1. The termination bit with value 1 signals that the codeword is terminated.

Figure 1: Example code that represents value 91

For the precise description of the parsing process, please refer to the section of 10.2.2, which describes the parsing process of byte-aligned variable-length codewords.

# Bitrate Comparison with Exp-Golomb code

Following two tables show the bitrate analysis for Exp-Golomb code and the proposed byte-aligned variable length code. Average lengths of the codes with byte alignment padding bits are given on the left columns. (For Exp-Golomb code please refer to Section 9.1 of the Working Draft.)

| Range of codeNum | Bit string form | Code length + 3.5 average byte alignment bits |
| --- | --- | --- |
| 0 | 1 | 1+3.5 = 4.5 |
| 1..2 | 0 1 x0 | 3+3.5 = 6.5 |
| 3..6 | 0 0 1 x1 x0 | 5+3.5 = 8.5 |
| 7..14 | 0 0 0 1 x2 x1 x0 | 7+3.5 = 10.5 |
| 15..30 | 0 0 0 0 1 x3 x2 x1 x0 | 9+3.5 = 12.5 |
| 31..62 | 0 0 0 0 0 1 x4 x3 x2 x1 x0 | 11+3.5 = 14.5 |
| 63..126 | … | 13+3.5 = 16.5 |
| 127..254 | … | 15+3.5 = 18.5 |
| 255..510 | … | 17+3.5 = 20.5 |
| 511..1022 | … | 19+3.5 = 22.5 |

Table 3 Bitrate analysis for Exp-Golomb codes

| Range of codeNum | Bit string form |  | Average code length |
| --- | --- | --- | --- |
|  | Largest codeword | Smallest codeword |  |
| 0..1 | 0 x0 0 0 0 0 0 0 1 | x0 1 | 5.5 |
| 2..3 | x0 0 x1 0 0 0 0 0 0 1 | x0 x1 1 | 6.5 |
| 4..7 | x0 x1 0 x2 0 0 0 0 0 0 1 | x0 x1 x2 1 | 7.5 |
| 8..15 | x0 x1 x2 0 x3 0 0 0 0 0 0 1 | x0 x1 x2 x3 1 | 8.5 |
| 16..31 | x0 x1 x2 x3 0 x4 0 0 0 0 0 0 1 | x0 x1 x2 x3 x4 1 | 9.5 |
| 32..63 | … | … | 10.5 |
| 64..125 |  |  | 11.5 |
| 126..255 |  |  | 13.5 |
| 256..511 |  |  | 14.5 |
| 512..1023 |  |  | 15.5 |

Table 4 Bitrate analysis byte aligned variable length codes

The proposed byte aligned variable length code achieves smaller bitrates in the coding of all values but 0. In the coding of the value 0, exp-Golomb code spends 4.5 bits (including the average 3.5 byte alignment padding bits), whereas the byte aligned variable length code spends 5.5 bits. However for all values other than 0, the byte-aligned variable length codes achieve smaller bitrates.

Moreover the bitrate savings increase as the value to be coded increases. Starting with the value 32, the byte-aligned variable code starts outperforming exp-golomb codes even in the absence of alignment padding bits. In other words, byte-aligned code achieves at least 4 bits smaller code lengths than exp-golomb code if the value to be coded is greater than 32.

# Simulation results

During the simulations the Revision 1423 branch of the HM4.0-dev-misc [2] software has been used due to the fact that the HM4.1 software was not delivered in a timely fashion. 2 sets of simulation results are provided as follows:

- Simulation results according to [4], with slice mode equal to 2.

- Simulation results with data points equal to QP 31, 34, 37, 40 and with slice mode equal to 2.

During the first set of simulations maximum slice size of 1500Bytes is used. The other test conditions are used from [4]. The simulation results of this test case are presented in Table 5. Please note that the encoding times are not reliable, more precise encoding time measurements are going to be provided as soon as they are available together with simulation results for the remaining test cases.

The second set of simulations is used to demonstrate low-bitrate applications. Compared to the first set of simulations the data points are changed to QP 31, 34, 37 and 40.

Table 5: Simulation results according to [4], with 1500Byte slices.

Table 6: Simulation results with data points QP 31, 34, 37, 40 and with 1500Byte slices.

# Conclusions

A new coding method which is called “byte-aligned variable length code” is introduced. The proposed coding method achieves byte alignment condition automatically, rendering any explicit byte alignment operation unnecessary. As a result an average coding gain of 0.1% is achieved. The new coding method is proposed to be included in the next version of HM.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- S. Wenger et al., “Adaptation Parameter Set (APS)”, JCTVC-F747, Torino, IT, August 2011

- HM 4.0-dev-brances Software, Revision: 1423, http://hevc.kw.bbc.co.uk/trac/browser#jctvc-hm/branches/HM-4.0-dev-misc

- M. Narroschke et al., “Restart of CABAC after coding of ALF and SAO slice header data”, JCTVC-F399, Torino, July 2011

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, Torino, July 2011

- B. Bross et al, “WD4: Working Draft 4 of High-Efficiency Video Coding”, JCTVC-F803_d5, Torino, July 2011

# Proposed changes in the Working Draft

## Syntax

### Adaptation Parameter Set RBSP syntax

| Line No | aps_rbsp( ) { | Descriptor |
| --- | --- | --- |
| 1 | aps_id | ue(v) |
| 2 | aps_sample_adaptive_offset_flag | u(1) |
| 3 | aps_adaptive_loop_filter_flag | u(1) |
| 4 | if( aps_sample_adaptive_offset_flag ¦¦ aps_adaptive_loop_filter_flag ) { |  |
| 5 | aps_cabac_use_flag | u(1) |
| 6 | if( aps_cabac_use_flag ) { |  |
| 7 | aps_cabac_init_idc | ue(v) |
| 8 | aps_cabac_init_qp_minus26 | se(v) |
| 9 | } |  |
| 10 | } |  |
| 11 | if(aps_sample_adaptive_offset_flag ) { |  |
| 12 | sao_data_byte_count | ub(v) u(12) |
| 13 | byte_align() |  |
| 14 | sao_param() | ae(v) |
| 15 | if(aps_adaptive_loop_filter_flag) |  |
| 16 | byte_align() |  |
| 17 | } |  |
| 18 | if(aps_adaptive_loop_filter_flag) { |  |
| 19 | alf_data_byte_count | ub(v) u(8) |
| 20 | byte_align () |  |
| 22 | alf_param() | ae(v) |
| 23 | } |  |
| 24 | rbsp_trailing_bits( ) |  |
| 25 | } |  |

Table 7 APS syntax structure

### Slice Header syntax

| Line No | slice_header( ) { | Descriptor |
| --- | --- | --- |
| 1 | lightweight_slice_flag | u(1) |
| 2 | if( !lightweight_slice_flag ) { |  |
| 3 | slice_type | ue(v) |
| 4 | pic_parameter_set_id | ue(v) |
| 5 | if( sample_adaptive_offset_enabled_flag ¦¦ adaptive_loop_filter_enabled_flag ) |  |
| 6 | aps_id |  |
| 7 | frame_num | u(v) |
| 8 | if( IdrPicFlag ) |  |
| 9 | idr_pic_id | ue(v) |
| 10 | if( pic_order_cnt_type = = 0 ) |  |
| 11 | pic_order_cnt_lsb /* | u(v) |
| 12 | if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| 13 | num_ref_idx_active_override_flag | u(1) |
| 14 | if( num_ref_idx_active_override_flag ) { |  |
| 15 | num_ref_idx_l0_active_minus1 | ue(v) |
| 16 | if( slice_type = = B ) |  |
| 17 | num_ref_idx_l1_active_minus1 | ue(v) |
| 18 | } |  |
| 19 | } |  |
| 20 | ref_pic_list_modification( ) |  |
| 21 | ref_pic_list_combination( ) |  |
| 22 | if( nal_ref_idc != 0 ) |  |
| 23 | dec_ref_pic_marking( ) |  |
| 24 | } |  |
| 25 | if( entropy_coding_mode_flag && slice_type != I) |  |
| 26 | cabac_init_idc | ue(v) |
| 27 | first_slice_in_pic_flag | u(1) |
| 28 | if( first_slice_in_pic_flag == 0 ) |  |
| 29 | slice_address | u(v) |
| 30 | if( !lightweight_slice_flag ) { |  |
| 31 | slice_qp_delta | se(v) |
| 32 | if( deblocking_filter_control_present_flag ) { |  |
| 33 | disable_deblocking_filter_idc |  |
| 34 | if( disable_deblocking_filter_idc != 1 ) { |  |
| 35 | slice_alpha_c0_offset_div2 |  |
| 36 | slice_beta_offset_div2 |  |
| 37 | } |  |
| 38 | } |  |
| 39 | if( slice_type = = B ) |  |
| 40 | collocated_from_l0_flag | u(1) |
| 41 | } |  |
| /* | if (entropyCodingSynchro) | */ |
| /* | Wavefront substream size information | */ |
| /* | if (tileBoundaryIndependanceFlag) | */ |
| /* | Tile location information | */ |
| 42 | if( !lightweight_slice_flag ) { |  |
| 43 | slice_qp_delta | ib(v) |
| 44 | } |  |
| 45 | byte_align() |  |
| 46 | if( adaptive_loop_filter_enabled_flag && aps_adaptive_loop_filter_flag ) { |  |
| 47 | alf_cu_control_param( ) |  |
| 48 | byte_align() |  |
| 49 | } |  |
| 50 | } |  |

Table 8 Slice Header syntax structure

## Semantics

### Specification of syntax functions and descriptors

- ib(v): signed integer byte-aligned variable-length-coded syntax element with sign bit first followed by the bits of the integer value in the order of increasing significance. The parsing process for this descriptor is specified in subclause 9.x.

- ub(v): unsigned integer byte-aligned variable-length-coded syntax element with least significant bit first. The parsing process for this descriptor is specified in subclause 9.x.

### Parsing process for byte-aligned variable-length codes

This process is invoked when the descriptor of a syntax element in the syntax tables in subclause Error: Reference source not found is equal to ib(v) or ub(v).

Inputs to this process are bits from the RBSP.

Outputs of this process are syntax element values.

The syntax elements that are coded with ib(v) have signed integer values, whereas the syntax elements coded with ub(v) have unsigned integer values.

The parsing process of the syntax elements that are coded with ib(v) begins with reading the first bit starting at the current location in the bitstream. The assignment of the variable codeNum is specified as follows:

signOfInteger = 1

signOfInteger = read_bits(1)

binaryDigit = 0

codeNum = 0

for (stopBit = 0; !stopBit; )

{

b = read_bits( 1 )

if (byte_aligned)

stopBit = b

else

{

codeNum += (signOfInteger) ? (b << binaryDigit) : - (b << binaryDigit)

binaryDigit++

}

}

The parsing process of ub(v) is defined same as the parsing process of ib(v), except that the second instruction, namely “signOfInteger = read_bits(1)”, is skipped.