| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G572 |
| --- | --- |

| Title: | AHG16: Reference sample padding harmonization for intra DC mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Viktor Wahadaniah ChongSoon Lim Block 1022 Tai Seng Avenue #06-3530 Singapore 534415 | Tel: Email: | (65) 6550 5415 viktor.wahadaniah@sg.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution reports the HM-4.0 simulation results of a revised reference sample padding scheme for intra DC mode initially presented in JCTVC-F414. The revised padding scheme is reported to give average BD-rate gains of 0.1% for intra-only setting with 1500-byte slices and constrained intra prediction enabled. It is reported that encoding and decoding runtimes are not affected by the revised padding scheme. It is suggested that JCT-VC consider the trade-off between design simplicity and conceptual accuracy of intra DC prediction and subsequently decide whether a revised padding scheme for intra DC prediction is desirable. It is further recommended that JCT-VC adopt the proposed scheme if conceptual accuracy of intra DC prediction is desired.

# Introduction

A scheme for padding unavailable intra reference samples using adjacent available reference samples was presented in JCTVC-E488 ([1]). A possible issue when intra DC prediction is performed using padded reference samples was reported in a joint contribution from NEC and Panasonic JCTVC-F414 ([2]). A possible solution was presented in JCTVC-F414 in the form of a revised padding scheme with minor modification to HEVC design.

Experimental results of the revised padding scheme were originally reported in JCTVC-F414 where BD-rate and BD-PSNR results were presented with reference to HM-3.1 anchor. It was reported that average BD-rate reduction of up to 0.1% resulted from the revised padding scheme when constrained intra prediction is enabled and 1500-byte slices are used.

The purpose of this contribution is to report the HM-4.0 experimental results of the revised padding scheme as described in JCTVC-F414.

In addition, a simple harmonization with the filtering process for intra DC prediction samples as described in HEVC Working Draft 4 ([3]) was implemented and tested. In the harmonized design, the filtering of the samples predSamples[ x, 0 ] is performed when and only when all neighbouring samples p[ x, –1 ] with x = 0..nS-1 are available prior to the reference sample substitution process. Similarly, the filtering of the samples predSamples[ 0, y ] is performed when and only when all neighbouring samples p[ –1, y ] with y = 0..nS-1 are available prior to the reference sample substitution process.

# Experimental Setting

The revised padding scheme was implemented into HM-4.0 software (revision 1354). Simulations were conducted on six out of eight test conditions as specified in JCTVC-F900 ([4]), namely:

- Intra, High Efficiency

- Intra, Low Complexity

- Random Access, High Efficiency

- Random Access, Low Complexity

- Low Delay, High Efficiency (B slices)

- Low Delay, Low Complexity (B slices)

Class A to class E test sequences and four quantization parameter values as specified in JCTVC-F900 were used. Three sets of simulations were conducted using a modified test setting with 1500-byte slices and constrained intra prediction enabled.

- HM-4.0 anchor

- Modified HM-4.0 using revised padding

- Modified HM-3.1 using revised padding and harmonized intra DC prediction filtering

# Experimental results

Experimental results are shown in Table 1 and Table 2. It is observed from the results that the revised padding scheme gives small but consistent BD-rate reduction of up to 0.1% in intra only setting, consistent with the previous results collected using HM-3.1. It is also observed that the simple harmonization with intra DC prediction filtering brings about a small additional amount of BD-rate reduction.

Table 1 - Coding efficiency of revised padding scheme compared to HM-4.0 ([b] vs. [a])

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% |
| Class B | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class C | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class D | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class E | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| All | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Enc Time | 100% |  |  | 100% |  |  |
| Dec Time | 100% |  |  | 100% |  |  |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1% | 0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | -0.1% | -0.2% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class E |  |  |  |  |  |  |
| All | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| Class C | 0.0% | -0.1% | 0.1% | 0.0% | 0.1% | -0.1% |
| Class D | 0.0% | -0.3% | -0.4% | 0.0% | 0.3% | 0.4% |
| Class E | 0.2% | 0.6% | 1.3% | 0.1% | 0.7% | 0.4% |
| All | 0.0% | 0.0% | 0.1% | 0.0% | 0.3% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 2 - Coding efficiency of revised padding scheme with harmonized intra DC prediction filtering compared to HM-4.0 ([c] vs. [a])

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | -0.2% |
| Class B | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.2% |
| Class C | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class D | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class E | -0.1% | -0.1% | -0.2% | -0.1% | -0.2% | -0.2% |
| All | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% | -0.1% |
| Class B | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% | 0.0% |
| Class C | 0.0% | 0.0% | -0.2% | -0.1% | 0.1% | 0.0% |
| Class D | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| All | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.2% | -0.1% | 0.0% | 0.1% | -0.2% |
| Class C | 0.0% | -0.1% | 0.1% | 0.0% | 0.1% | 0.0% |
| Class D | 0.0% | -0.5% | -0.4% | 0.0% | 0.4% | 0.0% |
| Class E | 0.0% | 0.2% | 1.0% | -0.1% | 0.8% | 0.0% |
| All | 0.0% | 0.0% | 0.1% | 0.0% | 0.3% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Conclusion

This contribution reports the HM-4.0 simulation results of a revised reference sample padding scheme for intra DC mode initially presented in JCTVC-F414. The revised padding scheme is reported to give average BD-rate gains of 0.1% for intra-only setting with 1500-byte slices and constrained intra prediction enabled. It is reported that encoding and decoding runtimes are not affected by the revised padding scheme. It is suggested that JCT-VC consider the trade-off between design simplicity and conceptual accuracy of intra DC prediction and subsequently decide whether a revised padding scheme for intra DC prediction is desirable. It is further recommended that JCT-VC adopt the proposed scheme if conceptual accuracy of intra DC prediction is desired.

# References

[1]	R. Sjöberg. C. Lai, K. Chono, V. Wahadaniah, “BoG report and proposal on padding of unavailable reference samples for intra prediction,” JCT-VC document JCTVC-E488, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 5th meeting, Geneva, CH, March 2011.

[2]	V. Wahadaniah, C. S. Lim, K. Chono, H. Aoki, “Reference sample padding harmonization for intra DC mode,” JCT-VC document JCTVC-F414, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 6th meeting, Torino, IT, July 2011.

[3]	B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “WD4: Working Draft 4 of High-Efficiency Video Coding,” JCT-VC document JCTVC-F803_d5, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 6th meeting, Torino, IT, July 2011.

[4]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC document JCTVC-F900, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 6th meeting, Torino, IT, July 2011.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix: Proposed text changes for HEVC Working Draft

Proposed working draft text is included below using the following text conventions:

•	Unchanged text that are not shown here is indicated with "…"

•	Removed text is shown with strikethrough.

•	Newly added text is shown in yellow highlight.

…

8.3.3.1.1		Reference sample substitution process for intra sample prediction

…

Outputs of this process are the modified reference samples p[ x, y ] with x = -1, y = -1..nS*2-1 and x = 0..nS*2-1, y = -1 for intra sample prediction.

Outputs of this process are:

- the variables leftSamplesAvailBeforePaddingFlag and aboveSamplesAvailBeforePaddingFlag.

- the modified reference samples p[ x, y ] with x = -1, y = -1..nS*2-1 and x = 0..nS*2-1, y = -1 for intra sample prediction.

If all samples p[ -1, y ], with y = 0..nS-1 are marked as “available for intra prediction,” the variable leftSamplesAvailBeforePaddingFlag is set to 1. Otherwise, leftSamplesAvailBeforePaddingFlag is set to 0.

If all samples p[ x, -1 ], with x = 0..nS-1 are marked as “available for intra prediction,” the variable aboveSamplesAvailBeforePaddingFlag is set to 1. Otherwise, aboveSamplesAvailBeforePaddingFlag is set to 0.

…

8.3.3.1.5		Specification of Intra_DC prediction mode

…

…

The values of the prediction samples predSamples[ x, y ], with x, y = 0..nS-1, are derived as the following ordered steps:

The values of the prediction samples predSamples[ x, y ], with x, y = 0..nS-1, are derived as follows.

–	If leftSamplesAvailBeforePaddingFlag is equal to 1 and aboveSamplesAvailBeforePaddingFlag is equal to 1, the values of the prediction samples predSamples[ x, y ], with x, y = 0..nS-1 are derived as:

- A variable DCVal is derived as:

DCVal =x'=0nS−1p[x', −1]+y'=0nS−1p[−1, y']+nS>>(k+1), with x, y = 0..nS-1	(8-1)
where k=log2(nS)

- Depending on the prediction block size nS and the chroma component index cIdx, the following applies.

- If cIdx is equal to 0 and nS is less than 64, the following applies.

predSamples[ 0, 0 ] = ( 1*p[ -1, 0 ] + 2*DCVal + 1*p[ 0, -1 ] + 2 ) >> 2	(8-2)
predSamples[ x, 0 ] = ( 1*p[ x, -1 ] + 3*DCVal + 2 ) >> 2, with x = 1..nS-1	(8-3)
predSamples[ 0, y ] = ( 1*p[ -1, y ] + 3*DCVal + 2 ) >> 2, with y = 1..nS-1	(8-4)
predSamples[ x, y ] = DCVal, with x, y = 1..nS-1	(8-5)

- Otherwise, the prediction samples predSamples[ x, y ] are derived as

predSamples[ x, y ] = DCVal, with x, y = 0..nS-1	(8-6)

–	Otherwise, if leftSamplesAvailBeforePaddingFlag is equal to 1 and aboveSamplesAvailBeforePaddingFlag is equal to 0, the values of the prediction samples predSamples[ x, y ], with x, y = 0..nS-1 are derived as

- A variable DCVal is derived as:

DCVal =, with y = 0..nS-1	(8-x)
where k=log2(nS)

- Depending on the prediction block size nS and the chroma component index cIdx, the following applies.

- If cIdx is equal to 0 and nS is less than 64, the following applies.

predSamples[ 0, 0 ] = ( 1*p[ -1, 0 ] + 2*DCVal + 1*p[ 0, -1 ] + 2 ) >> 2	(8-x)
predSamples[ 0, y ] = ( 1*p[ -1, y ] + 3*DCVal + 2 ) >> 2, with y = 1..nS-1	(8-x)
predSamples[ x, y ] = DCVal, with x = 1..nS-1, y = 0..nS-1	(8-x)

- Otherwise, the prediction samples predSamples[ x, y ] are derived as

predSamples[ x, y ] = DCVal, with x, y = 0..nS-1					(8-x)

–	Otherwise, if leftSamplesAvailBeforePaddingFlag is equal to 0 and aboveSamplesAvailBeforePaddingFlag is equal to 1, the values of the prediction samples predSamples[ x, y ], with x, y = 0..nS-1 are derived as

- A variable DCVal is derived as:

DCVal =, with x = 0..nS-1	(8-x)
where k=log2(nS)

- Depending on the prediction block size nS and the chroma component index cIdx, the following applies.

- If cIdx is equal to 0 and nS is less than 64, the following applies.

predSamples[ 0, 0 ] = ( 1*p[ -1, 0 ] + 2*DCVal + 1*p[ 0, -1 ] + 2 ) >> 2	(8-x)
predSamples[ x, 0 ] = ( 1*p[ x, -1 ] + 3*DCVal + 2 ) >> 2, with x = 1..nS-1	(8-x)
predSamples[ x, y ] = DCVal, with x = 0..nS-1, y = 1..nS-1	(8-x)

- Otherwise, the prediction samples predSamples[ x, y ] are derived as

predSamples[ x, y ] = DCVal, with x, y = 0..nS-1	(8-x)

–	Otherwise, the values of the prediction samples predSamples[ x, y ], with x, y = 0..nS-1 are derived as

predSamples[ x, y ] = 1 << ( ( BitDepthY + increased_bit_depth_luma ) – 1 )	(8-x)

…