| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G575 |
| --- | --- |

| Title: | Transform skip mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Mrak, Andrea Gabriellini, David Flynn BBC R&D, Centre House, 56 Wood Lane W12 7SB London, UK |  | marta.mrak@bbc.co.uk andrea.gabriellini@bbc.co.uk davidf@rd.bbc.co.uk |
| Source: | British Broadcasting Corporation |  |  |

_____________________________

# Abstract

This contribution is related to the work of AdHoc Group 19: Transform Skipping. Starting from the initial Transform Skip Mode (TSM) proposal (JCTVC-F077), this contribution reports a number of harmonization and evaluation aspects of TSM. The harmonization aspects include interaction with RQT, NSQT, scans, and signaling. Also, a detailed study of skip transform throughput is provided and a new implementation of transform skipping is presented achieving the same compression performance of the original implementation while providing stricter guarantees in terms of data not overflowing 16-bit registers for worst case input sets. The tests evaluate the compression performance of TSM at different levels of complexity. In all tested configurations gains are the highest for class F, showing BD-rate gains of up to 4.6% for the luma component.

# Introduction

In the context of HEVC, Transform Skip Mode (TSM) was first proposed in [F077]. It was demonstrated that for inter predicted blocks skipping one or both 1D DCT transforms could lead to better compression performance. Moreover, in this way a number of transform steps at the decoder is reduced. Previous works [R1-R4] on transform skipping and 1D transforms have also demonstrated suitability of such approaches for motion compensated blocks.

The release of a new version of the reference software, HM-4.0, and the addition of new techniques affecting the way transform units are managed for inter predicted blocks (in particular NSQT) have required the development of new code to deliver the same technique presented in [F077]. Additionally, the RDO search mechanism supporting any RQT setting has been developed. Once the transform skipping functionality has been added to HM-4.0 a number of tests has been performed to evaluate the compression performance of the technique in different configurations. In particular, different depths of RQT have been used. Finally, a new implementation of transform skipping has been presented, aiming to satisfy stricter guarantees on data not overflowing 16-bit registers for worst case set of inputs.

# TSM and block structures

TSM defines transform skip in one or both directions on which a transform would be applied under common conditions, Figure 1. As illustrated in Figure 1.a), TSM covers the traditional approach where the transform is applied on both rows and columns of a block.

| a) 2D transform | b) transform on rows only | c) transform on columns only | d) no transform |
| --- | --- | --- | --- |
| TSM; Transform on rows; Transform on columns; Code words (CABAC and CAVLC); Note. TS0; +; +; 1; 2D transform. TS1; +; -; 01; 1D transform. TS2; -; +; 001; 1D transform. TS3; -; -; 000; no transform. |

e) TSM modes with its corresponding transform choices and codewords

Figure 1 Transform choices enabled by the proposed transform skip mode

The choice of transform or its skip equivalent, as defined by TSM, is defined on the transform unit level, i.e. on RQT leaves. Moreover, TSM is applied on inter coded blocks only. Four transform skip modes illustrated in Figure 1 (a-d) are further summarized in Figure 1 (e). For each coded transform unit one of four skip modes is signalled.

To support a choice of different transform skip modes, the encoder performs a rate-distortion optimized search for the best TSM mode for each inter predicted transform unit (each RQT node that is a valid TU). As in [F077], the optimization is performed jointly for all color components. However, in this proposal the RDOQ is not used for the full transform skip (TS3). This is done because the TS3 retains coefficients of a block in their spatial positions, while the basic RDOQ in HM-4.0 assumes frequency domain ordering of coefficients.

In HM-4.0, NSQT introduces new rectangular transforms based on the existing, separable transforms. In this proposal, TSM is used on non-square blocks, and fully provides TSM modes for any level of NSQT. Implementation of NSQT in HM-4.0 utilizes one of two possible block types, 16x4 or 32x8. If a block of the equivalent size but different shape is to be processed, e.g. 4x16 or 8x32, the block is first transposed to match one of the two supported transform implementations. The implementation of TSM keeps track of this transposing action so that when required, the flags determining whether to skip the horizontal or vertical transform can be swapped, as shown in Figure 2.

Figure 2 Implementation of TSM for NSQT blocks taking into account NSQT transposing

One of the features of the transform skipping is the introduction of vertical and horizontal coefficient scanning for horizontal transform skipping and vertical transform skipping, respectively. In this contribution, both horizontal/vertical scans for non-square blocks with related TSM modes have been tested, in comparison to preserving zig-zag scans for non-square blocks.

## TSM and RQT levels

The first set of tests aims to evaluate the performance of TSM at different RQT depth level. To enable selection of different RQT depth levels, an NSQT bug fix [BF223] has been used. In the following text, HM-4.0* refers to HM-4.0 patched with this bug fix.

All tests used common test conditions, except the maximum depth for RQT on inter predicted blocks which is varied (taking values of 1, 2 and 3, where 3 is the default value in HM-4.0). The results show a varying level of additional complexity, due to the RDO search of more transform options enabled with deeper RQTs. Figure 3 shows a visual comparison of screen content coded with HM-4.0*, with and without TSM. The two sequences were encoded with test point low-delay B, high efficiency, QP=37. The comparison shows the type of compression artifacts usually attributed to strongly quantized transform coefficients appearing in greater number on the left-hand side, as expected, making for a noisier picture.

Table 1 Comparison of the TSM without RQT splitting for inter coded blocks 
(--QuadtreeTUMaxDepthInter=1), relatively to the HM-4.0* with the same RQT setting

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.2% | -0.2% | 0.5% | 0.1% | 0.1% |
| Class B | -0.4% | -0.1% | 0.2% | -0.2% | -0.4% | 0.1% |
| Class C | -0.6% | -0.2% | 0.0% | -0.4% | -0.3% | 0.1% |
| Class D | -0.9% | 0.0% | -0.2% | -0.7% | 0.0% | -0.1% |
| Class E |  |  |  |  |  |  |
| Class F | -2.0% | -1.2% | -1.2% | -1.3% | -0.8% | -0.8% |
| Overall | -0.8% | -0.3% | -0.2% | -0.4% | -0.3% | -0.1% |
|  | -0.8% | -0.3% | -0.2% | -0.4% | -0.3% | -0.1% |
| Enc Time[%] | 118% |  |  | 118% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.5% | 0.8% | 1.5% | 0.1% | 0.6% | 1.0% |
| Class C | -0.9% | 0.1% | 0.6% | -0.6% | -0.1% | 0.5% |
| Class D | -1.5% | 0.7% | 0.6% | -1.2% | 0.8% | 1.0% |
| Class E | -0.4% | 2.1% | 1.4% | 0.3% | 1.8% | 1.1% |
| Class F | -4.6% | -3.0% | -2.6% | -3.2% | -1.8% | -1.9% |
| Overall | -1.6% | 0.1% | 0.3% | -0.9% | 0.2% | 0.3% |
|  | -1.6% | 0.0% | 0.3% | -0.9% | 0.2% | 0.3% |
| Enc Time[%] | 114% |  |  | 113% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 2 Comparison of the TSM with only one level of RQT splitting for inter coded blocks 
(--QuadtreeTUMaxDepthInter=2), relatively to the HM-4.0* with the same RQT setting

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.2% | 0.6% | 0.0% | 0.2% |
| Class B | -0.3% | 0.1% | 0.4% | -0.1% | 0.0% | 0.3% |
| Class C | -0.4% | -0.2% | 0.1% | -0.4% | -0.2% | 0.0% |
| Class D | -0.8% | -0.2% | -0.2% | -0.8% | -0.2% | -0.1% |
| Class E |  |  |  |  |  |  |
| Class F | -2.0% | -1.1% | -1.0% | -1.4% | -0.8% | -0.9% |
| Overall | -0.7% | -0.3% | -0.2% | -0.4% | -0.2% | -0.1% |
|  | -0.7% | -0.3% | -0.2% | -0.4% | -0.2% | -0.1% |
| Enc Time[%] | 130% |  |  | 129% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.4% | 0.9% | 1.3% | 0.2% | 1.1% | 1.5% |
| Class C | -0.8% | 0.4% | 0.6% | -0.5% | 0.4% | 0.4% |
| Class D | -1.4% | 1.1% | 1.3% | -1.1% | 1.4% | 0.6% |
| Class E | -0.3% | 1.9% | 1.4% | 0.4% | 2.6% | 1.7% |
| Class F | -4.6% | -2.9% | -2.6% | -3.5% | -1.6% | -2.0% |
| Overall | -1.5% | 0.2% | 0.4% | -0.9% | 0.7% | 0.4% |
|  | -1.5% | 0.2% | 0.4% | -0.9% | 0.7% | 0.4% |
| Enc Time[%] | 123% |  |  | 122% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 3 Comparison of the TSM with the default level of RQT splitting for inter coded blocks 
(--QuadtreeTUMaxDepthInter=3), relatively to the HM-4.0

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.2% | -0.2% | 0.6% | 0.1% | 0.2% |
| Class B | -0.2% | 0.0% | 0.3% | 0.1% | 0.1% | 0.4% |
| Class C | -0.4% | -0.2% | 0.1% | -0.3% | -0.2% | 0.0% |
| Class D | -0.7% | -0.2% | -0.1% | -0.6% | 0.0% | 0.1% |
| Class E |  |  |  |  |  |  |
| Class F | -2.0% | -1.1% | -1.1% | -1.4% | -0.8% | -0.8% |
| Overall | -0.6% | -0.3% | -0.2% | -0.3% | -0.2% | 0.0% |
|  | -0.6% | -0.3% | -0.2% | -0.3% | -0.2% | 0.0% |
| Enc Time[%] | 141% |  |  | 141% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | 1.0% | 1.0% | 0.4% | 1.2% | 1.2% |
| Class C | -0.6% | 0.3% | 0.6% | -0.5% | 0.3% | 0.3% |
| Class D | -1.1% | 1.0% | 1.3% | -0.9% | 0.9% | 0.3% |
| Class E | -0.2% | 2.8% | 1.8% | 0.6% | 2.4% | 1.3% |
| Class F | -4.5% | -2.8% | -2.6% | -3.3% | -1.9% | -1.8% |
| Overall | -1.3% | 0.3% | 0.4% | -0.7% | 0.5% | 0.3% |
|  | -1.3% | 0.3% | 0.4% | -0.7% | 0.4% | 0.3% |
| Enc Time[%] | 133% |  |  | 131% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.4% | 0.6% | 1.2% | 0.3% | 0.9% | 0.9% |
| Class C | -0.8% | 0.1% | 0.6% | -0.6% | -0.1% | 0.4% |
| Class D | -1.4% | 0.7% | 0.2% | -1.2% | 0.6% | 0.3% |
| Class E | -0.3% | 2.7% | 1.4% | 0.5% | 2.1% | 1.2% |
| Class F | -5.1% | -3.6% | -3.5% | -3.8% | -2.3% | -2.2% |
| Overall | -1.6% | 0.0% | 0.0% | -0.9% | 0.2% | 0.1% |
|  | -1.6% | -0.1% | -0.1% | -0.9% | 0.1% | 0.1% |
| Enc Time[%] | 147% |  |  | 147% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

| HM-4.0* | TSM |
| --- | --- |

Figure 3 Visual comparison of "SlideEditing", QP 37, frames 9 and 14

## TSM mode-dependant scans for NSQT

In this experiment, the horizontal and vertical scans of transform coefficients are used for non square blocks with modes TS1 or TS2 (one dimensional transforms only). The choice of the scan depends on NSQT transposing state and given TSM mode. The same contexts as in the default scan are used here. In this way, TSM mode dependant scans are used for all block sizes.

Table 4 shows the comparison of the TSM with mode-dependant scans supporting all blocks sizes (including non square blocks), against the TSM with the same settings (tested in Table 3) but with adaptive scans on square blocks only. The results show that BD-rates are not significantly affected.

Table 4 Comparison of the TSM with the mode dependant scans for all blocks, relatively to the TSM with the mode dependant scans for square blocks only (tested from Table 3)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% | -0.1% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | -0.1% |
| Class E |  |  |  |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.1% | 0.2% | 0.0% | -0.1% | 0.0% |
| Class C | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class D | 0.0% | 0.0% | -0.3% | 0.0% | -0.1% | 0.0% |
| Class E | 0.0% | -1.1% | 0.6% | -0.1% | 0.2% | 0.0% |
| Class F | -0.2% | -0.2% | -0.2% | -0.1% | -0.1% | -0.2% |
| Overall | 0.0% | -0.2% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | -0.3% | 0.1% | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

## NxN inter CU partitioning

Among the changes introduced with the latest iteration of the reference software is the disabling of the NxN partitions for inter predicted blocks. Table 5 shows the results for a test where RQT is not used (--QuadtreeTUMaxDepthInter=1) and NxN inter partitioning is enabled. An equivalent configuration of HM-4.0* is used as a reference. This test shows a TSM configuration offering fairly low complexity while still achieving compression gains.

Table 6 shows results for the same configuration but here TSM is only applied to 4x4 blocks. This configuration of TSM shows effectively the same complexity as the anchor while still providing some coding gains, especially for class F sequences.

Table 5 Evaluation of TSM with NxN inter partitioning enabled, 
relatively to an equivalent configuration of HM-4.0*

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.1% | 0.5% | -0.1% | -0.2% |
| Class B | -0.4% | -0.2% | 0.2% | -0.2% | -0.3% | 0.2% |
| Class C | -0.5% | -0.3% | 0.1% | -0.4% | -0.3% | 0.0% |
| Class D | -0.9% | -0.1% | -0.2% | -0.7% | -0.2% | 0.0% |
| Class E |  |  |  |  |  |  |
| Class F | -2.0% | -1.2% | -1.2% | -1.2% | -0.8% | -0.8% |
| Overall | -0.7% | -0.3% | -0.2% | -0.4% | -0.4% | -0.1% |
|  | -0.7% | -0.4% | -0.2% | -0.4% | -0.3% | -0.1% |
| Enc Time[%] | 116% |  |  | 116% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.6% | 0.7% | 1.0% | 0.1% | 0.5% | 1.1% |
| Class C | -0.8% | 0.4% | 0.4% | -0.5% | 0.1% | 0.5% |
| Class D | -1.4% | 1.0% | 1.0% | -1.1% | 0.5% | 0.5% |
| Class E | -0.3% | 2.4% | 0.8% | 0.3% | 2.5% | 1.0% |
| Class F | -4.4% | -2.6% | -3.1% | -3.0% | -1.7% | -1.7% |
| Overall | -1.5% | 0.3% | 0.0% | -0.9% | 0.3% | 0.3% |
|  | -1.5% | 0.3% | 0.0% | -0.9% | 0.2% | 0.3% |
| Enc Time[%] | 112% |  |  | 111% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.6% | 0.6% | 1.1% | 0.1% | 0.3% | 1.0% |
| Class C | -0.9% | 0.2% | 0.5% | -0.7% | -0.3% | 0.1% |
| Class D | -1.6% | 0.0% | 0.8% | -1.3% | 0.1% | 0.4% |
| Class E | -0.6% | 2.3% | 1.9% | 0.2% | 2.3% | 0.8% |
| Class F | -5.0% | -3.7% | -3.5% | -3.5% | -2.3% | -2.0% |
| Overall | -1.7% | -0.2% | 0.1% | -1.0% | -0.1% | 0.1% |
|  | -1.7% | -0.3% | 0.2% | -1.0% | -0.1% | 0.0% |
| Enc Time[%] | 119% |  |  | 119% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 6 Evaluation of TSM enabled on 4x4 block only with NxN inter partitioning enabled, 
relatively to an equivalent configuration of HM-4.0*

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.2% | 0.1% | 0.0% | -0.2% | 0.0% |
| Class B | 0.0% | -0.1% | -0.1% | 0.0% | 0.2% | 0.4% |
| Class C | -0.1% | -0.1% | 0.0% | -0.2% | 0.1% | 0.2% |
| Class D | -0.3% | 0.2% | 0.0% | -0.4% | 0.4% | 0.5% |
| Class E |  |  |  |  |  |  |
| Class F | -0.9% | -0.4% | -0.4% | -0.8% | -0.4% | -0.3% |
| Overall | -0.2% | 0.0% | -0.1% | -0.3% | 0.0% | 0.1% |
|  | -0.2% | 0.0% | -0.1% | -0.3% | 0.0% | 0.1% |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.2% | -0.2% | 0.0% | 0.4% | 0.4% |
| Class C | -0.1% | -0.1% | -0.2% | -0.2% | 0.4% | 0.4% |
| Class D | -0.2% | 0.5% | 0.7% | -0.4% | 1.1% | 1.1% |
| Class E | 0.2% | 0.5% | -0.3% | 0.1% | 0.8% | 0.0% |
| Class F | -2.1% | -0.9% | -1.2% | -1.8% | -0.2% | 0.0% |
| Overall | -0.5% | -0.1% | -0.2% | -0.5% | 0.5% | 0.4% |
|  | -0.5% | -0.1% | -0.3% | -0.5% | 0.4% | 0.4% |
| Enc Time[%] | 102% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.2% | -0.2% | -0.1% | 0.2% | 0.3% |
| Class C | -0.2% | -0.1% | 0.0% | -0.4% | 0.2% | 0.3% |
| Class D | -0.5% | 0.4% | 0.2% | -0.8% | 0.8% | 1.0% |
| Class E | 0.1% | -0.2% | 0.2% | 0.0% | -0.3% | 0.2% |
| Class F | -2.6% | -1.6% | -1.5% | -2.3% | -0.6% | -0.8% |
| Overall | -0.7% | -0.3% | -0.3% | -0.7% | 0.1% | 0.2% |
|  | -0.7% | -0.4% | -0.2% | -0.7% | 0.1% | 0.2% |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

# New implementation of transform skipping

In its initial form [F077], TSM was performed using normalization, when a transform is skipped. The normalization was, at both encoder and decoder, performed as follows:

y = (x  scale + offset) >> shift,	(1)

where scale is the scaling factor (Table 7), shift and offset are the same as in the core transform [E243], i.e. the same values that would be used if the transform was not skipped. x is the value of an element of the row or column for which the transform is skipped, and y is its scaled equivalent.

Table 7 Scaling factors for vectors of N elements.

| N | 4 | 8 | 16 | 32 |
| --- | --- | --- | --- | --- |
| scale | 128 | 181 | 256 | 362 |

Note that the same scaling operation described above is carried out for the forward and inverse skipped transform. When implemented as in the described scaling fashion and on top of HM4.0 core transform, the proposed TSM approach does not require changes to the quantization stage.

AHG 7 on Transform dynamic range has been studying solutions guaranteeing 16 bit buffers in inverse transform without overflow and minimal clipping operations. For this reason, worst-case scenarios (not necessarily possible outputs of forward transform) were considered, as the inputs for inverse transforms. To prevent worst case overflow possibility, in this document an alternative implementation of TSM is proposed.

The expansion of coefficients' range within transform skipping is avoided by removing (1) from the transform skip, and by adapting the quantization step accordingly to the combination of the scale levels for the given block dimensions. The quantization (both forward and inverse) is performed using a modified quantization factor:

uiQ' = (uiQ  bscale) >> bshift, 	(1)

where uiQ' = uiQ for full 2D transform, uiQ is HM4.0 quantization factor (inverse or forward), and bscale and bshift are dependent on TSM modes and the current block dimensions. The block scale is defined as:

bscale=1,if TS0scaleheight,if TS1scalewidth,if TS2scaleheight⋅scalewidth,if TS3

where scale is defined as in Table 7. The block shift bshift corresponds to the shifts of skipped transforms. Therefore, for the inverse quantization in HM-4.0, the blocks shift is defined as:

bshift=0,if TS0SHIFT_INV_1ST,if TS1SHIFT_INV_2ND - g_uiBitIncrement,if TS2SHIFT_INV_1ST+SHIFT_INV_2ND - g_uiBitIncrement,if TS3

In this way a multiplication free TSM is achieved, since the normalization is moved to the quantization stage. Moreover, the worst-case overflow problem in the transform stage remains dependant only on the core transform design and clipping is not needed in the transform skipping stage.

Table 8 Comparison of the proposed new TSM implementation, relatively to the TSM with the normalization in the transform skipping stage (tested from Table 4)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | -0.1% | 0.1% | 0.1% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class C | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | 0.1% |
| Class D | 0.0% | 0.1% | -0.2% | 0.0% | 0.0% | 0.2% |
| Class E |  |  |  |  |  |  |
| Class F | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | 0.1% |
|  | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

# Summary

In this proposal new features have been added to the TSM design - support for any RQT level, support for NSQT and mode-dependant scans as well as an implementation of TSM with normalization moved to the quantization stage. The results presented show TSM interaction with different codec parameters.

# Bit-stream modifications and semantics

For related bit-stream modifications and semantics, see [F077].

# References

[F077]	M. Mrak, A. Gabriellini, N. Sprljan, D. Flynn, “Transform Skip Mode”, JCTVC-F077, July 2011

[F770]	M. Mrak, A. Gabriellini, D. Flynn, “Results for SCC with Transform Skip Mode”, JCTVC-F770, July 2011

[BF223]	"Wrong derivation of deblocking filter boundary for non-square transform unit", HEVC bug tracker http://hevc.kw.bbc.co.uk/trac/ticket/223

[R1]	M. Biswas, M. R. Pickering, M. R. Frater, "Improved H.264-based Video Coding using an Adaptive Transform", IEEE ICIP 2010

[R2]	F. Kamisli, J.S. Lim, "Video compression with 1-D directional transforms in H.264/AVC", IEEE ICASSP 2010

[R3]	C. Lan, J. Xu, F. Wu, G. Shi, "Screen content coding results using TMuC", JCTVC-C276, October 2010.

[R4]	M. Narroschke, H. G. Musmann, "Adaptive prediction error coding in spatial and frequency domain with a fixed scan in the spatial domain", VQEG 2006

# Patent rights declaration(s)

The British Broadcasting Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).