| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G576-r2 WG11 Number: m 22141 |
| --- | --- |

| Title: | CE1: Delayed state update for CABAC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rajan Joshi Joel Sole Rojals Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Email: | rajanj@qualcomm.com Tel. +1-858-658-4511 |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

Context update is a known bottleneck in hardware implementation of a CABAC decoder. This is because the state of the context is updated based on the decoded bin. Previous efforts to mitigate this problem have included delaying the context update by one during transform coefficient coding and delaying the update till the end of a TU. In this proposal we extend the state update delay to bins from all syntax elements. Results are presented for update delays of 1, 2 and 3 bins. For update delays of 1, 2, and 3 bins, average BD-rates of 0.1%, 0.6% and 1.0%, respectively, are reported for HE configurations.

# Introduction

Context update is a known bottleneck in hardware implementation of a CABAC decoder [1]. This is because the state of the context is updated based on the decoded bin. Previous efforts to mitigate this problem have included delaying the context update by one during transform coefficient coding [2] and delaying the update till the end of a TU in transform coefficient coding [3]. In this proposal we extend the state update delay to bins from all syntax elements. We also present results for update delays of 1, 2 and 3 bins.

# Delayed state update

Let the bins to be encoded or decoded be denoted as bin(0), bin(1), … , bin(n-1), bin(n), … and so on. Let the corresponding context for bin(n) be denoted by ctxIdx(n). In the current CABAC, the state corresponding to ctxIdx(n) needs to be updated before encoding or decoding of bin(n+1) can start. Instead, when the update is delayed by 1 bin, the update of the state corresponding to ctxIdx(n) needs to finished before the coding or decoding of bin(n+2) can be started. Stated another way, the context states when coding or decoding bin(n) should be dependent only on bin(0), bin(1), … , bin(n-2) . Similarly for a delay of L bins, the context states when coding or decoding bin(n) should depend only on bin(0), bin(1), … , bin(n-L-1).

Figure 1 shows a CABAC decoder [1] with a update delay of L bins. The advantage of a delayed update is that the context update and encoding of the next bin can take place in parallel, thus mitigating one of the bottlenecks in hardware implementation of CABAC.

It should be noted that the delayed update may have a negative effect on performance for delay of 1 only when two consecutive contexts are the same. Similarly, for a delay of L, if any of the previous L contexts is the same as the current context, rate-distortion performance is affected.

Figure 1: CABAC decoder with delayed state update

# Results

The proposed method is implemented in HM 4.0. Simulation results are presented for the common test conditions [4] for HE configurations. The encoding was done on a 64-bit LINUX cluster. The decoding was performed on a PC using Microsoft Visual C++ (64-bit). No output YUV files were written. Tables 1-3 show the results for delay of 1, 2, and 3 bins. For delay of one bin the BD-rate is 0.1℅, 0.1℅, and 0.1℅ for AI-HE, RA-HE, and LB-HE configurations, respectively. For delay of two bins the BD-rate is 0.2℅, 0.8℅, and 0.9℅ for AI-HE, RA-HE, and LB-HE configurations, respectively. For delay of three bins, the BD-rate is 0.2℅, 1.1℅, and 1.5℅ for AI-HE, RA-HE, and LB-HE configurations, respectively. There is a small but consistent  increase in run-time for encoding and decoding for AI-HE configuration.

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.2% | 0.3% | 0.4% |
| Class B | 0.1% | 0.2% | 0.3% |
| Class C | 0.1% | 0.2% | 0.2% |
| Class D | 0.1% | 0.2% | 0.1% |
| Class E | 0.1% | 0.3% | 0.2% |
| Overall | 0.1% | 0.2% | 0.2% |
|  | 0.1% | 0.2% | 0.2% |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 103% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | 0.2% | 0.3% | 0.1% |
| Class B | 0.1% | 0.3% | 0.1% |
| Class C | 0.1% | 0.1% | 0.1% |
| Class D | 0.1% | 0.1% | -0.2% |
| Class E |  |  |  |
| Overall | 0.1% | 0.2% | 0.0% |
|  | 0.1% | 0.1% | 0.0% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.1% | -0.1% | -0.2% |
| Class C | 0.1% | 0.1% | 0.0% |
| Class D | 0.0% | 0.2% | 0.2% |
| Class E | 0.0% | 0.1% | 1.2% |
| Overall | 0.1% | 0.1% | 0.2% |
|  | 0.1% | 0.0% | 0.1% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |

Table 1: BD-rate results for delay = 1

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.3% | 0.2% | 0.2% |
| Class B | 0.2% | 0.2% | 0.3% |
| Class C | 0.1% | 0.2% | 0.2% |
| Class D | 0.1% | 0.2% | 0.2% |
| Class E | 0.3% | 0.3% | 0.3% |
| Overall | 0.2% | 0.2% | 0.2% |
|  | 0.2% | 0.2% | 0.2% |
| Enc Time[%] | 106% |  |  |
| Dec Time[%] | 105% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | 1.0% | 0.7% | 0.0% |
| Class B | 0.9% | 0.5% | 0.5% |
| Class C | 0.7% | 0.5% | 0.5% |
| Class D | 0.5% | 0.6% | 0.2% |
| Class E |  |  |  |
| Overall | 0.8% | 0.6% | 0.3% |
|  | 0.8% | 0.6% | 0.3% |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 1.0% | 0.7% | 0.7% |
| Class C | 0.7% | 0.4% | 0.5% |
| Class D | 0.5% | 0.3% | 0.7% |
| Class E | 1.8% | 1.0% | 2.3% |
| Overall | 0.9% | 0.6% | 0.9% |
|  | 0.9% | 0.6% | 0.9% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 101% |  |  |

Table 2: BD-rate results for delay = 2

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.3% | 0.3% | 0.6% |
| Class B | 0.2% | 0.2% | 0.6% |
| Class C | 0.2% | 0.2% | 0.3% |
| Class D | 0.2% | 0.2% | 0.3% |
| Class E | 0.3% | 0.4% | 0.9% |
| Overall | 0.2% | 0.3% | 0.5% |
|  | 0.2% | 0.3% | 0.6% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 105% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | 1.4% | 0.5% | 0.4% |
| Class B | 1.3% | 0.4% | 0.5% |
| Class C | 0.9% | 0.3% | 0.7% |
| Class D | 0.8% | 0.4% | 0.4% |
| Class E |  |  |  |
| Overall | 1.1% | 0.4% | 0.5% |
|  | 1.1% | 0.4% | 0.5% |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 1.6% | 0.7% | 0.6% |
| Class C | 1.0% | 0.6% | 0.5% |
| Class D | 0.7% | 0.2% | 0.3% |
| Class E | 3.2% | 1.0% | 3.0% |
| Overall | 1.5% | 0.6% | 0.9% |
|  | 1.5% | 0.7% | 0.9% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 101% |  |  |

Table 3: BD-rate results for delay = 3

# Conclusions

In this contribution it is proposed to introduce a delay in state update for CABAC. Results are presented for update delay of 1, 2 and 3 bins. A delay of 1 bin results in 0.1℅ BD-rate loss while eliminating a bottleneck in hardware implementation of CABAC. We propose that delayed state update with a delay of one bin be adopted into the next working draft of HEVC.

# References

- V. Sze, “Context selection complexity in HEVC CABAC”, JCTVC-D244, Daegu, KR, January 2011.

- J. Sole, R. Joshi and M. Karczewicz, “Parallel Processing of Residual Data in HE”, JCTVC-F552, Torino, IT, July 2011.

- H. Sasai and T. Nushi, “Parallel Context Processing for Significance map using block-based context updates”, JCTVC-E226, Geneva, CH, March 2011.

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, Torino, IT, July 2011.

# Working draft text

9.3.3.2.2.1: State transition process

Inputs to this process are the current pStateIdx, the decoded value binVal and valMPS values of the context variable associated with ctxIdxTable and ctxIdx.

Outputs of this process are the updated pStateIdx and valMPS of the context variable associated with ctxIdx.

Depending on the decoded value binVal, the update of the two variables pStateIdx and valMPS associated with ctxIdx is derived as specified by the following pseudo-code:

if( binVal  = =  valMPS ) 
	pStateIdx = transIdxMPS( pStateIdx )
else {				(9-1)
	if( pStateIdx  = =  0 )
		valMPS = 1  valMPS
	pStateIdx = transIdxLPS( pStateIdx )
}

Error: Reference source not found specifies the transition rules transIdxMPS( ) and transIdxLPS( ) after decoding the value of valMPS and 1  valMPS, respectively.

The updated pStateIdx and valMPS values of the context variable associated with ctxIdxTable and ctxIdx are effective only after the next L bins have been decoded. This means that for the decoding of next L bins, if this context variable is used, the values of pStateIdx and valMPS before the update process are used.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).