| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G577 |
| --- | --- |

| Title: | Transform skipping dependant on block parameters |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Glenn Van Wallendael, Jan De Cock, Rik Van de Walle Ghent University – IBBT Gaston Crommenlaan 8 P.O. Box 201 9050 Ghent-Ledeberg, Belgium Marta Mrak BBC R&D, Centre House, 56 Wood Lane W12 7SB London, UK | Tel: Email: | +32 93314957 glenn.vanwallendael@ugent.be marta.mrak@bbc.co.uk |
| Source: | Ghent University, BBC |  |  |

_____________________________

# Abstract

In this contribution a mechanism for selection of transform skip mode (TSM) depending on current block parameters is presented. TSM was introduced in JCTVC-F077 and it enables skipping transform on a row and/or a column of motion compensated residuals. The transform skip mode choice is signalled to the decoder where inverse transforms of rows/ columns are performed or skipped.

In this contribution, TSM is enabled or disabled depending on the size or the QP of the TU. Results are presented when only applying TSM on TU sizes 4x4. Additionally TSM is restricted to QPs under a threshold. These two scenarios offer an improved BD-rate performance and a lower encoder's execution time compared to TSM that is applied on all block sizes or QPs. To support such more customized selection of TSM, SPS signaling for TSM on larger blocks is proposed.

# Introduction

The Transform Skip Mode (TSM) [1] defines transform skip in one or both directions on which a transform would be applied under common conditions, Figure 1. As illustrated in Figure 1.a), TSM covers the traditional approach where the transform is applied on both rows and columns of a block.

| a) 2D transform | b) transform on rows only | c) transform on columns only | d) no transform |
| --- | --- | --- | --- |

Figure 1. Transform choices enabled by the proposed transform skip mode

Previous contributions [1, 2] introduced TSM and evaluated its performance under common conditions and on screen content.

# TU size dependent TSM

The TSM implementation available from AHG19 on transform skipping [3] is analyzed for TSM usage on different sizes of TUs. In this implementation TSM choice is selected in an RDO search that evaluates all transform skipping modes. Table 1 shows percentages of 2D transform usage, averaged over all sequences under common conditions [5] for random access and low delay B settings. In Table 1, the percentages are relative to the number of TUs with all TSM options (2D transform, horizontal transform, vertical transform and full transform skip).

Table 1: Relative amount of TUs with 2D transform

| TU size min(width,height) | Relative usage of 2D transform |
| --- | --- |
| 4 | 53% |
| 8 | 70% |
| 16 | 80% |
| 32 | 84% |

TSM software [3] was modified to support TSM only for 4x4 blocks. The results are presented in Table 2.

Table 2 TSM on 4x4 blocks only (relative to HM 4.0)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,0% | -0,2% | -0,2% | 0,1% | 0,3% | 0,1% |
| Class B | -0,1% | 0,1% | 0,1% | -0,1% | 0,4% | 0,5% |
| Class C | -0,2% | -0,1% | 0,0% | -0,4% | 0,2% | 0,2% |
| Class D | -0,4% | 0,1% | 0,0% | -0,6% | 0,4% | 0,4% |
| Class E |  |  |  |  |  |  |
| Class F | -1,7% | -1,0% | -0,9% | -1,3% | -0,5% | -0,5% |
| Overall | -0,4% | -0,2% | -0,2% | -0,4% | 0,2% | 0,2% |
|  | -0,4% | -0,2% | -0,2% | -0,4% | 0,2% | 0,2% |
| Enc Time[%] | 111% |  |  | 110% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0,0% | 0,0% | -0,1% | 0,0% | 0,6% | 0,5% |
| Class C | -0,3% | 0,0% | -0,2% | -0,5% | 0,6% | 0,2% |
| Class D | -0,5% | 0,5% | 0,6% | -0,7% | 1,5% | 0,9% |
| Class E | 0,0% | 0,2% | 0,7% | 0,0% | 0,6% | 0,1% |
| Class F | -3,9% | -1,9% | -1,3% | -3,0% | -0,9% | -1,0% |
| Overall | -0,9% | -0,3% | -0,1% | -0,9% | 0,5% | 0,2% |
|  | -0,9% | -0,3% | -0,1% | -0,8% | 0,4% | 0,1% |
| Enc Time[%] | 109% |  |  | 108% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0,1% | 0,0% | -0,2% | -0,1% | 0,4% | 0,4% |
| Class C | -0,4% | -0,2% | -0,2% | -0,7% | 0,3% | 0,2% |
| Class D | -0,7% | -0,1% | 0,2% | -1,0% | 0,9% | 0,4% |
| Class E | -0,1% | 0,1% | 0,1% | 0,0% | 0,1% | -0,3% |
| Class F | -4,1% | -2,6% | -2,4% | -3,3% | -1,1% | -0,9% |
| Overall | -1,1% | -0,5% | -0,5% | -1,0% | 0,1% | 0,0% |
|  | -1,1% | -0,6% | -0,6% | -1,0% | 0,1% | 0,0% |
| Enc Time[%] | 113% |  |  | 112% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

Table 3 TSM on 4x4 blocks only (relative to HM 4.0) without class F

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,0% | -0,2% | -0,2% | 0,1% | 0,3% | 0,1% |
| Class B | -0,1% | 0,1% | 0,1% | -0,1% | 0,4% | 0,5% |
| Class C | -0,2% | -0,1% | 0,0% | -0,4% | 0,2% | 0,2% |
| Class D | -0,4% | 0,1% | 0,0% | -0,6% | 0,4% | 0,4% |
| Class E |  |  |  |  |  |  |
| Overall | -0,1% | 0,0% | 0,0% | -0,2% | 0,3% | 0,3% |
|  | -0,1% | 0,0% | 0,0% | -0,2% | 0,3% | 0,3% |
| Enc Time[%] | 111% |  |  | 110% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0,0% | 0,0% | -0,1% | 0,0% | 0,6% | 0,5% |
| Class C | -0,3% | 0,0% | -0,2% | -0,5% | 0,6% | 0,2% |
| Class D | -0,5% | 0,5% | 0,6% | -0,7% | 1,5% | 0,9% |
| Class E | 0,0% | 0,2% | 0,7% | 0,0% | 0,6% | 0,1% |
| Overall | -0,2% | 0,1% | 0,2% | -0,3% | 0,8% | 0,4% |
|  | -0,2% | 0,1% | 0,2% | -0,3% | 0,8% | 0,4% |
| Enc Time[%] | 109% |  |  | 108% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0,1% | 0,0% | -0,2% | -0,1% | 0,4% | 0,4% |
| Class C | -0,4% | -0,2% | -0,2% | -0,7% | 0,3% | 0,2% |
| Class D | -0,7% | -0,1% | 0,2% | -1,0% | 0,9% | 0,4% |
| Class E | -0,1% | 0,1% | 0,1% | 0,0% | 0,1% | -0,3% |
| Overall | -0,3% | 0,0% | 0,0% | -0,4% | 0,5% | 0,2% |
|  | -0,3% | -0,1% | -0,1% | -0,4% | 0,4% | 0,2% |
| Enc Time[%] | 113% |  |  | 112% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

When comparing these results to TSM on all sizes, it can be observed that faster encoding is obtained. TSM will be checked on fewer TUs (only of size 4x4) and therefore result in smaller encoding times. Compared to HM 4.0 reference encoding, a longer time to encode is taken up by the exhaustive RD search of skipping horizontal, vertical, or both directions.

# SPS signaling of TSM usage for larger blocks

As an extension of the previous findings, we propose to signal a QP in the SPS indicating the maximum QP on which TSM is used and signaled. As a result, for 4x4 TU sizes, TSM will always be used. For larger TU sizes, the usage of TSM depends on the QP of the TU and the signaled tsm_qp value. Backward compatibility with the common conditions is provided when a tsm_qp larger than 41 is signaled in the SPS. As an example of the proposed method, TSM is set to be used only on QPs smaller than 30. For frames with a higher QP, TSM is only used on 4x4 TU sizes.

Table 4 TSM on 4x4 blocks only or on TUs with a QP smaller than 30 (relative to HM 4.0)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,0% | -0,2% | -0,2% | 0,2% | 0,5% | 0,3% |
| Class B | -0,2% | 0,2% | 0,2% | -0,2% | 0,6% | 0,8% |
| Class C | -0,3% | 0,0% | 0,0% | -0,4% | 0,2% | 0,2% |
| Class D | -0,6% | 0,1% | 0,0% | -0,7% | 0,5% | 0,5% |
| Class E |  |  |  |  |  |  |
| Class F | -1,7% | -0,9% | -0,9% | -1,4% | -0,6% | -0,6% |
| Overall | -0,5% | -0,2% | -0,2% | -0,5% | 0,3% | 0,3% |
|  | -0,5% | -0,2% | -0,2% | -0,5% | 0,3% | 0,3% |
| Enc Time[%] | 122% |  |  | 120% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0,2% | 0,8% | 0,9% | -0,1% | 1,6% | 1,6% |
| Class C | -0,5% | 0,5% | 0,4% | -0,6% | 1,0% | 0,7% |
| Class D | -0,8% | 1,1% | 1,0% | -1,0% | 1,7% | 1,1% |
| Class E | -0,2% | 1,6% | 1,6% | 0,1% | 2,6% | 1,3% |
| Class F | -4,1% | -2,0% | -1,4% | -3,2% | -1,0% | -1,1% |
| Overall | -1,2% | 0,4% | 0,4% | -1,0% | 1,1% | 0,7% |
|  | -1,2% | 0,3% | 0,4% | -1,0% | 1,1% | 0,7% |
| Enc Time[%] | 118% |  |  | 118% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0,3% | 0,8% | 0,8% | -0,2% | 1,2% | 1,3% |
| Class C | -0,6% | 0,3% | 0,3% | -0,8% | 0,6% | 0,5% |
| Class D | -1,1% | 0,5% | 0,4% | -1,3% | 1,1% | 0,7% |
| Class E | -0,4% | 2,1% | 1,5% | -0,1% | 2,2% | 1,2% |
| Class F | -4,5% | -2,6% | -2,5% | -3,5% | -1,3% | -1,0% |
| Overall | -1,4% | 0,2% | 0,1% | -1,2% | 0,7% | 0,5% |
|  | -1,4% | 0,1% | 0,0% | -1,2% | 0,7% | 0,5% |
| Enc Time[%] | 126% |  |  | 125% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

# Slice header signaling of TSM usage for larger blocks

Instead of signaling the usage of TSM by means of a minimum QP in the SPS, simply a TSM enable flag can be sent in the slice header. This enable flag only decides about TSM being enabled or disabled for TU sizes larger than 4x4. The results are equivalent to the previous proposed method except that signaling happens with a flag in the slice header instead of with a QP in the SPS.

# Conclusions

In this proposal, we increased compression performance and reduced the encoder execution time of TSM. This was achieved by restricting TSM application to TU sizes and QPs where the profits of TSM are largest. For TU sizes 4x4, TSM is always used. For larger sizes TSM signaling can be enabled or disabled depending on a SPS signaled QP or a slice header signaled TSM enable flag.

# WD text

This section describes the modification to the HEVC syntax that this proposal requires. The changes are described in reference to the changes required to enable TSM [4].

## SPS signaling of TSM

To restrict TSM to a specific QP, a tsm_qp must be signaled in the SPS. With this tsm_qp, enabling and disabling the signaling of TSM is controlled.

Additionally, to enable TSM signaling for 4x4 TUs a second check must be performed during the parsing process of the decoder. The modifications to enable the proposed method are listed in red in the following tables.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| reserved_zero_8bits /* equal to 0 */ | u(8) |
| level_idc | u(8) |
| seq_parameter_set_id | ue(v) |
| max_temporal_layers_minus1 | u(3) |
| pic_width_in_luma_samples | u(16) |
| pic_height_in_luma_samples | u(16) |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| pcm_bit_depth_luma_minus1 | u(4) |
| pcm_bit_depth_chroma_minus1 | u(4) |
| log2_max_frame_num_minus4 | ue(v) |
| pic_order_cnt_type | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| else if( pic_order_cnt_type = = 1 ) { |  |
| delta_pic_order_always_zero_flag | u(1) |
| offset_for_non_ref_pic | se(v) |
| num_ref_frames_in_pic_order_cnt_cycle | ue(v) |
| for( i = 0; i < num_ref_frames_in_pic_order_cnt_cycle; i++ ) |  |
| offset_for_ref_frame[ i ] | se(v) |
| } |  |
| max_num_ref_frames | ue(v) |
| gaps_in_frame_num_value_allowed_flag | u(1) |
| log2_min_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| log2_min_pcm_coding_block_size_minus3 | ue(v) |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| chroma_pred_from_luma_enabled_flag | u(1) |
| loop_filter_across_slice_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| adaptive_loop_filter_enabled_flag | u(1) |
| pcm_loop_filter_disable_flag | u(1) |
| cu_qp_delta_enabled_flag | u(1) |
| temporal_id_nesting_flag | u(1) |
| inter_4x4_enabled_flag | u(1) |
| tsm_qp | ue(v) |
| rbsp_trailing_bits( ) |  |
| } |  |

| transform_coeff( x0, y0, log2TrafoSize, trafoDepth, cIdx ) { | Descriptor |
| --- | --- |
| if( ( ( cIdx = = 0 && cbf_luma[ x0 ][ y0 ][ trafoDepth ] ) ¦ ¦ ( cIdx = = 1 && cbf_cb[ x0 ][ y0 ][ trafoDepth ] ) ¦ ¦ ( cIdx = = 2 && cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) ) ) { |  |
| if( cu_qp_delta_enabled_flag && !IsCuQpDeltaCoded ) { |  |
| cu_qp_delta | se(v) ¦ ae(v) |
| IsCuQpDeltaCoded = 1 |  |
| } |  |
| if( split_transform_flag[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| x1 = x0 + ( ( 1 << log2TrafoSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2TrafoSize ) >> 1 ) |  |
| transform_coeff( x0, y0, log2TrafoSize − 1, trafoDepth + 1, cIdx ) |  |
| transform_coeff( x1, y0, log2TrafoSize − 1, trafoDepth + 1, cIdx ) |  |
| transform_coeff( x0, y1, log2TrafoSize − 1, trafoDepth + 1, cIdx ) |  |
| transform_coeff( x1, y1, log2TrafoSize − 1, trafoDepth + 1, cIdx ) |  |
| } else { |  |
| if ( PredMode != MODE_INTRA && cIdx = = 0 && |  |
| ( log2TrafoSize == 2 ¦ ¦ QPY<= tsm_qp)) |  |
| tsmIdx[ x0 ][ y0 ][ trafoDepth ] | ae(v) ¦ ce(v) |
| if ( PredMode = = MODE_INTRA ) { |  |
| if (cIdx = = 0) |  |
| scanIdx = ScanType[ log2TrafoSize -2][ IntraPredMode ] |  |
| else |  |
| scanIdx = ScanTypeC[ log2TrafoSize -2][ IntraPredModeC ] |  |
| } else if ( tsmIdx[ x0 ][ y0 ][ trafoDepth ] = = TS1 ) { |  |
| scanIdx = 2 |  |
| } else if ( tsmIdx[ x0 ][ y0 ][ trafoDepth ] = = TS2 ) { |  |
| scanIdx = 1 |  |
| } else { |  |
| scanIdx = 0 |  |
| } |  |
| if ( entropy_coding_mode_flag ) |  |
| residual_coding_cabac( x0, y0, log2TrafoSize, trafoDepth, scanIdx, cIdx ) |  |
| else if ( !entropy_coding_mode_flag ) |  |
| residual_coding_cavlc( x0, y0, log2TrafoSize, trafoDepth, scanIdx, cIdx ) |  |
| } |  |
| } |  |
| } |  |

## Slice header signaling of TSM

To enable or disable TSM, a flag must be signaled in the slice header.

| slice_header( ) { | Descriptor |  |  |
| --- | --- | --- | --- |
| lightweight_slice_flag | u(1) |  |  |
| if( !lightweight_slice_flag ) { |  |  |  |
| slice_type | ue(v) |  |  |
| pic_parameter_set_id | ue(v) |  |  |
| frame_num | u(v) |  |  |
| if( IdrPicFlag ) |  |  |  |
| idr_pic_id | ue(v) |  |  |
| if( pic_order_cnt_type = = 0 ) |  |  |  |
| pic_order_cnt_lsb /* | u(v) |  |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |  |  |
| num_ref_idx_active_override_flag | u(1) |  |  |
| if( num_ref_idx_active_override_flag ) { |  |  |  |
| num_ref_idx_l0_active_minus1 | ue(v) |  |  |
| if( slice_type = = B ) |  |  |  |
| num_ref_idx_l1_active_minus1 | ue(v) |  |  |
| } |  |  |  |
| } |  |  |  |
| ref_pic_list_modification( ) |  |  |  |
| ref_pic_list_combination( ) |  |  |  |
| if( nal_ref_idc != 0 ) |  |  |  |
| dec_ref_pic_marking( ) |  |  |  |
| } |  |  |  |
| if( entropy_coding_mode_flag && slice_type != I) |  |  |  |
| cabac_init_idc | ue(v) |  |  |
| first_slice_in_pic_flag | u(1) |  |  |
| if( first_slice_in_pic_flag == 0 ) |  |  |  |
| slice_address | u(v) |  |  |
| if( !lightweight_slice_flag ) { |  |  |  |
| slice_qp_delta | se(v) |  |  |
| if( sample_adaptive_offset_enabled_flag ) |  |  |  |
| sao_param() |  |  |  |
| if( deblocking_filter_control_present_flag ) { |  |  |  |
| disable_deblocking_filter_idc |  |  |  |
| if( disable_deblocking_filter_idc != 1 ) { |  |  |  |
| slice_alpha_c0_offset_div2 |  |  |  |
| slice_beta_offset_div2 |  |  |  |
| } |  |  |  |
| } |  |  |  |
| if( slice_type = = B ) |  |  |  |
| collocated_from_l0_flag | u(1) |  |  |
| if( adaptive_loop_filter_enabled_flag ) { |  |  |  |
| if( !shared_pps_info_enabled_flag ) |  |  |  |
| alf_param( ) |  |  |  |
| alf_cu_control_param( ) |  |  |  |
| } |  |  |  |
| } |  |  |  |
| tsm_enable_flag | u(1) |  |  |
| } |  |  |  |
| transform_coeff( x0, y0, log2TrafoSize, trafoDepth, cIdx ) { |  | Descriptor |  |
| if( ( ( cIdx = = 0 && cbf_luma[ x0 ][ y0 ][ trafoDepth ] ) ¦ ¦ ( cIdx = = 1 && cbf_cb[ x0 ][ y0 ][ trafoDepth ] ) ¦ ¦ ( cIdx = = 2 && cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) ) ) { |  |  |  |
| if( cu_qp_delta_enabled_flag && !IsCuQpDeltaCoded ) { |  |  |  |
| cu_qp_delta |  | se(v) ¦ ae(v) |  |
| IsCuQpDeltaCoded = 1 |  |  |  |
| } |  |  |  |
| if( split_transform_flag[ x0 ][ y0 ][ trafoDepth ] ) { |  |  |  |
| x1 = x0 + ( ( 1 << log2TrafoSize ) >> 1 ) |  |  |  |
| y1 = y0 + ( ( 1 << log2TrafoSize ) >> 1 ) |  |  |  |
| transform_coeff( x0, y0, log2TrafoSize − 1, trafoDepth + 1, cIdx ) |  |  |  |
| transform_coeff( x1, y0, log2TrafoSize − 1, trafoDepth + 1, cIdx ) |  |  |  |
| transform_coeff( x0, y1, log2TrafoSize − 1, trafoDepth + 1, cIdx ) |  |  |  |
| transform_coeff( x1, y1, log2TrafoSize − 1, trafoDepth + 1, cIdx ) |  |  |  |
| } else { |  |  |  |
| if ( PredMode != MODE_INTRA && cIdx = = 0 && |  |  |  |
| ( log2TrafoSize == 2 ¦ ¦ tsm_enable_flag)) |  |  |  |
| tsmIdx[ x0 ][ y0 ][ trafoDepth ] |  | ae(v) ¦ ce(v) |  |
| if ( PredMode = = MODE_INTRA ) { |  |  |  |
| if (cIdx = = 0) |  |  |  |
| scanIdx = ScanType[ log2TrafoSize -2][ IntraPredMode ] |  |  |  |
| else |  |  |  |
| scanIdx = ScanTypeC[ log2TrafoSize -2][ IntraPredModeC ] |  |  |  |
| } else if ( tsmIdx[ x0 ][ y0 ][ trafoDepth ] = = TS1 ) { |  |  |  |
| scanIdx = 2 |  |  |  |
| } else if ( tsmIdx[ x0 ][ y0 ][ trafoDepth ] = = TS2 ) { |  |  |  |
| scanIdx = 1 |  |  |  |
| } else { |  |  |  |
| scanIdx = 0 |  |  |  |
| } |  |  |  |
| if ( entropy_coding_mode_flag ) |  |  |  |
| residual_coding_cabac( x0, y0, log2TrafoSize, trafoDepth, scanIdx, cIdx ) |  |  |  |
| else if ( !entropy_coding_mode_flag ) |  |  |  |
| residual_coding_cavlc( x0, y0, log2TrafoSize, trafoDepth, scanIdx, cIdx ) |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

# References

[1] M. Mrak, A. Gabriellini, N. Sprljan, D. Flynn, “Transform Skip Mode”, JCTVC-F077, July 2011.

[2] M. Mrak, A. Gabriellini, D. Flynn, “Results for SCC with Transform Skip Mode”, JCTVC-F770, July 2011

[3] TSM software based on HM4.0. http://hevc.kw.bbc.co.uk/git/w/jctvc-hm/jctvc-hm-tsm.git/commit/a62af9fa8f9ebb20376fc910b486a8d0b0574930

[4] M. Mrak, A. Gabriellini, D. Flynn, “Transform skip mode”, JCTVC-G575, November 2011

[5] F. Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, CH, March. 2011.

# Patent rights declaration(s)

Ghent University and British Broadcasting Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, are prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).