| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G578 WG11 Number: m22143 |
| --- | --- |

| Title: | Compression and signaling of quantizer matrices |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rajan Joshi Joel Sole Rojals Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Email: | rajanj@qualcomm.com Tel. +1-858-658-4511 |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

A method for compression of quantizer matrices is presented. It uses raster scanning and prediction based on top and left neighbors. The prediction residual is biased towards positive values. A biased mapping of signed prediction residuals to unsigned values, followed by Golomb coding is used to code the prediction residual. This method is reported to work well with both symmetric and asymmetric matrices. Simulation results are presented to show that it reduces the bit-rate by roughly 25% when compared with the default AVC/H.264 method.

# Introduction

Core experiment 4, subtest 3 is studying proposals for inclusion of quantization matrices in HEVC and their compression. The proposals being studied include JCTVC-E073 [1] and JCTVC-F085 [2]. JCTVC-E073 allows the use of the AVC/H.264 method of quantization matrix compression as one of the options. JCTVC-F085 uses the AVC/H.264 method with additional signaling for symmetry and downsampling. Here we propose an alternate method for quantization matrix compression and also propose new signaling.

# An alternate method for quantizer matrix compression

In the AVC/H.264 method of quantizer matrix compression, the matrix entries are scanned using a zigzag scan. Then, the current matrix entry is predicted from the previous entry in the scan and the residual is encoded using exponential Golomb code with parameter 0. The zigzag scan does not work well when the quantization matrix is asymmetric. Also, for non-zero prediction residuals, the structure of the signed exponential Golomb code means that one bit is needed for sign.

The use of quantization matrices is motivated by the properties of the human visual system. One model used for the design of quantization matrices is the 2-D contrast sensitivity function (CSF) of the human visual system. The contrast sensitivity decreases with increasing frequency. Due to the decreased contrast sensitivity at higher frequencies, well-designed quantization matrices have the property that typically, quantization matrix entries  increase both in row and column directions. Our proposed method utilizes this property in such a way that the prediction residual is almost always positive. As a result, the overhead for the sign bit is reduced.

## Proposed method

The proposed method uses a simple raster scan. The maximum of the quantizer matrix entry above and to the left is used as the predictor for the current quantizer matrix entry. The prediction residual is remapped to a non-negative value and then the remapped residual is coded using an unsigned Golomb code. The remapping uses a parameter “offsetG” and can be described as follows:

Let the prediction residual be denoted by X and remapped prediction residual be denoted by Y. Then,

| For X<0 | Y=offsetG+(−X−1)∗2 | (1) |
| --- | --- | --- |
| For 0≤X<offsetG | Y=X |  |
| For X≥offsetG | Y=offsetG+1+(X−offsetG)∗2 |  |

Table 1 shows the remapping foroffsetG=4.

| Source symbol (X) | Remapped symbol (Y) |
| --- | --- |
| 0 | 0 |
| 1 | 1 |
| 2 | 2 |
| 3 | 3 |
| -1 | 4 |
| 4 | 5 |
| -2 | 6 |
| 5 | 7 |
| … | … |

Table 1: Remapping between X and Y

## Summary of Experimental Results

We have integrated our method into the joint software provided by Sony and TI for CE4 subtest 3. Our macro is implemented under the TI_QM macro. Table 2 shows results for our method combined with 3 of the flags proposed in JCTVC-F085 [2], namely, downsample, sym45 and sym135. These results have been generated used Golomb code (parameter=2, offset=4). These parameters are explicitly signaled in the bit-stream. The entries in the parentheses are percentages.

|  |  |  | Symmetric quant matrix |  |  | Asymmetric quant matrix |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Down-sample | Sym45 | Sym135 | Bits (AVC) | Bits (proposed) | Avg. abs error | Bits (AVC) | Bits (proposed) | Avg. abs error |
| 0 | 0 | 0 | 8856 | 7171 (81%) | 0.00 | 9512 | 6450 (68%) | 0.00 |
| 0 | 0 | 1 | 5020 | 3828 (76%) | 0.00 | 5056 | 3503 (69%) | 3.53 |
| 0 | 1 | 0 | 5052 | 4201 (83%) | 5.06 | 4558 | 3552 (78%) | 3.14 |
| 0 | 1 | 1 | 2861 | 2305 (81%) | 5.06 | 2626 | 2016 (77%) | 5.25 |
| 1 | 0 | 0 | 3631 | 2889 (80%) | 0.43 | 3784 | 2539 (67%) | 0.32 |
| 1 | 0 | 1 | 2339 | 1645 (70%) | 0.43 | 2323 | 1501 (65%) | 3.76 |
| 1 | 1 | 0 | 2263 | 1858 (82%) | 4.73 | 1923 | 1472 (77%) | 3.54 |
| 1 | 1 | 1 | 1477 | 1120 (76%) | 4.73 | 1249 | 955 (76%) | 5.52 |

Table 2: Comparison of the proposed method with the default AVC/H.264 method

# Signaling of a partial quantization matrix

When the encoder chooses a 32×32 block, typically, the probability of significant coefficients in higher frequencies is small. Thus, it may not be necessary to signal quantizer matrix entries for higher frequency coefficients very accurately. We propose to signal only a rectangular subset of the quantization matrix and predict the rest of the entries.

First a one bit flag is sent to indicate whether only a subset of the quantization matrix is being coded. If that is the case, the last row and the last column of the subset is indicated using a fixed number of bits depending on the block size. Then, the subset of the quantization matrix is coded using any of the proposed techniques. The quantization matrix entries for the subset are reconstructed and used to predict the quantization matrix entries outside the subset. The entries outside the subset are scanned in a raster scan. The predictor proposed in Section 2 (max(top, left)) is used. Tables 3 and 4 show an original 8×8 quantization matrix and the reconstructed matrix, when only the lowest 5×5 rectangle is signaled.

This signaling scheme can be thought of as a lossy compression scheme for quantization matrices, where compression error is introduced in the quantizer matrix entries at higher frequency. From the point of view of human visual system, this approach may be a preferable to introducing them randomly at any location.

| 6 | 9 | 13 | 18 | 25 | 35 | 36 | 37 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 9 | 10 | 15 | 21 | 32 | 35 | 37 | 41 |
| 13 | 15 | 18 | 23 | 35 | 55 | 58 | 59 |
| 18 | 21 | 23 | 26 | 65 | 58 | 64 | 66 |
| 25 | 32 | 35 | 65 | 66 | 66 | 67 | 70 |
| 35 | 35 | 55 | 58 | 66 | 68 | 70 | 73 |
| 36 | 37 | 58 | 64 | 67 | 70 | 76 | 80 |
| 37 | 41 | 59 | 66 | 70 | 73 | 80 | 85 |

Table 3: Original 8×8 quantization matrix

| 6 | 9 | 13 | 18 | 25 | 25 | 25 | 25 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 9 | 10 | 15 | 21 | 32 | 32 | 32 | 32 |
| 13 | 15 | 18 | 23 | 35 | 35 | 35 | 35 |
| 18 | 21 | 23 | 26 | 65 | 65 | 65 | 65 |
| 25 | 32 | 35 | 65 | 66 | 66 | 66 | 66 |
| 25 | 32 | 35 | 65 | 68 | 66 | 66 | 66 |
| 25 | 32 | 35 | 65 | 66 | 66 | 66 | 66 |
| 25 | 32 | 35 | 65 | 66 | 66 | 66 | 66 |

Table 4: Reconstructed quantization matrix when only the lowest 5×5 entries are coded

# Conclusions

A method for compression of quantizer matrices is presented. It uses raster scanning and prediction based on top and left neighbors. The prediction residual is biased towards positive values. A biased mapping of signed prediction residuals to unsigned values, followed by Golomb coding is used to code the prediction residual. This method is reported to work well with both symmetric and asymmetric matrices. Simulation results are presented to show that it reduces the bit-rate by roughly 25% when compared with the default AVC/H.264 method. We also propose a method to signal that only a subset of the quantization matrix is being signaled.

# References

- J. Tanaka, Y. Morigami, and T. Suzuki, “Quantization Matrix for HEVC”, JCTVC-E073, Geneva, CH, March 2011.

- M. Zhou, V. Sze, “Further study on compact representation of quantization matrices”, JCTVC-F085, Torino, IT, July 2011.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).