| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G579 WG11 Number: m22144 |
| --- | --- |

| Title: | CE10: Scaled integer transforms supporting recursive factorization structure |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rajan Joshi Joel Sole Rojals Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Email: | rajanj@qualcomm.com Tel. +1-858-658-4511 |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

In this contribution, modifications to the 8, 16, and 32 point transforms proposed previously are presented. These transforms are scaled integer transforms supporting a recursive factorization structure. The 4-point transform is unchanged from HM. The 8, 16 and 32-point transforms use 192, 1152, and 5888 multiplications, respectively, compared to 352, 2752, and 21888 multiplications for the corresponding transforms in HM. The luma BD-rate performance of the proposed transforms is on average (over 6 configurations) +0.02% for normal QP range, +0.07% for high QP range and +0.45% for low QP range when compared to HM 3.0 transforms.

# Introduction

In JCTVC-F352 [1], 8, 16, and 32 point transforms were proposed. These transforms were scaled orthogonal integer transforms supporting a recursive factorization structure. In this contribution, we modify these transforms so that the size of quantization and dequantization matrices is reduced. Another modification is that the 1st and 2nd stage of the inverse transform is 16 bit.

# Transform specification

Figure 1 shows the proposed 16-point transform. The factors on the right hand side are normalization factors (not including). Thus, the transform is a scaled transform. In order for the transform to be orthogonal, the butterfly factors have to satisfy:

Z4=A42+B42, Z8=A82+B82=C82+D82, and

Z16=A162+B162=C162+D162=E162+F162=G162+H162.

Similar relationship holds for butterfly factors in the odd part of 32-point transform, that is:

Z32=A322+B322=C322+D322 and so on.

In order to reduce the size of the quantization and dequantization matrices, we make some approximations. First, we choose Z16 to be equal to 1 and factors A16, B16, … , H16 are chosen such that the equality involving Z16 is satisfied only approximately. Similar strategy is followed for Z32. This has the effect of reducing the number of unique normalization factors to 5. These are:

1,12,1Z4,12×Z8,and1Z8

This makes it possible to reduce the size of quantization and dequantization matrices from 12×12×6 to 5×5×6. To make the approximation accurate, the bit-depth for constants A16, B16, … , H16 is increased to 10 bits from 8 bits as in [1]. The choice of butterfly factors is specified below.

Figure 1: Proposed 16-point transform

It should be noted that the upper (even) part of the 16-point transform already contains the specification of the 8-point transforms. The choice of butterfly factors for the 8, 16 and 32-point transforms is shown in Tables 1-3.

| A4 | B4 | A8 | B8 | C8 | D8 |
| --- | --- | --- | --- | --- | --- |
| 17/64 | 41/64 | 111/128 | 22/128 | 94/128 | 63/128 |

| A16 | B16 | C16 | D16 | E16 | F16 | G16 | H16 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1019/1024 | 100/1024 | 980/1024 | 297/1024 | 903/1024 | 483/1024 | 792/1024 | 650/1024 |

| A32 | C32 | E32 | G32 | I32 | K32 | M32 | O32 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1023/1024 | 1013/1024 | 993/1024 | 964/1024 | 926/1024 | 878/1024 | 822/1024 | 759/1024 |
| B32 | D32 | F32 | H32 | J32 | L32 | N32 | P32 |
| 50/1024 | 150/1024 | 249/1024 | 345/1024 | 438/1024 | 526/1024 | 610/1024 | 688/1024 |

Table 1-3: Butterfly factors for the proposed 16 and 32 point transforms

Tables 1-3 specify the butterfly factors for a 32-point transform. Tables 1-2 specify the butterfly factors for a 16-point transform. Table 1 specifies the butterfly factors for an 8-point transform.

# Multiplierless implementation

The proposed transforms can be implemented using only shifts and additions. This is accomplished by replacing a pair of multiplies in a butterfly by shifts and additions. Table 4 specifies a multiplierless design for the proposed 8 point transform.

| Pair of factors | 1st factor (A) | 2nd factor (B) | Algorithm: y = A*x; z = B*x; | Complexity |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Additions | Shifts |
| A4, B4 | 17 | 41 | x1 = x << 4; x2 = x << 3; x3 = x1 + x2; y = x + x1; z = x3 + x4; | 3 | 2 |
| A8, B8 | 111 | 22 | x1 = x << 3; x2 = x1 - x; x3 = x2 << 4; x4 = x2 << 1; y = x3 - x; z = x1 + x4; | 3 | 3 |
| C8, D8 | 94 | 63 | x1 = x << 5; x2 = x - x1; z = x1 - x2; y = z - x2; | 3 | 1 |

Table 4: Multiplierless design for an 8 point transform

- Complexity measurement

## Arithmetic operations

Table 5 compares the complexity of the proposed transforms with the existing transforms in HM in terms of multiplications and additions for 8, 16 and 32 point transforms. Here we have not counted the additions needed for adding the offset if bit-shifting is performed before or after a transform stage. Table 6 specifies the complexity of the proposed multiplierless transforms in terms of shifts and additions

|  |  | HM | Proposed transform |
| --- | --- | --- | --- |
| 8-point | Additions | 448 | 416 |
|  | Multiplications | 352 | 192 |
| 16-point | Additions | 3200 | 2304 |
|  | Multiplications | 2752 | 1152 |
| 32-point | Additions | 23808 | 11904 |
|  | Multiplications | 21888 | 5888 |

Table 5: Complexity comparison of the proposed transforms with HM 2.0 transforms

| Transform | Complexity of factorization (for 1-D) | Complexity of multiplications (for 1-D) | Complexity of full multiplierless transform |
| --- | --- | --- | --- |
| 8-point | 26 adds + 12 mults (6 pairs) | 2 * (9 adds + 6 shifts) | 44 adds + 12 shifts |
| 16-point | 72 adds + 36 mults (18 pairs) | 2 * (26 adds + 23 shifts) |  |
| 32-point | 186 adds + 92 mults (46 pairs) | 2 * (81 adds + 78 shifts) |  |

Table 6: Transform complexity estimates in terms of shifts and additions.

## Dynamic range analysis

The bitdepth expansion due to each stage of the proposed 8, 16 and 32-point transforms is, in the worst case, 3, 4 and 5 bits, respectively. This is true both for forward transforms and inverse transforms.

To reduce the bitdepths at the inputs of inverse transform stages, we applied the method described in JCTVC-D071 [2] in a modified form to the output of the dequantization stage and the output of the 1st inverse transform stage. The output of the dequantization stage was shifted to the right after adding a rounding offset. Then, the resulting values were clipped to 16 bits. The output of the 1st inverse transform stage was shifted to the right by  after adding a rounding offset. Then, the resulting values were clipped to 16 bits.

## Quantization matrix storage

The Q_BITS and DQ_BITS for various transform sizes was adjusted so that the quantization matrix for block size of 32×32 contained quantization matrices for block sizes of 16×16 and 8×8. DQ_BITS was also adjusted so that the dequantization matrix elements would fit within a byte. Thus, the following additional storage is needed for dequantization matrices:

- A 5×5×6 matrix (150 bytes).

- LUTs of sizes 32, 16, and 8 (56 bytes).

Thus, the total additional storage on the decoder side is 206 bytes.

## Number of sequential operations

The number of cascaded multipliers is 1 for 8-point transform and 2 for 16 and 32-point transforms. The number of sequential operations in the longest path is shown in Table 8 for each transform size.

| Transform size | Number of sequential operations in the longest path |
| --- | --- |
| 8 | 1 multiply and 4 additions |
| 16 | 2 multiplies and 5 additions |
| 32 | 2 multiplies and 7 additions |

Table 7: The number of sequential operations in the longest path for each transform size

# Results

The proposed transforms were implemented in HM3.0. Simulation results are presented for the common test conditions for normal QP range, low QP range and high QP range. Table 8 shows the results for normal QP range. On average, for 6 test configurations, the BD-rate is +0.02%. Table 10 shows the results for low QP range. On average, for 6 test configurations, the BD-rate is +0.25%. Table 11 shows the results for high QP range. On average, for 6 test configurations, the BD-rate is +0.05%.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.0% | -0.2% | 0.0% | 0.0% | 0.1% |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 105% |  |  | 106% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.1% | 0.3% | 0.1% | 0.0% | 0.1% |
| Class B | 0.0% | -0.1% | 0.2% | 0.0% | 0.1% | 0.2% |
| Class C | 0.0% | 0.2% | -0.1% | 0.0% | 0.1% | -0.1% |
| Class D | 0.0% | 0.1% | -0.2% | 0.0% | -0.3% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 106% |  |  | 107% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.4% | 0.5% | 0.1% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% |
| Class D | -0.1% | 0.1% | -0.1% | 0.0% | 0.2% | 0.0% |
| Class E | 0.1% | 0.8% | -0.4% | 0.0% | 1.3% | -0.1% |
| Overall | 0.0% | 0.3% | 0.1% | 0.1% | 0.3% | 0.0% |
|  | 0.0% | 0.3% | 0.0% | 0.1% | 0.3% | -0.1% |
| Enc Time[%] | 100% |  |  | 98% |  |  |
| Dec Time[%] | 106% |  |  | 108% |  |  |

Table 9: BD-rate results for normal QP range

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.7% | 0.1% | 0.1% | 1.0% | 1.2% | 1.2% |
| Class B | -0.1% | 0.1% | 0.2% | 1.5% | 1.1% | 1.1% |
| Class C | 0.0% | 0.1% | 0.1% | 0.9% | 0.7% | 0.7% |
| Class D | 0.0% | 0.1% | 0.1% | 0.9% | 0.6% | 0.6% |
| Class E | 0.1% | 0.3% | 0.3% | 0.9% | 0.6% | 0.6% |
| Overall | -0.2% | 0.2% | 0.2% | 1.1% | 0.9% | 0.9% |
|  | -0.2% | 0.1% | 0.1% | 1.1% | 0.6% | 0.6% |
| Enc Time[%] | 103% |  |  | 100% |  |  |
| Dec Time[%] | 107% |  |  | 109% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | 0.2% | 0.2% | 0.5% | 0.8% | 0.8% |
| Class B | 0.1% | 0.1% | 0.1% | 0.8% | 0.6% | 0.6% |
| Class C | 0.2% | 0.1% | 0.2% | 0.7% | 0.4% | 0.4% |
| Class D | 0.2% | 0.0% | 0.0% | 0.6% | 0.3% | 0.3% |
| Class E |  |  |  |  |  |  |
| Overall | 0.1% | 0.1% | 0.1% | 0.7% | 0.6% | 0.5% |
|  | 0.1% | 0.1% | 0.1% | 0.7% | 0.5% | 0.5% |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 109% |  |  | #NUM! |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.0% | 0.0% | 1.1% | 0.5% | 0.5% |
| Class C | 0.2% | 0.1% | 0.0% | 0.8% | 0.4% | 0.4% |
| Class D | 0.2% | 0.0% | 0.0% | 0.7% | 0.1% | 0.1% |
| Class E | 0.4% | 0.3% | 0.2% | 0.7% | 0.3% | 0.3% |
| Overall | 0.2% | 0.1% | 0.1% | 0.8% | 0.3% | 0.3% |
|  | 0.2% | 0.1% | 0.1% | 0.8% | 0.3% | 0.3% |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 110% |  |  | 107% |  |  |

Table 10: BD-rate results for low QP range

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -0.8% | -1.3% | 0.1% | 0.1% | 0.3% |
| Class B | 0.0% | -0.5% | -0.6% | 0.1% | 0.3% | 0.1% |
| Class C | -0.1% | 0.1% | 0.3% | 0.0% | 0.2% | 0.3% |
| Class D | 0.0% | 0.6% | 0.3% | 0.1% | -0.9% | 0.0% |
| Class E | 0.1% | -0.6% | -0.3% | 0.1% | 0.3% | 0.0% |
| Overall | 0.0% | -0.2% | -0.3% | 0.1% | 0.0% | 0.1% |
|  | 0.0% | -1.1% | 0.6% | 0.1% | 0.2% | 0.2% |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 105% |  |  | 105% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.4% | -0.6% | 0.0% | 0.7% | 0.8% |
| Class B | 0.1% | -0.4% | 2.0% | 0.1% | 1.0% | 0.8% |
| Class C | -0.2% | -0.1% | 1.0% | 0.0% | -1.6% | 0.9% |
| Class D | 0.0% | 2.2% | 1.2% | 0.0% | -0.1% | 0.6% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.3% | 0.9% | 0.0% | 0.0% | 0.8% |
|  | 0.0% | 0.4% | 1.0% | 0.0% | -0.1% | 0.7% |
| Enc Time[%] | 103% |  |  | 101% |  |  |
| Dec Time[%] | 108% |  |  | 107% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.5% | 0.8% | 0.2% | 1.9% | -0.8% |
| Class C | 0.0% | 2.2% | 2.1% | 0.1% | 0.4% | 0.4% |
| Class D | 0.3% | -10.0% | 0.5% | 0.1% | 3.4% | 0.6% |
| Class E | 0.8% | 3.5% | 1.4% | 0.0% | 1.7% | 0.5% |
| Overall | 0.2% | -1.1% | 1.1% | 0.1% | 1.8% | 0.1% |
|  | 0.2% | 2.4% | 0.8% | 0.1% | 2.5% | 0.0% |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 107% |  |  | 107% |  |  |

Table 11: BD-rate results for high QP range

# Conclusions

In this contribution, 8, 16, and 32 point transforms are proposed. These transforms are scaled integer transforms supporting a recursive factorization structure. The 4-point transform is unchanged from HM. The 8, 16 and 32-point transforms use 192, 1152, and 5888 multiplications, respectively, compared to 352, 2752, and 21888 multiplications for the corresponding transforms in HM. The luma BD-rate performance of the proposed transforms is on average (over 6 configurations), +0.02% for normal QP range, +0.07% for high QP range and +0.45% for low QP range when compared to HM 3.0 transforms.

# References

- R. Joshi, Y. Reznik, J. Sole, and M. Karczewicz, “CE10: Scaled orthogonal integer transforms supporting recursive factorization structure”, JCTVC-F352, Torino, IT, July 2011.

- K. Misra, L. Kerofsky, and A. Segall, “On transform dynamic range”, JCTVC-D079, Daegu, KR, January 2011.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).