| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G580 WG11 Number: m22145 |
| --- | --- |

| Title: | CE7: Crosscheck of rotational transform (JCTVC-G304) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Rajan Joshi 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Email: | rajanj@qualcomm.com Tel. +1-858-658-4511 |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution provides a summary of a cross-check performed by Qualcomm of Samsung’s rotational transform proposal in CE7 (JCTVC-G304). The results of the crosscheck matched those reported by Samsung. The encoding and decoding times also matched closely.

- Introduction

Samsung proposes rate-distortion optimized selection of an 8x8, secondary, rotational transform, applied after the HM 4.0 core transform. The secondary transform is applied to the upper-left, 8x8 DCT-coefficient block of 8x8, 16x16 and 32x32 luma TUs. The implementation uses a lifting factorization of the rotational transform.

It was verified that the maximum bit precision for intermediate storage is 16 bits. The functions which implement the lifting transforms use “short” datatype exclusively. The lifting multipliers use 5-bit precision.

RDOQ was modified so that the samples from 16×16 and 32×32 blocks that are not affected by the rotational transform are processed only once during RDOQ.

# Results

Results for 4 configurations, AI-HE, AI-LC, RA-HE, and RA-LC, were crosschecked, as specified in CE7 test conditions. Table 1 shows a summary of the BD-rate results. More detailed coding results can be found in the Excel attachment. No YUV files were written when the decoding times were measured. Encoding as well as decoding was done on a LINUX cluster (64 bit) by specifying only one type of CPU.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1.1% | -0.1% | -0.2% | -1.1% | 0.1% | -0.2% |
| Class B | -1.1% | 0.5% | 0.5% | -1.0% | 0.6% | 0.6% |
| Class C | -0.6% | 0.2% | 0.2% | -0.7% | 0.4% | 0.3% |
| Class D | -0.6% | 0.3% | 0.2% | -0.7% | 0.5% | 0.4% |
| Class E | -0.8% | 0.1% | 0.1% | -0.8% | 0.3% | 0.5% |
| Overall | -0.9% | 0.2% | 0.2% | -0.9% | 0.4% | 0.3% |
|  | -0.9% | 0.2% | 0.2% | -0.9% | 0.4% | 0.3% |
| Enc Time[%] | 127% |  |  | 158% |  |  |
| Dec Time[%] | 101% |  |  | 103% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.5% | -0.2% | -0.5% | -0.5% | -0.4% | -0.4% |
| Class B | -0.6% | 0.2% | 0.3% | -0.7% | 0.2% | 0.3% |
| Class C | -0.4% | 0.0% | 0.0% | -0.5% | 0.0% | 0.2% |
| Class D | -0.3% | 0.1% | -0.3% | -0.4% | 0.4% | 0.2% |
| Class E |  |  |  |  |  |  |
| Overall | -0.5% | 0.0% | -0.1% | -0.5% | 0.1% | 0.1% |
|  | -0.5% | 0.0% | -0.1% | -0.5% | 0.1% | 0.1% |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |

Table 1: BD-rate results for the proposed rotational transform

# Conclusions

We have verified results reported in Samsung’s proposed rotational transform implemented as a lifting factorization. For AI configuration, BD-rate of -0.9% and -0.9% is achieved for HE and LC, respectively. This is achieved at a cost of encoding time increase of 27% and 58%, respectively. For RA configuration, BD-rate of -0.5% and -0.5% is achieved for HE and LC, respectively. In this case, the encoding time increase is 1-2%. The encoding and the decoding times were within margin of error of those reported by Samsung.