| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G583 WG11 Number: m22148 |
| --- | --- |

| Title: | Reducing output delay for "bumping" process |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonatan Samuelsson Rickard Sjöberg | Tel: Email: | +46 761 26 35 91 jonatan.samuelsson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This document proposes to add a flag to the slice header called output_process_flag and that the “bumping” process is replaced by an output process invoked based on the value of output_process_flag of the current picture. The document claims that the use of output_process_flag reduces unnecessary picture output delay. It is also claimed to be well suited for temporal scalability, in which sub-sequences may use a smaller DPB size.

# Introduction and Problem Statement

In H.264/AVC, single-level temporal scalability is supported through usage of non-reference frames. Multi-level temporal scalability is supported through gaps_in_frame_num_allowed_flag and “non-existing” pictures. The “non-existing” pictures can be used to ensure that the decoded picture buffer (DPB) status is the same regardless of how many temporal layers are decoded and that the correct number of pictures is outputted in the right order at the right time.

At the Torino meeting it was proposed, in F493, to remove frame_num and the generation of “non-existing” pictures. It was shown that the introduction of absolute signalling of reference pictures made it possible to remove these elements and still guarantee a correct decoding process regardless of how many layers are decoded. Correct output order is also guaranteed. However, without changes to the output process it is not guaranteed that pictures are outputted at “the right” time. This causes the following problems:

- Output delay can increase when not all temporal layers are decoded.

- Pictures that would have been outputted if all temporal layers were decoded might be discarded if not all temporal layers are decoded before IDR pictures when no_output_of_prior_pics_flag is equal to 1 or is inferred to be equal to 1.

- It is not possible to signal that a temporally scaled sub-sequence requires a smaller DPB size (and thus may conform to a lower level).

In order to solve the problems mentioned above, an output process is needed that is not dependent on the value of MaxDpbSize. It could of course be possible to require one value of MaxDpbSize per temporal layer and redefine the “bumping” process so that each temporal layer uses its own MaxDpbSize but that would complicate the description and would for many coding structures increase the required total size of the DPB.

We propose to introduce a flag in the slice header of each picture; output_process_flag. Further, we propose to replace the “bumping” process with an output process that is invoked when the output_process_flag is set to 1. The proposed process does not depend on MaxDpbSize and is therefore well suited for temporal scalability, in which sub-sequences may use a smaller DPB size.

The output_process_flag should be set by the encoder to 1 only if the following condition is true:

- There is no picture with output_flag equal to 1 that follows the current picture in decoding order and precedes any picture X in display order where X precedes the current picture in decoding order and display order.

It is recommended for an encoder to set the flag to 1 for each picture that fulfils the above condition and to set the flag to 0 otherwise.

The HDR specification for output process is changed as follows (marked with “track changes”):

  - Decoder conformance

    - Operation of the output order DPB

The decoded picture buffer contains frame buffers. Each of the frame buffers contains a decoded picture that is marked as "used for reference" or is held for future output. At HRD initialisation, the DPB fullness, measured in frames, is set to 0. The following steps all happen instantaneously when an access unit is removed from the CPB, and in the order listed.

    - Removal of pictures from the DPB

The removal of pictures from the DPB before decoding of the current picture (but after parsing the slice header of the first slice of the current picture) proceeds as follows.

The decoding process for reference picture set as specified in subclause 8.2.2 is invoked.

–	If the current picture is an IDR picture, the following applies.

- When the IDR picture is not the first IDR picture decoded and the value of pic_width_in_luma_samples or pic_height_in_luma_samples or max_dec_frame_buffering derived from the active sequence parameter set is different from the value of pic_width_in_luma_samples or pic_height_in_luma_samples or max_dec_frame_buffering derived from the sequence parameter set that was active for the preceding picture, respectively, no_output_of_prior_pics_flag is inferred to be equal to 1 by the HRD, regardless of the actual value of no_output_of_prior_pics_flag.

NOTE – Decoder implementations should try to handle changes in the value of pic_width_in_luma_samples or pic_height_in_luma_samples or max_dec_frame_buffering more gracefully than the HRD.

- When no_output_of_prior_pics_flag is equal to 1 or is inferred to be equal to 1, all frame buffers in the DPB are emptied without output of the pictures they contain, and DPB fullness is set to 0.

–	Otherwise (the current picture is not an IDR picture), frame buffers containing a picture which are marked as "not needed for output" and "unused for reference" are emptied (without output), and the DPB fullness is decremented by the number of frame buffers emptied. When the output_process_flag of the current picture is set to 1 and there is one or more pictures in the DPB that are marked as "needed for output" with PicOrderCnt lower than the current picture the output process specified in subclause C.4.2.1 is invoked repeatedly until all pictures with PicOrderCnt lower than the PiOrderCnt of the current picture have been marked as "not needed for output".

When the current picture is an IDR picture for which no_output_of_prior_pics_flag is not equal to 1 and is not inferred to be equal to 1, the following two steps are performed.

1.	Frame buffers containing a picture that is marked as "not needed for output" and "unused for reference" are emptied (without output), and the DPB fullness is decremented by the number of frame buffers emptied.

2.	All non-empty frame buffers in the DPB are emptied by repeatedly invoking the output process specified in subclause C.4.2.1, and the DPB fullness is set to 0.

      - Output process

The output process is invoked in the following cases.

–	The current picture is an IDR picture and no_output_of_prior_pics_flag is not equal to 1 and is not inferred to be equal to 1, as specified in subclause C.4.2.

–	The output_process_flag of the current picture is set to 1 and there are pictures in the DPB marked as "needed for output" with PicOrderCnt lower than the current picture , as specified in subclause C.4.2.

The output process consists of the following ordered steps:

- The picture that is first for output is selected as the one having the smallest value of PicOrderCnt of all pictures in the DPB marked as "needed for output".

- The picture is cropped, using the cropping rectangle specified in the active sequence parameter set for the picture, the cropped picture is output, and the picture is marked as "not needed for output".

- If the frame buffer that included the picture that was cropped and output contains a picture marked as "unused for reference", the frame buffer is emptied and the DPB fullness is decremented by 1.

    - Picture decoding, marking and storage

The current picture is decoded and stored in an empty frame buffer in the DPB. The DPB fullness is incremented by one, and the following applies.

–	If the current decoded picture has OutputFlag equal to 1, it is marked as "needed for output".

–	Otherwise (the current decoded picture has OutputFlag equal to 0), it is marked as "not needed for output".

If the current decoded picture is a reference picture, it is marked as "used for reference", otherwise (the current decoded picture is a non-reference picture), it is marked as "unused for reference".

# Example

The coding structure for the Random Access case in Common Conditions with temporal layers turned on is shown in Figure 1.

Figure 1	. Coding structure for Random Access with output_process_flag

Table 1 shows which pictures are outputted at what time comparing the current HEVC scheme with the proposed. The pictures represented by POC in the leftmost column are ordered in decoding order. It can be seen that there is a difference between the two schemes, although not huge, when all temporal layers are decoded.

|  | current scheme | proposed scheme |
| --- | --- | --- |
| POC of received picture | POC of outputted pictures | POC of outputted pictures |
| 0 | - | - |
| 8 | - | 0 |
| 4 | - | - |
| 2 | - | - |
| 6 | - | - (since output_process_flag = 0) |
| 1 | - | - |
| 3 | - | 1, 2 |
| 5 | 0, 1 | 3, 4 |
| 7 | 2, 3 | 5, 6 |
| 16 | 4, 5 | 7, 8 |
| 12 | 6, 7 | - |
| 10 | - | - |
| 14 | - | - (since output_process_flag = 0) |
| 9 | - | - |
| 11 | 8, 9 | 9, 10 |
| 13 | 10, 11 | 11, 12 |
| 15 | 12, 13 | 13, 14 |
| 24 | 14, 15 | 15, 16 |

Table 1. Comparison of output when all temporal layers are decoded.

Table 2 shows which pictures are outputted at what time when only two temporal layers are decoded. It can be noted that there is a large difference in delay between the two schemes. The difference is even larger when only one temporal layer is decoded.

|  | current scheme | proposed scheme |
| --- | --- | --- |
| POC of received picture | POC of outputted pictures | POC of outputted pictures |
| 0 | - | - |
| 8 | - | 0 |
| 4 | - | - |
| 16 | - | 4, 8 |
| 12 | - | - |
| 24 | - | 12, 16 |
| 20 | 0 | - |
| 32 | 4 | 20, 24 |
| 28 | 8 | - |
| 40 | 12 | 28, 32 |
| 36 | 16 | - |
| 48 | 20 | 36, 40 |
| 44 | 24 | - |
| 56 | 28 | 44, 48 |

Table 2. Comparison of output when two temporal layers are decoded.

# Proposed syntax and semantics

### Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| lightweight_slice_flag | u(1) |
| if( !lightweight_slice_flag ) { |  |
| output_process_flag | u(1) |
| slice_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| … |  |
| } |  |

output_process_flag equal to 1 specifies that all decoded pictures that are marked as "needed for output" shall be outputted and marked as "not needed for output" if their picture order count value is lower than the current picture before the decoding of the current picture as specified in Annex C. output_process_flag equal to 0 specifies that no picture shall be outputted before the decoding of the current picture as specified in Annex C.

output_process_flag should be added to the list of syntax elements for which the value must be the same in all slice headers of a picture.

# Patent rights declaration(s)

Ericsson may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).