| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, 21-30 November, 2011 | Document: JCTVC-G589 m22154 |
| --- | --- |

| Title: | Encoder optimization: Reduced number of reference pictures for RA |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Per Wennersten Rickard Sjöberg Jonatan Samuelsson Färögatan 6 16480 Stockholm | Tel: Email: | +46 10 7172926 per.wennersten@ericsson.com |
| Source: | Ericsson AB |  |  |

_____________________________

# Abstract

This contribution consists of an alternative GOP structure for random access coding. Compared to the GOP structure currently in use for the random access JCT-VC common conditions test case, it requires fewer pictures to be kept in the decoder picture buffer. Only the encoder configuration is changed, and the average luma BD-rate result is -0.2% for random access high efficiency and -0.3% for random access low complexity. Calculating YUV BD-rate as (6Y+U+V)/8, the change is -0.2% for RA HE and -0.4% for RA LC.

# Introduction

In the current common condition random access configuration, each GOP of eight frames is encoded in a hierarchical fashion. The order in which the individual frames are encoded is shown in Figure 1. The contents of the reference picture buffer for each of these frames are shown in Table 1.

| Coding order | Display order | Reference pictures |
| --- | --- | --- |
| 1 | 8 | P4, P3, P2, P1 |
| 2 | 4 | 1, P4, P1 |
| 3 | 2 | 2, 1, P4, P1 |
| 4 | 6 | 3, 2, 1, P4, P1 |
| 5 | 1 | 4, 3, 2, 1, P4, P1 |
| 6 | 3 | 4, 3, 2, 1, P1 |
| 7 | 5 | 4, 3, 2, 1 |
| 8 | 7 | 4, 3, 2, 1 |

Figure 1: Coding order of original GOP

Table 1: Reference pictures used in original GOP.     Reference pictures are given in coding order.

As Table 1 shows, the maximum number of reference pictures which need to be kept at any time is 6, when encoding the first frame in display order.

# Proposed GOP structure

In the proposed GOP structure, coding order has been modified to the one shown in Figure 2. In addition, the reference pictures used for each frame have been changed, and the new contents of the reference picture buffer for each frame are shown in Table 2.

Figure 2: Coding order of modified GOP

| Coding order | Display order | Reference pictures |
| --- | --- | --- |
| 1 | 8 | P6, P2, P1, PP1 |
| 2 | 4 | 1, P6, P1 |
| 3 | 2 | 2, 1, P6, P1 |
| 4 | 1 | 3, 2, 1, P1 |
| 5 | 3 | 3, 2, 1, P1 |
| 6 | 6 | 3, 2, 1, P1 |
| 7 | 5 | 6, 2, 1, P1 |
| 8 | 7 | 6, 2, 1, P1 |

Table 2: Reference pictures used in modified GOP.     Reference pictures are given in coding order.

As seen in Table 2, the maximum number of reference pictures kept in the decoder buffer is now 4.

# Experimental results

Experiments have been performed on the HM branch used in AHG 21. No modifications to the source code have been made, but configuration files specifying the proposed GOP structure have been created, and are available together with this input document. Full results are summarized in Table 3. Encoding and decoding time estimates are not reliable as the simulations were run on a cluster.

Simulations for random access high efficiency coding give an average luma BD-rate change of -0.2% when using the proposed GOP structure. YUV BD-rate, calculated as (6Y+U+V)/8, is also changed by -0.2%.

For low complexity coding, luma BD-rate is changed by -0.3% and the YUV BD-rate by -0.4%.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,1% | 0,1% | 0,1% | 0,1% | -0,2% | -0,1% |
| Class B | -0,2% | -0,2% | -0,2% | -0,2% | -0,4% | -0,3% |
| Class C | -0,5% | -0,5% | -0,5% | -0,6% | -0,8% | -0,7% |
| Class D | -0,3% | -0,4% | -0,4% | -0,6% | -0,8% | -0,8% |
| Class E |  |  |  |  |  |  |
| Overall | -0,2% | -0,3% | -0,3% | -0,3% | -0,5% | -0,5% |
|  | -0,2% | -0,2% | -0,3% | -0,3% | -0,5% | -0,5% |
| Enc Time[%] | 99% |  |  | 103% |  |  |
| Dec Time[%] | 103% |  |  | 106% |  |  |

Table 3: Experimental results. Time estimates are not reliable.

# Conclusions

In this contribution, an alternative GOP structure for random access coding is suggested. The changes reduce the required decoder picture buffer size, and provide small coding efficiency gains. For these reasons, it is proposed to use the presented GOP structure for the JCT-VC common conditions.

# Patent rights declaration(s)

Ericsson may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).