| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G590 |
| --- | --- |

| Title: | CE12 Subtest 1: Results for modified decisions for deblocking |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Matthias Narroschke Thomas Wedi Semih Esenlik Panasonic R&D Center Germany Monzastr. 4c, 63225 Langen, Germany | Tel: Email: | +49 6103 766 204 Matthias.Narroschke@eu.panasonic.com +49 6103 766 1194 Thomas.Wedi@eu.panasonic.com +49 6103 766 1304 Semih.Esenlik@eu.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution is part of CE12. It presents the results for Modified decisions for deblocking which is based on JCTVC-E251 and JCTVC-F191. In HM4.0, a first decision for enabling the deblocking is performed for edge segments of eight lines. In the case of enabled deblocking, a subsequent second decision is performed for each individual line by which either a strong or a weak filter is selected. In this proposal, two modifications are introduced. The first decisions are performed for edge segments of four lines instead of 8 lines. The second decisions are also performed for edge segments of four lines instead of for each individual line. At the same quality, the following average bit rate reductions are achieved relative to HM4.0: I-HE: 0.0%, I-LC: 0.0%, RA-HE: 0.0%, RA-LC: 0.0%, LD(B)-HE: 0.2%, LD(B)-LC: 0.1%, LD(P)-HE: 0.2%, LD(P)-LC: 0.0%. The modifications reduce the number of operations required for these two decisions by around 20%. In addition, the size of line buffers is reduced. They allow parallel deblocking of all 8x8 blocks. It is proposed to adopt the proposed technique into the next version of the HM.

# Introduction

Improved Decisions for deblocking were previously presented in the proposal JCTVC-E251 [1] and JCTVC-F191 [2]. It had been decided to evaluate improvements of deblocking decisions within the core experiment CE12 [3].

In the following Section 2, a brief technical description of the decisions according to HM4.0 is given. Section 3 explains the proposed deblocking decisions, Section 4 shows the performed experiments and the corresponding results. Section 5 concludes this contribution.

# Deblocking decisions according to HM4.0

HM 4.0 [4][5] applies hybrid coding. In a first step, the input signal to be coded is predicted block-wise by either motion compensated prediction or Intra prediction. The resulting prediction error is block-wise transform coded by applying an approximation of the discrete cosine transform, also known as Integer DCT, followed by a quantization of the coefficients. Due to the block wise motion compensated prediction and a block wise prediction error coding, so called blocking artifacts often become visible in the decoded images. These blocking artifacts tend to be annoying for human observers. In order to reduce these annoying blocking artifacts, an adaptive deblocking filter is applied.

The deblocking is performed based on edge segments of 8 consecutive lines or columns. The respective sides of a segment are denoted as A and B. The segment A corresponds to the left neighboring partition to B for vertical edges and to the above neighboring partition to B for horizontal edges.

For each segment of 8 lines/columns, HM4.0 performs the decisions as follows. In a first step, the four values dp2,dp5,dq2, and dq5are calculated by the use of the samples of two lines/columns as illustrated in Figure 1:

Figure 1: Calculation of the values dp2,dp5,dq2, and dq5 required to decide whether to filter or not according to HM4.0 [4][5].

By the use of these four values dp2,dp5,dq2, and dq5, it is decided by the threshold operation

dp2+dp5+dq2+dq5<β

if all 8 lines/columns of the corresponding segment are filtered or not. The first decision requires 20 operations to be performed for each segment of 8 lines/columns.

If the above condition is true, a second decision is performed for each individual line/column to select between a so called strong and a so called weak filter as illustrated in Figure 2.

Figure 2: Illustration of samples used to perform the selection between a strong and a weak filter according to HM4.0 [4][5].

This second decision is of the form:

d<β>>2∧p3i−p0i+q0i−q3i<β>>3∧p0i−q0i<5⋅tc+1>>1

If the above condition is true, the strong filter is applied, otherwise the weak filter. The second decision requires 96 operations to be performed for each segment of 8 lines/columns. Hereby, it is considered that the terms β>>2, β>>3, and 5⋅tc+1>>1 are pre-calculated and stored in tables.

# Proposed deblocking decisions

Each segment of 8 lines/columns is further divided into 2 segments, each of 4 lines/columns. For each segment of 4 lines/columns, all decisions are performed independently. In a first step, the values dp0, dp3,dp4,dp7,dq0, dq3,dq4, and dq7 are calculated by the use of the samples of four lines/columns as illustrated in Figure 3:

Figure 3: Calculation of the values dp0, dp3,dp4,dp7,dq0, dq3,dq4, anddq7 required to decide whether to filter or not according to the proposal.

By the threshold operation

dp0+dq0+dp3+dq3<β

it is decided if the first 4 lines/columns with the indexes 0,1,2, and 3 of the corresponding segment are filtered or not.

By the threshold operation

dp4+dq4+dp7+dq7<β

it is decided if the second 4 lines/columns with the indexes 4,5,6, and 7 of the corresponding segment are filtered or not.

The first decision requires 2⋅20=40operations to be performed for each segment of 8 lines/columns.

If the above decision is true for a segment of 4 lines/columns, a second decision is performed. In contrast to HM4.0, this decision is not performed for each individual line/column but only one decision is performed for a segment of 4 lines/columns by the use of the samples of four lines/columns as illustrated in Figure 4.

Figure 4: Illustration of samples used to perform the selection between a strong and a weak according to the proposal.

The first 4 lines/columns with the indexes i=0,1,2, and 3 are filtered with the strong filter if both decisions

2⋅dp0+dq0<β>>2∧p30−p00+q00−q30<β>>3∧p00−q00<5⋅tc+1>>1

2⋅dp3+dq3<β>>2∧p33−p03+q03−q33<β>>3∧p03−q03<5⋅tc+1>>1

are positive.

The second 4 lines/columns with the indexes i=4,5,6, and 7 are filtered with the strong filter if the decision

2⋅dp4+dq4<β>>2∧p34−p04+q04−q34<β>>3∧p04−q04<5⋅tc+1>>1

2⋅dp7+dq7<β>>2∧p37−p07+q07−q37<β>>3∧p07−q07<5⋅tc+1>>1

are positive.

This second decision requires 52 operations to be performed for each segment of 8 lines/columns. Hereby, it is also considered that the terms β>>2, β>>3, and 5⋅tc+1>>1 are pre-calculated and stored in tables.

# Experiments and results

The proposed decisions have been integrated into the reference software of HM4.0 [4].

## BD-bit rate and run time ratios

Following the common conditions [6], the performance is evaluated for all six mandatory as well as for the two optional test cases, which is Intra, Random access, Low delay B, and Low delay P, each in high efficiency and low complexity operation mode. For all run time measurements, computers of the same configuration are used.

The BD-rates as well as the encoder-/decoder run time ratios compared to the reference HM4.0 are shown in Figure 5. Negative BD-rate numbers show a gain compared to the reference. Run-time ratios less than 100% show reflect that the run time is lower than the one of the reference. An average bit rate reduction at the same quality is achieved compared to HM4.0: I-HE: 0.0%, I-LC: 0.0%, RA-HE: 0.0%, RA-LC: 0.0%, LD(B)-HE: 0.2%, LD(B)-LC: 0.1%, LD(P)-HE: 0.2%, LD(P)-LC: 0.0%. The run times are approximately unchanged.

Figure 5: BD-bit rates and run time ratios of the proposed decisions compared to the reference HM4.0 [4].

## Line memory reduction

In HM4.0, the decisions to enable the deblocking filter and to select between the strong and the weak filter for a current block are performed based on four lines of the reconstructed signal as shown in the top image of Figure 6. In hardware implementations, these four lines are preferably stored in on-chip line buffers. The horizontal filtering of the vertical edges of these four lines is preferably delayed until the decisions have been performed. In HM4.0, the delay of the decisions requires buffering the decisions whether to filter or not for a whole line of blocks since these decisions can not be performed based on the stored four lines of samples. An additional line is necessary, see middle image of Figure 6. Alternatively to storing of decisions, it is possible to store the samples of the additional line in a line buffer. However, this is associated with a larger memory. By the use of the proposal, the decisions do not need to be buffered since they can be performed based on the stored four lines of samples, see bottom image of Figure 4.

Figure 6: Top image: Illustration of reconstructed samples used to perform decisions for the current block. Middle image: Illustration of reconstructed samples used to perform delayed decisions for the blocks above the current block according to HM4.0. Bottom image: Illustration of reconstructed samples used to perform delayed decisions for the blocks above the current block according to the proposal.

## Increased of parallelization

With the introduced decisions, the dependencies in the deblocking are limited to blocks of 8x8 samples as shown in Figure 7. As a consequence, each 8x8 block can be deblocked in parallel to the others.

Figure 7: Illustration of dependencies in the deblocking limited to 8x8 samples.

## Computational expense

For each segment of 8 lines/columns, HM4.0 requires 20+96=116 operations. The proposal requires only 40+52=92 operations, which is around 20% less.

# Conclusion

In this contribution, results for Modified decisions for deblocking are presented. Two modifications are introduced. The decisions to enable the deblocking are performed for edge segments of four lines instead of eight lines. The decisions between a strong and a weak filter are also performed for edge segments of four lines instead of for each individual line. At the same quality, the following average bit rate reductions are achieved relative to HM4.0: I-HE: 0.0%, I-LC: 0.0%, RA-HE: 0.0%, RA-LC: 0.0%, LD(B)-HE: 0.2%, LD(B)-LC: 0.1%, LD(P)-HE: 0.2%, LD(P)-LC: 0.0%. The modifications reduce the number of operations required for these two decisions by around 20%. They allow parallel deblocking of all 8x8 blocks. In addition, the size of line buffers is reduced. It is proposed to adopt the proposed technique into the next version of the HM.

# References

- M. Narroschke, T. Wedi, Decisions for deblocking, JCTVC-E251, Geneva, CH, March 2011.

- M. Narroschke, T. Wedi, CE12: Results for decisions for deblocking, JCTVC-F191, Torino, IT, July 2011.

- A. Norkin, Xun Guo, B. Jeon, M. Narroschke, Description of Core Experiment 12: Deblocking filtering, JCTVC-F912, Torino, IT, July 2011.

- HM4.0 software: http://hevc.kw.bbc.co.uk/trac/browser/jctvc-hm/tags/HM-4.0.

- T. Wiegand, W.-J. Han, J.-R. Ohm, G. J. Sullivan, WD4: Working Draft 4 of High-Efficiency Video Coding, JCTVC-E803, Torino, IT, July 2011.

- F. Bossen, Common test conditions and software reference configurations, JCTVC-F900, Torino, IT, July 2011.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).