| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G591 |
| --- | --- |

| Title: | Non CE 7: Supplementary Results for the Rotational Transform |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhan Ma Felix Fernandes Elena Alshina Alexander Alshin | Email: | zhan.ma@sta.samsung.com ffernandes@sta.samsung.com elena_a.alshina@samsung.com alexander_b.alshin@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution supplements the JCTVC-G304 CE 7 results for the Rotational Transform (ROT). It is shown that coding performance is unaffected by using only 2 sets of lifting multipliers instead of 3 sets. 
Additionally, using an encoder restriction, for the All-Intra (AI) test case, the ROT provides 0.5% (HE) / 0.5% (LC) performance gain. For the Random-Access (RA) test case, the ROT shows 0.2% (HE) / 0.3% (LC) performance gain. Encoder execution times relative to anchor are 110%, 120%, 102%, 102% for AI-HE, AI-LC, RA-HE, RA-LC cases respectively. Decoder execution times are identical to those of the anchor, within measurement-error margins, for all cases.

# Introduction

In CE 7, the Rotational Transform is tested in HM 4.0 and provides the following performance gains [1].  For the All-Intra (AI) test case, the ROT provides 0.9% (HE) / 0.9% (LC) performance gain. For the Random-Access (RA) test case, the ROT shows 0.5% (HE) / 0.5% (LC) performance gain. Encoder execution times relative to anchor are 129%, 158%, 103%, 104% for AI-HE, AI-LC, RA-HE, RA-LC cases respectively. Note that these results are achieved by using 3 sets of lifting multipliers. However, storage space can be reduced if only 2 sets of lifting multipliers are used. Also, most of the coding gains are retained under an encoder restriction that reduces the encoder runtime significantly.

- Eliminating a set of lifting multipliers

As explained in [1], the ROT is easily implemented with pseudocode that we reproduce here for the forward, horizontal transform. We use C-style syntax and the following notation:

short matrix[64] = 8x8 transform coefficients in row-major order

short temp[64] = intermediate storage

short Ai, Bi, Ci, Di, Fi, Gi, dum = temporary variables

short u[4][6] =

{//----vertical-|---horizontal---

{ -7,  -7,   5,  -4,  -2,  -2, },// indexROT = 0

{  4, -22,   9,  -7,  -4,   5, },// indexROT = 1

{ -3,  -5,  -3,   2, -29,   5, },// indexROT = 2

{-10,  -6,  -4,  -1, -29,   6, },// indexROT = 3

};

short v[4][6] =

{//-----vertical-|-horizontal--

{ 14,  13, -10,   8,   3,   4, },// indexROT = 0

{ -8,  30, -17,  13,   7, -10, },// indexROT = 1

{  5,   9,   6,  -5,  32, -10, },// indexROT = 2

{ 18,  11,   8,   1,  32, -12, },// indexROT = 3

};

w[][] is identical to u[][] except for w[0][4] = -1.

// Forward, horizontal ROT: Rh

for (n=0; n<64; n+=8){

// Rx

A0 = matrix[n  ] + (w[indexROT][3] * matrix[n+1])>>5

A3 = matrix[n+3] + (w[indexROT][3] * matrix[n+4])>>5

B1 = (v[indexROT][3] * A0)>>5 + matrix[n+1]

B4 = (v[indexROT][3] * A3)>>5 + matrix[n+4]

C0 = A0 + (u[indexROT][3]  *  B1)>>5

C3 = A3 + (u[indexROT][3]  *  B4)>>5

// Rz(π/2) swap and sign flip, Equation (17)

if ((indexROT == 2) || (indexROT == 3))

dum = B1

B1 = -matrix[n+2]

matrix[n+2] = dum

dum = B4

B4 = -matrix[n+5]

matrix[n+5] = dum

// Rz, Equation (7)

D1 = B1 + (w[indexROT][4]  *  matrix[n+2])>>5

D4 = B4 + (w[indexROT][4]  *  matrix[n+5])>>5

temp[n+2] = (v[indexROT][4]  *  D1)>>5 + matrix[n+2]

temp[n+5] = (v[indexROT][4]  *  D4)>>5 + matrix[n+5]

F1 = D1 + (u[indexROT][4] * temp[n+2])>>5

F4 = D4 + (u[indexROT][4] * temp[n+5])>>5

// Rx

G0 = C0 + (w[indexROT][5] * F1)>>5

G3 = C3 + (w[indexROT][5] * F4)>>5

temp[n+1] = (v[indexROT][5] * G0)>>5 + F1

temp[n+4] = (v[indexROT][5] * G3)>>5 + F4

temp[n  ] = G0 + (u[indexROT][5] * temp[n+1])>>5

temp[n+3] = G3 + (u[indexROT][5] * temp[n+4])>>5

} // for: horizontal ROT

To reduce storage space, we replace the w set of lifting multipliers by the u set as shown in the following pseudocode:

short u[4][6] =

{//----vertical-|---horizontal---

{ -7,  -7,   5,  -4,  -2,  -2, },// indexROT = 0

{  4, -22,   9,  -7,  -4,   5, },// indexROT = 1

{ -3,  -5,  -3,   2, -29,   5, },// indexROT = 2

{-10,  -6,  -4,  -1, -29,   6, },// indexROT = 3

};

short v[4][6] =

{//-----vertical-|-horizontal--

{ 14,  13, -10,   8,   3,   4, },// indexROT = 0

{ -8,  30, -17,  13,   7, -10, },// indexROT = 1

{  5,   9,   6,  -5,  32, -10, },// indexROT = 2

{ 18,  11,   8,   1,  32, -12, },// indexROT = 3

};

// Forward, horizontal ROT: Rh

for (n=0; n<64; n+=8){

// Rx

A0 = matrix[n  ] + (u[indexROT][3] * matrix[n+1])>>5

A3 = matrix[n+3] + (u[indexROT][3] * matrix[n+4])>>5

B1 = (v[indexROT][3] * A0)>>5 + matrix[n+1]

B4 = (v[indexROT][3] * A3)>>5 + matrix[n+4]

C0 = A0 + (u[indexROT][3]  *  B1)>>5

C3 = A3 + (u[indexROT][3]  *  B4)>>5

// Rz(π/2) swap and sign flip, Equation (17)

if ((indexROT == 2) || (indexROT == 3))

dum = B1

B1 = -matrix[n+2]

matrix[n+2] = dum

dum = B4

B4 = -matrix[n+5]

matrix[n+5] = dum

// Rz, Equation (7)

D1 = B1 + (u[indexROT][4]  *  matrix[n+2])>>5

D4 = B4 + (u[indexROT][4]  *  matrix[n+5])>>5

temp[n+2] = (v[indexROT][4]  *  D1)>>5 + matrix[n+2]

temp[n+5] = (v[indexROT][4]  *  D4)>>5 + matrix[n+5]

F1 = D1 + (u[indexROT][4] * temp[n+2])>>5

F4 = D4 + (u[indexROT][4] * temp[n+5])>>5

// Rx

G0 = C0 + (u[indexROT][5] * F1)>>5

G3 = C3 + (u[indexROT][5] * F4)>>5

temp[n+1] = (v[indexROT][5] * G0)>>5 + F1

temp[n+4] = (v[indexROT][5] * G3)>>5 + F4

temp[n  ] = G0 + (u[indexROT][5] * temp[n+1])>>5

temp[n+3] = G3 + (u[indexROT][5] * temp[n+4])>>5

} // for: horizontal ROT

# An Encoding Restriction to Reduce Execution Time

Consider the following pseudo code that describes the rate-distortion optimized search for optimal intra prediction mode and ROT index [1]

bestROTindex = -1

bestIntraMode = -1

rdCostMin = INT_MAX

for i in Intra_Pred_Mode_Candidate_Set

for j in ROT_Dictionary

rdCost = getRDcost(i, j)

if rdCost < rdCostMin

rdCostMin = rdCost

bestIntraMode = i

bestROTindex = j

Observe that this pseudo code incurs long execution times because       
    |Intra_Pred_Mode_Candidate_Set| * |ROT_Dictionary| iterations occur, where || indicates set 
    cardinality.

Now consider the following method in which the intra prediction mode search is decoupled from the 
    ROT index search.

bestIntraMode = -1

rdCostMin = INT_MAX

for i in Intra_Pred_Mode_Candidate_Set

rdCost = getRDcost(i, 0)

if rdCost < rdCostMin

rdCostMin = rdCost

bestIntraMode = i

bestROTindex = -1

rdCostMin = INT_MAX

for j in ROT_Dictionary

rdCost = getRDcost(bestIntraMode, j)

if rdCost < rdCostMin

rdCostMin = rdCost

bestROTindex = j

With this implementation, we expect shorter execution times because only |Intra_Pred_Mode_Candidate_Set| + |ROT_Dictionary| iterations occur.

# Experimental Results

Experiments are conducted in the HM4.0 software according to the common test conditions. In Table 1, the ROT implementation with 2 lifting-multiplier sets (Section 2) and WITHOUT the encoding restriction (Section 3) is compared against the HM4.0 anchor. Observe that these results are identical to the Table 1 results in [1], which used 3 lifting-multiplier sets. Therefore, no loss is incurred with 2 lifting multiplier sets.

Table 1: ROT performance with 2 lifting-multipler sets without encoding restriction, over HM4.0

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1.1% | -0.1% | -0.2% | -1.1% | 0.1% | -0.2% |
| Class B | -1.1% | 0.5% | 0.5% | -1.0% | 0.6% | 0.5% |
| Class C | -0.6% | 0.2% | 0.2% | -0.7% | 0.4% | 0.3% |
| Class D | -0.6% | 0.3% | 0.2% | -0.7% | 0.4% | 0.4% |
| Class E | -0.8% | 0.1% | 0.1% | -0.8% | 0.3% | 0.5% |
| Class F | -0.2% | 0.2% | 0.2% | -0.3% | 0.2% | 0.3% |
| Overall | -0.9% | 0.2% | 0.2% | -0.9% | 0.4% | 0.3% |
|  | -0.9% | 0.2% | 0.2% | -0.9% | 0.4% | 0.3% |
| Enc Time[%] | 130% |  |  | 155% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.5% | -0.2% | -0.5% | -0.5% | -0.2% | -0.3% |
| Class B | -0.6% | 0.2% | 0.3% | -0.7% | 0.1% | 0.2% |
| Class C | -0.4% | 0.0% | 0.0% | -0.5% | 0.1% | 0.3% |
| Class D | -0.3% | 0.1% | -0.3% | -0.4% | 0.3% | 0.2% |
| Class E |  |  |  |  |  |  |
| Class F | -0.3% | 0.2% | 0.0% | -0.2% | 0.2% | 0.1% |
| Overall | -0.5% | 0.0% | -0.1% | -0.5% | 0.1% | 0.1% |
|  | -0.5% | 0.0% | -0.1% | -0.5% | 0.1% | 0.1% |
| Enc Time[%] | 103% |  |  | 103% |  |  |
| Dec Time[%] | 102% |  |  | 100% |  |  |

In Table 2, the ROT implementation with 2 lifting-multiplier sets (Section 2) and an encoding restriction (Section 3) is compared against the HM4.0 anchor. Observe that, relative to [1], the encoding runtimes for AI-HE are reduced substantially from 129% to 110% and those for AI-LC are reduced considerably from 158% to 120%. Correspondingly, the coding gains reduce from 0.9% to 0.5% for AI-HE and AI-LC cases. Therefore moderately large coding gains are still attainable with the encoding restriction that reduces execution time significantly.

Table 2: ROT performance with 2 lifting-multipler sets and an encoding restriction, over HM4.0

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.8% | -0.2% | -0.1% | -0.8% | 0.3% | 0.0% |
| Class B | -0.7% | 0.4% | 0.3% | -0.7% | 0.7% | 0.6% |
| Class C | -0.3% | 0.2% | 0.1% | -0.4% | 0.4% | 0.4% |
| Class D | -0.3% | 0.2% | 0.1% | -0.4% | 0.4% | 0.4% |
| Class E | -0.4% | -0.1% | 0.0% | -0.4% | 0.4% | 0.5% |
| Class F | -0.2% | 0.1% | 0.2% | -0.1% | 0.2% | 0.3% |
| Overall | -0.5% | 0.1% | 0.1% | -0.5% | 0.5% | 0.4% |
|  | -0.5% | 0.1% | 0.1% | -0.5% | 0.5% | 0.4% |
| Enc Time[%] | 110% |  |  | 120% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -0.3% | -0.3% | -0.3% | 0.1% | -0.1% |
| Class B | -0.3% | 0.2% | -0.1% | -0.4% | 0.4% | 0.4% |
| Class C | -0.2% | 0.0% | 0.0% | -0.3% | 0.0% | 0.3% |
| Class D | -0.2% | 0.1% | -0.3% | -0.2% | 0.1% | 0.4% |
| Class E |  |  |  |  |  |  |
| Class F | -0.1% | 0.2% | 0.2% | -0.2% | 0.2% | 0.0% |
| Overall | -0.2% | 0.0% | -0.2% | -0.3% | 0.2% | 0.3% |
|  | -0.2% | 0.0% | -0.1% | -0.3% | 0.2% | 0.3% |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 102% |  |  | 100% |  |  |

Table 2: ROT performance with 2 lifting-multipler sets and an encoding restriction, over HM4.0

# Clipping

In CE 10 on Core Transforms, clipping to 16 bits is recommended after the vertical and horizontal transforms. Applying the same concept to the rotational transform, we get the results shown below. Note that they are virtually identical to those in Tables 1 and 2, except for slight encoder runtime differences attributed to cluster specifics.

Table 3: ROT performance with clipping, 2 lifting-multipler sets, no encoding restriction

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1.2% | -0.1% | -0.1% | -1.1% | 0.1% | -0.2% |
| Class B | -1.1% | 0.5% | 0.5% | -1.0% | 0.6% | 0.6% |
| Class C | -0.6% | 0.3% | 0.2% | -0.7% | 0.4% | 0.4% |
| Class D | -0.6% | 0.2% | 0.2% | -0.7% | 0.4% | 0.4% |
| Class E | -0.8% | 0.1% | 0.1% | -0.8% | 0.4% | 0.5% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -0.9% | 0.2% | 0.2% | -0.9% | 0.4% | 0.3% |
|  | -0.9% | 0.2% | 0.2% | -0.9% | 0.4% | 0.3% |
| Enc Time[%] | 131% |  |  | 157% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.5% | -0.2% | -0.3% | -0.5% | -0.3% | -0.6% |
| Class B | -0.6% | 0.1% | 0.2% | -0.7% | 0.1% | 0.2% |
| Class C | -0.4% | -0.1% | 0.1% | -0.5% | 0.1% | 0.1% |
| Class D | -0.3% | 0.2% | -0.1% | -0.4% | 0.1% | 0.0% |
| Class E |  |  |  |  |  |  |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -0.5% | 0.0% | 0.0% | -0.5% | 0.0% | -0.1% |
|  | -0.5% | 0.0% | 0.0% | -0.5% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  | 103% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |

Table 4: ROT performance with clipping, 2 lifting-multipler sets and an encoding restriction

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.8% | -0.2% | -0.1% | -0.8% | 0.3% | 0.0% |
| Class B | -0.7% | 0.4% | 0.3% | -0.7% | 0.7% | 0.6% |
| Class C | -0.3% | 0.2% | 0.1% | -0.4% | 0.4% | 0.4% |
| Class D | -0.3% | 0.2% | 0.1% | -0.4% | 0.4% | 0.4% |
| Class E | -0.4% | -0.1% | 0.0% | -0.4% | 0.4% | 0.5% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -0.5% | 0.1% | 0.1% | -0.5% | 0.5% | 0.4% |
|  | -0.5% | 0.1% | 0.1% | -0.5% | 0.5% | 0.4% |
| Enc Time[%] | 110% |  |  | 119% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -0.3% | -0.3% | -0.3% | 0.1% | -0.1% |
| Class B | -0.3% | 0.2% | -0.1% | -0.4% | 0.4% | 0.4% |
| Class C | -0.2% | 0.0% | 0.0% | -0.3% | 0.0% | 0.3% |
| Class D | -0.2% | 0.1% | -0.3% | -0.2% | 0.1% | 0.4% |
| Class E |  |  |  |  |  |  |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -0.2% | 0.0% | -0.2% | -0.3% | 0.2% | 0.3% |
|  | -0.2% | 0.0% | -0.1% | -0.3% | 0.2% | 0.3% |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 102% |  |  | 103% |  |  |

# Bit precision

The maximum bit precision for intermediate storage is 16 bits. The functions which implement the lifting transforms use “short” datatype exclusively. The lifting multipliers use 5-bit precision.

# Memory requirements

Additional storage is required for lifting multipliers. Each lifting multiplier has 5-bit precision and can be packed into a byte. There are 4*6*2 = 48 lifting multipliers. Therefore 48 bytes are required.

# Quantization

No additional quantization matrices are required.

# R-D optimization

Rate-distortion iterations are performed at the encoder to select the best rotation-angle set from a dictionary of 4 non-trivial sets and one trivial zero set.

# Other statistics

No memory transfers, reordering, or other data operations are performed.

# Subjective effects

We did not observe any subjective differences.

# Operation Counts

Table 2 provides the worst-case operation counts for the inverse ROT transform. Because the forward transform is identical to the inverse transform modulo changing adders to subtractors, the operations  count for the forward transform is identical.

To compute the worst case, recall that the ROT dictionary contains 4 non-trivial sets of rotation angles and one trivial set, the zero set (no ROT). Only one set is selected for a given TU. For adder counts, all 4 non-trivial sets have the same number of adders, 288. For multiplier counts, several multiplications are by powers of 2 and would be implemented using shifts. The worst-case multiplier count occurs for the third non-trivial set of rotation angles in which 240 actual multipliers are required. However, note that these are low-complexity multiplications because each multiplication is by a 5-bit constant. In a hardware implementation, these would be replaced with low-complexity adds and shifts as explained in [2]. For this worst-case the corresponding number of shifts is 336.

To satisfy the curious reader, we have also provided the operations count for the best-case, non-trivial rotation-angle set.

| Transform size |  | HM 2 core 2-D transform | JCTVC-E243 partial butterfly implementation of 2-D inverse DCT | ROT Inverse Transform (worst-case) | ROT Inverse Transform (best-case) |
| --- | --- | --- | --- | --- | --- |
| 4x4 | Mults | 0 | 48 | 0 | 0 |
|  | Adds | 80 | 96 | 0 | 0 |
|  | Shifts | 32 | 32 | 0 | 0 |
| 8x8 | Mults | 0 | 352 | +240 | +160 |
|  | Adds | 640 | 576 | +288 | +288 |
|  | Shifts | 352 | 128 | +336 | +416 |
| 16x16 | Mults | 1408 | 2752 | +240 | +160 |
|  | Adds | 2624 | 3712 | +288 | +288 |
|  | Shifts | 1600 | 512 | +336 | +416 |
| 32x32 | Mults | 7424 | 21888 | +240 | +160 |
|  | Adds | 13440 | 25856 | +288 | +288 |
|  | Shifts | 7296 | 2048 | +336 | +416 |

Table 2: Comparison of operation counts

# Conclusion

Based on the simulation results, 2 lifting-multiplier sets can be used instead of 3 multiplier-sets with no performance loss. By enforcing an encoding restriction, the encoder run time can be reduced significantly while retaining a moderately large coding gain. We recommend HM adoption of the ROT as an additional transform.

# References

- Z. Ma, F. Fernandes, E. Alshina, A. Alshin, “CE 7: Experimental Results for the ROT”, JCTVC-G304, Geneva, November 2011.

- F. Fernandes, “Low-Complexity Rotational Transform”, JCTVC-C096, Guangzhou, October 2011.

# Appendix: Draft Text

### Transform coefficient syntax

| transform_coeff( x0, y0, log2TrafoSize, trafoDepth, cIdx ) { | Descriptor |
| --- | --- |
| if( ( ( cIdx = = 0 && cbf_luma[ x0 ][ y0 ][ trafoDepth ] ) ¦ ¦ ( cIdx = = 1 && cbf_cb[ x0 ][ y0 ][ trafoDepth ] ) ¦ ¦ ( cIdx = = 2 && cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) ) ) { |  |
| if( cu_qp_delta_enabled_flag && !IsCuQpDeltaCoded ) { |  |
| cu_qp_delta | se(v) ¦ ae(v) |
| IsCuQpDeltaCoded = 1 |  |
| } |  |
| if( split_transform_flag[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| x1 = x0 + ( ( 1 << log2TrafoSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2TrafoSize ) >> 1 ) |  |
| transform_coeff( x0, y0, log2TrafoSize − 1, trafoDepth + 1, cIdx ) |  |
| transform_coeff( x1, y0, log2TrafoSize − 1, trafoDepth + 1, cIdx ) |  |
| transform_coeff( x0, y1, log2TrafoSize − 1, trafoDepth + 1, cIdx ) |  |
| transform_coeff( x1, y1, log2TrafoSize − 1, trafoDepth + 1, cIdx ) |  |
| } else { |  |
| if ( PredMode = = MODE_INTRA ) { |  |
| if (cIdx = = 0) |  |
| scanIdx = ScanType[ log2TrafoSize -2][ IntraPredMode ] |  |
| Else |  |
| scanIdx = ScanTypeC[ log2TrafoSize -2][ IntraPredModeC ] |  |
| } else { |  |
| scanIdx = 0 |  |
| } |  |
| if (log2TrafoSize > 2) { |  |
| rotational_transform_idx | ue(v)¦ae(v) |
| } |  |
| if ( entropy_coding_mode_flag ) |  |
| residual_coding_cabac( x0, y0, log2TrafoSize, trafoDepth, scanIdx, cIdx ) |  |
| else if ( !entropy_coding_mode_flag ) |  |
| residual_coding_cavlc( x0, y0, log2TrafoSize, trafoDepth, scanIdx, cIdx ) |  |
| } |  |
| } |  |
| } |  |

## Transform unit semantics

rotational_transform_idx specifies the rotational transform index used in the rotational transform process for the transform coefficients.

## Scaling and transformation process

Inputs to this process are:

–	a luma location ( xT, yT ) specifying the top-left luma sample of the current transform unit relative to the top-left luma sample of the current picture,

–	a variable trafoDepth specifying the hierarchy depth of the current block relative to the coding unit,

–	a variable cIdx specifying the chroma component of the current block,

–	a variable nS specifying the size of the current transform unit.

Output of this process is:

–	a modified version of the (nS)x(nS) array of residual samples r with elements rij.

The quantization parameter qP is derived as follows.

–	If cIdx is equal to 0,

qP = QP’Y			(15-1)

–	Otherwise,

qP = QP’C			(15-2)

The (nS)x(nS) array of residual samples r are derived as specified in the following ordered steps.

Depending on the values of rotational_transform_idx, the following applies.

- If rotational_transform_idx is equal to 1, the following ordered steps are specified:

- The rotational transformation process as specified in subclause xxxx is invoked with bitDepth, qP, and (nS)x(nS) array c as the inputs and the output is assigned to the (nS)x(nS) array dij of scaled transform coefficients.

- The scaling process for transform coefficients as specified in subclause Error: Reference source not found is invoked with the size of the transform unit nS, the (nS)x(nS) array d, the chroma component variable cIdx and the quantization parameter qP as the inputs and the output is a scaled transform coefficient (nS)x(nS) array e.

- The transformation process for scaled transform coefficients as specified in subclause Error: Reference source not found is invoked with the size of the transform unit nS, the scaled transform coefficient (nS)x(nS) array e and the chroma component variable cIdx as the inputs and the output is a residual samples (nS)x(nS) array r.

- Otherwise (rotational_transform_idx is equal to 0), the following ordered steps are specified:

- The inverse scanning process for transform coefficients as specified in subclause Error: Reference source not found is invoked with the size of the current transform unit nS, the list of transform coefficients transCoeffLevel[ xT ][ yT ][ trafoDepth ][ cIdx ] and the variable cIdx as the inputs and the output is a (nS)x(nS) array cij.

- The scaling process for transform coefficients as specified in subclause Error: Reference source not found is invoked with the size of the transform unit nS, the (nS)x(nS) array c, the chroma component variable cIdx and the quantization parameter qP as the inputs and the output is a scaled transform coefficient (nS)x(nS) array d.

- The transformation process for scaled transform coefficients as specified in subclause Error: Reference source not found is invoked with the size of the transform unit nS, the scaled transform coefficient (nS)x(nS) array d and the chroma component variable cIdx as the inputs and the output is a residual samples (nS)x(nS) array r.

## Rotational transform process for 8x8 transform coefficients

Input to this process is an 8x8 array of transform coefficients s with elements sij

Output of this process is transform coefficients as an 8x8 array d with elements dij.

When the value of variable idx is set to rotational_transform_idx – 1, output element dij is specified as follows.

- For i=0..7, a set of intermediate values tij is derived by

g3 = si,3 - (u[idx][2] * si,4)>>5

g0 = si,0 - (u[idx][2] * si,1)>>5

f4 = si,4 - (v[idx][2] * g3)>>5

f1 = si,1 - (v[idx][2] * g0)>>5

c3 = g3 - (u[idx][2]*f4)>>5

C0 = g0 - (u[idx][2]*f1)>>5

d4 = f4 - (u[idx][1] * si,5])>>5

d1 = f1 - (u[idx][1] * si,2])>>5

ti,5 = si,5 - (v[idx][1] * d4)>>5

ti,2 = si,2 - (v[idx][1] * d1)>>5

b4 = d4 - (u[idx][1] * ti,5)>>5

b1 = d1 - (u[idx][1] * ti,2)>>5

if (idx == 1)

e = b1

b1 = ti,2

ti,2 = -e

e = b4

b4 = ti,5

ti,5 = -e

a3 = c3 - (u[idx][0] * b4)>>5

a0 = c0 - (u[idx][0] * b1)>>5

ti,4 = b4 - (v[idx][0] * a3)>>5

ti,1 = b1 - (v[idx][0] * a0)>>5

ti,3 = a3 - (u[idx][0] * ti,4)>>5

ti,0 = a3 - (u[idx][0] * ti,1)>>5

ti,6 = si,6

ti,7 = si,7

- For j=0...7, the transformed result sij is computed from these intermediate values tij as follows:

g3 = t3,j - (u[idx][5] * t4,j)>>5

g0 = t0,j - (u[idx][5] * t1,j)>>5

f4 = t4,j - (v[idx][5] * g3)>>5

f1 = t1,j - (v[idx][5] * g0)>>5

c3 = g3 - (u[idx][5] * f4)>>5

c0 = g0 - (u[idx][5] * f1)>>5

// Rz, Equation (7)

d4 = f4 – (u[idx][4] * t5,j)>>5

d1 = f1 - (u[idx][4] * t2,j)>>5

d5,j = t5,j - (v[idx][4] * d4)>>5

d2,j = t2,j - (v[idx][4] * d1)>>5

b4 = d4 - (u[idx][4] * d5,j)>>5

b1 = d1 - (u[idx][4] * d2,j)>>5

// Rz(π/2) swap and sign flip, Equation (17)

if ((idx == 2) || (idx == 3))

e = b1

b1 = d2,j

d2,j = -e

e = b4

b4 = d5,j

d5,j = -e

// Rx, Equation (6)

a3 = c3 - (u[idx][3] * b4)>>5

a0 = c0 - (u[idx][3] * b1)>>5

d4,j = b4 - (v[idx][3] * a3)>>5

d1,j = b1 - (v[idx][3] * a0)>>5

d3,j = a3 - (u[idx][3] * d4,j)>>5

d0,j = a0 - (u[idx][3] * d1,j)>>5

d6,j = t6,j

d7,j = t7,j

The multiplier matrices u, and v are specified in Tables 1 and 2.

|  | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| idx = 0 | -7 | -7 | 5 | -4 | -2 | -2 |
| idx = 1 | 4 | -22 | 9 | -7 | -4 | 5 |
| idx = 2 | -3 | -5 | -3 | 2 | -29 | 5 |
| idx = 3 | -10 | -6 | -4 | -1 | -29 | 6 |

Table 1: u Lifting Multipliers

|  | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| idx = 0 | 14 | 13 | -10 | 8 | 3 | 4 |
| idx = 1 | -8 | 30 | -17 | 13 | 7 | -10 |
| idx = 2 | 5 | 9 | 6 | -5 | 32 | -10 |
| idx = 3 | 18 | 11 | 8 | 1 | 32 | -12 |

Table 2: v Lifting Multipliers

# Patent rights declaration(s)

Samsung Electronics Co. Ltd may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).