| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G594 |
| --- | --- |

| Title: | Improved Extrapolation for Intra Prediction Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jaeil Kim Sungho Bae Houari Sabirin Munchurl Kim Jungsun Kim Byungmoon Jeon Korea Advanced Institute of Science and Technology (KAIST) 291 Daehak-ro, Yuseong-gu, Daejeon 305-701, Korea | Tel: Email: | +82-42-350-7419 jaeil1203@ kaist.ac.kr bbb8484@ kaist.ac.kr houari@ kaist.ac.kr mkim@ee. kaist.ac.kr jungsun@ lge.com bm.jeon@lge.com |
| Source: | KAIST, LG Electronics |  |  |

_____________________________

# Abstract

This contribution proposes a new extrapolation to enhance intra prediction and constrained intra prediction in HM. The simulation results report that the proposed new extrapolation method provides a slight coding gain (0.1%~0.4%) of BDBR for chorma in “Low delay B HE” and “Low delay B LC”, but exhibits a slight coding loss of 0.1~0.2% BDBR for both “Random Access HE” and “Random Access HE”, and 0.4~0.6% losses in coding gain for “All Intra HE” and “All Intra LC.

# Introduction

In HM 4.0, the extrapolation for intra prediction is performed for reference pixels by copying the most outer pixel in the neighboring block when the reference pixels are not available. For the extrapolation in constrained intra prediction, the reference pixels in the inter-coded neighboring blocks are filled with the most outer pixel of an intra coded block or the average of the two most outer pixels of two intra coded neighboring blocks.

This simple extrapolation results in poor estimation without fully considering the signal characteristics of all the reference border pixels.

This contribution improves the HM 4.0 extrapolation to enhance the coding efficiency.

# Algorithm Description

## Current algorithm (HM4.0)

In HM4.0, the extrapolation for intra prediction is illustrated in Figure 1 and the extrapolation for constrained intra prediction is illustrated in Figure 2. It is noted in Figure 1 and Figure 2 that the extrapolation is simply performed by copying the most outer pixel of an intra coded block or the average of the two most outer pixels of the corresponding two intra coded blocks.

Figure 1 Extrapolation for intra prediction

Figure 2 Extrapolation for constrained intra prediction

## Proposed algorithm

- One-way extrapolation in I-/P-/S-slices for (constrained) intra prediction

Figure 3 One-way extrapolation

Extrapolation procedure:

CASE 1: Np<2Nr1

- Perform Nr1+Np-point DCT with pj=r1Nr1−1, j=0, 1,⋯, Np−1.

- Perform Nr1+Np-point inverse DCT with DC component.

- Replace inverse-DCTed r1(i) with r1(i).

CASE 2: Np≥2Nr1

- Perform Nr1+Np-point DCT with pj=r10+⋯+r1Nr1−1Nr1, j=0, 1,⋯, Np−1.

- Perform Nr1+Np-point inverse DCT with DC component.

- Replace inverse-DCTed r1(i) with r1(i).

- Two-way extrapolation in P-/S-slices for constrained intra prediction

Figure 3 One-way extrapolation

Extrapolation procedure:

CASE 1: Np<2Nr1+Nr2

- Perform Nr1+Np+Nr2-point DCT with pj=r1Nr1−1+r202, j=0, 1,⋯, Np−1.

- Perform Nr1+Np+Nr2-point inverse DCT with DC and AC(1) components.

- Replace inverse-DCTed r1(i) & inverse-DCTed r1(i) with r1(i) & r2(i).

CASE 2: Np≥2Nr1+Nr2

- Perform Nr1+Np+Nr2-point DCT with  pj=r10+⋯+r1Nr1−1+r20+⋯+r2Nr2−1Nr1+Nr2, j=0, 1,⋯, Np−1.

- Perform Nr1+Np-point inverse DCT with DC and AC(1) components.

- Replace inverse-DCTed r1(i) & inverse-DCTed r1(i) with r1(i) & r2(i).

## Implementation

### Software

This proposed technique is implemented into HM4.0 software.

# Experiments

## Experimental environment

Our computing platform used for cross-verification tests is a clustering system with 16 computing nodes, each of which contains:

- CPU: dual-socket quad-core Intel Xeon 2.5 GHz

- memory: 32 GB RAM

- storage (local): one 146 GB 2.5" 10k RPM SAS disk

The encoder and decoder executables were generated with g++ 4.1.2.

## Simulation results

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.8% | 0.6% | 0.5% | 0.9% | 1.0% | 0.9% |
| Class B | 0.4% | 0.4% | 0.5% | 0.5% | 0.6% | 0.7% |
| Class C | 0.3% | 0.2% | 0.2% | 0.4% | 0.4% | 0.4% |
| Class D | 0.2% | 0.2% | 0.2% | 0.3% | 0.3% | 0.3% |
| Class E | 0.8% | 0.6% | 0.6% | 0.9% | 1.0% | 1.0% |
| Overall | 0.5% | 0.4% | 0.4% | 0.6% | 0.6% | 0.6% |
|  | 0.5% | 0.4% | 0.4% | 0.6% | 0.6% | 0.6% |
| Enc Time[%] | 114% |  |  | 124% |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.3% | 0.0% | 0.0% | 0.3% | 0.1% | 0.1% |
| Class B | 0.1% | 0.1% | 0.0% | 0.2% | 0.3% | 0.3% |
| Class C | 0.2% | 0.2% | 0.1% | 0.2% | 0.2% | 0.2% |
| Class D | 0.1% | 0.3% | 0.1% | 0.1% | 0.3% | 0.2% |
| Class E |  |  |  |  |  |  |
| Overall | 0.2% | 0.2% | 0.0% | 0.2% | 0.2% | 0.2% |
|  | 0.2% | 0.1% | 0.0% | 0.2% | 0.2% | 0.2% |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | -0.2% |
| Class C | 0.0% | 0.1% | -0.1% | 0.0% | 0.1% | 0.0% |
| Class D | 0.0% | -0.2% | -0.1% | 0.1% | 0.0% | -0.2% |
| Class E | 0.1% | -1.4% | 0.6% | 0.0% | -1.0% | -1.1% |
| Overall | 0.0% | -0.3% | 0.1% | 0.0% | -0.1% | -0.3% |
|  | 0.0% | -0.3% | 0.1% | 0.0% | -0.1% | -0.4% |
| Enc Time[%] | 103% |  |  | 104% |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

# Conclusions

Our proposed new extrapolation method provides a slight coding gain (0.1%~0.4%) of BDBR for chorma in “Low delay B HE” and “Low delay B LC”, but exhibits a slight coding loss of 0.1~0.2% BDBR for both “Random Access HE” and “Random Access HE”, and 0.4~0.6% losses in coding gain for “All Intra HE” and “All Intra LC.

Due to the lack of time, the proposed method has not been cross-checked with its decoder implementation and cross verification. Further studies are necessarily to tune the proposed extrapolation method according to various signal characteristics, QP values etc.

# References

# Patent rights declaration(s)

KAIST and LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).