| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G597 |
| --- | --- |

| Title: | AHG 18: Cross-check of Adaptive Resolution Coding (ARC) (JCTVC-G264) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Glenn Van Wallendael, Jan De Cock, Rik Van de Walle Ghent University – IBBT Gaston Crommenlaan 8 P.O. Box 201 9050 Ghent-Ledeberg, Belgium David Flynn BBC R&D, Centre House, 56 Wood Lane W12 7SB London, UK | Tel: Email: | +32 93314957 glenn.vanwallendael@ugent.be davidf@rd.bbc.co.uk |
| Source: | Ghent University, BBC |  |  |

_____________________________

# Abstract

This informational contribution provides a cross verification of the Adaptive Resolution Coding proposal of Cisco (JCTVC-G264). Apart from rerunning the software to obtain bit exact results, some further inspection on the code and the used test setup was carried out. The code exactly executes what is written in the proposal. Some questions were asked about derivations from the common conditions (QP 32, 35, 39, 42; lost frames). The conclusion was that, in the proposed scenario, these test conditions provide more relevant results than when tested under common conditions. Additionally, although results of class F are not so relevant for this scenario, results from this configuration are included in this cross verification as well.

# Remarks

It was observed that test conditions are changed from QPs 22, 27, 32, 37 to QPs 32, 35, 39, 42. 
For the proposed method, this change is justifiable because when bandwidth availability on the network reduces, the first step an encoder would take is increasing the QP. When the situation gets more critical, a resolution decrease can be performed. As a result, it is more relevant to do simulations on a lower quality than the common conditions. The only point I would like to remark is that at a QP of 42, the anchor bitstreams are forced to an extreme coding situation. The anchors increase their QP with 6 after the frame loss. Therefore, a base QP of 48 is used. This implies that for the highest temporal level a QP of 51 is used.

It was also observed that a frame loss is simulated by making the frames black in the encoder. This can only be regarded as a frame loss when the encoder knows these frames are lost. For this point as well, this corresponds with the scenario that is described and is therefore a relevant way to test the scenario.

Although these 2 points are remarked, it must be added that both of them are well described in the proposal.

# Test conditions

Our computing platform used for cross-verification tests is shown below.

## Encoding

Super computer with 167 computing nodes (IBM iDataPlex dx360M2), each of which contains:

- CPU: dual-socket quad-core Intel Xeon L5520 (Intel Nehalem microarchitecture, 2.27 GHz, 8 MB L3 cache per quad-core chip), thus 8 cores / node

- memory: 12 GB RAM (DDR3 PC3-8500S 1067 MHz)

- storage (local): one 156 GB 3.5" 15k RPM SAS disk

and 194 computing nodes (IBM HS 21 XM blade), each of which contains:

- CPU: dual-socket quad-core Intel Xeon L5420 (Intel Core microarchitecture, 2.5 GHz, 6 MB L2 cache per quad-core chip), thus 8 cores / node

- memory: 16 GB RAM (DDR2 FB-DIMM PC-5300 CL5)

- storage (local): one 146 GB 2.5" 10k RPM SAS disk

The encoder executables were generated with g++ 4.1.2.

## Decoding

- CPU: quad-core Intel Core 2 Q6600 (Intel Core microarchitecture, 2.40GHz, 8MB L2 cache)

- Memory: 4 GB RAM (DDR2 PC2-6400 400MHz)

# Code

The code is conforming the proponants description.

# Simulation results

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -7,0% | 74,1% | 67,8% | -14,3% | 76,5% | 82,9% |
| Class B | -4,3% | 23,5% | 26,5% | -11,4% | 13,4% | 18,5% |
| Class C | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class D | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class E |  |  |  |  |  |  |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -2,9% | 24,3% | 23,8% | -6,7% | 21,9% | 24,9% |
|  | -2,9% | 78,5% | 61,3% | -6,7% | 74,4% | 62,5% |
| Enc Time[%] | 69% |  |  | 69% |  |  |
| Dec Time[%] | 66% |  |  | 65% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -3,5% | 36,5% | 35,4% | -11,0% | 25,2% | 21,9% |
| Class C | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class D | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class E | -0,5% | 11,4% | 9,4% | -6,8% | 6,9% | 3,8% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -1,2% | 13,5% | 12,8% | -4,7% | 9,2% | 7,5% |
|  | -1,2% | 13,8% | 12,8% | -4,7% | 9,3% | 7,2% |
| Enc Time[%] | 79% |  |  | 79% |  |  |
| Dec Time[%] | 66% |  |  | 65% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -3,5% | 36,5% | 28,9% | -11,5% | 23,8% | 20,8% |
| Class C | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class D | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class E | -0,8% | 10,7% | 7,8% | -6,9% | 6,9% | 3,7% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -1,3% | 13,4% | 10,5% | -4,9% | 8,7% | 7,2% |
|  | -1,2% | 13,7% | 9,9% | -4,9% | 8,3% | 6,8% |
| Enc Time[%] | 79% |  |  | 78% |  |  |
| Dec Time[%] | 66% |  |  | 65% |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -6,9% | 76,6% | 69,8% | -13,9% | 81,7% | 89,2% |
| Class B | -4,5% | 25,1% | 27,2% | -11,5% | 15,6% | 20,1% |
| Class C | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class D | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class E |  |  |  |  |  |  |
| Class F | #NAME? | #NAME? | #NAME? | #NAME? | #NAME? | #NAME? |
| Overall | -2,9% | 25,4% | 24,4% | -6,7% | 23,8% | 26,9% |
|  | -2,9% | 81,4% | 63,7% | -6,6% | 80,9% | 67,7% |
| Enc Time[%] | 70% |  |  | 70% |  |  |
| Dec Time[%] | 66% |  |  | 65% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -3,6% | 37,7% | 36,4% | -11,0% | 27,3% | 23,7% |
| Class C | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class D | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class E | -1,7% | 11,8% | 9,9% | -7,6% | 7,7% | 4,7% |
| Class F | #NAME? | #NAME? | #NAME? | #NAME? | #NAME? | #NAME? |
| Overall | -1,5% | 14,0% | 13,2% | -4,9% | 10,0% | 8,3% |
|  | -1,4% | 14,3% | 13,3% | -4,9% | 10,2% | 8,0% |
| Enc Time[%] | 80% |  |  | 80% |  |  |
| Dec Time[%] | 66% |  |  | 65% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -3,6% | 37,6% | 29,8% | -11,6% | 25,9% | 22,5% |
| Class C | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class D | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class E | -2,1% | 11,0% | 8,3% | -7,8% | 7,7% | 4,6% |
| Class F | #NAME? | #NAME? | #NAME? | #NAME? | #NAME? | #NAME? |
| Overall | -1,5% | 13,8% | 10,9% | -5,1% | 9,5% | 7,9% |
|  | -1,5% | 14,2% | 10,2% | -5,1% | 9,1% | 7,5% |
| Enc Time[%] | 81% |  |  | 80% |  |  |
| Dec Time[%] | 66% |  |  | 65% |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -8,6% | 189,9% | 153,3% | -14,5% | 72,7% | 159,3% |
| Class B | -12,7% | 26,2% | 28,4% | -12,9% | 20,7% | 27,6% |
| Class C | -13,4% | -2,6% | 2,2% | -11,7% | -3,5% | 1,5% |
| Class D | -12,6% | 8,1% | 5,8% | -10,7% | 5,7% | 1,1% |
| Class E |  |  |  |  |  |  |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -11,9% | 53,7% | 46,3% | -12,5% | 23,7% | 46,2% |
|  | -11,9% | 53,9% | 47,3% | -12,4% | 56,8% | 47,2% |
| Enc Time[%] | 69% |  |  | 69% |  |  |
| Dec Time[%] | 66% |  |  | 66% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -12,4% | 36,8% | 31,8% | -11,4% | 35,0% | 29,3% |
| Class C | -15,4% | -5,7% | -3,5% | -12,1% | -3,4% | -1,9% |
| Class D | -10,0% | 0,7% | -0,3% | -7,1% | 0,7% | -0,3% |
| Class E | -9,8% | 13,5% | 13,3% | -11,9% | 10,1% | 5,9% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -12,0% | 12,8% | 11,5% | -10,6% | 12,1% | 9,7% |
|  | -12,0% | 12,5% | 11,2% | -10,6% | 12,0% | 9,2% |
| Enc Time[%] | 79% |  |  | 79% |  |  |
| Dec Time[%] | 69% |  |  | 68% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -12,8% | 36,2% | 32,3% | -11,7% | 33,3% | 30,1% |
| Class C | -16,0% | -5,0% | -4,0% | -12,0% | -3,5% | -1,9% |
| Class D | -12,5% | 5,9% | 2,0% | -8,7% | -2,8% | -2,5% |
| Class E | -11,1% | 14,1% | 10,0% | -11,6% | 9,5% | 6,0% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -13,2% | 14,2% | 11,4% | -11,0% | 10,6% | 9,4% |
|  | -13,1% | 14,6% | 11,2% | -11,0% | 9,8% | 9,2% |
| Enc Time[%] | 80% |  |  | 78% |  |  |
| Dec Time[%] | 66% |  |  | 66% |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -8,6% | 197,7% | 159,7% | -14,2% | 77,9% | 172,1% |
| Class B | -12,9% | 27,9% | 29,1% | -13,0% | 23,5% | 29,5% |
| Class C | -13,4% | -2,6% | 2,2% | -11,7% | -3,5% | 1,5% |
| Class D | -12,6% | 8,1% | 5,8% | -10,7% | 5,7% | 1,1% |
| Class E |  |  |  |  |  |  |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -11,9% | 56,0% | 48,0% | -12,4% | 25,8% | 49,8% |
|  | -11,9% | 56,2% | 49,2% | -12,4% | 62,0% | 51,2% |
| Enc Time[%] | 70% |  |  | 70% |  |  |
| Dec Time[%] | 66% |  |  | 66% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -12,5% | 38,2% | 32,9% | -11,5% | 37,6% | 31,7% |
| Class C | -15,4% | -5,7% | -3,5% | -12,1% | -3,4% | -1,9% |
| Class D | -10,0% | 0,7% | -0,3% | -7,1% | 0,7% | -0,3% |
| Class E | -11,2% | 13,8% | 13,9% | -12,8% | 10,9% | 6,9% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -12,3% | 13,3% | 11,9% | -10,8% | 13,1% | 10,6% |
|  | -12,3% | 13,0% | 11,7% | -10,7% | 13,1% | 10,1% |
| Enc Time[%] | 80% |  |  | 79% |  |  |
| Dec Time[%] | 69% |  |  | 68% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -12,9% | 37,4% | 33,3% | -11,8% | 35,9% | 32,3% |
| Class C | -16,0% | -5,0% | -4,0% | -12,0% | -3,5% | -1,9% |
| Class D | -12,5% | 5,9% | 2,0% | -8,7% | -2,8% | -2,5% |
| Class E | -12,5% | 14,3% | 10,5% | -12,5% | 10,4% | 7,0% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -13,5% | 14,6% | 11,9% | -11,2% | 11,6% | 10,3% |
|  | -13,4% | 15,1% | 11,6% | -11,2% | 10,9% | 10,1% |
| Enc Time[%] | 81% |  |  | 79% |  |  |
| Dec Time[%] | 66% |  |  | 66% |  |  |

All results are bit exact with the proposal from Cisco. The only point where this cross verification differs from the original proposal is encoding and decoding times. Difference in encoding time is probably caused by variations on the cluster and a small difference in collecting the data. In the original proposal encoding times were taken only from frames occurring after the frame loss. The encoding times in this cross verification are measured over all frames.

# Conclusion

The code and results are verified and are conformant to the results stated by Cisco.

# Acknowledgement

The research activities described in this proposal were funded by Ghent University, the Interdisciplinary Institute for Broadband Technology (IBBT), the Institute for the Promotion of Innovation by Science and Technology in Flanders (IWT), the Fund for Scientific Research-Flanders (FWO-Flanders), and the European Union. Furthermore, this work was carried out using the Stevin Supercomputer Infrastructure at Ghent University.