| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, Suisse, 21-30 November, 2011 | Document: JCTVC-G599 |
| --- | --- |

| Title: | Non-CE2: Cross-check of Non-Square Hadamard transform for motion estimation and merge estimation JCTVC-G521 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Cross-check Report |  |  |
| Author(s) or Contact(s): | Simon Oudin Benjamin Bross Fraunhofer HHI Einsteinufer 37 D-10587 Berlin | Tel: Email: | +49 30 31002 167 simon.oudin@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document reports test results of JCTVC-G521, a study on non-square Hadamard transform for estimation and merge estimation. This study has been combined with the proposal JCTVC-G518, a study on non-square quadtree transform (NSQT) using 2x8 and 8x2 transform blocks for symmetric partitions (2NxN and Nx2N) and for asymmetric partitions (AMP) used in inter prediction. The technique has been studied, proposed code has been checked, compiled and the results reported by Huawei can be confirmed. Furthermore additional results are provided for informative purposes.

# Tested Proposals

For the tested proposal JCTVC-G521 [1], one software package has been released. It is based on the software including 2x8 and 8x2 non-square transform blocks from JCTVC-G518 [2] as cross-checked in JCTVC-G571 [3]. Thus, only changes related to non-square Hadamard transform will be studied in this document.

## Non-square Hadamard transform

The tested proposal has been integrated into JCTVC-G518 [2] software adding 2x8 and 8x2 transform blocks to NSQT. One macro has been introduced to integrate this proposal:

- NS_HAD

In HM4.0 [4], hadamard based distortion functions are used in inter prediction, i.e. motion estimation and merge estimation. Thus, distortion is calculated for various CU partitionings (PUs). The different HAD size used for different PU sizes are shown in Table 1. HM4.0 only uses 8x8 and 4x4 transforms while in the tested proposal, non-square transform sizes of 16x4, 4x16, 8x2 and 2x8 are added.

Different test configurations can be found in Table 2. In the software provided there is no possibility implemented to switch 8x2 and 2x8 HAD transform on and off. The following macros are used to configure the provided software (red shaded macro settings in Table 2):

#define NSQT        1       // F410 & F412 : Non-Square Quadtree Transform

#if NSQT

#define NSQT_W2x8   1

#endif

#define NS_HAD      1       // Non-Square Hadamard

Enabling 2x8/8x2 HAD transforms is implicitly done. In combination with NSQT and 8x2/2x8 blocks, HAD transforms of size 16x4, 4x16, 8x2 and 2x8 are used (JCTVC-G521) while NSQT without 8x2/2x8 block, e.g., is only used in combination with HAD transforms of size 16x4 and 4x16.

In order to be able to perform all of the test in Table 2, we added a macro NS_HAD_W2x8 that, if defined as 1, enables 8x2/2x8 HAD transforms in addition to. (light green overlay macro setting in Table 2).

#define NSQT           1       // F410 & F412 : Non-Square Quadtree Transform

#if NSQT

#define NSQT_W2x8      1

#endif

#define NS_HAD         1       // Non-Square Hadamard (16x4, 4x16)

#if NS_HAD

#define NS_HAD_W2x8    1       // Non-Square Hadamard (16x4, 4x16, 8x2, 2x8)

#endif

Table 1 Hadamard transform size (HAD) for different PU sizes in HM4.0 and when using non square hadamard transform (NSHAD) of different sizes.

Table 2 Different configurations of NSQT and NSHAD combinations as used in HM4.0, NSQT with 2x8 and 8x2 blocks proposal JCTVC-G518, the proposal JCTVC-G521 tested in this document and additional informative tests 1-5.

# Simulation Environment and Results

## Software

The binaries were compiled on a 64 bit Ubuntu 10.10 Linux using the included make files and gcc 4.4.5. The coding conditions used for the simulations are specified in [5]. The HM4.0 default settings, both for configuration file and compile-time, are used in this test as anchor.

## Hardware

For all tests configurations one computer type has been used to run the simulations running a 64 bit Ubuntu 10.10 Linux operating system. It is equipped with Intel(R) Xeon(R) CPU X5570 @ 2.93GHz quadcore CPUs and 32 GB of memory.

## Results

The coding efficiency results in this section are given in terms of BD-rate. The values are averaged over all sequences from one class. BD-PSNR, PSNR values and bit rates for every test point can be found in the attached Excel spreadsheets JCTVC-G599_cc1.xls, JCTVC-G599_cc2.xls, JCTVC-G599_cc3.xls and JCTVC-G599_cc4.xls. The list of tests can be found in Table 3 which refers to the configurations of Table 2. The encoding and decoding times shown shouldn’t be considered for this cross-check.

The tests cc1 and cc2 should be compared in order to observe the influence of the 2x8/8x2 HAD transforms when NSQT is using 2x8 and 8x2 transform blocks. The 2x8/8x2 HAD transforms are present for cc1 in the opposite of cc2 where they are inactive, as shown respectively in Table 4 and in Table 5.

The tests cc3 and cc4 should be compared in order to observe the influence of the 2x8/8x2 HAD transforms when NSQT is not using 2x8 and 8x2 transform blocks. The 2x8/8x2 HAD transforms are present for cc3 in the opposite of cc4 where they are inactive, as shown respectively in Table 6 and in Table 7.

The best configurations found, through the BD-PNSR performances, are the cc1 and cc4 which corresponds to the 2 first tests of JCTVC-G521 [1].

Table 3 List of tests

Table 4 cross-check 1(cc1)

Table 5 cross-check 2(cc2)

Table 6 cross-check 3(cc3)

Table 7 cross-check 4(cc4)

# References

- X. Zheng, L. Liu, Y. Yuan, Y. He, “Non-CE2: Non-square hadamard transform for motion estimation and merge estimation”, JCTVC-G521, Geneva, November, 2011.

- Y. Yuan, X. Zheng, Y. He, “CE2 subtest C.1: Non-square quadtree (NSQT) with 2x8 and 8x2 transfor”, JCTVC-G518, Geneva, November, 2011.

- S. Oudin, B. Bross “CE2: Cross-check of Non-Square Quadtree Transform (NSQT) JCTVC-G517 & JCTVC-G571”, JCTVC-G518, Geneva, Nove,ber, 2011.

- CT-VC, “HEVC Test Model Software 4.0”, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-4.0

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, Turin, July, 2011.