| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G600r1 |
| --- | --- |

| Title: | Non-CE3: Adaptive Motion Vector Resolution based on the PU Size |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jiwook Jung Jin Heo Sehoon Yea 221, Yangjae-Dong, Seocho-Gu Seoul 137-130, Korea | Email: Tel: | jiwook.jung@lge.com jin78.heo@lge.com sehoon.yea@lge.com +82-2-6912-6477 |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution proposes an adaptive mechanism for threshold selection at a PU level in the Progressive Motion Vector Resolution (PMVR) method proposed by MediaTek. The result shows it improves the coding efficiency of the PMVR method thanks to its PU-level adaptation of threshold values. This contribution presents the result of the proposed scheme implemented on the PMVR method without 1/8 MV resolution. The Y BD-rate gains with respect to HM4.0 were -0.2% for RA HE, -0.2% for RA-LC, -0.1% for LB-HE, -0.2% for LB-LC, 0.0% for LP-HE, and 0.0% for LP-LC. The Y BD-rate gains with respect to the PMVR without 1/8-pel with Th=2 were 0.0% for RA HE, 0.0% for RA-LC, 0.0% for LB-HE, 0.0% for LB-LC, -0.1% for LP-HE, and -0.1% for LP-LC.

# Introduction

A progressive MV resolution (PMVR) was proposed in JCTVC-F125 [2], where MV resolution is progressively adjusted based on the magnitude of MV difference (MVD) and predefined thresholds so that side-information indicating the pel-precision of a MV needs not be sent to the decoder.

In this contribution, an improvement of the PMVR scheme is proposed which incorporates the use of PU-size to adaptively change the threshold values in determining the pel-precision for each PU. The proposed scheme capitalizes on the tendency that higher pel-precision is more beneficial in terms of coding efficiency as the PU size increases. While the original PMVR scheme signals a predefined threshold value per each slice, the proposed scheme adjusts the threshold on a PU basis so that not only higher coding gain but also reduced encoding complexity can be achieved by avoiding unnecessary high pel-precision search. The PMVR method has two versions: ones with and without 1/8 MV resolution. This contribution presents the result of the proposed scheme implemented on top of latter, i.e, the PMVR method without 1/8 MV resolution.

# Proposed Method

Extensive experiments indicate that (optimal) MV resolution is likely to depend also on the PU size. In Figures 1 and 2, the z-axis corresponds to the number of PU’s that have 1/4-pel and 1/8-pel optimal MV resolutions, respectively (the x-axis and y-axis correspond to PU_width and PU_height). The results were obtained by coding the BQSquare sequence with TMuC-0.8-qualcomm (AMVRES [1]) under the LB-HE condition.

Note the two tallest peaks in each figure show up in the small PU region in the case of 1/4-pel (Figure 1) and the other way around in the case of 1/8-pel (Figure 2). Similar tendencies were observed in other test cases as well. Therefore it seems more effective to employ higher MV resolution for large PU’s and vice versa.

Figure 1. 1/4-pel accuracy

Figure 2. 1/8-pel accuracy

Table 1 shows how much of the optimal MV’s were chosen as 1/4-pel and 1/8-pel as a function of the PU size for the PartyScene and BQSqaure sequences.

Table 1. Selection Rate of 1/4 pel MV vs. 1/8 pel MV

| PU size | Selection Rate |  |  |  |
| --- | --- | --- | --- | --- |
|  | PartyScene |  | BQSquare |  |
|  | 1/4 pel | 1/8 pel | 1/4 pel | 1/8 pel |
| 4x4 | 0.83 | 0.17 | 0.7 | 0.3 |
| 4x8 | 0.79 | 0.21 | 0.64 | 0.36 |
| 8x4 | 0.79 | 0.21 | 0.65 | 0.35 |
| 8x8 | 0.76 | 0.24 | 0.59 | 0.41 |
| 8x16 | 0.7 | 0.3 | 0.48 | 0.52 |
| 16x8 | 0.7 | 0.3 | 0.49 | 0.51 |
| 16x16 | 0.61 | 0.39 | 0.39 | 0.61 |
| 16x32 | 0.52 | 0.48 | 0.37 | 0.63 |
| 32x16 | 0.49 | 0.51 | 0.38 | 0.62 |
| 32x32 | 0.43 | 0.57 | 0.33 | 0.67 |
| 32x64 | 0.31 | 0.69 | 0.32 | 0.68 |
| 64x32 | 0.35 | 0.65 | 0 | 1 |
| 64x64 | 0.35 | 0.67 | 0 | 1 |

As mentioned earlier, in PMVR without 1/8 MV resolution, one fixed default threshold value (Th = 2) is used to determine the MV resolution. The proposed method selects a proper threshold value based on the PU size and then determines the MV resolution adaptively. The PU size is determined by the product of the horizontal and vertical lengths of the PU (i.e., PU_width * PU_height). More specifically, if the PU size is smaller than ThPU, which was set to 256 in the experiments, the threshold value for MV resolution (ThMV) is kept to the predefined default value of 2 as with PMVR, otherwise ThMV is increased to 4. The decision process is summarized as follows and Figure 3 illustrates the proposed method based upon such process applied to the PMVR without 1/8-pel resolution case:

PUsize = PU_width * PU_height If (PUsize < ThPU) Use default ThMV (ThMV = 2) Else Increase ThMV (ThMV = 4)

(a) ThMV = 2 for PUsize < ThPU                               (b) ThMV = 4 for PUsize ≥ ThPU

Figure 3. Proposed method

Obviously it is straightforward to extend the proposed scheme to the PMVR with 1/8-pel resolution. In addition, it is readily understood that the proposed threshold adaptation at PU level can not only lead to higher coding gain but also, depending upon the default threshold settings, potentially reduce encoding complexity by avoiding unnecessary high pel-precision search.

# Experimental Results

Table 2.  Performance of the proposed algorithm with respect to HM 4.0

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -0.5% | -0.4% | -0.2% | -0.4% | -0.3% |
| Class B | -0.2% | -0.5% | -0.4% | -0.3% | -0.5% | -0.3% |
| Class C | -0.2% | -0.6% | -0.6% | -0.3% | -0.5% | -0.5% |
| Class D | 0.0% | -0.4% | -0.5% | -0.1% | -0.4% | -0.2% |
| Class E |  |  |  |  |  |  |
| Overall | -0.2% | -0.5% | -0.5% | -0.2% | -0.5% | -0.3% |
| Enc Time[%] | XX% |  |  | XX% |  |  |
| Dec Time[%] | XX% |  |  | XX% |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | -0.4% | -0.4% | -0.2% | -0.3% | -0.2% |
| Class C | -0.2% | -0.4% | -0.6% | -0.2% | -0.2% | -0.6% |
| Class D | 0.0% | -0.1% | -0.6% | 0.0% | 0.0% | -0.2% |
| Class E | 0.0% | -0.1% | 0.3% | -0.1% | -0.2% | -0.1% |
| Overall | -0.1% | -0.3% | -0.4% | -0.2% | -0.2% | -0.3% |
| Enc Time[%] | XX% |  |  | XX% |  |  |
| Dec Time[%] | XX% |  |  | XX% |  |  |

|  | Low delay P HE |  |  | Low delay P LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -0.1% | -0.2% | -0.1% | 0.1% | 0.1% |
| Class C | -0.1% | -0.3% | -0.1% | 0.0% | -0.2% | 0.0% |
| Class D | 0.2% | -0.3% | -0.7% | 0.1% | 0.1% | 0.1% |
| Class E | -0.1% | 0.1% | -0.3% | -0.1% | 0.1% | -0.4% |
| Overall | 0.0% | -0.2% | -0.3% | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | XX% |  |  | XX% |  |  |
| Dec Time[%] | XX% |  |  | XX% |  |  |

Table 3.  Performance of the proposed algorithm compared to PMVR w/o 1/8-pel with Th=2

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.1% |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.1% | -0.1% | 0.0% | -0.1% | 0.0% |
| Class E | 0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.1% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | -0.1% | 0.0% | -0.2% | -0.1% |
| Class C | 0.0% | -0.1% | -0.2% | 0.0% | -0.1% | -0.2% |
| Class D | 0.0% | -0.1% | 0.0% | -0.1% | 0.2% | -0.2% |
| Class E | 0.1% | 0.2% | -0.1% | 0.0% | -0.9% | -0.3% |
| Overall | 0.0% | 0.0% | -0.1% | 0.0% | -0.2% | -0.2% |
|  | 0.0% | 0.0% | -0.2% | 0.0% | -0.2% | -0.3% |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

|  | Low delay P HE |  |  | Low delay P LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.1% | -0.2% | 0.0% | 0.0% | 0.0% |
| Class C | -0.1% | -0.2% | 0.0% | 0.0% | -0.2% | -0.1% |
| Class D | -0.1% | -0.5% | -0.7% | -0.1% | 0.1% | 0.2% |
| Class E | -0.1% | 0.2% | -0.6% | -0.2% | 0.1% | -0.3% |
| Overall | -0.1% | -0.2% | -0.3% | -0.1% | 0.0% | 0.0% |
|  | -0.1% | -0.1% | -0.4% | -0.1% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

# Conclusion

In this contribution, an adaptive motion vector resolution method based on the size of prediction unit (PU) is proposed. The proposed method introduces an adaptive mechanism for threshold selection at a PU level in PMVR without incurring any extra side-information cost. The result shows it improves the coding efficiency of the PMVR method thanks to its PU-level adaptation of threshold values. The proposed method was implemented on top of the PMVR without 1/8-pel resolution. The Y BD-rate gains with respect to the PMVR without 1/8-pel (Th=2) were 0.0% for RA HE, 0.0% for RA-LC, 0.0% for LB-HE, -0.0% for LB-LC, -0.1% for LP-HE, and -0.1% for LP-LC. Experiments are underway for the case with the PMVR with 1/8-pel resolution as well as other refinements of the proposed scheme.

# Patent rights declaration(s)

LGE may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- M. Karczewicz, P. Chen, R. Joshi, X. Wang, W.-j. Chien, and R. Panchal, “Video coding technology proposal by Qualcomm In”, JCT-VC meeting contribution JCTVC-A121, Dresden, DE, April, 2010.

- J. An, X. Li, W. Guo, and S. Lei, “Progressive MV Resolution,” JCT-VC meeting contribution JCTVC-F125, Torino, IT, July, 2011.