| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G603 |
| --- | --- |

| Title: | Non-CE9: Cross-check of Bi-prediction for low delay coding (JCTVC-G305) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Philipp Helle Benjamin Bross Fraunhofer HHI Einsteinufer 37 D-10587 Berlin | Tel: Email: | philipp.helle@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document reports test results of JCTVC-G305 that proposes a modified bi-prediction for low delay conditions by not signaling the motion vector difference for list 1 and setting it to (0,0). The technique has been studied, proposed code has been checked, compiled and the results reported by NTT DOCOMO can be confirmed.

# Modified bi-prediction for low delay coding

In the proposal JCTVC-G305 [1], the encoder enables the proposed technique, i.e. sets the mvd_l1_zero_flag in the slice header to true for all B-slices that use identical reference picture lists list 0 and list 1 (identical in length, values and order of values). For all other slices, the flag is set to false.

While checking the code, these were observed:

- In TEncSearch.h line 302:
The parameter puiDist has the same name as the one declared when ZERO_MVD_EST is true. Other places in the code suggest that it was intended to allow for both LOWCOST_BIPRED and ZERO_MVD_EST being true. Also, I think it essentially breaks a function to add a parameter with a default value and at the same time add code that crashes when run with that default value.

- In TEncSearch.cpp line 3388:
#if ZERO_MVD_EST will always evaluate to false here.

- In TEncSearch.cpp line 3413:
The #if ZERO_MVD_EST block would not compile as the variable uiDist would not be defined. However, the software doesn't even compile with ZERO_MVD_EST set to true when the macros are false.

- In TDecCAVLC.cpp line 1820:
The Symbol BLOCK_ADAPT_LC_BIPRED is not defined.

# Simulation Environment and Results

## Software

The binaries were compiled on a 64 bit Ubuntu 10.10 Linux using the included make files and gcc 4.4.5. The coding conditions used for the simulations are specified in [3]. The HM4.0 default settings, both for configuration file and compile-time, are used in this test as anchor.

## Results

The coding efficiency results in this section are given in terms of BD-rate. The values are averaged over all sequences from one class. BD-PSNR, PSNR values and bit rates for every test point can be found in the attached Excel spreadsheet JCTVC-G603.xls. The encoding and decoding times shown in Table 2-1 shouldn’t be considered due to test running on different platforms.

Table 2-1  cross-check results

# References

- Y. Suzuki, A. Fujibayashi, “Non-CE9: Bi-prediction for low delay coding”, JCTVC-G305, Geneva, November, 2011.

- JCT-VC, “HEVC Test Model Software 4.0”, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-4.0

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, Turin, July, 2011.